MINERALOGICAL REPORT No.  8274

by Alan C. Purvis,  PhD

PRELIMINARY SUMMARY

October 24,  2002

TO :	Mr Peter Simpson

Giant’s Reef Mining Ltd

PO  Box 1244

TENNANT CREEK  NT  0861

YOUR REFERENCE :	Order No. 200981

MATERIAL :	RC Drill chip samples

EL8882, 8883,

Sample Nos. 81642 to 81665 (total 24)

WORK REQUESTED :	Thin section preparation, description and report with comments and interpretations as specified.

SAMPLES & SECTIONS :	Returned to you with this report.

DIGITAL COPY :	Enclosed with hard copy of this report.

PONTIFEX & ASSOCIATES PTY. LTD.

# SUMMARY COMMENTS

Twenty-four samples of RC-chips are described in this report from thin sections made from multiple chips mounted in epoxy.  They are all from the Bluebush area southwest of Tennant Creek in the Northern Territory.

Various partly variolitic and/or pyroxene-porphyritic basalt lavas, hyaloclastites, breccias and tuffs are seen in BBRC-029 to 032, mostly with chlorite-sericite-quartz-carbonate alteration.  Veins, with quartz, carbonate or adularia are common and seem to have an epithermal character.  It is not clear from the present study whether these have any economic potential.  Interbedded sediments include dolomite and carbonaceous slate (BBRC-029, 103-104m) sandy dolomite (BBRC-030, 70-71m) and sandy shale or greywacke (BBRC-031, 119-120m).

There is also a sheared, quartz-poor, fine-grained felsic igneous lithology in BBRC032, with albite, chlorite and muscovite as well as rutile or leucoxene.  This may represent a quartz microdiorite or andesitic intrusion.

The following petrographic summaries for each sample will basically constitute headers to the individual descriptions in the final report.

### TABLE 1:    Samples described in Report No. 8274

| ### 81657 ### BBRC-030, 87-88m | Chlorite-carbonate-leucoxene schists with very minor quartz. The chlorite has replaced plagioclase to 1.5mm in grainsize, with carbonate after pyroxene and possibly late magmatic quartz in a fine-grained metadolerite, sheared and altered. |
| --- | --- |
| ### 81658 ### BBRC-031, 79-80m | Chlorite-quartz-sericite-leucoxene-altered basaltic rocks with poor textural preservation: aphyric, pyroxene porphyritic and fragmented, locally flooded by quartz and/or carbonate. Several chips contain or consist of coarse-grained deformed carbonate from large veins. |
| ### 81659 ### BBRC-031, 89-90m | Partly brecciated altered mafic rocks with albitised plagioclase and chlorite ± quartz after pyroxene as well as leucoxene after partly dendritic oxides. Some variolitic areas. Matrix of chlorite and hydrothermal quartz in the breccias. One chip of coarse carbonate with lenses or fragments of granular to prismatic hydrothermal quartz, |
| ### 81660 ### BBRC-031, 99-100m | Massive and brecciated plagioclase-porphyritic, partly variolitic basalts with albite, chlorite, quartz and leucoxene. Leucoxene has replaced partly dendritic opaque oxides. The brecciated chips have a chlorite-quartz matrix as in the previous sample. |
| ### 81661 ### BBRC-031, 119-120m | Schistose matrix-rich greywacke or sandy shale with single-crystal quartz grains to 0.5mm (medium-grained sandstone) and rare lithic grains: also has slate composed of sericite ± quartz. Narrow veins of sparry quartz. |
| ### 81662 ### BBRC-032, 115-117m | Albite-chlorite-muscovite-leucoxene/anatase schist with minor quartz, accessory apatite and zircon. Veins of chlorite and of albite. Metamorphosed felsic igneous rock. |