MINERALOGICAL REPORT No.  8142

by Ian R. Pontifex  MSc.  &  A.C. Purvis  PhD.

22 October,  2001

TO :	Mr Peter Simpson

Exploration Manager

Giants Reef Mining Ltd

PO  Box 1244

TENNANT CREEK  NT  0861

YOUR REFERENCE :	Order No.  200534 (9 samples)

Order No.  200549 (6 samples)

MATERIAL :	Drill Core Samples,  BBRD-002

IDENTIFICATION :	Sample Nos. 424164 to 424178 (15 in all)

WORK REQUESTED :	Thin section preparation, description and report with comments as specified.

SAMPLES & SECTIONS :	Temporarily retained pending possible follow-up analyses.

DIGITAL COPY :	Enclosed.

PONTIFEX & ASSOCIATES PTY. LTD.

SUMMARY COMMENTS

Fifteen samples from a drillhole BBRD-002 in unknown Proterozoic rocks south west of Tennant Creek are described in this report from normal thin sections.  Selected photomicrographs accompany some descriptions.  The upper six samples from 424164 at 78m to 424169 at 276m are RC precollar drill chips, with nine deeper samples of NQ2 core to the bottom of hole at 549.7m.

The uppermost seven samples from 78m to 306.3m seem to represent a contiguous comagmatic suite of broadly basaltic lavas and mafic volcanic fragmental rocks, including lapilli tuff and possible pillow breccias.  These are completely altered/retrograded to various combinations of tremolite-actinolite, with locally abundant uralitised pyroxene phenocrysts (clinopyroxene-augite).  Patches of chlorite to tremolite or of actinolite alone are derived from other phenocrysts, (possibly olivine, less probably plagioclase).  Some also have abundant-accessory opaque oxides, magnetite ± ilmenite.

Primary compositions throughout these seven samples vary from normal basalt, towards more mafic ankaramite or picrite with more abundant phenocrysts.  This trend suggests an accumulation of mafic phenocryst material and the host rocks may be richer in chromium or nickel than typical basalts, but selected geochemistry is necessary to check this.

The mineralogy in these seven samples, with tremolite-actinolite, epidote, albite, chlorite and leucoxene or sphene, is typical of greenschist-facies metabasalts, metamorphosed at 300-500C.  Veins of prehnite and of calcite indicate that deuteric fluid circulation continued into lower temperatures, below 300C.

Geochemistry would also be useful in determining the nature and setting of these basalts, (for example, rifting seems to be the most common trigger for volcanic activity in Proterozoic sequences in northern Australia).  Recommendations in this regard are presented at the end of these summary comments, consistent with a memo to Peter Simpson, 26/10/01.

The eight deeper samples from 424171 (334.3m) to end of hole 424178 (549.7m) mostly represent a different sequence of lithologies, summarised below by way of a listing of the headers from each of the individual descriptions.

| 424171 BBRD-002, 334.3m | Complex breccia, with chaotic fragments of recrystallised and tectonised (carbonaceous) chert, lesser sericitic shale, and (separate?) vein quartz. Matrix of graphite and chlorite which seem likely to be hydrothermal. Accessory pyrite. |
| --- | --- |
| 424172 BBRD-002, 335.9m | Microporphyritic (?weakly alkaline) basalt, cut by chaotic fractures healed by random threads and stringers of quartz, K-spar, rarer chlorite. Also, pervasive sericitic and (reddened) clay-sericite > chloritic (± potassic?) alteration. |
| 424173 BBRD-002, 372.4m | Layered metapelite. Different bands have different concentrations of sericite and chlorite (shale), some with very small spots of sericite, chlorite, carbonate, rarer ?garnet. Leucoxene dispersed and in stylolites. One chlorite-rich band of cherty (silicified?) with disrupted vein quartz and pyrite. |
| 424174 BBRD-002, 406.1m | Dacitic-porphyry (or porphyritic-dacite). Numerous phenocrysts of quartz > sericitised plagioclase > biotite-altered to chlorite + muscovite and chloritised hornblende. Fine sericite-altered groundmass fractured with stringers and veins of carbonate > chlorite. May be subvolcanic intrusive or lava? |
| 424175 BBRD-002, 456.8m | Mafic hyaloclastite (coarse vitric-tuff). Numerous attenuated fragments of mafic glass, completely altered to carbonate (+ leucoxene?) within a matrix of secondary carbonate and chlorate forming green foliae and white lenticular veins. |
| 424176 BBRD-002, 484.9m | Rhyolitic to rhyodacitic-porphyry. Extensive micro-granophyric groundmass and bipyramidal quartz phenocrysts to suggest a subvolcanic genesis (cf. Gawler Range Porphyries). Alteration : clay-sericite-muscovite in groundmass feldspar phenocrysts sericitised, and biotite phenocrysts chloritised. Minor carbonate veinlets. |
| 424177 BBRD-002, 514.6m | Dacite-porphyry with coarse bipyramidal quartz-phenocrysts, also phenocrysts of largely sericite-altered plagioclase and of chloritised biotite. Very fine sericite-chlorite alteration, also in groundmass. |
| 424178 BBRD-002, 549.7m | Homogeneous sericite-rich shale in sharp contact with a wacke containing detrital quartz, altered feldspar and altered biotite, probably largely volcanically derived. Also minor pelitic lithic detritus and a matrix of weakly schistose meta silty pelite. Minor quartz stringers in both facies. |

# CONCLUSIONS

Within this whole sample suite, the chert-breccia with extensive graphite matrix at 334.3m seems to separate an upper sequence of metamorphically retrograded mafic volcanics, and a deeper intercept through more varied lithologies, including three acid porphyries, two basaltic rocks, and two metapelites.  This division may be a fault, with a hydrothermal component.

It is of interest that below this fault?, sample 424175 at 456.8m is a hyaloclastic coarse vitric tuff, with a similar gross mafic lava composition as for the mafic volcanic sequence above, but with a different alteration assemblage of chlorite and carbonate, without the extensive amphibole alteration seen above.  The sample 414172 at 335.9m is an altered basalt, but probably partly alkaline, also without amphibole alteration.  These two deeper ‘basaltic’ samples therefore seem unlikely to be genetically related to the upper suite, which suggests that they, and the sequence of rocks in which they occur, represent a different geological entity to the upper part of this drill hole.

The three (altered) biotite-feldspar quartz porphyries may be related, but as noted in the descriptions, the dacitic rocks 424174 at 406.1m and 424.77 at 514.6m are comparable, and both somewhat different to the rhyolitic/rhyodacitic porphyry 424176 at 484.9m, which may be subvolcanic.  (These samples compare with Gawler Range Volcanics, Gawler Craton, South Australia).

The sericite-shale in contact with wacke in 424178 at the bottom of the hole, compares with Warramunga Formation facies, but the wacke does include considerable detritus derived from an altered acid porphyritic source.

RECOMMENDATION  (re follow-up geochemical assessment)

The following recommendation is a repeat of a separate memo sent ot Peter Simpson 26/10/01.

Following a review of the 15 drill hole samples described in this report, together with Geoff Hudson, 24/10/01, it was agreed that the mafic (ankaramitic) volcanics in the upper part of this drill hole are of special interest, mainly because they probably have not been previously recognised in the Tennant Creek area.  Although they may not offer immediate potential for ore mineralisation, their relative uniqueness probably justifies further analysis in the interests of broader geological understanding of the project area, by an interpretation of major and immobile trace element chemistry, in the context of our petrology.

This recommendation is briefly suggested by Alan Purvis in the Summary to this report, given his particular interest and expertise in the petrology and integrated geochemistry of mafic/ultramafic rocks.  The aim would be for Alan to interpret chemical data as specified below in the context of his petrology, to refine the rock type classifications, also ideas on the origin and models of geological setting of these rocks.  The samples are metamorphosed, but alteration is only minor and is not expected to prevent meaningful interpretations of the chemistry.

Also as noted above, an upper suite of seven mafic samples to 306.3m in this drill hole seems to be separated by a graphite-rich chert breccia with mylonitic and hydrothermal components at 334.3m (probably representing a fault or shear), from a deeper and apparently separate package of metasediments, porphyries, also two samples of mafic volcanics.  These mafic volcanics are altered (?alkaline) basalt at 335.9m, and altered glassy-mafic, vitric-tuff, at 456.8m, both probably different from the upper mafic sequence, but it would be useful to subject these to the same geochemistry as for the above suite to check this.

In the light of the above, the following geochemical analyses are recommended, to be followed by an interpretation and report by Alan.  Seven samples 424164 to 424170, (or a selected 3 or 4 of these 7 samples).  Two samples 424172 and 424175.

•	Major elements  Si, Ti, Al, Fe, Mn, Mg, Ca, Na, K, P  Analytical technique  ICP-MS

•	Minor elements :  Zr, Cr, V, Cd, Nb, Y, La, Ce, Nd, Ni, Cu, Zn, Pb, analysed as

follows:

•	Zr, Nb by XRF (1ppm detection limited)

•	All other minor elements by various ICP schemes, detection limit 1ppm or less.

Estimated cost of the above analyses to be carried out by AMDEL is $86.50 per sample, inclusive of GST.

INDIVIDUAL DESCRIPTIONS

| 424164 BBRD-2, 78-79m | Original pyroxene-olivine porphyritic volcanic rock (ankaramite or picrite), retrograde metamorphosed to massive tremolite. Minor chlorite replacing some phenocrysts and in veins. Minor disseminated opaque oxide (oxidised magnetite and limonite). |
| --- | --- |

Field Note:	A brownish and greenish non-foliated rock, possibly a volcanic?

Rock chips to 25mm occur in this thin section, with abundant flooding by limonite or earthy hematite apparently indicating deep weathering (or possibly a palaeo-oxidation surface of the underlying mafic sequence?).

Most of the chips contain uralitic tremolite in masses from 0.5 to 5mm long, clouded with extremely fine secondary opaque oxide, apparently derived from pyroxene phenocrysts (15-20%).  This includes poorly oriented fine fibrous tremolite and chlorite in patches that may represent former olivine phenocrysts or, less probably, plagioclase phenocrysts.  The chlorite is optically positive and rich in Mg and Al.  Rounded inclusions, also possibly derived from olivine, are enclosed in some of the uralitic patches.

Small dense aggregates of fine opaque oxide may be (partly) derived from magnetite microphenocrysts.  The groundmass is rich in decussate fine-grained tremolite with abundant fine-grained opaque oxide, probably oxidised to hematite and/or goethite.  Veins, composed largely of optically negative, aluminium-poor chlorite occur in some chips, but most of the chlorite is in the altered phenocrysts described above.

Interpretation

This seems to represent a mafic to ultramafic volcanic rock originally rich in pyroxene and olivine phenocrysts, possibly ankaramite or picrite.  It may contain anomalous chromium and nickel.

| Fig 1 | 424164 | _____ 0.45 mm |
| --- | --- | --- |

Thin section [TS].  Crossed nicols [Xnic].  Original pyroxene-olivine-porphyritic ankaramitic picritic lava.  Completely retrograded to uralitic tremolite after numerous small pyroxene phenocrysts clouded with very fine opaque oxide in a tremolite-rich metamorphosed groundmass.

| 424165 BBRD-002, 95-97m | Pyroxene-porphyritic basalt, completely altered to intergrown tremolite-actinolite-chlorite-sphene. Less porphyritic than 424164, also has carbonate veins and carbonate flooding. |
| --- | --- |

Field Note:	Dark grey-green fine-grained rock with calcite veins.

These chips are to some extent fresher versions of the weathered chips seen in the previous sample, original phenocrysts are more sparsely scattered (5-10%), although clearly defined uralitic tremolite-actinolite pseudomorphs after pyroxene phenocrysts are from 1 to 4mm long.  There are also inequigranular largely rectilinear areas, to 5mm long, that range from chlorite-rich to tremolite-actinolite-rich, the amphibole being decussate or in fan-like aggregates, rather than being uralitic.  It is possible that these represent phenocrysts of olivine and/or plagioclase, as suggested for the previous sample.

The groundmass is completely altered to decussate fine tremolite but also has areas rich in sericite, clinozoisite or chlorite, as well as irregular masses of opaque oxide and/or microcrystalline sphene, in different proportions in different areas.  Some of the chips contain narrow carbonate veins and one has been largely flooded by carbonate.

Interpretation

These chips are interpreted to represent porphyritic mafic-basaltic volcanics, with original pyroxene phenocrysts and olivine and/or plagioclase phenocrysts altered to tremolite-actinolite ± chlorite, in a tremolite to chlorite-rich groundmass with opaque oxide and sphene.

| Fig 2 | 424165 | _____ 0.45 mm |
| --- | --- | --- |

TS.  [Crossed nicols] Xnic.  Meabasalt with uralitised clinopyroxene phenocrysts and lenticular crosscutting carbonate veins.

| Fig 3 | 424165 | _____ 0.018 mm |
| --- | --- | --- |

TS.  Plane polarised light. [PPL].  Altered probable olivine phenocryst (top right), now composed of quartz and actinolite, within a heterogeneous completely altered groundmass containing actinolite after prismatic pyroxene and fine-grained albite.  Uralitised small pyroxene crystals clustered in the bottom left corner.

| 424166 BBRD-002, 150m | Lapilli tuff of pyroxene porphyritic lavas and plagioclase ± olivine-porphyritic lavas. Metamorphosed and completely altered to tremolite-clinozoisite-albite-chlorite. Veins of tremolite, clinozoisite-albite and prehnite occur. |
| --- | --- |

Field Note:	Single chunk, probably similar to 424167

The larger chips in this sample contain primary volcanic rock fragments to 15mm long, which therefore include lapilli.  The smaller chips are more homogeneous but may represent portions of larger fragments in a pyroclastic lithology.  Most areas, including fragments and the matrix, have uralitised pyroxene phenocrysts comprising 5% of the rock.  The fragments also contain cuspate masses of opaque oxide and leucoxene, partly interstitial to poorly or well-preserved tremolite-rich microspherulites.  One fragment has a contorted flow-layering defined by fibrous tremolite.  Another fragment has large phenocrysts, to 3mm in diameter, altered to various combinations of albite and tremolite-actinolite, suggesting plagioclase and pyroxene or olivine phenocrysts.  This fragment has a more massive (not spherulitic) tremolite-rich groundmass than in other basaltic rock fragments.

The matrix in the fragmental chips is rich in very fine minerals other than tremolite, possibly clinozoisite and albite ± chlorite ± leucoxene.  Veins rich in tremolite-actinolite occur in several chips, and there are narrow veins of albite and clinozoisite, especially in the more obviously fragmental chips.  The chip with the larger phenocrysts has a vein of decussate prehnite laths.

Interpretation

This sample represents a lapilli tuff dominated by primary fragments of pyroxene and plagioclase ± olivine-porphyritic lava.  These are completely altered to tremolite-actinolite, chlorite, clinozoisite, albite and leucoxene. Veins rich in tremolite occur, as well as albite-clinozoisite veins and veins of prehnite.

| Fig 4 | 424166 | _____ 0.45 mm |
| --- | --- | --- |

TS.  Xnic.  Whole photo represents a pyroxene-porphyritic fragment, with uralitised ex-pyroxene phenocrysts, in tremolite-rich altered groundmass material with cuspate lenses of opaque oxide.

| Figs 5 & 6 | 424166 | _____ 0.45 mm |
| --- | --- | --- |

TS.  PPL [Fig 5].  Xnic. [Fig 6]  Fragment with clustered and scattered ex-olivine phenocrysts completely altered, within an amphibole-rich altered groundmass with abundant opaque material, now largely leucoxene.  Altered phenocrysts highlighted in Xnic Fig 6.

| Fig 7 | 424166 | _____ 0.18 mm |
| --- | --- | --- |

TS.  Xnic.  Veins of prehnite along the margin of a metabasalt chip.

| 424167 BBRD-002, 177-181m | Porphyritic basalt completely altered to tremolite-albite-epidote-carbonate-sphene, also with veins containing calcite-epidote-actinolite-albite, and prehnite. |
| --- | --- |

Field Note:	Greenish grey chloritic rock with calcite veins

This sample is a relatively well-preserved porphyritic basalt with sparse, small uralitised clinopyroxene phenocrysts to 1.5mm long and more abundant phenocrysts to 3mm long, altered to various combinations of tremolite-actinolite, epidote, albite and quartz, commonly with minor opaque oxide.  The larger phenocrysts were possibly plagioclase, less probably olivine.

The groundmass has well-preserved, largely albitised plagioclase microlites with a contorted flow pattern, and interstitial areas rich in tremolite with minor epidote and microcrystalline sphene.  There is also a large patch of carbonate, possibly calcite, 4 x 3mm, possibly representing a vesicle.  Veins with various proportions of carbonate, epidote, albite and actinolite occur as well as later narrow prehnite veins and pure carbonate veins.

Interpretation

This sample represents a possibly pyroxene-plagioclase porphyritic basalt with good textural preservation and abundant veins of (deuteric) minerals :  calcite, epidote, actinolite, albite, prehnite.

| Fig 8 | 424167 | _____ 0.45 mm |
| --- | --- | --- |

TS.  Xnic.  Altered probable olivine phenocrysts, now composed largely of quartz and episode, with rare uralitised clinopyroxene (right hand side below centre) in a dark, altered, fine-grained groundmass.  A carbonate vein is seen on the left hand side, cutting large altered phenocrysts.

| 424168 BBRD-002, 222-223m | Pyroxene and olivine/plagioclase-porphyritic lava, completely altered to actinolite-clinozoisite-albite-leucoxene. Also veined and flooded by carbonate. |
| --- | --- |

Field Note:	Looks similar to 424167

Uralitised clinopyroxene phenocrysts, mostly less than 2mm long, rarely 5mm long, are more abundant in this sample compared with the previous chips in 424168.  There are also less abundant ex-phenocrysts, mostly to 2mm long, now composed of poorly oriented tremolite with albite and leucoxene, largely in resorption channels.  These phenocrysts may have been olivine, as their shapes are not as typical of plagioclase as they are of olivine.  The groundmasses in these chips include abundant plagioclase microlites, now clinozoisite ± albite, with a flow texture and interstitial tremolite-actinolite and sphene.  Veins of carbonate, probably calcite, are common and some chips have been flooded by carbonate.

Interpretation

This represents a pyroxene and olivine or plagioclase porphyritic lava, completely altered to tremolite-actinolite, clinozoisite, albite and sphene, veined and flooded by carbonate.

| Figs 9 & 10 | 424168 | _____ 0.45 mm |
| --- | --- | --- |

TS.  PPL.  Xnic.  Polycrystalline actinolite-rich pseudomorphs of probable olivine phenocrysts are more abundant than uralitised clinopyroxene, suggesting a former picrite, with a dark, altered fine-grained groundmass.

| 424169 BBRD-002, 275-276m | Pyroxene and olivine/plagioclase porphyritic basalt, extensively altered to tremolite-actinolite-chlorite-opaque oxide. Rare quartz veins containing minor tremolite. |
| --- | --- |

Field Note:	Dark green, probably similar to above but has no calcite veins.

Like some of the previous samples, this has 20% uralitised clinopyroxene phenocrysts less than 1.5mm in diameter and 15-20% chlorite-rich aggregates to 2mm in diameter, with minor to common tremolite-actinolite and very minor opaque oxide, derived from plagioclase or olivine phenocrysts.  The groundmass is mostly decussate fine-grained tremolite-actinolite with dendrites of opaque oxide and very minor interstitial chlorite.  The only veins are very narrow and appear to contain mostly quartz and minor fibrous tremolite.

Interpretation

This seems to be a metamorphosed pyroxene-rich basalt, passing towards ankaramite, depending on the original nature of the chlorite-rich altered phenocrysts.

| Fig 11 | 424169 | _____ 0.45 mm |
| --- | --- | --- |

TS.  Xnic.  The dark areas on the left had side represent chlorite-rich altered olivine phenocrysts, with uralitised clinopyroxene suggesting a former picrite, with a dark, altered fine-grained groundmass.

| 424170 BBRD-002, 306.3m | Metamorphosed pyroxene and olivine/plagioclase porphyritic basalt band (pillow fragment?), between chlorite-rich layers with tremolite. Carbonate veins, ± quartz, partly along microfaults offsetting the central band or fragment. |
| --- | --- |

Field Note:	Piece of half NQ core chloritised volcanic breccia, probably same as most above.

This thin section includes a band of the altered metabasalt lithology commonly as seen in the previous samples, from 12 to 25mm wide and offset along veins.  Along either side is an area rich in schistose chlorite with altered tremolite or with carbonate veins.  The central band has abundant (20%) uralitised pyroxene phenocrysts to 1.5mm in diameter and larger patches of fine-grained schistose tremolite to 3mm in diameter, possibly after olivine or plagioclase phenocrysts.  The groundmass areas are rich in decussate fine-grained tremolite but also have irregular masses of opaque oxide and leucoxene.  Irregular lenses of carbonate are present and the veins that offset the central band are rich in carbonate with less abundant quartz.  More irregular carbonate veins also occur, with no obvious dislocation of the host rock.

Interpretation

It is possible that this is a pillow breccia with lenticular fragments of pyroxene and olivine/plagioclase porphyritic basalt in a chlorite-rich schistose matrix with carbonate-quartz veins along microfaults.

| Fig 12 | 424170 | _____ 0.45 mm |
| --- | --- | --- |

TS.  Xnic.  Uralitised clinopyroxene phenocrysts, with pale polycrystalline pseudomorphs of olivine phenocryst on the right-hand side.

| 424171 BBRD-002, 334.3m | Complex breccia, with chaotic fragments of recrystallised and tectonised (carbonaceous) chert, lesser sericitic shale, and (separate?) vein quartz. Matrix of graphite and chlorite which seem likely to be hydrothermal. Accessory pyrite. |
| --- | --- |

Field Note :	Graphite matrix breccia

Approximately 25% of this sample consists of black opaque, extremely fine crystalline and compact graphite matrix material as highly irregular networks discontinuous between breccia fragments.  The breccia fragments consist of a chaotic loose-packed aggregate of mostly quartzose material ranging from 0.2mm single crystal quartz grains, to fragments up to 8mm across of fine heterogeneous quartz mosaic, lesser fragments of sericitic shale and there are discontinuous chlorite-rich layers.

Most quartz fragments appear to be essentially chert, largely recrystallised and many have been tectonised with mylonitic and microbreccia textures.  Fragments of patchy vein quartz seem to be mostly independent of the chert.  Many of the cherty fragments are carbonaceous, some weakly chlorite and rarely they contain pyrite.

The sericitic shale fragments occur independently, also composite with (in contact with) chert.  Chloritic domains occur in a poorly defined layer, and are locally crowded with microbrecciated quartz.

# Interpretation

This sample seems primarily to involve a cherty facies, with associated sericitic shale (metapelite), probably within a fault or shear zone to produce the gross breccia, also with microbrecciation (and mylonitisation) within fragments.  Disrupted vein quartz, also probably the chlorite are probably hydrothermal contemporaneous with tectonism.  The graphite matrix may also be hydrothermal (?exhalative) or seemingly less likely, remobilised from carbonaceous chert.

| 424172 BBRD-002, 335.9m | Microporphyritic (?weakly alkaline) basalt, cut by chaotic fractures healed by random threads and stringers of quartz, K-spar, rarer chlorite. Also, pervasive sericitic and (reddened) clay-sericite > chloritic (± potassic?) alteration. |
| --- | --- |

Field Note :	Brownish brecciated ?volcanic, within the graphite breccia zone represented by 424172 above.

This core piece consists basically of altered massive microporphyritic basalt, with an extensive chaotic stockwork of stringers and veinlets of quartz and lesser very fine chlorite healing discontinuous microfractures.

Minor scattered microphenocrysts (about 15% of the whole rock) and <1mm in size and almost completely altered to ultrafine sericite ± quartz.  Relict morphologies indicating ex-pyroxene and rarer possible ex-plagioclase, and staining with sodium cobalti-nitrite indicates numerous scattered apparent relicts of K-spar, also stringers of K-spar, all suggesting an alkaline component to this basalt, (and therefore different to other basalts elsewhere in this drill hole).

These components are scattered through a massive microcrystalline groundmass of former apparent micritic plagioclase, minor diffuse extremely fine K-spar, and pyroxene, extensively altered to (reddened) clay-sericite.  There is no inherent quartz.  Minute titaniferous grains are dispersed (typical of altered basalt).

The chaotic micronetwork of quartz and of the lesser poorly defined and finer chlorite, partly dissipate into interstices throughout the groundmass.

| Fig 13 | 424172 | _____ 0.18 mm |
| --- | --- | --- |

Thin section (TS), ordinary light (OL).  Relatively low magnification (x50) dominated by groundmass of microcrystalline basalt with randomly interlocking feldspar microlites (?including K-spar) altered and reddened by limonite-hematite, also altered ex-pyroxene scattered poorly defined small altered phenocrysts.  Clear veinlets of quartz.

| Fig 14 | 424172 | _____ 0.18 mm |
| --- | --- | --- |

TS.  Crossed nicols (Xnic) equivalent of Fig 13 to highlight particularly sericite alteration of phenocrysts.

| 424173 BBRD-002, 372.4m | Layered metapelite. Different bands have different concentrations of sericite and chlorite (shale), some with very small spots of sericite, chlorite, carbonate, rarer ?garnet. Leucoxene dispersed and in stylolites. One chlorite-rich band of cherty (silicified?) with disrupted vein quartz and pyrite. |
| --- | --- |

Field Note :	Grey-greenish banded altered siltstone or fine grained wacke with pyrite aggregates.

This piece of core is indeed banded on a scale of about 2mm to 10mm, with the essential lithology being meta-pelite manifest in the purest sense as fissile sericite-shale and is equally fine and fissile sericitic-chloritic (± kaolinitic?) shale.  Minute grains of leucoxene are typically disseminated, minor very thin thread of (darker) chlorite occur in the pale sericite-shale, rare small spots of carbonate occur in both of these facies.

The several darkest greenish bands (including the thinnest bands) consist of relatively coarser chlorite, which is foliated and weakly crenulated with leucoxenitic stylolites, also incorporating very fine muscovite commonly as clustered very small spots, ± cherty and/or meta/silty quartz.  Accessory very small spots of garnet occur in these bands and minor very fine carbonate is strung out along the schistosity in some bands.

Several chert-rich bands incorporate closely spaced foliae of muscovite, minute spots of darkish chlorite (± some garnet?).

Threads and stringers of very fine quartz occur along the foliation in mostly the chloritic bands.  The thickest (and most ‘altered’) chlorite-rich band has internal oblique foliae, also it incorporates somewhat more concentrated vein quartz, which is disrupted into several lenses, and several clusters of pyrite, partly with a disconformable and partly a conformable relationship.  The lenses of disrupted vein quartz, individually 1 to 2.5mm, may be the wacke layers referred to in the field note.

| 424174 BBRD-002, 406.1m | Dacitic-porphyry (or porphyritic-dacite). Numerous phenocrysts of quartz > sericitised plagioclase > biotite-altered to chlorite + muscovite and chloritised hornblende. Fine sericite-altered groundmass fractured with stringers and veins of carbonate > chlorite. May be subvolcanic intrusive or lava? |
| --- | --- |

Field Note :	Light brown-grey porphyritic igneous rock, with quartz augen and calcite-chlorite veins.

The petrography confirms this as a porphyry moderately altered, and with local fractures, fine breccia zones and veins.  Randomly scattered phenocrysts are somewhat heterogeneous in composition and size as follows :

|  |  | Approx. % of whole rock |
| --- | --- | --- |
| * | Quartz phenocrysts, 0.2mm to 5mm, with the finer ones as subrounded microphenocrysts and part of the groundmass, and the coarser ones subrounded to vaguely bipyramidal and mostly strongly embayed. | 30% |
| * | Plagioclase phenocrysts, subhedral, mostly 0.5mm to 2mm, some to 4mm, microfissured and sericitised | 10-15% |
| * | Ex-hornblende phenocrysts to 2mm, completely altered to chlorite | 5% |
| * | Ex-biotite phenocrysts to 2mm, completely altered to chlorite ± muscovite | 5-7% |

The groundmass is massive microcrystalline sericitised feldspar and quartz, together with microphenocrysts of the above species and of accessory leucoxene-altered opaque oxide grains.

Numerous microfissures/fractures are healed by carbonate > chlorite, microprismatic quartz.  Several wider (to 3mm) and more continuous veins consist of sparry carbonate (?dolomite) with lesser dark green chlorite and quartz.  A breccia corridor to 6mm wide is crowded with fragments of the host rock, with networks between of chlorite threads and stringers.

| Fig 15 | 424174 | _____ 0.45 mm |
| --- | --- | --- |

TS.  Xnic.  Lowest magnification (x20).  Dacitic-porphyry, phenocrysts of quartz of various size;  also of completely sericitised plagioclase (PL), and completely altered biotite (Bi).  Groundmass is sericite-altered cryptocrystalline quartzo-feldspathic.

| 424175 BBRD-002, 456.8m | Mafic hyaloclastite (coarse vitric-tuff). Numerous attenuated fragments of mafic glass, completely altered to carbonate (+ leucoxene?) within a matrix of secondary carbonate and chlorate forming green foliae and white lenticular veins. |
| --- | --- |

Field Note :	Dark grey green streaky chloritic breccia with calcite veins.

This rock can be classified as a hyaloclastite, i.e. a volcanic breccia (or coarse vitric tuff) composed largely of glassy lava fragments.  The fragments and the matrix within which they occur are all of altered glassy mafic lava.   There is no primary quartz and no recognisable feldspar.

The fragments which form about 50% of the rock are more or less elongate, some with fine shredded margins, commonly about 3mm x 10mm, attenuated and similarly oriented.  These have a cloudy groundmass of indefinite ultrafine carbonate (± leucoxene?), incorporating minor microphenocrysts which are altered to chlorite, also threads and stringers of quartz, carbonate, chlorite.  Many have internal fine relict perlitic to ignimbritic (glassy-flow) textures.

The whole rock matrix consists of relatively clearer micromosaic of carbonate, commonly in white vein-like lenses, also subequal patches, stringers and threads of pale green chlorite, commonly in streaky foliae, but some in random veinlets cutting across the prevailing (flow) layering.

Minor cryptocrystalline secondary silica is present.  Sparse extremely fine pyrite and pyrrhotite is disseminated and occurs in very small clusters.

| Fig 16 | 424175 | _____ 0.45 mm |
| --- | --- | --- |

TS.  Xnic (x20).  Mafic hyaloclastite (or coarser vitric tuff).  Top fragment is ex-mafic glass with relict shredded-lenticular, fluid-flow structures within chloritic alteration.  Bottom fragment has relict micro-perlitic textures in alteration products.  Matrix is fine carbonate incorporating fine shredded-streaky pale chlorite.

| Fig 17 | 424175 | _____ 0.18 mm |
| --- | --- | --- |

TS.  Xnic (x50).  Shredded-lenticular mafic-glass fragments altered to cloudy ultrafine carbonate ± leucoxene, with vesicles or phenocrysts altered to blue chlorite.  Matrix largely extremely fine carbonate.

| 424176 BBRD-002, 484.9m | Rhyolitic to rhyodacitic-porphyry. Extensive micro-granophyric groundmass and bipyramidal quartz phenocrysts to suggest a subvolcanic genesis (cf. Gawler Range Porphyries). Alteration : clay-sericite-muscovite in groundmass feldspar phenocrysts sericitised, and biotite phenocrysts chloritised. Minor carbonate veinlets. |
| --- | --- |

Field Note :	Altered igneous rock with brown-orange matrix and epidotised phenocrysts.

The petrography confirms this rock as a porphyry, with a distinctive quartz potassic feldspar-rich, micro-spherulitic to micro-granophyric groundmass, (i.e. not massive and microcrystalline as in the dacitic porphyries 424174 above and 424177 below).  This groundmass forms about 50% of the whole rock, as a compact microspherulitic mosaic about 0.5mm grain size of intricate quartz and K-spar intergrowths.  The K-spar has a pale reddish colour due to sparse ultrafine hematitic dust, also it is dusted by clay-sericite.  Minor relatively more granular quartz micromosaic is intergranular, also minor scattered decussate muscovite (which may be a late-stage greisen-related alteration).

Randomly scattered phenocrysts from the other approximate 50% of this rock, ranging in size from microphenocrysts about 0.2mm basically as part of the groundmass, to 1.5mm.  These consist of :

| * | Quartz crystals, bi-pyramidal to subrounded, commonly embayed | 20% |
| --- | --- | --- |
| * | Ex-feldspar phenocrysts (probably mostly plagioclase), completely altered to cloudy clay-sericite (rarely to 4mm) | 20% |
| * | Ex-biotite phenocrysts, usually <1mm size, and completely altered to chlorite + leucoxene | 7% |

There are minor narrow carbonate veins, which carry accessory microprismatic quartz crystals.  Trace very small pyrite grains are present.

| Fig 18 | 424176 | _____ 0.18 mm |
| --- | --- | --- |

TS.  Xnic (x50), quartz phenocrysts, bipyramidal and embayed.  Randomly scattered through massive groundmass of very fine microgranophyric intricately intergrown quartz and feldspar.  Altered biotite phenocrysts in top NW corner.

| 424177 BBRD-002, 514.6m | Dacite-porphyry with coarse bipyramidal quartz-phenocrysts, also phenocrysts of largely sericite-altered plagioclase and of chloritised biotite. Very fine sericite-chlorite alteration, also in groundmass. |
| --- | --- |

Field Note :	May be similar to 424176, but coarser with grey matrix

In thin section, this core piece is identified as a dacitic-porphyry, without any K-spar (as indicated by negative response to staining).  The groundmass is a microcrystalline mosaic, and there are prominent large quartz phenocrysts, all of which compare more with sample 424174 rather than to 424176 questioned in the field note.

The groundmass which forms at least 50% of the sample consists of massive extremely fine crystalline micromosaic of essential quartz, and of former feldspar (plagioclase) which is completely altered to sericite > ultrafine chlorite.

Phenocrysts range in size from 0.3mm to 5mm, average size about 3mm, which is generally coarser than in the porphyries above, and these consist of :

| * | Quartz crystals, mostly more or less bipyramidal, some embayed, some as composites | 10-15% |
| --- | --- | --- |
| * | Plagioclase crystals, clouded by clay-sericite alteration, but some with relict (twinned) crystals visible in the thin section | 15-20% |
| * | Biotite crystals, to 2.5mm, extensively altered to chlorite ± muscovite, with fine granular leucoxene. These seem to be most of the green altered phenocrysts seen in handspecimen | 10% |
| * | Hornblende crystals, completely altered to an unusually very pale chlorite crowded with minute rutile/anatase crystals | 3% |

Accessory (1%) very small pyrite crystals are disseminated and selectively within altered biotite crystals.  Minor braided stringers of fine chlorite and/or quartz occur locally.

| Fig 19 | 424177 | _____ 0.45 mm |
| --- | --- | --- |

TS.  Xnic (x20).  Dacite porphyry with clear quartz phenocrysts, several altered plagioclase phenocrysts, in microcrystalline to cryptocrystalline, sericite-altered, quartzo-feldspathic groundmass.

| 424178 BBRD-002, 549.7m | Homogeneous sericite-rich shale in sharp contact with a wacke containing detrital quartz, altered feldspar and altered biotite, probably largely volcanically derived. Also minor pelitic lithic detritus and a matrix of weakly schistose meta silty pelite. Minor quartz stringers in both facies. |
| --- | --- |

Field Note :	Greywacke-shale graded bed contact.  Possible Tennant Creek Warramunga Formation

The petrography confirms that this sample has a contact between sericite-rich shale and a wacke, but there is no apparent graded bedding.

The shale consists of compact, massive, albeit weakly layered/bedded, sericite/fine muscovite, very minor chlorite and disseminated titaniferous ‘spots’.  Several localised small tension-gash-type partings, filled by microcrystalline quartz carrying minor chlorite and hematite, are confined to the shale layer.

The greywacke consists of about 50% of poorly sorted, weakly layered/bedded detrital grains of fine sandsize quartz, to minor coarse size (0.5mm), and one lithic fragment of vein quartz 3mm across.  Some of these quartz grains have a morphology indicating derivation from quartz phenocrysts (?possibly reworked tuff).  Detrital grains of clouded clay-sericite-altered feldspar are co-dominant with the quartz grains, together with minor detrital chlorite-altered biotite, and these components seem to be derived from a volcanic source.  Other lithic grains appear to be derived from metapelitic siltstone however.

The whole rock matrix is weakly foliated sericite-rich very fine schist, basically metasilty-pelitic.  Several random veinlets of microfine crystalline quartz cut through this wacke.