MINERALOGICAL REPORT No.  8160

by Alan C. Purvis,  PhD

3 December,  2001

TO :	Mr Peter Simpson

Exploration Manager

Giants Reef Mining Ltd

PO  Box 1244

TENNANT CREEK  NT  0861

YOUR REFERENCE :	Order No. 200633

MATERIAL :	Chemical Assay Data

Drill core samples, 002

WORK REQUESTED :	Assessment of assay data provided and report as discussed.

DIGITAL COPY :	Enclosed with hard copy of this report.

PONTIFEX & ASSOCIATES PTY. LTD.

# INTRODUCTION

Alan Purvis has recently petrologically described and discussed core samples from drill holes BBRD-001 and 002 for Giants Reef, Tennant Creek  [Pontifex Report Nos. 8124  (28/8/01) and 8142 (22/10/01)].  The presence of unusual mafic rocks in BBRD-002 prompted the suggestion to Peter Simpson that several of these samples, (also several mafic samples from 001 for comparison) should be chemically analysed for selected elements, and that this data be assessed by Alan, with the aim of assisting interpretations of their geological setting.

Chemical data was subsequently received from Giants Reef Mining/AMDEL.  An assessment of these data in the context of the above, follow in this report (presented separately of BBRD-001 and BBRD002, since it transpires that there is some question regarding the validity of some of the BBRD-001 data.)

ASSESSMENT OF CHEMICAL DATA BBRD-002

The chemical information for BBRD-002 core refer to the following sample numbers, best-fit depth, and corresponding petrographic classification (Report 8142) as follows :

| Number | Depth Interval (m) | Essential petrography |
| --- | --- | --- |
| 80523 | 78-84 | Ankaramite or picrite |
| 80534 | 144-150 | Ankaramite or picrite |
| 80547 | 222-228 | Ankaramitic volcanic flooded by carbonate |
| 424721 | 306-308 | Ankaramite or picrite |
| 424741 | 335.5-336.5 | Sericite-rich altered volcanic |
| 424803 | 456-458 | Mafic hyaloclastite, extensive carbonate-chlorite-altered matrix. |

As indicated above, these samples include four described as ankaramite or picrite, one sericite-rich altered volcanic rock and an altered mafic hyaloclastite.  The analyses were recalculated to 100% anhydrous, assuming 25% of total iron as Fe2O3.  Only iron provided as Fe2O3 was used, as iron reported as Fe and manganese reported as Mn, also provided, did nor correlate with iron provided as Fe2O3 or manganese as MnO, both being lower, indicating incomplete solution of the sample.  These lavas are altered, so that methods requiring fresh rocks, such as the total alkali-silica diagram, are not appropriate in clarifying the nature and setting of these samples

# Discussion

CIPW norms were calculated for the BBRD-002 samples and these showed that the normative mineralogy was in accord with the petrography.  The results are given in Table 2, below.  Four samples described as ankaramite or picrite have abundant normative pyroxene ± olivine, but one sample (80547), is flooded by carbonate, and excess CaO and low SiO2, meaning that the geochemistry is a poor indicator of the original mineralogy.  However the textures seen in thin section, together with high MgO, Ni and Cr suggest an ankaramite.  The sericitised sample (424741) was described as possibly basaltic, but with very poor textural preservation.  The composition is closer to that of dacites from extensional terranes, suggesting a more felsic protolith than was interpreted from thin section petrology.  Data on elements such as Zr, Nb and Y would be useful in further clarifying the original lithology for this sample.

The more mafic lavas mostly have low Al2O3 and high TiO2 contents as well as high MgO, Cr and Ni, and similar lavas are known to the writer from work carried out by Xiaowen Sun at Adelaide University on mafic lavas in an extensional basin in the northeast of South Australia.  This basin is called the Warburton Basin and formed inboard of the Tasman Line, separating Proterozoic Australia from the Tasman Orogenic Belt.  Lavas in this area are of late Neoproterozoic to Cambrian in age, but it is the tectonic setting that is similar to the samples from the Tennant Creek area, rather than the age.  Charts showing comparisons between the Tennant Creek and Warburton Basin sample are given as Fig 1 (MgO against TiO2), and Fig.2 (Al2O3 against TiO2).  The more felsic sample is also similar to felsic volcanics in the same basin, with similarly abundant sericite.  The felsite from Tennant Creek has 9% normative corundum (present in sericite) with as much as 12% in those from South Australia.

The titanium contents and Al2O3/TiO2 ratios of the more mafic lavas suggest picrite and ankaramite of alkali olivine basalt or transitional tholeiite suites.  Further refining of this suggestion would require data on Zr, Nb and Y, with lower Ti/Zr ratios in alkali olivine basalts and Nb/Y ratios of more than 0.7.  In tholeiites, the Nb/Y ratio would be less than 0.7 and higher Ti/Zr ratios would be seen.

# Conclusions

These samples include basalts, ankaramites and felsic lavas that seem to represent an extensional tectonic regime and may be related to alkali olivine basalt or tholeiite magma suites.  Similar lavas, with similar alteration, occur in the Warburton Basin in northeastern South Australia, probably related to the breakup of Rodinia and the formation of the Oceanic Tasman Orogenic zone.  The older samples from near Tennant Creek would also be in a significant extensional terrane, but the exact events to which they may be related are unclear.

# Table 2: Analyses from BBRD002, and similar lithologies from South Australia

|  | 80523 | 80534 | 80547 | 424721 | 424803 | Warburton |  |  |  | 424741 | Warburton |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 78 | 144 | 222 | 306 | 456 |  |  |  | G1 | 335.5 | Co-1 | G-4 |
|  | 84 | 150 | 228 | 308 | 458 | J-6 | J-6 | NU27 | 3376.5 | 336.5 | 3637.48 | 2211.93 |
| SiO2 | 43.80 | 48.34 | 44.03 | 47.07 | 47.07 | 45.83 | 46.53 | 42.80 | 44.96 | 63.28 | 67.17 | 64.62 |
| TiO2 | 1.19 | 2.24 | 2.68 | 1.86 | 1.79 | 1.91 | 1.61 | 1.14 | 3.82 | 0.72 | 0.57 | 0.91 |
| Al2O3 | 6.37 | 8.91 | 10.50 | 8.67 | 11.40 | 6.26 | 6.92 | 6.20 | 13.68 | 16.45 | 15.25 | 18.97 |
| Fe2O3 | 3.00 | 3.18 | 2.48 | 3.20 | 3.18 | 3.45 | 3.32 | 4.07 | 3.47 | 2.23 | 1.36 | 2.79 |
| FeO | 9.49 | 8.92 | 7.62 | 9.39 | 10.17 | 9.32 | 8.97 | 10.98 | 9.36 | 6.33 | 3.32 | 3.26 |
| MnO | 0.19 | 0.18 | 0.19 | 0.23 | 0.19 | 0.15 | 0.14 | 0.21 | 0.22 | 0.07 | 0.11 | 0.11 |
| MgO | 21.67 | 11.85 | 7.31 | 18.32 | 13.45 | 20.02 | 20.70 | 27.38 | 7.11 | 4.23 | 2.67 | 1.96 |
| CaO | 12.53 | 13.09 | 20.97 | 9.38 | 11.21 | 11.03 | 9.70 | 5.01 | 11.57 | 0.29 | 3.98 | 0.14 |
| Na2O | 0.29 | 1.52 | 2.51 | 0.64 | 0.06 | 0.54 | 0.60 | 0.57 | 3.86 | 1.78 | 1.16 | 0.27 |
| K2O | 0.10 | 0.47 | 0.47 | 0.05 | 0.01 | 0.22 | 0.29 | 0.23 | 0.46 | 3.90 | 4.26 | 6.45 |
| P2O5 | 0.10 | 0.22 | 0.19 | 0.23 | 0.18 | 0.20 | 0.19 | 0.21 | 0.29 | 0.15 | 0.13 | 0.25 |
| Total | 98.74 | 98.92 | 98.94 | 99.05 | 98.70 | 98.93 | 98.97 | 98.80 | 98.79 | 99.44 | 99.97 | 99.71 |
| Al2O3/TiO2 | 5.37 | 3.98 | 3.91 | 4.65 | 6.37 | 3.29 | 4.29 | 5.44 | 3.58 | 22.73 | 26.98 | 20.82 |
| Alk | 0.40 | 1.99 | 2.98 | 0.69 | 0.07 | 0.76 | 0.89 | 0.80 | 4.32 | 5.68 | 5.43 | 6.72 |

Table 2: Continued

| CIPW | Norm |  |  |  |  |  |  |  | G1 | 424741 | Co-1 | G-4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 80523 | 80534 | 80547 | 424721 | 424803 | J-6 | J-6 | NU27 | 3376.5 | 336.50 | 3637.48 | 2211.93 |
| Qtz | 0.00 | 0.00 | 0.00 | 0.00 | 2.44 | 0.00 | 0.00 | 0.00 | 0.00 | 27.04 | 29.54 | 34.28 |
| Or | 0.60 | 2.80 | 2.80 | 0.31 | 0.07 | 1.33 | 1.74 | 1.36 | 2.69 | 23.07 | 25.20 | 38.12 |
| Ab | 2.48 | 12.82 | -1.51 | 5.41 | 0.48 | 4.53 | 5.07 | 4.82 | 19.75 | 15.06 | 9.83 | 2.27 |
| An | 15.75 | 16.10 | 15.99 | 20.63 | 30.81 | 14.02 | 15.32 | 13.68 | 18.63 | 0.43 | 18.89 | -0.92 |
| Ne | 0.00 | 0.00 | 12.30 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 7.01 | 0.00 | 0.00 | 0.00 |
| C | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 9.13 | 1.79 | 11.88 |
| Cpx | 35.97 | 37.54 | 61.52 | 19.21 | 18.79 | 30.91 | 24.78 | 7.70 | 29.44 | 0.00 | 0.00 | 0.00 |
| Wo | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Opx | 5.44 | 17.44 | 0.00 | 36.27 | 37.79 | 21.31 | 26.24 | 20.17 | 0.00 | 18.29 | 9.91 | 6.32 |
| Ol | 33.43 | 3.25 | 0.00 | 9.06 | 0.00 | 18.74 | 18.45 | 44.51 | 0.00 | 0.00 | 0.00 | 0.00 |
| Mt | 4.35 | 4.60 | 3.60 | 4.64 | 4.60 | 5.01 | 4.82 | 5.90 | 5.03 | 3.23 | 1.98 | 4.04 |
| Il | 2.25 | 4.25 | 5.09 | 3.54 | 3.40 | 3.62 | 3.06 | 2.17 | 7.25 | 1.37 | 1.07 | 1.73 |
| Ap | 0.24 | 0.52 | 0.44 | 0.54 | 0.42 | 0.47 | 0.43 | 0.49 | 0.67 | 0.36 | 0.30 | 0.57 |
| Total | 100.52 | 99.32 | 100.23 | 99.60 | 98.80 | 99.94 | 99.91 | 100.79 | 90.48 | 99.92 | 100.46 | 100.20 |
|  | 80523 | 80534 | 80547 | 424721 | 424803 | Warburton |  |  |  | 424741 |  |  |
| Olivine | 33.43 | 3.25 | 0.00 | 9.06 | 0.00 | 18.74 | 18.45 | 44.51 | 0.00 | 0.00 | 0.00 | 0.00 |
| Pyroxenes | 41.41 | 54.99 | 61.52 | 55.48 | 56.58 | 52.22 | 51.02 | 27.86 | 29.44 | 20.23 | 11.84 | 8.23 |
| Feldspars | 18.84 | 31.72 | 29.59 | 26.35 | 31.36 | 19.88 | 22.13 | 19.86 | 48.09 | 38.56 | 53.93 | 39.47 |
| Accessories | 6.84 | 9.37 | 9.13 | 8.72 | 8.43 | 9.10 | 8.31 | 8.55 | 12.95 | 4.96 | 3.35 | 6.34 |
| Quartz | 0.00 | 0.00 | 0.00 | 0.00 | 2.44 | 0.00 | 0.00 | 0.00 | 0.00 | 27.04 | 29.54 | 34.28 |
| Corundum | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 9.13 | 1.79 | 11.88 |

### Figure 1,  8160.xls goes here

## Figure 2,  8160.xls goes here