MINERALOGICAL REPORT No.  8256

by Alan C. Purvis,  PhD

September 02,  2002

TO :	Mr Peter Simpson

Giant’s Reef Mining Limited

PO  Box 1244

TENNANT CREEK  NT  0861

YOUR REFERENCE :

Order No. 200938

(EL8883, Sample Nos. 80596 and 80597)

MATERIAL :	RC Drill chip samples

WORK REQUESTED :	Thin section preparation, description and report with comments and interpretations as specified.

SAMPLES & SECTIONS :	Returned to you with this report.

DIGITAL COPY :	Enclosed with hard copy of this report.

PONTIFEX & ASSOCIATES PTY. LTD.

# INTRODUCTION

Two samples of drill chips from EL8883 (O/N 200938) are described in this report from the chips mounted in epoxy and prepared as thin sections.

The samples are numbered consecutively from 80585 to 80597  and a list of the essential petrography of each sample (as repeats of headers to the individual descriptions) appears in this report after the location Fig 1.  These are basically the same as faxed to Giant’s Reef, Tennant Creek office 30/8/02.

# SUMMARY COMMENTS

Chips from two samples in drillhole BBRC-004 consist of tremolite-actinolite-rich schists with minor chlorite and accessory fine opaque oxide and these are interpreted to be primarily of ultramafic origin (metapyroxenite?).  Chips in the adjacent hole BBRC010, (2 samples) consist of quartz-biotite-plagioclase schist and microcline-biotite-schist, representing metasediments partly with potassium enrichment and an attenuated quartz fabric suggesting some shearing.  These metasediments may be related to metasandstones described in Pontifex Report No. 8158.

Igneous lithologies are more abundant in the southern area (BBRC15-016), and pyroxene ± plagioclase-porphyritic lavas and breccias in BBRC-015-016.  The lava in BBRC-015 has abundant actinolite and albite/sericite-altered plagioclase as well as disseminated leucoxene.  The breccia in BBRC-016 has abundant epidote with various amounts of actinolite, albite and calcite.  These lavas may relate to the more pyroxene-rich ankaramites and picrites described in Pontifex Report No. 8142, with similar metamorphism and alteration.

# Table 1:  summary of headers to each petrographic description

| 80592 BBRC-015, 70-72m | Pyroxene ± plagioclase-porphyritic basalt with alteration involving albite-smectite-limonite-leucoxene. Veins contain clay, quartz and sparse limonite after sulphide. |
| --- | --- |
| 80596 BBRC-016, 47-48m | Partly weathered mafic lavas, with plagioclase and mafic phenocrysts, xenolithic vesicles, and cloudy actinolite-albite-sericite-leucoxene alteration. |
| 80597 BBRC-016, 52-54n | Brecciated probable mafic lava, with extensive alteration to clouded epidote, actinolite, albite, limonite and calcite. Cut by limonite-filled micro-fractures and carbonate stringers. |

**	If the reported Cu (2000ppm) and P (2.53%) were to occur together in the one phase, then possible minerals could include turquoise [CuAl6(PO4)4(OH)8.5H2O], pseudomalachite [Cu5(PO4)2(OH)4] and cornetite [Cu3PO4(OH)3], but with excess phosphorus (a relative to Cu) possibly in apatite or in iron-bearing phosphates such as vivianite [Fe3(PO4)2.8H2O] or vauxite [FeAl2(PO4)2(OH)2.5H2O].  No such coloured Cu-phosphates were seen in these chips however.  Fe-rich phosphates may be extremely fine and camouflaged by the dominant limonite throughout these chips, but probe analysis would be required to investigate this.

# INDIVIDUAL DESCRIPTIONS

| 80592 BBRC-015, 70-72m | Pyroxene ± plagioclase-porphyritic basalt with alteration involving albite-smectite-limonite-leucoxene. Veins contain clay, quartz and sparse limonite after sulphide. |
| --- | --- |

Field Note:	Intermediate Volcanic

These chips are all altered/weathered with minor to abundant altered phenocrysts, mostly less than 1mm long, largely altered to smectite, but some albitised plagioclase phenocrysts occur in one chip. One chip has an altered phenocryst composed of microcline and smectite, adjacent to a clay-filled vein, suggesting that the microcline is secondary.  The smectite seen in this thin section is most commonly derived from amphibole, but any amphibole in this sample may have formed from former pyroxene.  Some of the chips have abundant fine-grained albitised plagioclase in the groundmass, mostly unoriented, as well as clays, limonite and leucoxene, but most are poor in, or lack, albite.  The abundance of disseminated leucoxene suggests a reasonably titaniferous lithology, such as basalt, rather than an intermediate volcanic as suggested in your notes.

One chip has a vein with minor quartz in massive smectite possibly derived from chlorite.  Veins containing clays, quartz and sparse limonite, possibly after sulphide, occur and there are small, deformed quartz-rich chips that may represent silicified material that has been sheared.  Overall, the major lithology represented is interpreted as pyroxene (± plagioclase)-porphyritic basalt.

| 80596 BBRC-016, 47-48m | Partly weathered mafic lavas, with plagioclase and mafic phenocrysts, xenolithic vesicles, and cloudy actinolite-albite-sericite-leucoxene alteration. |
| --- | --- |

Field Note:	Basic volcanic?

Up to 15% of these dark chips consist of uralitic amphibole, locally replaced by smectite, replacing scattered primary phenocrysts of clinopyroxene to 3mm long.  Rarer plagioclase phenocrysts (3%), to 2mm long, have been replaced by sericite ± albite.  Clouded aggregates of amphibole and albite possibly fill vesicles (5-7%).  Small aggregates of composite uralitised pyroxene (partly altered to smectite) and albitised plagioclase seem to represent xenoliths.

The groundmasses are rich in clouded extremely fine actinolite and leucoxene, with lesser sericite in interstitial areas, apparently derived from plagioclase.  Early threads and stringers with actinolite and smectite are cut by later micro-fractures, largely filled by probable adularia.

These chips represent altered pyroxene and plagioclase-porphyritic mafic lavas as described in previous reports on samples from this area.

| 80597 BBRC-016, 52-54m | Brecciated probable mafic lava, with extensive alteration to clouded epidote, actinolite, albite, limonite and calcite. Cut by limonite-filled micro-fractures and carbonate stringers. |
| --- | --- |

Field Note:	Felsic volcanic?

The chips in this sample seem to represent breccias with millimetre to centimetre-scale clasts altered to clouded very fine to fine-grained epidote.  Some clasts have clear granular epidote patches apparently replacing phenocrysts, but it is not entirely clear whether the original phenocrysts were plagioclase or pyroxene.  Some seem to have been pyroxene, however, and more obvious small plagioclase phenocrysts have been replaced by albite with minor epidote, actinolite and limonite.  Irregular limonite-filled microfractures are abundant in many of the fragments.

Minor local interstitial areas are rich in granular to prismatic epidote, with minor to abundant albite, actinolite or quartz.  In some chips the interstitial material is largely granular albite with less abundant granular to prismatic epidote, but in others epidote is more abundant, with minor to abundant actinolite and/or with common to abundant calcite.

One chip has parallel stringers of granular and fibrous carbonate, separated by lamellae of limonite.

The clouded nature of the epidote makes specific identification of any leucoxene quite difficult.  Some of this clouding must surely be dispersed leucoxenitic dust, and considered together with the other gross mineralogy, a brecciated mafic volcanic would seem to be the most likely protolith.