MINERALOGICAL REPORT No.  8198

by Alan C. Purvis,  PhD

March 18,  2002

TO :	Mr Peter Simpson

Exploration Manager

Giant’s Reef Mining Ltd

PO  Box 1244

TENNANT CREEK  NT  0861

YOUR REFERENCE :	Original Order No. 200633

MATERIAL :	Additional Chemical Assay Data

Drill core samples,  BBRD-001, 002

WORK REQUESTED :	Re-assessment of assay data provided and report as discussed.

DIGITAL COPY :	Enclosed with hard copy of this report.

PONTIFEX & ASSOCIATES PTY. LTD.

INTRODUCTION

This report is basically an addendum to Pontifex Report No. 8160 (3/12/01) by Alan Purvis, which previously assessed chemical data provided by Giants Reef Mining Ltd in relation to even earlier petrographic identification of core samples from BBRD-001, 002.  Some of the previous chemistry, particularly for Fe, was incompatible with the petrology, resulting in new Fe assay data provided by Giants Reef (memo/fax dated 8/03/02) and which is discussed in this new report.

## DISCUSSION

It was noted in the previous report that there were two sets of analytical data of Fe for samples from drillhole BBRD002, with no correlation between the two sets.  Both sets of data were provided to Giants Reef Mining Limited by AMDEL, but were obtained by different analytical methods, one being IC2E.  The other data was listed as providing total iron as Fe2O3, but the method is not stated definitely on the assay sheet. The IC2E data provided Fe values that indicated less iron than the total Fe2O3 data and not consistent with the petrography, so that these lower values were not used in preparing the previous report.

Non-chemical analyses for Fe recently received from Giants Reef Mining Limited (also from AMDEL), were obtained by IC4 rather than IC2E, and these seem to be more consistent with the petrography.  The data for BBRD002 correlate with the total Fe2O3 values from the set used in preparing our previous report No. 8160, within a few % of the previously obtained values. Fig. 1 shows the correlation between new Fe data for BBRD002 (recalculated as Fe2O3) with the previous Fe2O3 data as used in Report No. 8160.  Table 1, below, provides newly recalculated analyses for the samples from BBRD001 (olivine gabbro, gabbronorite, norite and quartz diorite).

The previous Fe determinations analysed by IC2E, were not regarded as reliable and were not used in preparing the previous report on these samples, therefore very little modification is required to the interpreted lithologies for BBRD002 as given in that report.  The total iron in the samples from BBRD001 are less than the ‘guessed’ values used in the previous report, but indicate the presence of normative olivine in only one sample (424841/422797).  This calculated mineralogy is consisted with the petrography and with conclusions reached in that report, based on the geochemistry.

The overall geochemistry suggests that the samples from BBRD002 are within-plate, probably alkaline, and from an extensional environment, as discussed in the previous report.  Although the samples from BBRD001 are poorer in titanium and may be more tholeiitic in character, these also seem to suggest an extensional environment.  The interpreted original lithologies in BBRD001 include pyroxene-rich norite (424838/422791), mafic quartz diorite with tholeiitic affinities (424839/422793), tholeiitic, quartz-bearing gabbronorite (424840/422795) and olivine-pyroxene-rich gabbronorite (424841/422797) with brown hornblende in the last sample.  The high Fe/Mg ratio of samples 424839 would specifically suggest a tholeiitic, rather than calcalkaline character to this suite of samples.  The mineralogy, with biotite and/or hornblende, suggests a water-rich magma possibly formed from enriched lithosphere.

Fig. 1:	Correlation between previous ferric iron and new Fe data for drillhole BBRD002.

Note that the thin section numbers and the analysis numbers are not the same for this drillhole, so that these numbers present the analysis number followed by the thin section number,