Daly River, Northern Territory

Airborne Magnetic and Radiometric Geophysical Survey

Acquisition and Processing Report

for

Territory Uranium Company Limited

Prepared by :	P. Chambers  	…………………………..

L. Stenning	…………………………..

Authorised for release by :		…………………………..

…………………………..

Survey flown: July 2008

by

Fugro Airborne Surveys

65 Brockway Road, Floreat. WA 6014, Australia

Tel: (61-8) 9273 6400      Fax: (61-8) 9273 6466

FAS JOB # 1988

CONTENTS

1.	SURVEY OPERATIONS AND LOGISTICS	4

1.1	Introduction	4

1.2	Survey Base	4

1.3	Survey Personnel	4

1.4	Survey Equipment	4

1.5	Area Map	5

1.6	General Disclaimer	6

2.	SURVEY SPECIFICATIONS AND PARAMETERS	7

2.1	Area Co-ordinates	7

2.2	Survey Area Parameters	7

2.3	Data Sample Intervals	7

2.4	Survey Tolerances	8

3.	AIRCRAFT EQUIPMENT AND SPECIFICATIONS	9

3.1	Aircraft	9

3.2	Navigation System	9

3.3	Aircraft Magnetometers	9

3.4	Automatic Compensator	9

3.5	Gamma Ray Spectrometer System	9

3.6	Radar Altimeter	10

3.7	Barometric Altimeter	10

3.8	Flight Data Recording	10

3.9	Flight Following	10

4.	GROUND DATA ACQUISITION EQUIPMENT AND SPECIFICATIONS	11

4.1	Magnetic Base Station	11

4.2	GPS Base Station	11

5.	EQUIPMENT CALIBRATIONS AND DATA ACQUISITION CHECKS	12

5.1	Survey Calibrations	12

5.1.1	Dynamic Magnetometer Compensation	12

5.1.2	Parallax	12

5.1.3	Pad Calibrations	12

5.1.4	Background and Cosmic Calibration Stacks	12

5.1.5	Height Attenuation Calibrations	13

5.1.6	Daily Calibrations	13

6.	DATA VERIFICATION AND FIELD PROCESSING	14

6.1	Magnetic Diurnal Data	14

6.2	Height Data	14

6.2.1	Radar Altimeter Data	14

6.2.2	GPS Height Data	14

6.2.3	Barometric Altimeter Data	14

6.2.4	Topographical Data	14

6.2.5	Gridding and Inspection	14

6.3	Flight Path Data	14

6.4	Magnetic Data	15

6.4.1	Diurnal Correction	15

6.4.2	Parallax Correction	15

6.4.3	Preliminary Gridding and Inspection	15

6.5	Spectrometer Data	15

6.5.1	Parallax Correction	15

6.5.2	Preliminary Gridding and Inspection	15

7.	FINAL DATA PROCESSING	16

7.1	Aircraft Location	16

7.2	Magnetic Data Processing	16

7.2.1	Gridding	16

7.3	Radiometric Data Processing	16

7.3.1	NASVD Filtering	17

7.3.2	Energy Recalibration	17

7.3.3	Dead Time	17

7.3.4	STP Altitude	17

7.3.5	Cosmic and Aircraft Background Removal	17

7.3.6	Window Definitions	18

7.3.7	Radon Correction	18

7.3.8	Spectral Stripping	18

7.3.9	Height Correction	18

7.3.10	Gridding	18

7.4	Digital Elevation Model	19

7.4.1	Gridding	19

APPENDIX I – Weekly Operations Report	20

APPENDIX II – Button Calibration Data	23

APPENDIX III – Low Level Statistics	24

APPENDIX IV – Final Located Data Formats	25

APPENDIX V – List Of All Supplied Data	34

LIST OF TABLES

Table 1: Magnetometer Compensation Details	12

Table 2: Parallax Values	12

Table 3: Diurnal Base Values	16

Table 4: IGRF Base Values	16

Table 5: Aircraft Background and Cosmic Stripping Ratios	17

Table 6: IAEA Window Definitions	18

Table 7: Radon Stripping Values	18

Table 8: Spectral Stripping Ratios	18

Table 9: STP Altitude Coefficients	18

# SURVEY OPERATIONS AND LOGISTICS

## Introduction

Between the 15th of July 2008 and the 29th of July 2008, Fugro Airborne Surveys Pty. Ltd. (FAS) undertook an airborne magnetic and radiometric survey for Territory Uranium Company Limited, over the Daly River Project area, in the Northern Territory. The survey consisted of one area, flown in 13 flights. Total coverage of the survey area amounted to 7,248 line kilometres. The survey was flown using a Cresco 750 aircraft, registration VH-KPY owned and operated by FAS. This report summarises the procedures and equipment used by FAS in the acquisition, verification and processing of the airborne geophysical data.

## Survey Base

The survey was based out of Daly River, Northern Territory. The survey aircraft was operated from the Daly River airstrip with the aircraft fuel available on site. A temporary office was set up at the Daly River Roadside inn, where all survey operations were run and the post-flight data verification was performed.

## Survey Personnel

The following personnel were involved in this project:

Project Supervision	- Acquisition	Bart Anderson

- Processing	Adam Shales

On-site Crew Leader	Murray Johnston

Pilot/s	Terry Miller, Tim Masefield

System Operator/s	Murray Johnston, Dave Hart, P Bombardieri

Data Processing	Peter Chambers

## Survey Equipment

Survey Platform	-	PAC Cresco 750 VH-KPY

Data Acquisition System	-	FAS digital acquisition system

Total Field Magnetometer	- 	Scintrex CS-2 Caesium vapour

Vector Magnetometer	-	Develco 3-Axis M9202

Magnetometer Compensator	-	Fugro FASDAS Mag Decoupler Unit Aeromagnetic Digital

Gamma-ray Spectrometer	-	Exploranium GR820 256 Channels

Gamma-ray Detector	-	8 Nal(Tl) crystals; 33.56 L down

Navigation System GPS	-	Fugro Omnistar in VBS (Virtual Base Station) mode,

Novatel OEM4 GPS receiver

Base Station Magnetometers	-	2 x Scintrex Envi Mag

Altimeter	                                      	-       	Sperry Stars AA-200 radio altimeter

Barometer	                                      	-      	Rosemount 1241M barometric altimeter

Thermometer                                     	-      	Vaisala HMY 133 temperature and humidity sensor

## Area Map

## 1.6	General Disclaimer

It is Fugro Airborne Survey’s understanding that the data and report provided to the client is to be used for the purpose agreed between the parties. That purpose was a significant factor in determining the scope and level of the Services being offered to the Client. Should the purpose for which the data and report is used change, the data and report may no longer be valid or appropriate and any further use of, or reliance upon, the data and report in those circumstances by the Client without Fugro Airborne Survey's review and advice shall be at the Client's own or sole risk.

The Services were performed by Fugro Airborne Survey exclusively for the purposes of the Client. Should the data and report be made available in whole or part to any third party, and such party relies thereon, that party does so wholly at its own and sole risk and Fugro Airborne Survey disclaims any liability to such party.

Where the Services have involved Fugro Airborne Survey's use of any information provided by the Client or third parties, upon which Fugro Airborne Survey was reasonably entitled to rely, then the Services are limited by the accuracy of such information. Fugro Airborne Survey is not liable for any inaccuracies (including any incompleteness) in the said information, save as otherwise provided in the terms of the contract between the Client and Fugro Airborne Survey.

# SURVEY SPECIFICATIONS AND PARAMETERS

## Area Co-ordinates

The survey area was located within MGA Zone 52, Central Meridian = 129

(Note - Co-ordinates in GDA94/MGA Zone 52)

Easting		Northing

700072  	8488412

709045  	8488342

709194  	8475663

722172  	8475567

722069  	8451033

708998  	8451519

708605  	8426789

722543  	8415947

724598  	8415868

724549  	8410387

713850  	8410542

713899  	8416324

710272  	8416355

710303  	8420029

708670  	8420042

708504  	8421911

706811  	8421925

706765  	8423732

705012  	8423807

705026  	8425614

703212  	8425689

703286  	8434843

694272  	8434974

694723  	8486706

700118  	8486484

## Survey Area Parameters

Job Number	-	1988

Survey Company	-	Fugro Airborne Surveys Pty Ltd

Date Flown	-	15th July – 29th July 2008

Client	-	Territory Uranium Company Limited

Project Name	-	Daly River, Northern Territory

Nominal Terrain Clearance	-	60 m

Traverse Line Spacing	-	200 m

Traverse Line Direction	-	000 – 180 deg

Traverse Lines	-	20001 – 20226

Tie Line Spacing	-	2000 m

Tie Line Direction	-	090 – 270 deg

Tie Lines	-	29001 – 29039

Total Line Kilometres	-	7,248km

## Data Sample Intervals

Nominal data sample intervals.

Magnetometer	-	6.6 m (@10 Hz)

Radar Altimeter	-	6.6 m (@10 Hz)

Thermometer	-	66 m (@1 Hz)

Barometer	-	66 m (@1 Hz)

GPS	-	66 m (@1 Hz)

Spectrometer	-	66 m (@1 Hz)

Magnetic Base Station	 (ENVI Mag)	-	5 s

## Survey Tolerances

As specified in the contract the following tolerances were used:

Traverse line deviation	-	+/- 50% of nominated line spacing over 1 km or more

Tie line deviation	-	+/- 50% of nominated tie line spacing over 1 km or more

Terrain clearance deviation	-	+/-10 m of nominal terrain clearance over 1 km or more,

except where such lines breach air regulations, or in the

opinion of the pilot, put aircraft and crew at risk.

Total magnetometer system noise	-	More than 0.1 nT continuously for more than 1 km

Magnetic diurnal variation	-	More than 10 nT in 10 minutes non-linear either on flight

lines or tie lines.

# AIRCRAFT EQUIPMENT AND SPECIFICATIONS

## Aircraft

Manufacturer	-	PAC

Model	-	Cresco 750

Registration	-	VH-KPY

Ownership	-	Fugro Airborne Surveys Pty Ltd

## Navigation System

The GPS receiver was integrated as part of the acquisition system. Navigation displays were generated by the acquisition system software that displayed to the pilot a graphical representation of the line being flown. A pre-defined flight plan, with area boundaries and the start and end of the line co-ordinates, was loaded into memory and used for real-time navigation information. Position co-ordinates and other relevant GPS information were output and recorded by the acquisition computer.

## Aircraft Magnetometers

The survey was flown using a Scintrex CS-2 ultra-high sensitivity Caesium vapour magnetometer sensor with the sensor mounted in the tail stinger of the aircraft. The sensor provides a Larmor signal that is processed by high precision counters embedded within the FASDAS to provide an operating range of 20,000 to 100,000 nT.

Specifications

Nominal Sensitivity:	-	0.001 nT

Still Air RMS Noise:	-	0.05 nT

Digital Recording Resolution:	-	0.001 nT

Magnetic Gradient Tolerance 	-	>20,000 nT/m

## Automatic Compensator

The magnetometer data, together with data from the 3-axis fluxgate, was integrated in the acquisition system to produce real time compensation for the effects of the aircraft’s motion, i.e. from changes in attitude and heading. The compensation coefficients were calculated from compensation flights carried out before the survey commenced. The compensated output data, with a resolution and sensitivity of 0.001 nT at a sampling rate of 10 times per second, were recorded digitally.

## Gamma Ray Spectrometer System

The radiometric acquisition system consisted of a 256 channel gamma-ray spectrometer and detector system with the following specifications:

Manufacturer:			Exploranium Inc.

Model:				GR-820

Number of channels:		256

Crystal	Volume:		33.56 L downward looking (thermally insulated)

Sampling interval:		1 s

Windows (keV):			Potassium: 	1370 to 1570

Uranium: 	1660 to 1860

Thorium: 	2410 to 2810

Total Count: 	  410 to 2810

Cosmic:           4000 to >6000

Data checking in the survey system was carried out by the use of resolution procedures using known radiometric sources. To verify the system, real time display of individual crystal resolutions and system resolutions, real time display peak channel tracking information, real time display of the energy spectrum showing counts, cosmic level and system livetime were available. The survey system displayed to the operator any errors encountered in the spectrometer system.

## Radar Altimeter

A Sperry Stars AA-200 radio altimeter system was used to measure ground clearance. The radio altimeter indicator provides an absolute altitude display from 0 - 750 metres (0 - 2,500 feet) with a sensitivity of 4 mV/ft. Radar altimeter data were digitally recorded every 0.1 seconds.

Specifications

Range:	-	0 - 2500 ft

Accuracy:	-	1%

Resolution:	-	4 mV/ft

## Barometric Altimeter

The output of the Rosemount 1241M Barometric Altimeter was used for calculating the barometric altitude of the aircraft. The atmospheric pressure was taken from a probe and fed to the transducer. The transducer uses a precise quartz crystal resonator whose frequency of oscillation varies with pressure induced stress. The temperature of the pressure sensor was also recorded. In conjunction with the area QNH pressure and ambient temperature, the barometric altitude was calculated.

Specifications

Range:	-	sea level to 10,000 ft

Accuracy:	-	5 ft

Resolution:	-	1 mV/ft

## Flight Data Recording

All data recorded by the data acquisition system were stored in a digital format on the removable media drive located in the DAS. This data were then transferred to the field office computers for post-flight quality control examination.

## Flight Following

An integral part of the Safety Management System provides for the installation of a Flight Following System that transmits a position via satellite at pre determined intervals. The Fugro OmniTRACK system is fitted to the aircraft and position information is transmitted every 4 minutes to the Omnistar Network control centre. This information can be monitored by accessing the Fugro web page where the updated flight path is displayed. The aircraft is also fitted with an emergency switch and activation of this by the pilot or crew will notify the Omnistar Network control centre immediately. They in turn will contact FAS personnel as per the Emergency Response Plan.

# GROUND DATA ACQUISITION EQUIPMENT AND SPECIFICATIONS

## Magnetic Base Station

Two Scintrex Envi Mag magnetometers were used to measure the daily variations of the Earth’s magnetic field. The base stations were established in an area of low gradient, away from cultural influences. The base stations were run continuously throughout the survey flying period with a sampling interval of 5 seconds at a sensitivity of 0.01 nT. The base station data were closely examined after each day’s production flying to determine if any data had been acquired during periods of out-of-specification diurnal variation. The base stations were located on the grassed area north of the runway, of Daly River airstrip, approximately 100 m apart.

## GPS Base Station

A GPS base logging station was set up at the Daly River Roadside Inn. The GPS antenna was located on the gas pipeline on the roof of Donga 5.

The GPS base system was comprised of a GPS receiver, a logging computer, an antenna and a power supply. Data was logged and displayed in real time on the logging computer screen. The logged base data was processed with the airborne GPS data to calculate the differentially post-processed position of the aircraft.

The GPS base station position was calculated by logging data continuously at the base position over a period of approximately 36 hours. These data were then statistically averaged to obtain the position of the base station.

The calculated GPS base position was (in WGS84):

13° 45’ 54.84299” S, 130° 42’ 37.30558” E, 61.726 m.

# EQUIPMENT CALIBRATIONS AND DATA ACQUISITION CHECKS

## Survey Calibrations

A series of calibrations were performed as follows:

### Dynamic Magnetometer Compensation

Carrying a magnetometer through a varying field in a non-uniform orientation produces manoeuvre noise. To compensate for this manoeuvre noise a standard compensation test flight called a “comp box” was flown. The compensation file produced also removed the majority of the heading error. Aircraft compensation tests were flown on the 4 survey line headings and also at +/- 7½ and 15o to the line headings (to accommodate for cross wind flying conditions). The data for each heading consists of a series of aircraft manoeuvres with large angular excursions: specifically pitches, rolls and yaws. This was done to artificially create the worst possible attitudes and rates of attitudinal change likely to be encountered while on line and compensate for any magnetic noise created by the aircraft’s motion within the earth’s magnetic field. The data was processed to obtain the real-time compensation terms. These coefficients were applied in real-time or later during post-processing if required. Note that this form of compensation will only remove those noise effects modelled in the manoeuvre test flight. Random motions of the stinger with respect to the aircraft airframe generally establish the noise floor for this type of installation. Details of the comp boxes flown for this survey are shown in the table below.

| Flown | Flights covered |
| --- | --- |
| 15/7/2008 | Flight 1 – 3 |
| 20/7/2008 | Flights 4 – 13 |

Table 1: Magnetometer Compensation Details

### Parallax

Parallax error is caused by the physical difference in distance between the various sensors, the electronic delay and software timing in the acquisition system. Hence all variables are subjected to a displacement from the GPS co-ordinates. If these variables are processed without a position offset a parallax error will usually occur. The most suitable way to treat this problem is to use the 1 second radiometric data as a base with a zero correction. This will prevent interpolation of important variables (a filtering process). The co-ordinates were moved by linear interpolation and other data variables were displaced onto the radiometric data, without change.

| Data | Parallax |
| --- | --- |
| Radiometrics | 0 second |
| GPS | -0.5 second |
| Magnetics | 0 second |
| Radar Altitude | 0 second |
| Pressure | 0.1 second |
| Temperature | 0 second |

Table 2: Parallax Values

### Pad Calibrations

A series of tests were taken using a set of radiometric pads of known concentrations of Potassium, Uranium and Thorium. Each crystal pack was tested individually, with data accumulated for 15 minutes. The pad calibration data were processed to determine the radiometric stripping coefficients for each crystal pack. Where aircraft had more that one crystal pack installed, the average of the stripping coefficients were used in final data processing.

### Background and Cosmic Calibration Stacks

High-level stacks were flown over the ocean away form the effects of any land based radon. Data were collected for ten minutes at altitudes starting at 1000 feet above sea level and incrementing to 10000 feet above sea level. The high-level stack data were processed to determine the cosmic and aircraft background coefficients.

### Height Attenuation Calibrations

Low-level stacks were flown over the Carnamah Dynamic Test Range, Western Australia. Data were collected at altitudes of 130 feet above sea level (ASL), 200 ft ASL, 260 ft ASL, 330 ft ASL, 400 ft ASL and 650 ft ASL. The neighbouring salt lake was flown at the same altitudes, and the data were used as a radon test. A ground survey was carried out on the same day using a calibrated gamma-ray spectrometer.

The airborne and ground data were processed to determine radioelement sensitivity and height attenuation coefficients.

### Daily Calibrations

A set of calibrations were performed each survey day as follows:

Magnetic base station time check

Spectrometer resolution test

Spectrometer button test

Low level test line

#### Magnetic Base Station Time Check

Prior to each day’s survey all magnetic base stations were time checked and synchronised with the time on the aircraft survey system GPS receiver.

#### Spectrometer Resolution Test

Once the spectrometer had stabilised a Thorium source resolution check was carried out by placing the source in a cradle specially designed to ensure precisely repeatable locations.

#### Spectrometer Button Test

Thorium sample checks were performed on the spectrometer before and after each day’s survey acquisition. Each sample was placed in a predetermined location and data recorded for 180 sec. Relative count rates above background were within +/- 5% of the average sample checks for the duration of the survey.

#### Low Level Test line

To monitor the effects of soil moisture and radon and to verify the system was functioning correctly a low level test line was flown at survey altitude prior to and after each day’s production. The collected data were checked by the operator to ensure the Thorium for the low level test line was within +/- 10% of the initial average. The low level test line was located crossing the north west corner of the survey block.

A	692461E	8479273N

B	695930E	8488623N

(Coordinates in GDA94 MGA52)

# DATA VERIFICATION AND FIELD PROCESSING

All data verification was conducted at the field office in Daly River for the duration of the survey. At the conclusion of each days survey all magnetic, radiometric, altimeter, flight path and diurnal data were downloaded onto the field office computer for preliminary verification. All raw aircraft data were backed up at the end of each day’s survey. One copy was sent to the FAS office in Perth, the other copy remaining at the field office.

## Magnetic Diurnal Data

Diurnal data recorded from the primary base station was downloaded onto the field office computer. The data was checked for spikes and erroneous readings. If invalid diurnal data occurred whilst survey data was being acquired the affected section was re-flown. The diurnal data was also checked to see that the change in diurnal readings during the course of the survey did not exceed the specified tolerances. When this occurred the affected part of the survey line was re-flown. The diurnal data was merged with the aircraft data and used in the verification of the magnetic data. Diurnal data recorded on the secondary base station was also downloaded onto the field office computer.

## Height Data

Radar altimeter, barometric altimeter and GPS height data from the aircraft were transferred onto the field office computer.

### Radar Altimeter Data

The radar altimeter data was verified to check that a reasonably constant height above the terrain was flown, readings during the course of the survey did not exceed the specified tolerances and for equipment reliability.

### GPS Height Data

The aircraft’s height above the WGS84 ellipsoid each second was determined by differentially post-processing the synchronised GPS data from the aircraft and GPS base station data. The GPS height of the aircraft was verified to check for data masking and for equipment reliability.

### Barometric Altimeter Data

As a backup to the aircraft’s GPS height, barometric height was also recorded. The barometric height of the aircraft was verified to check for equipment reliability. The barometric data were also used in the processing of the radiometric data.

### Topographical Data

After verification parallax corrections were applied, the radar altitude was subtracted from the GPS height to give the elevation of the terrain above the WGS84 ellipsoid. It was not considered necessary to make any further corrections as this data was for verification purposes only.

### Gridding and Inspection

The topographical data was gridded and grid image enhancements were computed and displayed on screen. These were inspected for inconsistencies and errors.

## Flight Path Data

The flight path data from the aircraft and the GPS base station were transferred onto the field office computer. The aircraft’s precise location each second was determined by differentially post-processing the synchronised GPS data from the aircraft and GPS base station data. The flight path was recovered and plotted daily to ensure it was within specification. Any data not within specification was re-flown. The flight path data was then merged with the rest of the aircraft and diurnal data. Both the aircraft and GPS base station recorded the data in the WGS84 datum.

## Magnetic Data

The real-time compensated and uncompensated magnetic data from the aircraft recorded every 0.1 second were transferred onto the field office computer. The raw magnetic data was checked to identify noise and spikes. If the noise exceeded the specified tolerances the part of the line affected was re-flown. After the magnetic data were merged with the digital flight path the following sequence of operations were carried out to allow inspection and verification of the data:

### Diurnal Correction

The synchronised digital diurnal data collected by the base station was first subtracted from the corresponding airborne magnetic readings to calculate a difference. The resultant difference was then subtracted from the base value to produce diurnally corrected magnetic data.

### Parallax Correction

The diurnally corrected magnetic data was corrected for system parallax using the calculated value.

### Preliminary Gridding and Inspection

The magnetic data were gridded and grid image enhancements were computed and displayed on screen. These were inspected for inconsistencies and errors.

## Spectrometer Data

Spectrometer data from the aircraft were transferred onto the field office computer. The data was verified to check that readings during the course of the survey did not exceed the specified tolerances and for equipment reliability.

### Parallax Correction

The raw window data were corrected for system parallax using the calculated value.

### Preliminary Gridding and Inspection

The spectrometer data were gridded and grid image enhancements were computed and displayed on screen. These were inspected for inconsistencies and errors.

# FINAL DATA PROCESSING

## Aircraft Location

The aircraft’s location each second was determined by differentially post-processing the synchronised GPS data recorded on both the aircraft and GPS base station. This data is recorded in the WGS84 datum.

## Magnetic Data Processing

The processing procedures applied to the magnetic data are summarised below:

- Apply any spike corrections to the compensated magnetic variables.

- Interpolate undefined magnetic values.

- Co-ordinate the data with post-processed GPS data.

- Filter diurnal values and subtract them from individual compensated magnetic readings.

| Area | Base Value |
| --- | --- |
| Daly River | 47110 nT |

Table 3: Diurnal Base Values

- Apply parallax correction.

- Correct for regional effects of the earth’s magnetic field by calculating the IGRF value at each fiducial using IGRF model 2005 and secular variation model. A base value was added back.

| Area | IGRF Model | Base Value |
| --- | --- | --- |
| Daly River | 15/7/2008 | 47270 nT |

Table 4: IGRF Base Values

- Using the tie lines (flown at 90 degrees to the traverse lines) a set of miss-tie values were determined. These miss-tie values reflected the differences in the magnetic value between the tie lines and traverse lines over the same geographical point. Using a least squares fit algorithm, which also takes into account the statistical variation inherent in DGPS positioning, a series of corrections were applied to the traverse line data. These allowed the data to be levelled to the same base value.

- Following this, a FAS proprietary microlevelling process was applied in order to more subtly level the data.

### Gridding

- The final levelled magnetic data were gridded using a bi-directional spline algorithm. The data was gridded with a cell size of 50 m.

## Radiometric Data Processing

The radiometric data was processed using the standard IAEA window processing technique as summarised below.

- Co-ordinate the data with post-processed GPS data.

- Apply spike corrections to the radar altimeter, temperature and pressure values.

- Apply parallax corrections to altimeter, temperature and pressure values.

- Apply NASVD filtering to the 256 channel radiometric data.

- Correct for dead time.

- Calculate the equivalent terrain clearance at STP (standard temperature and pressure).

- Remove aircraft background.

- Remove cosmic background.

- Window the 256 channel data using the IAEA standard energy windows.

- Remove radon background.

- Apply stripping ratios.

- Apply height corrections.

- Following this, a Fugro proprietary micro-levelling process was applied in order to more subtly level the data.

### NASVD Filtering

The radiometrics were produced with NASVD smoothing. Using the NASVD technique, the raw spectra were first smoothed using 6 principal components. Eigenvectors and statistics on the NASVD processing results were used for analysis.

### Energy Recalibration

The spectral drift was checked by monitoring the position of the Potassium, Uranium and Thorium peaks on average spectra along flight lines. The peak positions were determined by using a Gaussian fitting method. Energy recalibration was not required on any data.

### Dead Time

Gamma-ray spectrometers require a finite time to process each pulse from the detectors. While one pulse is being processed, any other pulse that arrives will be rejected. Consequently the ‘live time’ of a spectrometer is reduced by the time taken to process all pulses reaching the spectrometer. The spectra are normalised to counts per second by dividing by the live time.

### STP Altitude

The radar altimeter data was converted to effective height at standard temperature and pressure using the expression:

STPAlt = RAlt * (P/1013) * (273 / (T+273))

where:

RAlt =	the observed radar altitude in m

T =	the measured air temperature in deg C

P =	the barometric pressure in hPa

### Cosmic and Aircraft Background Removal

The 256 channel aircraft and cosmic spectra for the aircraft were calculated from the high-level test data with the aircraft and cosmic backgrounds derived using least squares fitting applied on a channel by channel basis.

The aircraft background was removed by subtracting the computed aircraft background spectra from the dead time corrected spectra. The 256 channel cosmic background spectrum that is removed is calculated by multiplying the 256 channel cosmic factor values by the cosmic counts recorded. The effect of cosmic radiation is removed from the spectra by subtracting the resultant cosmic spectrum.

| Window | Aircraft Background | Cosmic Stripping Ratio |
| --- | --- | --- |
| Total Count | 27.0 | 1.0300 |
| Potassium | 9.4 | 0.0540 |
| Uranium | 0.5 | 0.0495 |
| Thorium | 1.4 | 0.0520 |

Table 5: Aircraft Background and Cosmic Stripping Ratios

### Window Definitions

The 256 channel data were summed into the standard IAEA windows.

| Window | Peak Energy (keV) | Energy Window (keV) |  |  | GR-820 Channel Window |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total Count | - | 410 | - | 2810 | 34 | - | 234 |
| Potassium | 1460 | 1370 | - | 1570 | 115 | - | 131 |
| Uranium | 1765 | 1660 | - | 1860 | 139 | - | 155 |
| Thorium | 2614 | 2410 | - | 2810 | 201 | - | 234 |
| Cosmic | - | 4000 | - | 6000 |  | - |  |

Table 6: IAEA Window Definitions

### Radon Correction

Radon corrections were applied using the spectral ratio method.

| Stripping | Value |
| --- | --- |
| Total Count | 15.00 |
| Potassium | 0.78 |
| Thorium | 0.06 |
| Radon | 1.88 |
| Ground | - 0.72 |

Table 7: Radon Stripping Values

### Spectral Stripping

Spectral stripping was applied to the Potassium, Uranium and Thorium windows. The stripping coefficients were corrected for STP altitude.

| Stripping | Value | STP adjustment (/m) |
| --- | --- | --- |
| Alpha | 0.2638 | 0.00049 |
| Beta | 0.4279 | 0.00065 |
| Gamma | 0.7658 | 0.00069 |
| A | 0.0631 | 0 |
| B | 0.0018 | 0 |
| G | -0.0167 | - 0 |

Table 8: Spectral Stripping Ratios

### Height Correction

The background corrected and stripped window data were corrected for variations in the density altitude of the detector.

| Window | Attenuation coefficient (m-1) |
| --- | --- |
| Total Count | -0.0072 |
| Potassium | -0.0096 |
| Uranium | -0.0076 |
| Thorium | -0.0071 |

Table 9: STP Altitude Coefficients

### Gridding

The final radiometric data were gridded using a minimum curvature algorithm. The data was gridded with a cell size of 50 m.

## Digital Elevation Model

The processing procedures applied to the terrain data are summarised below:

- Apply any spike corrections to the raw radar altimeter data.

- Interpolate undefined values.

- Co-ordinate the data with post-processed GPS data.

- Apply parallax corrections.

- Subtract the aircraft’s height above ground from the aircraft’s height above the WGS84 ellipsoid and correct for radar altimeter/GPS sensor separation.

- Derive surface topography values with respect to mean sea level (referenced to the geoid) by correcting the WGS84 ellipsoid values with geoid-ellipsoid separation values.

- Using the tie lines (flown at 90 degrees to the traverse lines) a set of miss-tie values were determined. These miss-tie values reflected the differences in the value between the tie lines and the traverse lines over the same geographical point. Using a least squares fit algorithm, which also takes into account the statistical variation inherent in DGPS positioning, a series of corrections were applied to the traverse line data. These allowed the data to be levelled to the same base value.

- Following this, a FAS proprietary micro-levelling process was applied in order to more subtly level the data.

### Gridding

The final levelled elevation data were gridded using a bi-directional spline algorithm. The data was gridded with a cell size of 50 m.

The accuracy of the elevation calculation is directly dependent on the accuracy of the two input parameters, radar altitude and GPS altitude. The radar altitude value may be erroneous in areas of heavy tree cover, where the altimeter reflects the distance to the tree canopy rather than the ground. The GPS altitude value is primarily dependent on the number of available satellites. Although post-processing of GPS data will yield X and Y accuracies in the order of 1-2 metres, the accuracy of the altitude value is usually much less, sometimes in the ±5 metre range. Further inaccuracies may be introduced during the interpolation and gridding process.

Because of the inherent inaccuracies of this method, no guarantee is made or implied that the information displayed is a true representation of the height above sea level. Although this product may be of some use as a general reference, THIS PRODUCT MUST NOT BE USED FOR NAVIGATION PURPOSES.

APPENDIX I – Weekly Operations Report

APPENDIX II – Button Calibration Data

AIRCRAFT VH-KPY

| Day | Flight | K Peak | K Res | U Peak | U Res | Th Peak | Th Res |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 197 | 2 | 121.854 | 5.692 | 149.036 | 4.205 | 218.393 | 4.879 |
| 197 | 2.5 | 121.951 | 5.774 | 149.196 | 3.997 | 218.589 | 4.869 |
| 198 | 3 | 121.77 | 5.786 | 148.735 | 4.623 | 218.414 | 4.841 |
| 198 | 3.5 | 121.858 | 5.887 | 148.669 | 4.783 | 218.57 | 4.886 |
| 203 | 5 | 121.832 | 5.747 | 148.764 | 4.548 | 218.511 | 4.866 |
| 203 | 5.5 | 121.913 | 5.759 | 149.158 | 4.152 | 218.542 | 4.894 |
| 204 | 6 | 121.899 | 5.752 | 148.231 | 5.454 | 218.542 | 4.896 |
| 204 | 6.5 | 121.799 | 5.791 | 148.837 | 4.359 | 218.41 | 4.862 |
| 205 | 7 | 121.848 | 5.774 | 148.231 | 5.454 | 218.714 | 4.866 |
| 205 | 7.5 | 121.747 | 5.86 | 148.676 | 4.716 | 218.49 | 4.879 |
| 206 | 8 | 121.845 | 5.755 | 148.231 | 5.454 | 218.641 | 4.819 |
| 206 | 8.5 | 121.892 | 5.834 | 148.912 | 4.445 | 218.699 | 4.863 |
| 207 | 9 | 121.718 | 5.709 | 148.689 | 4.476 | 218.535 | 4.805 |
| 207 | 9.5 | 121.775 | 5.777 | 148.772 | 4.578 | 218.512 | 4.855 |
| 208 | 10 | 121.636 | 5.801 | 148.573 | 4.781 | 218.529 | 4.823 |
| 208 | 10.5 | 121.824 | 5.717 | 149.143 | 4.003 | 218.45 | 4.837 |
| 209 | 11 | 121.602 | 5.894 | 148.689 | 4.668 | 218.623 | 4.8 |
| 209 | 11.5 | 121.871 | 5.748 | 149.026 | 4.214 | 218.52 | 4.879 |
| 210 | 12 | 121.666 | 5.866 | 148.474 | 4.931 | 218.557 | 4.822 |
| 210 | 12.5 | 121.782 | 5.718 | 148.849 | 4.526 | 218.397 | 4.893 |
| 211 | 13 | 121.664 | 5.728 | 148.694 | 4.841 | 218.455 | 4.879 |
| 211 | 13.5 | 121.801 | 5.796 | 148.613 | 4.846 | 218.531 | 4.83 |

APPENDIX III – Low Level Statistics

TESTLINE RADIOMETRIC DATA VH-KPY

| Day | Flight | % Difference | Bottom Limit (%) | Top Limit (%) | Mean Value (cps) | Window(cps) |
| --- | --- | --- | --- | --- | --- | --- |
| 197 | 2 | -3.49269 | -10 | 10 | 59.99545 | 57.9 |
| 197 | 2.5 | -0.49246 | -10 | 10 | 59.99545 | 59.7 |
| 198 | 3 | -0.32578 | -10 | 10 | 59.99545 | 59.8 |
| 198 | 3.5 | 1.841049 | -10 | 10 | 59.99545 | 61.1 |
| 203 | 5 | -3.49269 | -10 | 10 | 59.99545 | 57.9 |
| 203 | 5.5 | 0.340935 | -10 | 10 | 59.99545 | 60.2 |
| 204 | 6 | -0.65914 | -10 | 10 | 59.99545 | 59.6 |
| 204 | 6.5 | 4.841276 | -10 | 10 | 59.99545 | 62.9 |
| 205 | 7 | 2.341086 | -10 | 10 | 59.99545 | 61.4 |
| 205 | 7.5 | 1.50769 | -10 | 10 | 59.99545 | 60.9 |
| 206 | 8 | -0.9925 | -10 | 10 | 59.99545 | 59.4 |
| 206 | 8.5 | 2.674445 | -10 | 10 | 59.99545 | 61.6 |
| 207 | 9 | -1.32586 | -10 | 10 | 59.99545 | 59.2 |
| 207 | 9.5 | 0.840973 | -10 | 10 | 59.99545 | 60.5 |
| 208 | 10 | -2.32593 | -10 | 10 | 59.99545 | 58.6 |
| 208 | 10.5 | 1.841049 | -10 | 10 | 59.99545 | 61.1 |
| 209 | 11 | 0.007576 | -10 | 10 | 59.99545 | 60 |
| 209 | 11.5 | -1.32586 | -10 | 10 | 59.99545 | 59.2 |
| 210 | 12 | -1.99258 | -10 | 10 | 59.99545 | 58.8 |
| 210 | 12.5 | -1.99258 | -10 | 10 | 59.99545 | 58.8 |
| 211 | 13 | 0.507614 | -10 | 10 | 59.99545 | 60.3 |
| 211 | 13.5 | 1.674369 | -10 | 10 | 59.99545 | 61 |

APPENDIX IV – Final Located Data Formats

- Headers for final data files

Description File for 0.1 sec Magnetics and Elevation Data

COMM JOB NUMBER:                                                         1988

COMM AREA NUMBER:                                                           2

COMM SURVEY COMPANY:                                   Fugro Airborne Surveys

COMM CLIENT:                                Territory Uranium Company Limited

COMM SURVEY TYPE:                             Magnetic and Radiometric Survey

COMM AREA NAME:                                                    Daly River

COMM STATE:                                                Northern Territory

COMM COUNTRY:                                                       Australia

COMM SURVEY FLOWN:                                                  July 2008

COMM LOCATED DATA CREATED:                           Wed Aug 13 09:29:32 2008

COMM

COMM SURVEY SPECIFICATIONS:

COMM

COMM TRAVERSE LINE SPACING:                                             200 m

COMM TRAVERSE LINE DIRECTION:                                   000 - 180 deg

COMM TIE LINE SPACING:                                                 2000 m

COMM TIE LINE DIRECTION:                                        090 - 270 deg

COMM NOMINAL TERRAIN CLEARANCE:                                          60 m

COMM FINAL LINE KILOMETRES:                                             7,263

COMM

COMM LINE NUMBERING

COMM

COMM TRAVERSE LINE NUMBERS:                                     20001 - 20220

COMM TIE LINE NUMBERS:                                          29001 - 29039

COMM

COMM AREA BOUNDARY

COMM DATUM                                                              GDA94

COMM PROJECTION                                                           MGA

COMM ZONE                                                                  52

COMM

COMM easting northing

COMM 700072  8488412

COMM 709045  8488342

COMM 709194  8475663

COMM 722172  8475567

COMM 722069  8451033

COMM 708998  8451519

COMM 708605  8426789

COMM 722543  8415947

COMM 724598  8415868

COMM 724549  8410387

COMM 713850  8410542

COMM 713899  8416324

COMM 710272  8416355

COMM 710303  8420029

COMM 708670  8420042

COMM 708504  8421911

COMM 706811  8421925

COMM 706765  8423732

COMM 705012  8423807

COMM 705026  8425614

COMM 703212  8425689

COMM 703286  8434843

COMM 694272  8434974

COMM 694723  8486706

COMM 700118  8486484

COMM

COMM SURVEY EQUIPMENT

COMM

COMM AIRCRAFT:                                              VH-KPY Cresco 750

COMM

COMM MAGNETOMETER:                              Scintrex CS-2 CV Magnetometer

COMM INSTALLATION:                                  Tail stinger installation

COMM RESOLUTION:                                                     0.001 nT

COMM RECORDING INTERVAL:                                                0.1 s

COMM

COMM BASE MAGNETOMETER:                                     Scintrex Envi-mag

COMM RECORDING INTERVAL:                                                  5 s

COMM LOCATION:                                            Daly River Airstrip

COMM

COMM RADAR ALTIMETER:                     Sperry Stars AA-200 Radar Altimeter

COMM RECORDING INTERVAL:                                                0.1 s

COMM

COMM NAVIGATION:                                   real-time differential GPS

COMM RECORDING INTERVAL:                                                1.0 s

COMM

COMM ACQUISITION SYSTEM:                                            Fugro DAS

COMM

COMM DATA PROCESSING

COMM

COMM CO-ORDINATES

COMM PARALLAX CORRECTION APPLIED:                                      -0.5 s

COMM

COMM GPS HEIGHT DATA (HAE)

COMM PARALLAX CORRECTION APPLIED:                                      -0.5 s

COMM

COMM HEIGHT ABOVE GROUND DATA (HAG)

COMM PARALLAX CORRECTION APPLIED:                                         0 s

COMM

COMM MAGNETIC DATA

COMM DIURNAL CORRECTION APPLIED:                          base value 47110 nT

COMM PARALLAX CORRECTION APPLIED:                                         0 s

COMM IGRF CORRECTION APPLIED:                             base value 47270 nT

COMM IGRF MODEL 2005 extrapolated to:                              2008/07/15

COMM DATA HAVE BEEN TIE LINE LEVELLED

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM DIGITAL TERRAIN DATA

COMM DTM CALCULATED [DTM = HAE - (HAG + SENSOR SEPARATION)]

COMM DATA CORRECTED TO AUSTRALIAN HEIGHT DATUM

COMM DATA HAVE BEEN TIE LINE LEVELLED

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM

COMM ----------------------------------------------------------------------

COMM DISCLAIMER

COMM ----------------------------------------------------------------------

COMM It is Fugro Airborne Survey’s understanding that the data provided to

COMM the client is to be used for the purpose agreed between the parties.

COMM That purpose was a significant factor in determining the scope and

COMM level of the Services being offered to the Client. Should the purpose

COMM for which the data is used change, the data may no longer be valid or

COMM appropriate and any further use of, or reliance upon, the data in

COMM those circumstances by the Client without Fugro Airborne Survey's

COMM review and advice shall be at the Client's own or sole risk.

COMM

COMM The Services were performed by Fugro Airborne Survey exclusively for

COMM the purposes of the Client. Should the data be made available in whole

COMM or part to any third party, and such party relies thereon, that party

COMM does so wholly at its own and sole risk and Fugro Airborne Survey

COMM disclaims any liability to such party.

COMM

COMM Where the Services have involved Fugro Airborne Survey's use of any

COMM information provided by the Client or third parties, upon which

COMM Fugro Airborne Survey was reasonably entitled to rely, then the

COMM Services are limited by the accuracy of such information. Fugro

COMM Airborne Survey is not liable for any inaccuracies (including any

COMM incompleteness) in the said information, save as otherwise provided

COMM in the terms of the contract between the Client and Fugro Airborne

COMM Survey.

COMM

COMM With regard to DIGITAL TERRAIN DATA, the accuracy of the elevation

COMM calculation is directly dependent on the accuracy of the two input

COMM parameters, radar altitude and GPS altitude. The radar altitude value

COMM may be erroneous in areas of heavy tree cover, where the altimeter

COMM reflects the distance to the tree canopy rather than the ground. The

COMM GPS altitude value is primarily dependent on the number of available

COMM satellites. Although post-processing of GPS data will yield X and Y

COMM accuracies in the order of 1-2 metres, the accuracy of the altitude

COMM value is usually much less, sometimes in the ±5 metre range. Further

COMM inaccuracies may be introduced during the interpolation and gridding

COMM process. Because of the inherent inaccuracies of this method, no

COMM guarantee is made or implied that the information displayed is a true

COMM representation of the height above sea level.  Although this product

COMM may be of some use as a general reference,

COMM THIS PRODUCT MUST NOT BE USED FOR NAVIGATION PURPOSES.

COMM ----------------------------------------------------------------------

COMM

COMM

COMM LINE DATA FORMAT

COMM A space is left between fixed fields so that a field of, for example,

COMM A8 should only ever have a maximum of 7 characters in it, even when it

COMM is a null, thus:

COMM

COMM FIELD                        UNITS            NULL                FORMAT

COMM Line number                                   -99999              I6

COMM Flight number                                 -99                 I4

COMM Date (YYYYMMDD)                               -9999999            I9

COMM Fiducial number                               -9999.9             F8.1

COMM Time (local)                 s                -9999.9             F8.1

COMM Easting                      m                -99999.99           F10.2

COMM Northing                     m                -999999.99          F11.2

COMM Longitude                    deg              -999.99999999       F14.8

COMM Latitude                     deg              -99.99999999        F13.8

COMM GPS height                   m                -999.99             F8.2

COMM Radar altitude               m                -999.99             F8.2

COMM Compensated TMI              nT               -99999.999          F11.3

COMM Magnetic Diurnal             nT               -9999.999           F10.3

COMM Final TMI                    nT               -99999.999          F11.3

COMM Digital Terrain Model        m                -999.99             F8.2

Description File for 1.0 sec Windowed Radiometrics Data

COMM JOB NUMBER:                                                         1988

COMM AREA NUMBER:                                                           2

COMM SURVEY COMPANY:                                   Fugro Airborne Surveys

COMM CLIENT:                                Territory Uranium Company Limited

COMM SURVEY TYPE:                             Magnetic and Radiometric Survey

COMM AREA NAME:                                                    Daly River

COMM STATE:                                                Northern Territory

COMM COUNTRY:                                                       Australia

COMM SURVEY FLOWN:                                                  July 2008

COMM LOCATED DATA CREATED:                           Wed Aug 13 09:31:54 2008

COMM

COMM SURVEY SPECIFICATIONS:

COMM

COMM TRAVERSE LINE SPACING:                                             200 m

COMM TRAVERSE LINE DIRECTION:                                   000 - 180 deg

COMM TIE LINE SPACING:                                                 2000 m

COMM TIE LINE DIRECTION:                                        090 - 270 deg

COMM NOMINAL TERRAIN CLEARANCE:                                          60 m

COMM FINAL LINE KILOMETRES:                                             7,248

COMM

COMM LINE NUMBERING

COMM

COMM TRAVERSE LINE NUMBERS:                                     20001 - 20220

COMM TIE LINE NUMBERS:                                          29001 - 29039

COMM

COMM AREA BOUNDARY

COMM DATUM                                                              GDA94

COMM PROJECTION                                                           MGA

COMM ZONE                                                                  52

COMM

COMM easting northing

COMM 700072  8488412

COMM 709045  8488342

COMM 709194  8475663

COMM 722172  8475567

COMM 722069  8451033

COMM 708998  8451519

COMM 708605  8426789

COMM 722543  8415947

COMM 724598  8415868

COMM 724549  8410387

COMM 713850  8410542

COMM 713899  8416324

COMM 710272  8416355

COMM 710303  8420029

COMM 708670  8420042

COMM 708504  8421911

COMM 706811  8421925

COMM 706765  8423732

COMM 705012  8423807

COMM 705026  8425614

COMM 703212  8425689

COMM 703286  8434843

COMM 694272  8434974

COMM 694723  8486706

COMM 700118  8486484

COMM

COMM SURVEY EQUIPMENT

COMM

COMM AIRCRAFT:                                              VH-KPY Cresco 750

COMM

COMM SPECTROMETER:                             Exploranium GR820 Spectrometer

COMM CRYSTAL VOLUME:                                                  33.56 l

COMM RECORDING INTERVAL:                                                1.0 s

COMM

COMM RADAR ALTIMETER:                     Sperry Stars AA-200 Radar Altimeter

COMM RECORDING INTERVAL:                                                0.1 s

COMM

COMM NAVIGATION:                                   real-time differential GPS

COMM RECORDING INTERVAL:                                                1.0 s

COMM

COMM ACQUISITION SYSTEM:                                            Fugro DAS

COMM

COMM DATA PROCESSING

COMM

COMM CO-ORDINATES

COMM PARALLAX CORRECTION APPLIED:                                      -0.5 s

COMM

COMM GPS HEIGHT DATA

COMM PARALLAX CORRECTION APPLIED:                                      -0.5 s

COMM

COMM RADAR ALTIMETER DATA

COMM PARALLAX CORRECTION APPLIED:                                         0 s

COMM

COMM RADIOMETRIC DATA

COMM NASVD FILTERING APPLIED TO 256 CHANNEL DATA

COMM WINDOW DATA EXTRACTED USING IAEA STANDARD WINDOWS

COMM PARALLAX CORRECTION APPLIED:                                         0 s

COMM COSMIC, AIRCRAFT AND RADON BACKGROUNDS REMOVED

COMM STRIPPING CORRECTIONS APPLIED

COMM HEIGHT CORRECTED TO:                                            60 m AGL

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM AIRCRAFT BACKGROUND COEFFICIENTS

COMM TOTAL COUNT:                                                        27.0

COMM POTASSIUM                                                            9.4

COMM URANIUM                                                              0.5

COMM THORIUM                                                              1.4

COMM COSMIC COEFFICIENTS

COMM TOTAL COUNT:                                                      1.0300

COMM POTASSIUM:                                                        0.0540

COMM URANIUM:                                                          0.0495

COMM THORIUM:                                                          0.0520

COMM STRIPPING COEFFICIENTS

COMM ALPHA:                                                            0.2638

COMM BETA:                                                             0.4279

COMM GAMMA:                                                            0.7658

COMM DELTA:                                                            0.0631

COMM b:                                                                0.0018

COMM g:                                                               -0.0167

COMM RADON STRIPPING COEFFICIENTS

COMM TOTAL COUNT:                                                       15.00

COMM POTASSIUM:                                                          0.78

COMM THORIUM:                                                            0.06

COMM SPECTRAL RATIOS

COMM RADON:                                                              1.88

COMM GROUND:                                                             0.72

COMM ALTITUDE COEFFICIENTS

COMM TOTAL COUNT:                                                     -0.0072

COMM POTASSIUM:                                                       -0.0096

COMM URANIUM:                                                         -0.0079

COMM THORIUM:                                                         -0.0071

COMM

COMM

COMM ----------------------------------------------------------------------

COMM DISCLAIMER

COMM ----------------------------------------------------------------------

COMM It is Fugro Airborne Survey’s understanding that the data provided to

COMM the client is to be used for the purpose agreed between the parties.

COMM That purpose was a significant factor in determining the scope and

COMM level of the Services being offered to the Client. Should the purpose

COMM for which the data is used change, the data may no longer be valid or

COMM appropriate and any further use of, or reliance upon, the data in

COMM those circumstances by the Client without Fugro Airborne Survey's

COMM review and advice shall be at the Client's own or sole risk.

COMM

COMM The Services were performed by Fugro Airborne Survey exclusively for

COMM the purposes of the Client. Should the data be made available in whole

COMM or part to any third party, and such party relies thereon, that party

COMM does so wholly at its own and sole risk and Fugro Airborne Survey

COMM disclaims any liability to such party.

COMM

COMM Where the Services have involved Fugro Airborne Survey's use of any

COMM information provided by the Client or third parties, upon which

COMM Fugro Airborne Survey was reasonably entitled to rely, then the

COMM Services are limited by the accuracy of such information. Fugro

COMM Airborne Survey is not liable for any inaccuracies (including any

COMM incompleteness) in the said information, save as otherwise provided

COMM in the terms of the contract between the Client and Fugro Airborne

COMM Survey.

COMM ----------------------------------------------------------------------

COMM

COMM

COMM LINE DATA FORMAT

COMM A space is left between fixed fields so that a field of, for example,

COMM A8 should only ever have a maximum of 7 characters in it, even when it

COMM is a null, thus:

COMM

COMM FIELD                        UNITS            NULL                FORMAT

COMM Line number                                   -99999              I6

COMM Flight number                                 -99                 I4

COMM Date (YYYYMMDD)                               -9999999            I9

COMM Fiducial number                               -9999.9             F8.1

COMM Time (local)                 s                -9999.9             F8.1

COMM Easting                      m                -99999.99           F10.2

COMM Northing                     m                -999999.99          F11.2

COMM Longitude                    deg              -999.99999999       F14.8

COMM Latitude                     deg              -99.99999999        F13.8

COMM GPS height                   m                -999.99             F8.2

COMM Radar altitude               m                -999.99             F8.2

COMM Uncorrected Total Count      cps              -9999.9             F8.1

COMM Uncorrected Potassium        cps              -999.9              F7.1

COMM Uncorrected Uranium          cps              -999.9              F7.1

COMM Uncorrected Thorium          cps              -999.9              F7.1

COMM Raw cosmic                   cps              -99                 I4

COMM Final Total Count            cps              -9999.9             F8.1

COMM Final Potassium              cps              -999.9              F7.1

COMM Final Uranium                cps              -999.9              F7.1

COMM Final Thorium                cps              -999.9              F7.1

Description File for 1.0 sec Raw 256 Channel Radiometrics Data

COMM JOB NUMBER:                                                         1988

COMM AREA NUMBER:                                                           2

COMM SURVEY COMPANY:                                   Fugro Airborne Surveys

COMM CLIENT:                                Territory Uranium Company Limited

COMM SURVEY TYPE:                             Magnetic and Radiometric Survey

COMM AREA NAME:                                                    Daly River

COMM STATE:                                                Northern Territory

COMM COUNTRY:                                                       Australia

COMM SURVEY FLOWN:                                                  July 2008

COMM LOCATED DATA CREATED:                           Wed Aug 13 09:33:06 2008

COMM

COMM SURVEY SPECIFICATIONS:

COMM

COMM TRAVERSE LINE SPACING:                                             200 m

COMM TRAVERSE LINE DIRECTION:                                   000 - 180 deg

COMM TIE LINE SPACING:                                                 2000 m

COMM TIE LINE DIRECTION:                                        090 - 270 deg

COMM NOMINAL TERRAIN CLEARANCE:                                          60 m

COMM FINAL LINE KILOMETRES:                                             7,248

COMM

COMM LINE NUMBERING

COMM

COMM TRAVERSE LINE NUMBERS:                                     20001 - 20220

COMM TIE LINE NUMBERS:                                          29001 - 29039

COMM

COMM AREA BOUNDARY

COMM DATUM                                                              GDA94

COMM PROJECTION                                                           MGA

COMM ZONE                                                                  52

COMM

COMM easting northing

COMM 700072  8488412

COMM 709045  8488342

COMM 709194  8475663

COMM 722172  8475567

COMM 722069  8451033

COMM 708998  8451519

COMM 708605  8426789

COMM 722543  8415947

COMM 724598  8415868

COMM 724549  8410387

COMM 713850  8410542

COMM 713899  8416324

COMM 710272  8416355

COMM 710303  8420029

COMM 708670  8420042

COMM 708504  8421911

COMM 706811  8421925

COMM 706765  8423732

COMM 705012  8423807

COMM 705026  8425614

COMM 703212  8425689

COMM 703286  8434843

COMM 694272  8434974

COMM 694723  8486706

COMM 700118  8486484

COMM

COMM SURVEY EQUIPMENT

COMM

COMM AIRCRAFT:                                              VH-KPY Cresco 750

COMM

COMM SPECTROMETER:                             Exploranium GR820 Spectrometer

COMM CRYSTAL VOLUME:                                                  33.56 l

COMM RECORDING INTERVAL:                                                1.0 s

COMM

COMM THERMOMETER:                Vaisala HMY133 Temperature & Humidity Sensor

COMM RECORDING INTERVAL:                                                1.0 s

COMM

COMM BAROMETER:                          Rosemount 1241M Barometric Altimeter

COMM RECORDING INTERVAL:                                                1.0 s

COMM

COMM RADAR ALTIMETER:                     Sperry Stars AA-200 Radar Altimeter

COMM RECORDING INTERVAL:                                                0.1 s

COMM

COMM NAVIGATION:                                   real-time differential GPS

COMM RECORDING INTERVAL:                                                1.0 s

COMM

COMM ACQUISITION SYSTEM:                                            Fugro DAS

COMM

COMM DATA PROCESSING

COMM

COMM CO-ORDINATES

COMM PARALLAX CORRECTION APPLIED:                                      -0.5 s

COMM

COMM GPS HEIGHT DATA

COMM PARALLAX CORRECTION APPLIED:                                      -0.5 s

COMM

COMM RADAR ALTIMETER DATA

COMM PARALLAX CORRECTION APPLIED:                                         0 s

COMM

COMM TEMPERATURE DATA

COMM PARALLAX CORRECTION APPLIED:                                         0 s

COMM

COMM PRESSURE DATA

COMM PARALLAX CORRECTION APPLIED:                                       0.1 s

COMM

COMM RADIOMETRIC DATA

COMM

COMM AIRCRAFT BACKGROUND COEFFICIENTS

COMM TOTAL COUNT:                                                        27.0

COMM POTASSIUM                                                            9.4

COMM URANIUM                                                              0.5

COMM THORIUM                                                              1.4

COMM COSMIC COEFFICIENTS

COMM TOTAL COUNT:                                                      1.0300

COMM POTASSIUM:                                                        0.0540

COMM URANIUM:                                                          0.0495

COMM THORIUM:                                                          0.0520

COMM STRIPPING COEFFICIENTS

COMM ALPHA:                                                            0.2638

COMM BETA:                                                             0.4279

COMM GAMMA:                                                            0.7658

COMM DELTA:                                                            0.0631

COMM b:                                                                0.0018

COMM g:                                                               -0.0167

COMM RADON STRIPPING COEFFICIENTS

COMM TOTAL COUNT:                                                       15.00

COMM POTASSIUM:                                                          0.78

COMM THORIUM:                                                            0.06

COMM SPECTRAL RATIOS

COMM RADON:                                                              1.88

COMM GROUND:                                                             0.72

COMM ALTITUDE COEFFICIENTS

COMM TOTAL COUNT:                                                     -0.0072

COMM POTASSIUM:                                                       -0.0096

COMM URANIUM:                                                         -0.0079

COMM THORIUM:                                                         -0.0071

COMM

COMM

COMM ---------------------------------------------------------------------

COMM DISCLAIMER

COMM ----------------------------------------------------------------------

COMM It is Fugro Airborne Survey’s understanding that the data provided to

COMM the client is to be used for the purpose agreed between the parties.

COMM That purpose was a significant factor in determining the scope and

COMM level of the Services being offered to the Client. Should the purpose

COMM for which the data is used change, the data may no longer be valid or

COMM appropriate and any further use of, or reliance upon, the data in

COMM those circumstances by the Client without Fugro Airborne Survey's

COMM review and advice shall be at the Client's own or sole risk.

COMM

COMM The Services were performed by Fugro Airborne Survey exclusively for

COMM the purposes of the Client. Should the data be made available in whole

COMM or part to any third party, and such party relies thereon, that party

COMM does so wholly at its own and sole risk and Fugro Airborne Survey

COMM disclaims any liability to such party.

COMM

COMM Where the Services have involved Fugro Airborne Survey's use of any

COMM information provided by the Client or third parties, upon which

COMM Fugro Airborne Survey was reasonably entitled to rely, then the

COMM Services are limited by the accuracy of such information. Fugro

COMM Airborne Survey is not liable for any inaccuracies (including any

COMM incompleteness) in the said information, save as otherwise provided

COMM in the terms of the contract between the Client and Fugro Airborne

COMM Survey.

COMM ----------------------------------------------------------------------

COMM

COMM

COMM LINE DATA FORMAT

COMM A space is left between fixed fields so that a field of, for example,

COMM A8 should only ever have a maximum of 7 characters in it, even when it

COMM is a null, thus:

COMM

COMM FIELD                        UNITS            NULL                FORMAT

COMM Line number                                   -99999              I6

COMM Flight number                                 -99                 I4

COMM Date (YYYYMMDD)                               -9999999            I9

COMM Fiducial number                               -9999.9             F8.1

COMM Time (local)                 s                -9999.9             F8.1

COMM Easting                      m                -99999.99           F10.2

COMM Northing                     m                -999999.99          F11.2

COMM Longitude                    deg              -999.99999999       F14.8

COMM Latitude                     deg              -99.99999999        F13.8

COMM GPS height                   m                -999.99             F8.2

COMM Radar altitude               m                -999.99             F8.2

COMM Raw cosmic                   cps              -99                 I4

COMM Barometric Pressure          hPa              -999.99             F8.2

COMM Temperature                  deg C            -9.9                F5.1

COMM Live time                    s                -9.999              F7.3

COMM Raw 256 channel radiometrics counts           -999                256I5

APPENDIX V – List Of All Supplied Data

Final Located Data – for each area

- 0.1 second magnetics and digital elevation data

- 1.0 second windowed radiometrics data

- 1.0 second raw 256 channel radiometric data

Final located data is in ASCII format. Contents of each are shown in Appendix IV.

Raw Initial Products

Raw initial grids were produced in Geo-referenced TIFF format GDA94/MGA52

- Raw total magnetic intensity (nT)

- Raw total count (cps)

- Raw Potassium count (cps)

- Raw Uranium count (cps)

- Raw Thorium count (cps)

- Raw digital elevation model (m AHD)

Preliminary Gridded Data

Preliminary gridded data was produced in ERMapper format in GDA94/MGA52

- Total magnetic intensity (nT)

- Total count (cps)

- Potassium count (cps)

- Uranium count (cps)

- Thorium count (cps)

- Digital elevation model (m AHD)

Final Gridded Data

Final gridded data was produced in ERMapper format in GDA94/MGA52

- Total magnetic intensity (nT)

- Total count (cps)

- Potassium count (cps)

- Uranium count (cps)

- Thorium count (cps)

- Digital elevation model (m AHD)

Additional Products

A logistics and processing report