NEWEXCO PTY LTD Mining and Exploration Services A.B.N. 890 883 169 01 15 Joel Terrace, East Perth, 6004 Western Australia Ph: +61 8 9227 1266 Fax: +61 8 9227 1677 Email: hrh@newexco.com.au Website: www.newexco.com.au

Memorandum

| To: | John Hicks | Client: | Panoramic Resources |
| --- | --- | --- | --- |
| c.c: |  | Prospect: | Bluebush |
| From: | Brett Adams | Job: |  |
| Date: | 29th July 2009 | File: |  |
| Subject: | Gravity and magnetic modelling |  |  |

The Bluebush prospect is located within the Tennant Creek mineral field. This memo covers a selection of gravity and magnetic models within Bluebush.

The gravity data was collected at 500m line spacing and 100m station spacing. Both ground and airborne magnetic data have been used in the modelling where appropriate. Airborne magnetic data was downloaded from the Geological Survey web site and has been collected using 200m line spacing.

Ground magnetic data was collected by Territory Uranium over the gravity highs using a line spacing of 200m. The ground data has been processed to remove poor readings then filtered to remove the worst of the noise.

Eastern Anomalies

Three gravity anomalies have been identified in the eastern section (Figure 1). Bluebush 1 represents the best of these anomalies exhibiting the largest anomaly in terms of both lateral extent and amplitude.

All three anomalies appear to be structurally related and almost certainly represent the same rock type.  Magnetically a relationship is not clear, possibly implying the alteration environment at each anomaly is not related?

Each model has been targeted with a series of 3 drill holes. The centre hole targets the upper model edge with the others testing approximately 50m above and below.

Bluebush 2

Bluebush 1

Bluebush 3

Figure 1: Bluebush Gravity 1VD image showing the location of the ground magnetic traverses.

Bluebush 3

Bluebush 2

Bluebush 1

Figure 2: Bluebush magnetic analytic signal derived from GSWA airborne data highlights magnetic sources.

Bluebush 1

Bluebush 1 is gravity high detected over 3 lines (Figure 3 and Figure 4) suggesting a strike length of over 1000m. The higher amplitude seen at the northern end is interpreted to indicate a slightly shallower source.

Ground magnetic data (Figure 5) has defined 3 anomalies with the southernmost anomaly coinciding with the location of the northern gravity anomaly (see Figure 6). The magnetic anomaly is a potential indicator of alteration within the iron stone and is therefore thought to represent a good target for drilling. Proposed drill holes are demonstrated in Figure 6 and detailed in Table 1.

Gravity                                                            1VD Gravity

Figure 3: Modelling of gravity data over Bluebush 1

Figure 4: Gravity modelling profiles over Bluebush 1.

Plan View                                Eastern Line                                       Western Line

Figure 5: Modelling of the ground magnetic data over Bluebush 1.

Plan View                                   Looking south west along strike of the gravity model.

Figure 6: Bluebush anomaly 1. Red and blue models are derived from ground gravity and ground magnetic survey data. Proposed drill holes shown as traces.

Table 1: Proposed drill holes targeting Bluebush anomaly 1.

| Anomaly | Hole # | East | North | Dip | Azi | Depth | Intersection |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB grav 1 | Drillhole 1 | 391545 | 7808640 | 73 | 310 | 425 | 325 |
| BB grav 1 | Drillhole 2 | 391545 | 7808640 | 70 | 310 | 450 | 370 |
| BB grav 1 | Drillhole 3 | 391545 | 7808640 | 67 | 310 | 475 | 430 |

Bluebush 2

Bluebush 2 is gravity high detected over 2 lines (Figure 7 and Figure 8) suggesting a strike length of approximately 1000m.

Ground magnetic data (Figure 9 and Figure 10) has not covered the best part of the magnetic anomaly as defined by the airborne data. Ground data is also very noisy despite filtering making identification of anomalies difficult. The airborne data has also been modelled (Figure 11 and Figure 12) and this is thought to represent a more realistic set of models.

The gravity model coincides with the magnetic models (see Figure 13) potentially indicating alteration within the iron stone. This is thought to represent a good target for drilling.

Proposed drill holes are demonstrated in Figure 13 and detailed in Table 2.

Gravity                                                                                       1VD Gravity

Figure 7: Modelling of gravity data over Bluebush 2.

Figure 8: Gravity modelling profiles over Bluebush 2.

Airborne  TMI                                                                            Analytic Signal

Figure 9: Bluebush 2 ground magnetic model locations over airborne data.

Plan View                                           Eastern Line                                                            Western Line

Figure 10: Modelling of the ground magnetic data over Bluebush 2.

TMI                                                                                      Analytic Signal

Figure 11: Airborne magnetic model locations over Bluebush 2.

Figure 12: Profiles of airborne magnetic modelling over Bluebush 2

Plan View                          Looking south west along strike of the gravity model.

Figure 13: Bluebush anomaly 2. Red and blue models are derived from ground gravity and ground magnetic survey data. Proposed drill holes shown as traces.

Table 2:  Proposed drill holes targeting Bluebush anomaly 2.

| Anomaly | Hole # | East | North | Dip | Azi | Depth | Intersection |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB grav 2 | Drillhole 4 | 392485 | 7809400 | 66 | 320 | 425 | 340 |
| BB grav 2 | Drillhole 5 | 392485 | 7809400 | 62 | 320 | 400 | 295 |
| BB grav 2 | Drillhole 6 | 392485 | 7809400 | 58 | 320 | 375 | 270 |

Bluebush 3

Bluebush 3 is gravity high detected over 3 lines (Figure 14 and Figure 15) suggesting a strike length of over 1000m. The  anomaly implies a smaller source located above a larger, deeper source.

The gravity anomaly does not coincide with a magnetic anomaly as demonstrated in Figure 18. Magnetic features located to the south (Figures 16 and 17) are unlikely to be related to the gravity.

The lack of magnetic support over this anomaly does not comply with the traditional model associated with Tennant Creek making this target difficult to recommend for drilling.

If this anomaly is to be drilled, the proposed holes are detailed in Table 3.

Gravity                                                            1VD Gravity

Figure 14: Modelling of gravity data over Bluebush 3.

Figure 15: Gravity modelling profiles over Bluebush 3.

TMI                                                                                      Analytic Signal

Figure 16: Airborne magnetic model locations over Bluebush 3.

Figure 17: Profiles of airborne magnetic modelling over Bluebush 3.

Plan View                          Looking south west along strike of the gravity model.

Figure 18: Bluebush anomaly 3. Red and blue models are derived from ground gravity and airborne magnetic survey data. Proposed drill holes shown as traces.

Table 3: Proposed drill holes targeting Bluebush anomaly 3.

| Anomaly | Hole # | East | North | Dip | Azi | Depth | Intersection |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB grav 3 | Drillhole 7 | 393750 | 7809800 | 67 | 350 | 440 | 390 |
| BB grav 3 | Drillhole 8 | 393750 | 7809800 | 64 | 350 | 400 | 350 |
| BB grav 3 | Drillhole 9 | 393750 | 7809800 | 60 | 350 | 380 | 310 |

Bluebush 4

Gravity data indicates a large regional high on the western side of the Bluebush prospect. The source of this anomaly has been simulated with a very large (10km X 2.5km) and deep (approx 3.5km) body.  This does not represent the primary gravity target.

Application of the first vertical derivative (1VD) to the gravity data highlights an anomaly at 382000E / 7810700N (see Figure 19). This anomaly has been modelled and may potentially represent ironstone.

Numerous smaller anomalies can be seen on the eastern side of the larger anomaly. The significance of these is unknown however they may potentially represent targets worthy of attention in the future.

Gravity                                                                               1VD Gravity

Figure 19: Bluebush 4 Gravity modelling

1VD

Gravity

Figure 20: Gravity modelling profiles over Bluebush 4.

The primary gravity anomalies are not well supported by the airborne magnetic data which indicate only minor responses as seen in Figure 21 and Figure 22. Smaller gravity anomalies seen on the eastern side exhibit more magnetic activity potentially implying greater opportunity for alteration. Not all of the eastern magnetic anomalies have been modelled as they were not the focus of this work.

TMI                                                                                                  1VD TMI

Figure 21: Airborne magnetic model locations over Bluebush 4.

Figure 22: Model locations of the Bluebush gravity (red) and Airborne magnetic (blue) anomalies.

The primary gravity anomalies have modelled at depths of 530m and 620m. These have been targeted with vertical drill holes described in Table 4. The lack of magnetic response associated with these anomalies means they do not conform to the traditional Tennant Creek model.

Figure 23: Bluebush 4 gravity and airborne magnetic model locations.

Table 4: Proposed drill holes targeting Bluebush anomaly 1.

| Anomaly | Hole # | East | North | Dip | Azi | Depth | Intersection |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BB grav 4 | Drillhole 10 | 381932 | 7810945 | 90 | 000 | 700 | 620 |
| BB grav 4 | Drillhole 11 | 381956 | 7810166 | 90 | 000 | 600 | 530 |