Woolner Airborne

Electromagnetic (AEM) Mapping Survey

Acquisition and Processing Report

for

Geoscience Australia

Prepared by :	M. Lawrence

L. Stenning

Authorised for release by :		…………………………..

…………………………..

Survey flown: October 2008 – December 2008

by

Fugro Airborne Surveys

435 Scarborough Beach Road, Osborne Park. WA 6017, Australia

Tel: (61-8) 9273 6400      Fax: (61-8) 9273 6466

FAS PROJECT # 2017

GA PROJECT # 1196

CONTENTS

SURVEY OPERATIONS AND LOGISTICS	4

1.1	Introduction	4

1.2	Survey Base	4

1.3	Survey Personnel	4

1.4	Area Map	5

SURVEY SPECIFICATIONS AND PARAMETERS	6

1.5	Area Co-ordinates	6

1.6	Survey Area Parameters	9

1.7	Job Safety Plan	10

1.8	General Disclaimer	10

2.	AIRCRAFT EQUIPMENT AND SPECIFICATIONS	11

2.1	Aircraft	11

2.2	TEMPEST System Specifications	11

2.2.1	EM Receiver and Logging Computer	11

2.2.2	TEMPEST Transmitter	12

2.2.3	TEMPEST 3-Axis Towed Bird Assembly	12

2.3	FASDAS Survey Computer	12

2.3.1	Caesium Vapour Magnetometer Sensor	12

2.3.2	Magnetometer Processor Board	12

2.3.3	Fluxgate Magnetometer	12

2.3.4	GPS Receiver	12

2.3.5	Differential GPS Demodulator	12

2.4	Navigation System	13

2.5	Altimeter System	13

2.5.1	Radar Altimeter	13

2.5.2	Laser Altimeter	13

2.5.3	Barometric Altimeter	13

2.6	Video Tracking System	13

2.7	Data Recorded by the Airborne Acquisition Equipment	13

3.	GROUND DATA ACQUISITION EQUIPMENT AND SPECIFICATIONS	14

3.1	Magnetic Base Station	14

3.2	GPS Base Station	14

4.	EM AND OTHER CALIBRATIONS AND MONITORING	15

4.1	Pre/Post-Flight GPS Repeat Point: Line 505FFFFAA	15

4.2	Pre/Post-Flight Transmitter-off: Lines 900FFFFAA , 906FFFFAA	15

4.3	Pre/Post-Flight Noise Additive: Lines 901FFFFAA, 904FFFFAA	15

4.4	Pre/Post-Flight Zero: Line 902FFFFAA, 905FFFFAA	15

4.5	Pre-Flight Swoops: Line 903FFFFAA	15

4.6	Dynamic Magnetometer Compensation	15

4.7	Parallax Checks	16

4.8	Radar Altimeter Calibration	16

4.9	Laser Altimeter Calibration	17

4.10	Heading Error Checks	18

4.11	Repeat Point GPS Check	19

5.	DATA PROCESSING	22

5.1	Field Data Processing	22

5.1.1	Quality Control Specifications	22

5.1.2	In-Field Data Processing	24

5.2	Final Data Processing	24

5.2.1	Flight Path Recovery	24

5.2.2	Magnetics	24

5.2.3	Altimeters	25

5.2.4	Derived Ground Elevation	25

5.2.5	Electromagnetic Data Processing	26

5.2.6	Conductivity Depth Images (CDI)	29

5.2.7	System Specifications for Modelling TEMPEST Data	32

5.2.8	CDI Depth Slicing	33

5.2.9	Delivered Products	33

REFERENCES	34

APPENDIX I – Weekly Acquisition Reports VH-TEM	35

APPENDIX II – Flight Summary (Survey Line Listing)	43

APPENDIX III – Pre and Post-flight Zero Statistics	55

APPENDIX IV – Located Data Format	63

APPENDIX V – Standby Days	86

APPENDIX VI – List of all Supplied Data and Products	87

APPENDIX VII – EM Repeat line CDIs	89

# SURVEY OPERATIONS AND LOGISTICS

## Introduction

Between the 19th of October 2008 and the 1st of December 2008, Fugro Airborne Surveys Pty. Ltd., (FAS) undertook an airborne TEMPEST electromagnetic and magnetic survey for Geoscience Australia, over the Woolner Project area in the Northern Territory. The survey was flown over one large block with a number of tighter line spaced infill areas. Total coverage of the Woolner survey amounted to 6862 line kilometres, flown in 32 flights. The survey was flown using a Casa C212-200 Turbo Prop aircraft, registration VH-TEM, owned and operated by FAS. This report summarises the procedures and equipment used by FAS in the acquisition, verification and processing of the airborne geophysical data.

## Survey Base

The survey was based out of Bachelor, Northern Territory. The survey aircraft was operated from Bachelor airport, with the aircraft fuel available on site. A temporary office was set up at the Bachelor Resort, from where all survey operations were run and the post-flight data verification was performed.

## Survey Personnel

The following personnel were involved in this project:

Project Supervision	- Acquisition	Bart Anderson

- Processing	Adam Shales

Pilot/s	Grant Hamilton

Marcus Tapp

Tymon Dyer

System Operator/s	Michael Wirski

Michael Githinji

Luke Kelly

James Levarre

Aircraft Engineer/s	Richard Cardin

Clint Hazelwood

Field Data Processing	Paul Evans, Kah Tho Lee

Office Data Processing	Matt Lawrence

## Area Map

The following figure shows a breakdown of the areas flown, with the shaded regions being tighter line spaced infill areas. Note that some of the originally flight planned Rum Jungle areas (area names beginning with RJ) were subsequently included in the acquisition and delivery of this Woolner block.

# SURVEY SPECIFICATIONS AND PARAMETERS

## Area Co-ordinates

The survey area was located within Map Grid of Australia zones 52 and 53. All data was delivered in MGA zone 52.

As per the diagram above there are a number of areas infilling the larger 5000m spaced GA regional lines. The approximate co-ordinates of each of those areas (labelled as per the previous diagram) are as follows:

Coordinates of Woolner regional area WGA5000 (5000m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 726952 | 8656683 | 21 | 657798 | 8580159 |
| 2 | 739092 | 8652564 | 22 | 657798 | 8582543 |
| 3 | 752749 | 8642158 | 23 | 663001 | 8585795 |
| 4 | 752966 | 8639557 | 24 | 663651 | 8594683 |
| 5 | 834693 | 8639991 | 25 | 674491 | 8594900 |
| 6 | 835560 | 8624165 | 26 | 674057 | 8575173 |
| 7 | 829490 | 8618529 | 27 | 700938 | 8574956 |
| 8 | 815616 | 8613760 | 28 | 700721 | 8599886 |
| 9 | 815399 | 8609207 | 29 | 719148 | 8602921 |
| 10 | 819735 | 8604221 | 30 | 718931 | 8609858 |
| 11 | 819951 | 8587312 | 31 | 739525 | 8609858 |
| 12 | 823854 | 8587312 | 32 | 739308 | 8613977 |
| 13 | 824070 | 8580159 | 33 | 728253 | 8613977 |
| 14 | 782015 | 8579942 | 34 | 728253 | 8622648 |
| 15 | 784399 | 8573655 | 35 | 718931 | 8622431 |
| 16 | 788735 | 8569320 | 36 | 718931 | 8639340 |
| 17 | 790252 | 8563250 | 37 | 718931 | 8645627 |
| 18 | 623764 | 8562599 | 38 | 714595 | 8652997 |
| 19 | 623764 | 8564550 | 39 | 717630 | 8656683 |
| 20 | 648693 | 8579725 |  |  |  |

Coordinates of Woolner area WGA1666 (1666m line spacing).

(Note: not all lines were flown in the southern portion of this block by the CASA, those lines will be completed and delivered with the Rum Jungle survey dataset to be flown by VH-WGT (Skyvan))

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 718244 | 8656524 | 22 | 767982 | 8615404 |
| 2 | 725744 | 8656392 | 23 | 755679 | 8615338 |
| 3 | 726139 | 8655274 | 24 | 755613 | 8613562 |
| 4 | 737455 | 8652379 | 25 | 754495 | 8613233 |
| 5 | 742587 | 8646852 | 26 | 754561 | 8609483 |
| 6 | 747389 | 8645800 | 27 | 755547 | 8609549 |
| 7 | 750810 | 8638694 | 28 | 755547 | 8608496 |
| 8 | 748771 | 8636786 | 29 | 740350 | 8608496 |
| 9 | 752718 | 8636918 | 30 | 740481 | 8614154 |
| 10 | 752587 | 8637642 | 31 | 732192 | 8614286 |
| 11 | 758376 | 8640339 | 32 | 732126 | 8617049 |
| 12 | 779561 | 8640010 | 33 | 729165 | 8616917 |
| 13 | 779627 | 8621062 | 34 | 729231 | 8622510 |
| 14 | 791535 | 8621194 | 35 | 719889 | 8622575 |
| 15 | 791667 | 8639944 | 36 | 719954 | 8645997 |
| 16 | 815483 | 8639944 | 37 | 720349 | 8646984 |
| 17 | 815220 | 8613430 | 38 | 717060 | 8650866 |
| 18 | 786732 | 8613496 | 39 | 716599 | 8652445 |
| 19 | 786601 | 8601917 | 40 | 716599 | 8654287 |
| 20 | 771929 | 8601917 | 41 | 718047 | 8656195 |
| 21 | 772127 | 8615470 |  |  |  |

Coordinates of Woolner infill area W1 (555m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 719100 | 8639798 | 19 | 729353 | 8616985 |
| 2 | 751182 | 8639760 | 20 | 729391 | 8622508 |
| 3 | 751523 | 8638890 | 21 | 720046 | 8622470 |
| 4 | 750388 | 8637301 | 22 | 719857 | 8624059 |
| 5 | 748496 | 8636582 | 23 | 720046 | 8624513 |
| 6 | 747853 | 8635069 | 24 | 719857 | 8625724 |
| 7 | 754739 | 8635031 | 25 | 720008 | 8626254 |
| 8 | 754587 | 8614412 | 26 | 719857 | 8627389 |
| 9 | 755609 | 8614374 | 27 | 719819 | 8629016 |
| 10 | 755609 | 8613731 | 28 | 719895 | 8630718 |
| 11 | 754625 | 8613693 | 29 | 719857 | 8632345 |
| 12 | 754587 | 8609759 | 30 | 719932 | 8633972 |
| 13 | 740438 | 8609759 | 31 | 719895 | 8635636 |
| 14 | 740551 | 8610704 | 32 | 719932 | 8637301 |
| 15 | 740513 | 8613466 | 33 | 719932 | 8638966 |
| 16 | 740438 | 8614147 | 34 | 719478 | 8639457 |
| 17 | 732266 | 8614147 |  |  |  |
| 18 | 732379 | 8616947 |  |  |  |

Coordinates of Woolner infill area W2 (555m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 767999 | 8640024 | 7 | 771938 | 8601920 |
| 2 | 779625 | 8639976 | 8 | 771938 | 8605146 |
| 3 | 779720 | 8620996 | 9 | 772080 | 8615444 |
| 4 | 786648 | 8620901 | 10 | 768047 | 8615491 |
| 5 | 786743 | 8619952 | 11 | 768142 | 8639692 |
| 6 | 786648 | 8601920 |  |  |  |

Coordinates of Woolner infill area W3 (333m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 758545 | 8626336 | 4 | 758545 | 8624602 |
| 2 | 766657 | 8626364 | 5 | 758545 | 8626252 |
| 3 | 766685 | 8624518 |  |  |  |

Coordinates of Woolner infill area W4 (333m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 756811 | 8622588 | 4 | 756783 | 8618113 |
| 2 | 764923 | 8622588 | 5 | 756783 | 8622560 |
| 3 | 764979 | 8618085 |  |  |  |

Coordinates of Woolner infill area W5 (1666m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 815512 | 8639974 | 13 | 832796 | 8623504 |
| 2 | 833786 | 8639916 | 12 | 830876 | 8621642 |
| 3 | 833902 | 8638112 | 14 | 829828 | 8620012 |
| 4 | 834077 | 8636948 | 15 | 829246 | 8619314 |
| 5 | 834193 | 8635202 | 16 | 829014 | 8618266 |
| 6 | 834310 | 8633398 | 17 | 823427 | 8617684 |
| 7 | 834426 | 8631885 | 18 | 823485 | 8616986 |
| 8 | 834659 | 8630197 | 19 | 820633 | 8616462 |
| 9 | 834775 | 8628451 | 20 | 820691 | 8615240 |
| 10 | 834950 | 8626996 | 21 | 815163 | 8615240 |
| 11 | 833844 | 8624843 | 22 | 815454 | 8639683 |

Coordinates of Woolner infill area W6 (1666m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 801585 | 8604862 | 11 | 788049 | 8581259 |
| 2 | 818935 | 8604819 | 12 | 789377 | 8584857 |
| 3 | 818464 | 8588798 | 13 | 789891 | 8585371 |
| 4 | 819149 | 8587170 | 14 | 790576 | 8587856 |
| 5 | 823304 | 8586828 | 15 | 804198 | 8588027 |
| 6 | 823347 | 8580231 | 16 | 804198 | 8589098 |
| 7 | 800386 | 8579674 | 17 | 805998 | 8592525 |
| 8 | 800386 | 8578603 | 18 | 806383 | 8593082 |
| 9 | 791818 | 8578560 | 19 | 801671 | 8602506 |
| 10 | 791604 | 8581002 | 20 | 801885 | 8604305 |

The following areas were originally planned as part of the Rum Jungle survey but since they ended up being flown by VH-TEM (CASA), it was requested the data be processed and delivered as part of the Woolner survey.

There were four additional survey lines – 11022, 11023, 32002 and 32003, that were flown by the CASA at the southern end of the Woolner areas, but for data continuity purposes these will be reflown and delivered as part of the Rum Jungle survey dataset.

Coordinates of Woolner infill area RJNGA1666 (1666m line spacing).

(Note: not all lines were flown in the southern portion of this block by the CASA, those lines will be completed and delivered with the Rum Jungle survey dataset to be flown by VH-WGT (Skyvan)

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 704480 | 8591612 | 6 | 720680 | 8542095 |
| 2 | 737066 | 8591357 | 7 | 720667 | 8540496 |
| 3 | 736592 | 8536028 | 8 | 708892 | 8540589 |
| 4 | 727558 | 8536105 | 9 | 709042 | 8560228 |
| 5 | 727427 | 8542039 | 10 | 704250 | 8560264 |

Coordinates of Woolner infill area RJ8 (333m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 751336 | 8597690 | 7 | 769572 | 8588422 |
| 2 | 756830 | 8597672 | 8 | 769550 | 8589313 |
| 3 | 761748 | 8594153 | 9 | 751506 | 8589634 |
| 4 | 761726 | 8591785 | 10 | 751613 | 8591478 |
| 5 | 785565 | 8591124 | 11 | 748629 | 8591873 |
| 6 | 785535 | 8588080 | 12 | 748656 | 8595009 |

Coordinates of Woolner infill area RJ9 (333m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 770321 | 8584786 | 3 | 782221 | 8579688 |
| 2 | 782182 | 8584731 | 4 | 770274 | 8579867 |

Coordinates of Woolner infill area RJ10 (333m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 800560 | 8570608 | 3 | 787436 | 8576344 |
| 2 | 787649 | 8570653 | 4 | 800711 | 8576204 |

Coordinates of Woolner infill area RJ11 (333m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 704399 | 8580426 | 3 | 711578 | 8568662 |
| 2 | 711426 | 8580374 | 4 | 704313 | 8568717 |

Coordinates of Woolner infill area RJ13 (238m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 710260 | 8569809 | 3 | 719550 | 8562760 |
| 2 | 719455 | 8569738 | 4 | 710264 | 8562401 |

Coordinates of Woolner infill area RJ21 (1666m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 659210 | 8582840 | 6 | 669820 | 8577123 |
| 2 | 666298 | 8583106 | 7 | 669864 | 8574480 |
| 3 | 666371 | 8595397 | 8 | 656118 | 8574560 |
| 4 | 671862 | 8595426 | 9 | 656133 | 8577295 |
| 5 | 671751 | 8577265 | 10 | 659028 | 8577340 |

Coordinates of Woolner infill area RJ22 (1666m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 686211 | 8575451 | 3 | 694302 | 8568235 |
| 2 | 695258 | 8575605 | 4 | 686162 | 8568076 |

Coordinates of Woolner infill area RJ23 (1666m line spacing).

| Vertex | Easting (m) | Northing (m) | Vertex | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 678986 | 8567724 | 3 | 686038 | 8559329 |
| 2 | 686159 | 8567615 | 4 | 678717 | 8558844 |

Repeat Line Coordinates

| Line Number | Easting (m) | Northing (m) | End | Easting (m) | Northing (m) |
| --- | --- | --- | --- | --- | --- |
| 910FFFFAA | 758600 | 8625930 | 910 | 766600 | 8625930 |
| 911FFFFAA | 749700 | 8568950 | 911 | 760000 | 8568950 |

(where FFFF is the 4 digit flight number and AA is the two digit attempt number)

## Survey Area Parameters

Specifications for the Woolner AEM survey:

| Line spacing: Regional lines WGA5000 Area WGA1666 Area W1 Area W2 Area W3 Area W4 Area W5 Area W6 Area RJNGA1666 Area RJ8 Area RJ9 Area RJ10 Area RJ11 Area RJ13 Area RJ21 Area RJ22 Area RJ23 | 5000 metres 1666 metres 555 metres 555 metres 333 metres 333 metres 1666 metres 1666 metres 1666 metres 333 metres 333 metres 333 metres 333 metres 238 metres 1666 metres 1666 metres 1666 metres |
| --- | --- |
| Tie line spacing | n/a |
| Line direction | 090 – 270° grid (E-W) |
| Tie line direction | n/a |
| Terrain clearance | 121.1m (transmitter terrain clearance) |
| Total line kilometres | 6862 kilometres |

FAS Job Number	-	2017

GA Job Number	-	1196

Survey Company	-	Fugro Airborne Surveys Pty Ltd

Date Flown	-	19th October 2008 – 1st December 2008

Client	-	Geoscience Australia

EM System	-	25 Hz TEMPEST

Navigation	-	Real-time differential GPS

Datum	-	GDA94 (MGA 52)

Nominal Terrain Clearance	-	121.1m (transmitter terrain clearance)

## Job Safety Plan

A Job Safety Plan was prepared and implemented in accordance with the Fugro Airborne Surveys Occupational Safety & Health Management System.

## General Disclaimer

It is Fugro Airborne Survey’s understanding that the data and report provided to the client is to be used for the purpose agreed between the parties. That purpose was a significant factor in determining the scope and level of the Services being offered to the Client. Should the purpose for which the data and report is used change, the data and report may no longer be valid or appropriate and any further use of, or reliance upon, the data and report in those circumstances by the Client without Fugro Airborne Survey's review and advice shall be at the Client's own or sole risk.

The Services were performed by Fugro Airborne Survey exclusively for the purposes of the Client. Should the data and report be made available in whole or part to any third party, and such party relies thereon, that party does so wholly at its own and sole risk and Fugro Airborne Survey disclaims any liability to such party.

Where the Services have involved Fugro Airborne Survey's use of any information provided by the Client or third parties, upon which Fugro Airborne Survey was reasonably entitled to rely, then the Services are limited by the accuracy of such information. Fugro Airborne Survey is not liable for any inaccuracies (including any incompleteness) in the said information, save as otherwise provided in the terms of the contract between the Client and Fugro Airborne Survey.

# AIRCRAFT EQUIPMENT AND SPECIFICATIONS

## Aircraft

Manufacturer	-	CASA

Model	-	C212-200 Turbo Prop

Registration	-	VH-TEM

Ownership	-	Fugro Airborne Surveys Pty Ltd

## TEMPEST System Specifications

Specifications of the TEMPEST Airborne EM System (Lane et al., 2000) are:

- Base frequency			-	25 Hz

- Transmitter area			-	221 m2  (TEM)

- Transmitter turns			-	1

- Waveform				-	Square

- Duty cycle				-	50%

- Transmitter pulse width		-	10 ms

- Transmitter off-time			-	10 ms

- Peak current			-	280 A (TEM)

- Peak moment			-	61880 Am2  (TEM)

- Average moment			-	30940 Am2  (TEM)

- Sample rate      			-	75 kHz on X and Z

- Sample interval			-	13.333 microseconds

- Samples per half-cycle		-	1500

- System bandwidth			-	25 Hz to 37.5 kHz

- Tx Loop Flying height nominal	-	121.1 m (subject to safety considerations)

- Tx Loop Flying height average 	-	124.3 m (TEM)

- EM sensor				-	Towed bird with 3 component dB/dt coils

- Tx-Rx horizontal separation	average-	-121.0 m (TEM)

- Tx-Rx vertical separation average 	-	-33.9 m (TEM)

- Tx-Rx horizontal separation standard-	-120.0 m (geometry corrected standard)

- Tx-Rx vertical separation standard	-	-35.0 m (geometry corrected standard)

- Stacked data output interval		-	200 ms (~12 m)

- Number of output windows		-	15

- Window centre times		-	13 ms to 16.2 ms

- Magnetometer 			-	Stinger-mounted caesium vapour

- Magnetometer compensation	-	Fully digital

- Magnetometer output interval	-	200 ms (~12 m)

- Magnetometer resolution		-	0.001 nT

- Typical noise level			-	0.5 nT

- GPS cycle rate			-	1 second

### EM Receiver and Logging Computer

The EM receiver computer was an EMFASDAS. The EM receiver computer execute a proprietary program for system control, timing, data acquisition and recording. Control, triggering and timing is provided to the TEMPEST transmitter and Digital Signal Processing (DSP) boards by the timing card, which ensures that all waveform generation and sampling is accomplished with high accuracy. The timing card is synchronised to the Global Positioning System (GPS) through the use of the Pulse Per Second (PPS) output from the system GPS card. Synchronisation is also provided to the magnetometer processor card for the purpose of accurate magnetic sampling with respect to the EM transmitter waveform.

The EM receiver computer displays information on the main screen during system calibrations and survey line acquisition to enable the airborne operator to assess the data quality and performance of the system.

### TEMPEST Transmitter

The transmitted waveform is a square wave of alternating polarity, which is triggered directly from the EM receiver computer. The nominal transmitter base frequency was 25 Hz with a pulse width of 10ms (50 % duty cycle). Loop current waveform monitoring is provided by a current transformer located directly in the loop current path to allow for full logging of the waveform shape and amplitude, which is sampled by the EM receiver.

### TEMPEST 3-Axis Towed Bird Assembly

The TEMPEST 3-axis towed bird assembly provides accurate low noise sampling of the X (horizontal in line), Y (horizontal transverse) and Z (vertical) components of the electromagnetic field. Note that the Y component data were not processed or delivered in the dataset for this survey.  The receiver coils measure the time rate of change of the magnetic field (dB/dt). Signals from each axis are transferred to the aircraft through a tow cable specifically designed for its electrical and mechanical properties.

## FASDAS Survey Computer

The Survey computer executes a proprietary program for acquisition and recording of location, magnetic and ancillary data. Data are presented both numerically and graphically in real time on the Video Graphics Array (VGA) Liquid Crystal Display (LCD) display, which provides an on-line display capability. The operator may alter the sensitivity of the displays on-line to assist in quality control. Selected EM data are transferred from the EM receiver computer to the SURVEY computer for quality control (QC) display.

### Caesium Vapour Magnetometer Sensor

A caesium vapour magnetometer sensor is utilised on the aircraft and consists of the sensor head, cable and the sensor electronics. The sensor head is housed at the end of a composite material tail stinger.

### Magnetometer Processor Board

A FASDAS magnetometer processor board was used for de-coupling and processing the Larmor frequency output of the magnetometer sensor. The processor board interfaces with the survey computer, which initiates data sampling and transfer for precise sample intervals and also with the EM receiver computer to ensure that the magnetic samples remain synchronised with the EM system.

### Fluxgate Magnetometer

A tail stinger mounted Bartington MAG-03MC three-axis fluxgate magnetometer is used to provide information on the attitude of the aircraft. This information is used for compensation of the measured magnetic total field.

### GPS Receiver

A Novatel GPScard 951R is utilised for airborne positioning and navigation. Satellite range data are recorded for generating post processed differential solutions.

### Differential GPS Demodulator

The OMNISTAR differential GPS service provides real time differential corrections.

## Navigation System

A FASDAS Navigation Computer was used for real-time navigation. These computers load a pre-programmed flight plan from disk which contains boundary co-ordinates, line start and end co-ordinates, local co-ordinate system parameters, line spacing, and cross track definitions. The World Geodetic System 1984 (WGS84) latitude and longitude positional data received from the Novatel GPS card contained in the SURVEY computer is transformed to the local co-ordinate system for calculation of the cross track and distance to go values. This information, along with ground heading and ground speed, is displayed to the pilot numerically and graphically on a two line LCD display, and on an analogue Horizontal Strip Indicator (HSI). It is also presented on a LCD screen in conjunction with a pictorial representation of the survey area, survey lines, and ongoing flight path.

The Navigation computers are interlocked to the SURVEY computer for auto selection and verification of the line to be flown. The GPS information passed to the navigation computer is corrected using the received real time differential data from the OMNISTAR service, enabling the aircraft to fly as close to the intended track as possible.

## Altimeter System

### Radar Altimeter

Model:			Sperry Stars RT-220 radar altimeter system (VH-TEM)

Sample interval:		0.2 second

Accuracy:		± 1.5 % of indicated altitude

The radar altimeters fitted to the aircraft are high quality instruments whose output is factory calibrated.  The aircraft radar altitude is recorded onto hard drive as well as displayed on the aircraft chart recorder.  The recorded value is the average of the altimeter’s output during the previous 0.2 seconds.

### Laser Altimeter

Model:			Optech 501SB (TEM)

Sample interval:		0.2 second

Accuracy:		± 0.05m at survey altitude

### Barometric Altimeter

Output of a Digiquartz 215A-101pressure transducer is used for calculating the barometric altitude of the aircraft. The atmospheric pressure is taken from a gimbal-mounted probe projecting 0.5 metres from the wing tip of the aircraft and fed to the transducer mounted in the aircraft wingtip.

## Video Tracking System

The video file recorded by the digital video system is synchronised with the geophysical record by a digital fiducial display. It is also labelled with GPS latitude and longitude information and survey line number.

## Data Recorded by the Airborne Acquisition Equipment

With the FASDAS acquisition system the raw EM data including fiducial, local time, X and Z axis sensor response, current monitor and bird auxiliary sensor output are recorded on the EM receiver computer as “*.raw” EM files. Logging to the files is continuous, however, a new *.raw EM file is created when the size of the previous one reaches 1Gb.

The FASDAS Survey computer records a continuous MSD file which contains all other ancillary data including magnetic, altimeter, GPS and analogue channels.

# GROUND DATA ACQUISITION EQUIPMENT AND SPECIFICATIONS

## Magnetic Base Station

A CF1 and a Scintrex ENVI magnetometer were used to measure the daily variations of the Earth’s magnetic field.  The base stations were established in an area of low gradient, away from cultural influences. The base stations were run continuously throughout the survey flying period with a sampling interval of 1 and 2 seconds respectively at a sensitivity of 0.1 nT. The magnetometer base stations were set up at Bachelor airstrip, in scrub, north of the graded taxiway.

## GPS Base Station

A GPS base logging station integrated with the CF1 unit was used throughout the survey, setup at Bachelor airstrip.

The GPS base station position was calculated by logging data continuously at the base position over a period of approximately 24 hours. Data were then averaged to obtain the position of the base station using GrafNav software.

The calculated GPS base position was (in GDA94):

Lat:	-13° 02’ 29.04164” S

Long:	131° 01’ 25.91224” E

Height:	159.114 m. (ellipsoidal height). Sensor approximately 2m above ground surface.

# EM AND OTHER CALIBRATIONS AND MONITORING

At the beginning and end of each individual survey flight, the EM system is checked for background noise levels and performance. The airborne checks are conducted at a nominal terrain clearance of 1100 m (3600 ft) to eliminate ground response.

These checks include:-

## Pre/Post-Flight GPS Repeat Point: Line 505FFFFAA

Where possible, the aircraft is parked in the same position after every flight and the GPS position recorded pre and post flight, to allow for checks on GPS quality and repeatability.  Note: FFFF is the flight number and AA is the attempt number

## Pre/Post-Flight Transmitter-off: Lines 900FFFFAA , 906FFFFAA

These lines are recorded in straight and level flight with the system in standard survey geometry, with the transmitter turned off and bird response turned on to observe ambient noise and to check for noise in the receiver system (bird/coils  tow cable  winch  computer). Note: FFFF is the flight number and AA is the attempt number

## Pre/Post-Flight Noise Additive: Lines 901FFFFAA, 904FFFFAA

These lines are recorded in straight and level flight with the system in standard survey geometry, with the transmitter on and the bird response turned off at the tow cable winch.  This is to check the noise contribution from the acquisition system and is used in deconvolution of survey line data. Note: FFFF is the flight number and AA is the attempt number

## Pre/Post-Flight Zero: Line 902FFFFAA, 905FFFFAA

These lines are recorded in straight and level flight with the system in standard survey configuration with transmitter and receiver turned on. This is used to determine the system’s response in the absence of ground signal and is used to determine a standard waveform for deconvolution of survey lines. Note: FFFF is the flight number and AA is the attempt number

Additionally, through all these calibrations the airborne operator can assess the system and ambient noise levels.

## Pre-Flight Swoops: Line 903FFFFAA

This line is recorded immediately after the pre-flight zero. During this manoeuvre the pilot conducts a series of ‘swoop’ manoeuvres (pitch up/pitch down) over approximately 30-40 seconds to vary the position of the towed sensor relative to the aircraft. The EM data are monitored by the airborne operator to confirm correct operation of the system during the manoeuvre. This data is used to determine coefficients used in the processing to compensate for such variations in the survey data. Note: FFFF is the flight number and AA is the attempt number

## Dynamic Magnetometer Compensation

To limit aircraft manoeuvre effects on the magnetic data that can be of the same spatial wavelength as the signals from geological sources, compensation calibration lines are flown as high as practical in a low magnetic gradient area close to the survey. This involves flying a series of tests at 2500m or higher on the survey line heading and approximately 15 degrees either side to accommodate small heading variations whilst flying survey lines. The data for each heading consists of a series of aircraft manoeuvres, including pitches, rolls and yaws. This is done to artificially create the most extreme possible attitude the aircraft may encounter whilst on survey. Data from these lines are used to derive compensation coefficients for removing magnetic noise induced by the aircraft’s attitude in the naturally occurring magnetic field.

Compensation data were acquired on the following dates:

| Aircraft | Compensation Date | Flights Covered |
| --- | --- | --- |
| VH-TEM | 19/10/2008 | All flights |

Compensation data acquired the following statistics:

StdDev UnComp	2.189

StdDev Comp		0.089

Improvement Ratio	24.6

## Parallax Checks

Due to the relative positions of the EM towed bird and the magnetometer instruments on the aircraft and to processing / recording time lags, raw readings from each vary in position. To correct for this and to align selected anomaly features on lines flown in opposite directions, magnetics, EM data and the altimeters are ‘parallaxed’ with respect to the position information. System parallax is checked by flying in opposing directions over known geophysical features.  This is also monitored routinely during processing of jobs and specifically checked following any major changes in the aircraft system which are likely to affect the parallax values.

The last parallax check was performed in August 2008 to confirm GPS, Mag and altimeter parallax. Parallax values for the X and Z EM components were chosen to optimise the gridded display and for aligning, from line to line, the EM response amplitudes for horizontal or broad steeply dipping conductors, which account for the majority of responses in regolith-dominated terrains such as this.

Because of the change in acquisition system (from Picodas to EMFasdas) these values will vary from previous results.

| Variable | Parallax Value |
| --- | --- |
| Magnetics | -1.0 s |
| GPS | 0 s |
| Radar Altimeter | 0 s |
| Laser Altimeter | 0 s |
| EM – X | -1.6 s |
| EM – Z | -0.2 s |

Note that a positive parallax value in the tables above indicates that samples in that data stream are moved to lower fiducial numbers.

## Radar Altimeter Calibration

The radar altimeter is checked for accuracy and linearity every 12 months or when any change in a key system component requires this procedure to be carried out. This calibration involves flying a number of lines at a range of constant altitudes to allow the radar altimeter data to be compared to and assessed with other height data (GPS and barometric) to confirm the accuracy of the radar altimeter over its operating range.

Absolute radar calibrations for VH-TEM were carried out over the ocean on the 2nd October 2008..

The graphs below show the results of these calibrations as Radar Altimeter output (m) versus the GPS height normalised to altitude above the ocean (based on average GPS along the lowest altitude pass). This chart shows the linear behaviour of the radar altimeter in each range.

Comparison of Radar Altimeter and GPSZ    2nd October 2008

Regression equation: GPSZ=0.9981*ralt + 0.263		R-squared = 1

## Laser Altimeter Calibration

The Laser altimeter was checked based on the same process as that described above for the radar altimeters. The data used was from the same flights. The plots below show the laser altimeter heights compared to normalised GPS heights.

Pitch and roll manoeuvres were also conducted to determine coefficients to verify and/or correct for the laser’s deviation from the vertical.

The following equation was used to correct the laser altimeter for changes in pointing direction:

lc=lmcospm+p0cosrm+r0−hlsinpm+p0

Where lc is the corrected altimeter value, lm the raw measured altimeter value, pm and rm are the measured transmitter loop pitch and roll respectively, p0 and r0 are the laser altimeter pointing pitch and roll offsets relative to the transmitter loop orientation respectively, and h0 is the horizontal offset between the laser altimeter and the aircraft’s centre of rotation. Based on the data acquired during the calibration flights, the following values for p0, r0 and h0 were used for corrections throughout the survey.

|  | p0 | r0 | h0 |
| --- | --- | --- | --- |
| VH-TEM | 0.90 | -0.10 | 0.42 |

Comparison of Laser Altimeter and GPSZ – 2nd October 2008

Regression equation: GPSZ=0.9984*lidar + 0.0289		R-squared = 1

The following plot shows the radar altimeter compared to the laser altimeter, corrected for aircraft pitch and roll.

Comparison of Laser and Radar Altimeters – 2nd October 2008

Regression equation: lidar=1.0015*radalt + 0.1988		R-squared = 1

## Heading Error Checks

Historically, heading error checks have been part of the aeromagnetic data acquisition procedure but they are no longer used. Fugro Airborne Surveys now calculates these effects using the aircraft magnetic compensation system and specially developed software. The precision to which these effects are now calculated and corrected for is far in excess of the manual methods used in the past.

## Repeat Point GPS Check

At the end of each flight the aircraft were parked as close to the same position as possible.  Before and after the flight 90-120 seconds of data was recorded in this location to provide a check for consistency in navigation data.  The following pages show plots of the average GPS height, northing and easting for each ground calibration during the survey.

Woolner Flts 1-14

Note: the aircraft was required to be moved following flt 01

Note: the aircraft was required to be moved following flt 01

Note: the aircraft was required to be moved following flt 01

Woolner Flts 14-32

# DATA PROCESSING

## Field Data Processing

### Quality Control Specifications

#### Navigation Tolerance

The re-flight specifications applied for the duration of the survey were:

Electronic Navigation - absence of electronic navigation data (e.g. GPS base station fails).

Flight Path – flight path deviates by more than 40 metres over a continuous distance of 1500 metres or more unless the deviation is required by civil aviation requirements.

Altitude – the average terrain clearance for any one flight line shall be within ±5 metres of the nominal aircraft terrain clearance (121m). Portions of survey lines that are unable to be flown at the nominal survey height due to Australian Civil Aviation Safety Authority regulations of safety considerations shall be excluded from the average. Where the terrain clearance varies from that nominated by more than 20 metres over a continuous distance of two kilometres or more, a fill-in line will be flown at the Contractor’s expense unless it can be reasonably demonstrated that such flying would put pilot and crew at risk.

The following plot show the average transmitter ground clearance for all survey lines. A number of lines appear to violate the above specification for average height. In all cases this was a result of rougher terrain.

Woolner Flts 1-14

Woolner Flts 14-32

#### Electromagnetic Data

Based on the high altitude reference (zero) line flown at the start and end of each flight, the quality control checks on the electromagnetic data were:

Noise – For any flight, if the standard deviation of the processed high altitude data for a window exceeds the corresponding Additive Noise specified in the Noise Characteristics table below, then that window will be deemed to be ‘noisy’. If more than 25% of the windows are deemed to be noisy in either component, then that flight must be reflown at the Contractor’s expense. See Appendix III for full record of zero-line standard deviation statistics.

Bias – For any flight, if the absolute value of the mean of the processed high altitude data for a window exceeds the corresponding Bias specified in the Noise Characteristics table below, then that window will be deemed to be ‘biased’. If more than 25% of the windows are deemed to be biased in either component, then that flight must be reflown at the Contractor’s expense. See Appendix III for full record of zero-line bias statistics.

| Window | Additive Noise (standard deviation of high altitude data) (fT) |  | Bias (absolute value of mean of high altitude data) (fT) |  |
| --- | --- | --- | --- | --- |
|  | X component | Z component | X component | Z component |
| 1 | 0.0362 | 0.0267 | 0.0151 | 0.0145 |
| 2 | 0.0348 | 0.0160 | 0.0336 | 0.0248 |
| 3 | 0.0315 | 0.0140 | 0.0266 | 0.0195 |
| 4 | 0.0260 | 0.0134 | 0.0114 | 0.0081 |
| 5 | 0.0238 | 0.0122 | 0.0172 | 0.0132 |
| 6 | 0.0206 | 0.0123 | 0.0126 | 0.0096 |
| 7 | 0.0190 | 0.0117 | 0.0112 | 0.0093 |
| 8 | 0.0182 | 0.0118 | 0.0110 | 0.0090 |
| 9 | 0.0176 | 0.0110 | 0.0106 | 0.0087 |
| 10 | 0.0174 | 0.0102 | 0.0102 | 0.0087 |
| 11 | 0.0170 | 0.0099 | 0.0104 | 0.0081 |
| 12 | 0.0163 | 0.0084 | 0.0108 | 0.0078 |
| 13 | 0.0146 | 0.0075 | 0.0090 | 0.0066 |
| 14 | 0.0126 | 0.0070 | 0.0066 | 0.0054 |
| 15 | 0.0134 | 0.0087 | 0.0056 | 0.0051 |

Repeat lines – these were flown regularly to check system repeatability.  Section 1.5 lists the co-ordinates for the test line used throughout the survey. The repeat line was flown once every day for the first four successful production days, and once every three production days after that. Comparison plots of derived conductivity for both repeat lines are included as an attachment (see Appendix VII).

### In-Field Data Processing

Following acquisition, multiple copies of the EM data are made onto DVDs or CDs. The EM, location, magnetic and ancillary data are then processed at the field base to the point that the quality of the data from each flight can be fully assessed. Copies of the raw and processed data were then transferred to Perth for final data processing. A more comprehensive statement of EM data processing is given in section 5.2.3.

## Final Data Processing

### Flight Path Recovery

The GPS position of the aircraft at every point along the survey line was post-processed (differentially corrected) by applying the same X, Y and Z positional changes (deviations from averaged position) as seen at the base GPS unit (see 3.2 for a description of establishing the base GPS position).

The post-processed flight path (X and Y co-ordinates) and GPS height were then checked for spikes and level shifts, and if required, edited or improved by re-running the GPS post-processing. Section 4.12 describes the GPS repeat point test we conducted on every flight to confirm the repeatability of the GPS system. No other calibration procedures are performed for the GPS.

### Magnetics

Magnetic data were compensated for aircraft manoeuvre noise using coefficients derived from the appropriate compensation flight (see 4.7). Base station data was edited so that all significant spikes, level shifts and null data were eliminated.

A diurnal base value was then added.

| Area | Diurnal Base Value |
| --- | --- |
| All | 47436 nT |

A lag was applied to synchronise the magnetic data with the navigation data.

The International Geomagnetic Reference Field (IGRF) 2005 model (updated for secular variation 2008.8) was removed from the levelled total field magnetics. An IGRF base value, calculated at a central point within the survey area, was then added to the data.

| Area | IGRF Base Value |
| --- | --- |
| All | 46836 nT |

Following this, microlevelling was applied in order to subtly level the data. The algorithm is a FAS proprietary operation used to remove the small across-line corrugations that may appear in any gridded data. The process attempts to de-corrugate the data without destroying the data’s integrity. This is achieved by confining the changes to small values and applying them as a correction to the along-line data.

### Altimeters

Radar altimeter data are recorded by the data acquisition system as a value in millivolts. This value is converted to metres using the relationships determined during the altimeter calibration flights. This data has a parallax applied followed by a short smoothing filter to eliminate short-wavelength system noise. Due to the dense vegetation in the survey area some false returns, due to reflection of the radar altimeter signal from the tree tops and not the ground, remain in the data. As this is not the primary altimeter used for the DEM and the EM terrain clearance corrections, manual correcting of these effects was only applied to the laser altimeter data.

The laser altimeter (lidar) data are recorded directly as a height in metres. As a first step all spurious values, and values of 0m (non-returns) were removed, followed by a routine that used local maxima and minima to remove small sharp steps & spikes, resulting from vegetation and other cultural features. Because of the particularly dense vegetation in the Woolner survey area, the automated spike removal routines could not remove all false returns, hence, further manual editing of spurious responses thought to be from tree tops and not the ground, was required for the laser altimeter data. There was also an instance on line 6103101 where, due to a tower on the ground, a large flight path deviation and associated aircraft roll was required. This resulted in the lidar returning spurious values for this section which had to be manually removed in final processing.

The resulting channel from this process was splined and filtered, then finally the expression defined in section 4.10 was applied to correct for the changing pointing angle of the altimeter due to aircraft pitch and roll.

### Derived Ground Elevation

Aircraft navigation whilst in survey mode is via real time differential GPS, obtained by combining broadcast differential corrections with on-board GPS measurements. Terrain clearance is measured with a laser altimeter.

The ground elevation, relative to the WGS84 spheroid used by GPS receiver units, is obtained by finding the difference between the terrain clearance (from the final processed and edited laser altimeter) and the aircraft GPS antenna altitude above the ellipsoid (GPS height derived from post-processing of the DGPS data using the field base station data), and taking into account that the laser altimeter is mounted 2.3 metres below the GPS antenna.

The digital elevation model derived from this survey can be expected to have an absolute accuracy of +/- several metres in areas of low to moderate topographic relief. Sources of error include uncertainty in the height of the GPS base station, variations in the laser altimeter characteristics over ground of varying surface characteristics (ie. false and non-returns are more prevalent over dense vegetation and water, respectively), and the finite footprint of the laser altimeter.

Following this, microlevelling  was applied in order to more subtly level the data. The algorithm is a FAS proprietary operation used to remove the small across-line corrugations that may appear in any gridded data. The process attempts to de-corrugate the data without destroying the data’s integrity. This is achieved by confining the changes to very small values and applying them as a correction to the along-line data.

An N-Value is subtracted to correct the final data to the Australian Height Datum (AHD).

The final digital elevation model was then compared to the GEODATA 9 second DEM (DEM-9S) Version 3, which is a grid of ground elevation points covering the whole of Australia, with a grid spacing of 9 seconds in longitude and latitude (approximately 250m) in the GDA94 coordinate system. The DEM-9S grid is freely available through the Geophysical Archive Data Delivery System (GADDS). The following is a summary of the FASP final DEM relative to the DEM-9S data:

FASP DEM for Woolner:

Average value from final line data = 19.8m (relative to AHD)

DEM-9S for Woolner:

Average value from grid-sampled line data = 22.4m (relative to AHD)

Note:

The accuracy of the elevation calculation is directly dependent on the accuracy of the two input parameters, laser altitude and GPS altitude.  The GPS altitude value is dependent on the number of available satellites, plus the accuracy of the averaged GPS base position. Although post-processing of GPS data will yield X and Y accuracies in the order of 0.5 metres, the accuracy of the altitude value is usually much less, but generally still within 1-2 metres.  Further inaccuracies may be introduced during the interpolation and gridding process as only 1 out of every 5 points across-line is real data. Furthermore, along line obstructions may cause the pilot to veer laterally and so data interpolated between lines may vary significantly from real topography, and do not show artificial vertical obstructions.

Because of the inherent inaccuracies of this method, no guarantee is made or implied that the information displayed is a true representation of the height above sea level.  Although this product may be of some use as a general reference,  THIS PRODUCT MUST NOT BE USED FOR NAVIGATION PURPOSES.

### Electromagnetic Data Processing

Details of the pre-processing applied to TEMPEST data can be found in Lane et al. (2000), and are summarised below.

Calibration

High altitude pre and post flight zero line data (Section 4.4) are used to characterise the system response in the absence of any ground response. These calibration lines were acquired pre and post flight and were linearly interpolated during processing for use at individual transients during the flight.

Cleaning and Stacking

Routines to suppress sferic noise, powerline noise, VLF noise and coil motion noise (collectively termed “cleaning”) and to stack the data are applied to the survey line data. Output from the stacking filter is drawn at 0.2 second intervals. A cosine shaped filter making use of 152 transients (approximately 3 sec) is used in the stacking process.

Deconvolution

The survey height stacked data are deconvolved in the frequency domain using the interpolated high altitude reference waveform, to yield a quantity that is independent of system characteristics. This procedure accounts for slow variations in the transmitted current waveform’s amplitude and shape during the flight. It also accounts for the effect of eddy currents induced in the transmitter loop and airframe. The output of the deconvolved data is the summed effect of the direct coupling between the transmitter loop and receiver coils (primary field) and the coupling between currents induced in the ground and the receiver (secondary field).

Primary Field Estimation

Since the receiver’s orientation and position (relative to the transmitter) is not precisely known, the primary field cannot simply be theoretically computed and subtracted from the deconvolved data to yield the desired pure ground response. The primary field is instead estimated using knowledge of the asymptotic behaviour at the low frequency in-phase component of the deconvolved spectrum. The estimation of the primary field requires some assumptions to be made regarding the conductivity structure of the ground at depth. Once estimated the primary field is subtracted from the deconvolved data to yield the estimated pure ground response.

Transmitter-Receiver Separation Estimation

Once the primary field and coupling terms are estimated it is then possible to estimate the position of the receiver coils relative to the transmitter loop via basic dipole theory. Equations (1) and (2) define the coupling terms for an infinitesimal vertical magnetic dipole transmitter and an ideal receiver located at co-ordinates (x,z) with respect to the transmitter.  The horizontal (or X) component coupling is defined by,

gx=3xzx2+z25/2,	(1)

and for the vertical (or Z) component data;

gz=2z2−x2x2+z25/2.	(2)

The above equations are inverted to solve for the coil set position defined by the co-ordinates (x,z) as follows.  From equations (1) and (2),

gzgx=r=(2z2−x2)3xz	(3)

Therefore,

x2+3rxz−2z2=0	(4)

Therefore,

x=−(3rz±9r2z2+8z2)2=z(−3r±9r2+8)2=zr1	(5)

Substituting back into the expression for gx, we get

gx=3r1z3(r12+1)5/2	(6)

and

z=3r1gx(r12+1)5/213, and x=r13r1gx(r12+1)5/213	(7)

where

r1=−3(gzgx)+9(gzgx)2+82	(8)

The +/- solutions collapse to a single solution due to a basic knowledge that the bird is always going to be below and behind the transmitter.  Therefore equations (7) and (8) provide the necessary calculation to convert gx and gz values to x and z values which define the position of the receiver with respect to the transmitter.

An estimate of transmitter-receiver separation is made for every 0.2 second sample drawn from the stacking filter. Along with other system geometry variables (either measured or assumed) the survey wide averages of the system geometry is shown in the table following.

| Geometry Variable |  | VH-TEM |
| --- | --- | --- |
| Transmitter loop pitch | measured | 1.03 deg |
| Transmitter loop roll | measured | 0.59 deg |
| Transmitter loop yaw | assumed | 0.00 deg |
| Transmitter loop terrain clearance | measured | 124.3 m |
| Transmitter-receiver in-line horizontal separation | estimated | -121.0 m |
| Transmitter-receiver vertical separation | estimated | -33.9 m |
| Transmitter-receiver transverse horizontal separation | assumed | 0.00 m |
| Receiver pitch | assumed | 0.00 deg |
| Receiver roll | assumed | 0.00 deg |
| Receiver yaw | assumed | 0.00 deg |

Transformation to B-field Response

The pure ground response data are transformed from dB/dt to B-field responses equivalent to that which would be observed for a perfect 100% duty cycle square wave waveform with a 1 A peak to peak step.

Windowing

Finally, the evenly spaced samples are binned into a number of windows.

Table of TEMPEST window information for 25Hz base frequency

| Window # | Start sample | End sample | No of samples | start time (s) | End time (s) | centre time (s) | centre time (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 2 | 0.000007 | 0.000020 | 0.000013 | 0.013 |
| 2 | 3 | 4 | 2 | 0.000033 | 0.000047 | 0.000040 | 0.040 |
| 3 | 5 | 6 | 2 | 0.000060 | 0.000073 | 0.000067 | 0.067 |
| 4 | 7 | 10 | 4 | 0.000087 | 0.000127 | 0.000107 | 0.107 |
| 5 | 11 | 16 | 6 | 0.000140 | 0.000207 | 0.000173 | 0.173 |
| 6 | 17 | 26 | 10 | 0.000220 | 0.000340 | 0.000280 | 0.280 |
| 7 | 27 | 42 | 16 | 0.000353 | 0.000553 | 0.000453 | 0.453 |
| 8 | 43 | 66 | 24 | 0.000567 | 0.000873 | 0.000720 | 0.720 |
| 9 | 67 | 102 | 36 | 0.000887 | 0.001353 | 0.001120 | 1.120 |
| 10 | 103 | 158 | 56 | 0.001367 | 0.002100 | 0.001733 | 1.733 |
| 11 | 159 | 246 | 88 | 0.002113 | 0.003273 | 0.002693 | 2.693 |
| 12 | 247 | 384 | 138 | 0.003287 | 0.005113 | 0.004200 | 4.200 |
| 13 | 385 | 600 | 216 | 0.005127 | 0.007993 | 0.006560 | 6.560 |
| 14 | 601 | 930 | 330 | 0.008007 | 0.012393 | 0.010200 | 10.200 |
| 15 | 931 | 1500 | 570 | 0.012407 | 0.019993 | 0.016200 | 16.200 |

Geometry Corrections to EM Data

The final EM dataset includes both “non-geometry corrected” and geometry-corrected” located EM data. The non-geometry corrected EM amplitudes reflect, not only the variations in ground conductivity, but the variations in geometry of the various parts of the EM measurements (i.e. transmitter loop pitch, transmitter loop roll, transmitter loop terrain clearance, transmitter loop to receiver coil horizontal longitudinal separation, transmitter loop to receiver coil horizontal transverse separation, and transmitter loop to receiver coil vertical separation) during the survey. For example, the largest influence on the early time EM amplitude is the terrain clearance of the transmitter loop. The larger the terrain clearance, the smaller the amplitude. Later window times (larger window number) show diminished variations due to terrain clearance.

Geometry-corrected located data are produced for optimum presentation of the EM amplitude data in image format (e.g. window amplitude images, principal component analysis images derived from the window amplitudes (Green,1998b)). Between non-geometry and geometry corrected states, the ground response data undergo an approximate correction to produce data that would be measured if the system had always maintained a nominated standard (constant) geometry. A dipole-image method (Green, 1998a) is used to adjust the data to the response that would be expected at a standard terrain clearance (120m), standard transmitter loop pitch and roll (zero degrees), and a standard transmitter loop to receiver coil geometry (120m behind and 35 below the aircraft). These geometry variables have been set to their respective standard values in the geometry corrected located data. Zero parallax is applied to the transmitter loop pitch, roll, terrain clearance, X component EM and Z component EM data prior to geometry correction. Note that the final delivered non-geometry corrected EM data has had the parallax values, defined in section 5.2.7.1, applied.

Over extremely conductive ground (e.g. > 100 S conductance), the estimates for transmitter loop to receiver coil separation determined from the primary field coupling factors may be in error at the metre scale due to uncertainty in the estimation of the primary field. This will influence the accuracy of very early time window amplitude information in the geometry-corrected located data. Receiver coil pitch has a significant effect on early time Z component response and late time X component response (Green and Lin, 1996). Receiver coil roll impacts early time Z component response.

Values used to standardise transmitter height, pitch and roll and transmitter-receiver geometry

| Geometry Variable | Standard Value |
| --- | --- |
| Transmitter loop pitch | 0.0 deg |
| Transmitter loop roll | 0.0 deg |
| Transmitter loop yaw | 0.0 deg |
| Transmitter loop terrain clearance | 120.0 m |
| Transmitter-receiver in-line horizontal separation | -120.0 m |
| Transmitter-receiver vertical separation | -35.0 m |
| Transmitter-receiver transverse horizontal separation | 0.0 m |
| Receiver pitch | 0.0 deg |
| Receiver roll | 0.0 deg |
| Receiver yaw | 0.0 deg |

Levelling

Once the full dataset had been corrected to the same standard geometry, the following levelling procedure was applied:

- small amplitude DC shifts to the window data to remove base-level shifts related to slight imperfections in the deconvolution stage of the EM data processing. This type of levelling is termed ‘noise-levelling’, and it is designed to improve the presentation and remove the small amplitude ‘block’ shifts in the later EM windows that may occur from flight to flight.

- limited range micro-levelling was applied to all windows for presentation purposes and to ensure the input data for CDI processing was free of striping.

### Conductivity Depth Images (CDI)

CDI conductivity sections for TEMPEST data were calculated using EMFlow and then modified to reflect the finite depth of investigation using an in-house routine, Sigtime.

The Sigtime routine removes many of the spurious conductive features that appear at depth as a result of fitting long time constant exponential decays to very small amplitude features in the late times. For each observation, the time when the response falls below a signal threshold amplitude is determined. This time is transformed into a diffusion depth with reference to the conductivity values determined for that observation. Anomalous conductivity values below this depth are replaced by background values or set to undefined, reflecting the uncertainty in their origin. The settings and options applied are indicated in the appropriate header files for Sigtime output. This procedure is different to that which would be obtained by filtering conductivity values using either a constant time or constant depth across the entire line.

The “final” X and Z EM data were simultaneously input into version 5.10 of EMFlow to calculate Conductivity Depth Images (CDI). Conductivity values were calculated at each point then run through Sigtime.

EMFlow was developed within the CRC-AMET through AMIRA research projects (Macnae et al, 1998, Stolz and Macnae, 1998). The software has been commercialised by Encom Technology Pty Ltd. Examples of TEMPEST conductivity data can be seen in Lane et al. (2000), Lane et al. (1999), and Lane and Pracillio (2000).

Conductivity values were calculated to a depth of 200m below surface at each point, using a depth increment of 5m and a conductivity range of 1-3000mS/m.

#### Factors and Corrections

Geometric Factor

The geometric factor gives the ratio of the strength of the primary field coupling between the transmitter loop and the receiver coil at each observation relative to the coupling observed at high altitude during acquisition of reference waveform data. Variations in this factor indicate a change in the attitude and/or relative separation of the transmitter loop and the receiver coil.

Transmitter-Receiver Geometry

Transmitter-to-receiver geometry values for each observation are derived from the high altitude reference waveforms and knowledge of the system characteristics. The exact derivation of the primary field values and transmitter-to-receiver separation are described in section 5.2.5 above. The transmitter-to-receiver geometry values are available in the located data (see section 5.2.5 for “standardised” values).

GPS Antenna, Laser Altimeter and Transmitter Loop Offset Corrections

The transmitter loop was mounted 0.1m above the GPS antenna. And the GPS antenna is 2.3m above the belly of the aircraft. The laser altimeter sensor is mounted in the belly. Therefore a total of 2.4m was added to the laser altimeter data to determine the transmitter loop height above the ground.

Transmitter Loop Pitch and Roll Correction

Measured vertical gyro aircraft pitch and roll attitude measurements are converted to transmitter loop pitch and roll by adding -0.9 degrees for pitch and 0.1 degrees for roll. Nose up is positive for pitch, and left wing up is positive for roll.

#### Primary Sources of EM Noise

A number of “monitor” values are calculated during processing to assist with interpretation. They generally represent quantities that have been removed as far as is practical from the data, but may still be present in trace amounts. These are more significant for interpretation of discrete conductors than for general mapping applications.

Sferic Monitor

Sferics are the electromagnetic signals associated with lightning activity. These signals travel large distances around the Earth. Background levels of sferics are present at all times from lightning activity in tropical areas of the world (eg tropical parts of Asia, South America and Africa). Additional higher amplitude signals are produced by "local" lightning activity (ie at distances of kilometres to hundreds of kilometres).

The sferic monitor is the sum of the absolute differences brought about by the sferic filter operations, summed over 0.2 second intervals, normalised by the receiver effective area. It is given in units of uV/sq.m/0.2s. Many sferics have a characteristic form that is well illustrated by figure 2 in Garner and Thiel (2000), shown below. The high frequency, initial part of a sferic event can be detected and filtered more easily than the later, low frequency portion. The sferic monitor indicates where at least the high frequency portion of a sferic has been successfully removed, but it is quite possible that lower frequency elements of the sferic event may have eluded detection, passing through to the window amplitude data. Thus, discrete anomalies coincident with sferic activity as indicated by the sferic monitor should be down-weighted relative to features clear of any sign of sferic activity.

An electric field time-series sampled at 48 kilo samples per second using MIMDAS. The top panel exhibits the entire event, while the lower panel depicts a close up view of an individual sferic from that event. The sample rate and resolution in time are denoted by fs and tres, respectively. (Garner & Thiel, 2000.)

Low Frequency Monitor

The Low Frequency Monitor (LFM) makes use of amplitudes at frequencies below the base frequency which are present in the streamed data to estimate the amplitude of coil motion noise at the base frequency in log10(pV/sqrt(Hz)/sq.m). This noise is primarily induced by the coil’s motion through the earth’s magnetic field – a change in coupling between the receiver coil and the ambient magnetic field will induce a voltage in the receiver coil. This noise is referred to as coil motion or Earth field noise. Receiver coils in the towed bird are suspended in a fashion that attempts to keep this noise below the noise floor at frequencies equal to and above the base frequency of the system. Severe turbulence, however, can result in ‘coil knock events’ that introduce noise into the processed data. Note that the LFM will also respond to sferic events with an appreciable low frequency (sub-base frequency) component. This situation can be inferred when both the LFM and sferic monitors show a discrete kick.

The coil motion noise below the base frequency is rejected through the use of tapered stacking, but the coil motion noise at the base frequency itself is not easily removed.

Powerline Monitor

The powerline monitor gives the amplitude of the received signal at the powerline frequency (50 or 60 Hz) in log10(pV/sqrt(Hz)/sq.m). Careful selection of the base frequency (such that the powerline frequency is an even harmonic of the base frequency) and tapered stacking combine to strongly attenuate powerline signals. When passing directly over a powerline, the rapid lateral variations in the strength and direction of the magnetic fields associated with the powerline can result in imperfect cancellation of the powerline response during stacking. Some powerline-related interference can manifest itself in a form that is similar to the response of a discrete conductor. The exact form of the monitor profile over a powerline depends on the line direction, powerline direction, powerline current, and receiver component, but the monitor will show a general increase in amplitude approaching the powerline.

Grids (or images) of the powerline monitor reveal the location of the transmission lines. Note that the X component (horizontal receiver coil axis parallel with the flight line direction) does not register any response from powerlines parallel to the flight line direction since the magnetic fields associated with powerlines only vary in a direction perpendicular to the powerline. Note also that the Z component (vertical receiver coil axis) shows a narrow low directly over the powerline where the magnetic fields are purely horizontal.

Very Low Frequency Monitors

Wide area VLF communication signals in the 15 to 25 kHz frequency band are monitored by the TEMPEST system. In the Australian region, signals at 18.2 kHz, 19.8 kHz, 21.4 kHz and 22.2 kHz are monitored as the amplitude of the received signal at these frequencies in log10(pV/sqrt(Hz)/sq.m). The strongest signal comes from North West Cape (19.8 kHz). The signal at 18.2 kHz is often observed to pulse in a regular sequence. These strong narrow band signals have some impact on the high frequency response of the system, but they are strongly attenuated by selection of the base frequency and tapered stacking. The VLF transmissions are strongest in amplitude, in the horizontal direction at right angles to the direction to the VLF transmitter. This directional dependence enables the VLF monitors to be used to indicate the receiver coil attitude.

#### Other Sources of EM Noise

Man-made periodic discharges

If an image of the Z component sferic monitor shows the presence of spatially coherent events, then pulsed cultural interference would be strongly suspected. Since sferic signals are much stronger in the horizontal plane than in the vertical plane, few sferics of significant amplitude are recorded in Z component data. In contrast, evidence of cultural interference is generally swamped by true sferics in X component sferic monitor images.

Electric fences are the most common source of pulsed cultural interference. Periodic discharges (eg every second or so) into a large wire loop (fence) produce very large spikes in raw data. These are attenuated to a large degree by the sferic filter, but a residual artefact can still be present in the processed data.

Grounded metal objects

Grounded extensive metal objects such as pipelines and rail lines can qualify as conductors and may produce a response that is visible in processed data. Grounded metal objects produce a response similar to shallow, highly conductive, steeply dipping conductors. These objects can sometimes be identified from good quality topographic maps, from aerial photographs, by viewing the tracking video, from their unusual spatial distribution (ie often a series of linear segments) and in some circumstances from their effect on the powerline monitor. A powerline running close to a long metal object will induce a 50 Hz response in the object.

### System Specifications for Modelling TEMPEST Data

Differences between the specifications for the acquisition system, and those of the virtual system for which processed results are given, must be kept in mind when forward modelling, transforming or inverting TEMPEST data.

Acquisition is carried out with a 50% duty cycle square transmitter current waveform and dB/dt sensors.

During processing, TEMPEST EM data are transformed to the response that would be obtained with a B-field sensor for a 100% duty cycle square waveform at the base frequency, involving a 1A change in current (from -0.5A to +0.5A to -0.5A) in a 1sq.m transmitter. Data are given in units of femtoTesla (fT = 10-15 Tesla). It is this configuration, rather than the actual acquisition configuration, which must be specified when modelling fully pre-processed TEMPEST data.

Window timing information is given above (see section 5.2.5).

The geometry-corrected EM data have been standardised through an approximate transformation to a standard transmitter loop terrain clearance, transmitter loop pitch and roll of zero degrees, and a fixed transmitter loop to receiver coil geometry (roughly equal to the average estimated geometry values). Transmitter loop pitch, transmitter loop roll and transmitter loop terrain clearance values for each observation have been modified to reflect the standard values. Hence, the standardised geometry values should be used if modelling with the geometry corrected X and Z-component amplitude data (see table in section 5.2.5).

#### Parallax

The located data files utilise the following parallax values :-

- magnetics = -1.0 fiducials (5 observations from the zero parallax position),

- radar altimeter = 0 fiducials

- laser altimeter = 0 fiducials

- GPS co-ordinates = 0 fiducials

- EM X-component = -1.6 fiducials (8 observation from the zero parallax position),

- EM Z-component = -0.2 fiducials (1 observations from the zero parallax position),

For the Tempest Airborne EM system, due to the asymmetry in the transmitter loop-receiver coil geometry with respect to flight direction, there is no single EM parallax value which will align the peak response for all conductivity distributions for lines flown in opposite directions.

The choice of EM parallax value depends on the intended usage, but with the predominance of broad, shallowly dipping conductors, and the client’s desire to grid the data, parallax has been applied so that data are optimised for gridding. The ‘optimum’ depends on the conductor depth, the acquisition geometry and the delay time, and hence, the selected value will be a compromise.

(NB negative parallax values are defined in this case as shifting the indicated quantity forward along line to larger fiducial values. Location information remains in the zero parallax state)

### CDI Depth Slicing

Following calculation of CDI data as described in section 5.2.6, conductivity depth slices were derived by averaging conductivity data over the following depth intervals:

0 –  5m

5 – 10m

10 – 15m

15 – 20m

20 – 30m

30 – 40m

40 – 60m

60 – 100m

100 – 150m

150 – 200m

The conductivity depth slice data were gridded for each of the regional and infill areas using a grid cell size of 1/5th of the line spacing. The areas were gridded using a bi-directional spline interpolation (with no anti-aliasing) algorithm with a square cell size.

Finally, a 5-cell median filter and a 5-cell mean filter were applied to the conductivity depth slice grids to improve their appearance and smooth the blocky nature of the raw grids, which is a result of using 20 discretely defined conductivities in the CDI calculation.

### Delivered Products

Appendix VI contains a complete list of all data supplied digitally.

Digital located data in ASCII format was produced containing the non-geometry corrected and geometry corrected X and Z EM data as well as magnetics, digital elevation and derived conductivity data. The header file can be found in Appendix III.

Grids (in ER Mapper format) of selected conductivity slices, total magnetic field and digital elevation were also produced. Acquisition and processing report in hardcopy and digital format.

# REFERENCES

Garner, S.J., Thiel, D.V., 2000, Broadband (ULF-VLF) surface impedance measurements using MIMDAS: Exploration Geophysics, 31, 173-178.

Green, A., 1998. Altitude correction of time domain AEM data for image display and geological mapping, using the Apparent Dipole Depth (ADD) method. Exploration Geophysics. 29, 87-91.

Green, A., 1998. The use of multivariate statistical techniques for the analysis and display of AEM data. Exploration Geophysics. 29, 77-82.

Green, A., Lin, Z., 1996. Effect of uncertain or changing system geometry on airborne transient electromagnetic data: CSIRO Exploration. and Mining Research News No. 6, August 1996, 9-11, CSIRO Division of Exploration and Mining.

Lane, R., 2000, Conductive unit parameters : summarising complex conductivity distributions: Paper accepted for presentation at the SEG Annual Meeting, August 2000.

Lane, R., Green, A., Golding, C., Owers, M., Pik, P., Plunkett, C., Sattel, D., Thorn, B., 2000, An example of 3D conductivity mapping using the TEMPEST airborne electromagnetic system: Exploration Geophysics, 31, 162-172.

Lane, R., Leeming, P., Owers, M., Triggs, D., 1999, Undercover assignment for TEMPEST: Preview, Issue 82, 17-21.

Lane, R., Pracilio, G., 2000: Visualisation of sub-surface conductivity derived from airborne EM, SAGEEP 2000, 101-111.

Macnae, J.C., King, A., Stolz, N., Osmakoff, A. and Blaha, A., 1998, Fast AEM data processing and inversion: Exploration Geophysics, 29, 163-169.

Stolz, E., and Macnae, J., 1998, Evaluating EM waveforms by singular-value decomposition of exponential basis functions: Geophysics, 63, 64–74.

APPENDIX I – Weekly Acquisition Reports VH-TEM

APPENDIX II – Flight Summary (Survey Line Listing)

Notes on line numbers included below:

505FFFFPP – pre-flight barometer calibration for flight FFFF and part PP

605FFFFPP -- post=-flight barometer calibration for flight FFFF and part PP

902FFFFPP – pre-flight ‘zero’ calibration line for flight FFFF and part PP

905FFFFPP – post-flight ‘zero’ calibration line for flight FFFF and part PP

900FFFFPP-906FFFFPP – other pre/post flight EM calibrations (See Chapter 4)

810FFFFPP – 814FFFFPP – magnetic compensation lines

910FFFFPP-911FFFFPP – repeat line from flight FFFF and part PP

Casa 212-200 – VH-TEM

| Flight | Date (yyyymmdd) | Line | Direction | Start Fid | End Fid | Start Time (UTC) | End Time (UTC) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20081019 | 505000101 |  | 456 | 578 | 21:46:11 | 21:48:13 |
| 1 | 20081019 | 900000101 |  | 1429 | 1550 | 22:02:24 | 22:04:25 |
| 1 | 20081019 | 901000101 |  | 1578 | 1702 | 22:04:53 | 22:06:57 |
| 1 | 20081019 | 902000101 |  | 1719 | 1832 | 22:07:14 | 22:09:07 |
| 1 | 20081019 | 903000101 |  | 1848 | 1948 | 22:09:23 | 22:11:03 |
| 1 | 20081019 | 910000101 |  | 3381 | 3533 | 22:34:56 | 22:37:28 |
| 1 | 20081019 | 5200101 | W | 4280 | 4418 | 22:49:55 | 22:52:13 |
| 1 | 20081019 | 5200201 | E | 4504 | 4735 | 22:53:39 | 22:57:30 |
| 1 | 20081019 | 5200301 | W | 4908 | 5202 | 23:00:23 | 23:05:17 |
| 1 | 20081019 | 5200401 | E | 5313 | 5655 | 23:07:08 | 23:12:50 |
| 1 | 20081019 | 5200501 | W | 5785 | 6109 | 23:15:00 | 23:20:24 |
| 1 | 20081019 | 5200601 | E | 6230 | 6589 | 23:22:25 | 23:28:24 |
| 1 | 20081019 | 5200701 | W | 6763 | 7166 | 23:31:18 | 23:38:01 |
| 1 | 20081019 | 5200801 | E | 7280 | 7708 | 23:39:55 | 23:47:03 |
| 1 | 20081019 | 904000101 |  | 9075 | 9200 | 00:09:50 | 00:11:55 |
| 1 | 20081019 | 905000101 |  | 9220 | 9347 | 00:12:15 | 00:14:22 |
| 1 | 20081019 | 906000101 |  | 9376 | 9497 | 00:14:51 | 00:16:52 |
| 1 | 20081019 | 810000101 |  | 10199 | 10952 | 00:28:34 | 00:41:07 |
| 1 | 20081019 | 811000101 |  | 11001 | 11144 | 00:41:56 | 00:44:19 |
| 1 | 20081019 | 812000101 |  | 11185 | 11325 | 00:45:00 | 00:47:20 |
| 1 | 20081019 | 813000101 |  | 11367 | 11498 | 00:48:02 | 00:50:13 |
| 1 | 20081019 | 814000101 |  | 11552 | 11682 | 00:51:07 | 00:53:17 |
| 1 | 20081019 | 605000101 |  | 12855 | 12974 | 01:12:50 | 01:14:49 |
| 2 | 20081020 | 505000201 |  | 108 | 231 | 20:54:34 | 20:56:37 |
| 2 | 20081020 | 900000201 |  | 1358 | 1480 | 21:15:24 | 21:17:26 |
| 2 | 20081020 | 901000201 |  | 1520 | 1642 | 21:18:06 | 21:20:08 |
| 2 | 20081020 | 902000201 |  | 1670 | 1801 | 21:20:36 | 21:22:47 |
| 2 | 20081020 | 903000201 |  | 1986 | 2058 | 21:25:52 | 21:27:04 |
| 2 | 20081020 | 910000201 |  | 3376 | 3529 | 21:49:02 | 21:51:35 |
| 2 | 20081020 | 5200901 | W | 3996 | 4434 | 21:59:22 | 22:06:40 |
| 2 | 20081020 | 5201001 | E | 4546 | 5020 | 22:08:32 | 22:16:26 |
| 2 | 20081020 | 6100301 | E | 5828 | 6320 | 22:29:54 | 22:38:06 |
| 2 | 20081020 | 6100401 | W | 5258 | 5668 | 22:20:24 | 22:27:14 |
| 2 | 20081020 | 6100601 | W | 6423 | 6840 | 22:39:49 | 22:46:46 |
| 2 | 20081020 | 6100701 | E | 6990 | 7450 | 22:49:16 | 22:56:56 |
| 2 | 20081020 | 904000201 |  | 9149 | 9273 | 23:25:15 | 23:27:19 |
| 2 | 20081020 | 905000201 |  | 9297 | 9420 | 23:27:43 | 23:29:46 |
| 2 | 20081020 | 906000201 |  | 9442 | 9565 | 23:30:08 | 23:32:11 |
| 2 | 20081020 | 605000201 |  | 10627 | 10749 | 23:49:53 | 23:51:55 |
| 3 | 20081021 | 505000301 |  | 177 | 321 | 20:44:50 | 20:47:14 |
| 3 | 20081021 | 900000301 |  | 1060 | 1183 | 20:59:33 | 21:01:36 |
| 3 | 20081021 | 901000301 |  | 1221 | 1346 | 21:02:14 | 21:04:19 |
| 3 | 20081021 | 902000301 |  | 1372 | 1495 | 21:04:45 | 21:06:48 |
| 3 | 20081021 | 903000301 |  | 1510 | 1557 | 21:07:03 | 21:07:50 |
| 3 | 20081021 | 910000301 |  | 2964 | 3100 | 21:31:17 | 21:33:33 |
| 3 | 20081021 | 6101001 | W | 3364 | 3826 | 21:37:57 | 21:45:39 |
| 3 | 20081021 | 6101301 | E | 4395 | 4956 | 21:55:08 | 22:04:29 |
| 3 | 20081021 | 6101201 | W | 5120 | 5588 | 22:07:13 | 22:15:01 |
| 3 | 20081021 | 6101501 | E | 5710 | 6259 | 22:17:03 | 22:26:12 |
| 3 | 20081021 | 6101601 | W | 6417 | 6889 | 22:28:50 | 22:36:42 |
| 3 | 20081021 | 904000301 |  | 8095 | 8217 | 22:56:48 | 22:58:50 |
| 3 | 20081021 | 905000301 |  | 8243 | 8366 | 22:59:16 | 23:01:19 |
| 3 | 20081021 | 906000301 |  | 8390 | 8512 | 23:01:43 | 23:03:45 |
| 3 | 20081021 | 605000301 |  | 9567 | 9693 | 23:21:20 | 23:23:26 |
| 4 | 20081023 | 505000401 |  | 104 | 259 | 20:37:08 | 20:39:43 |
| 4 | 20081023 | 900000401 |  | 1019 | 1142 | 20:52:23 | 20:54:26 |
| 4 | 20081023 | 901000401 |  | 1164 | 1290 | 20:54:48 | 20:56:54 |
| 4 | 20081023 | 902000401 |  | 1309 | 1432 | 20:57:13 | 20:59:16 |
| 4 | 20081023 | 903000401 |  | 1447 | 1503 | 20:59:31 | 21:00:27 |
| 4 | 20081023 | 910000401 |  | 2621 | 2746 | 21:19:05 | 21:21:10 |
| 4 | 20081023 | 6101801 | W | 3010 | 3508 | 21:25:34 | 21:33:52 |
| 4 | 20081023 | 6101901 | E | 4307 | 4823 | 21:47:11 | 21:55:47 |
| 4 | 20081023 | 6102201 | W | 5055 | 5570 | 21:59:39 | 22:08:14 |
| 4 | 20081023 | 6102101 | E | 5662 | 6174 | 22:09:46 | 22:18:18 |
| 4 | 20081023 | 6102401 | W | 6293 | 6810 | 22:20:17 | 22:28:54 |
| 4 | 20081023 | 6102501 | E | 6893 | 7400 | 22:30:17 | 22:38:44 |
| 4 | 20081023 | 6102801 | W | 7520 | 8037 | 22:40:44 | 22:49:21 |
| 4 | 20081023 | 6102701 | E | 8112 | 8620 | 22:50:36 | 22:59:04 |
| 4 | 20081023 | 6103001 | W | 8748 | 9259 | 23:01:12 | 23:09:43 |
| 4 | 20081023 | 6103101 | E | 9346 | 9858 | 23:11:10 | 23:19:42 |
| 4 | 20081023 | 6103401 | W | 9991 | 10363 | 23:21:55 | 23:28:07 |
| 4 | 20081023 | 6103301 | E | 10448 | 10825 | 23:29:32 | 23:35:49 |
| 4 | 20081023 | 904000401 |  | 11211 | 11344 | 23:42:15 | 23:44:28 |
| 4 | 20081023 | 905000401 |  | 11358 | 11483 | 23:44:42 | 23:46:47 |
| 4 | 20081023 | 906000401 |  | 11515 | 11638 | 23:47:19 | 23:49:22 |
| 4 | 20081023 | 605000401 |  | 13086 | 13209 | 00:13:30 | 00:15:33 |
| 5 | 20081024 | 900000501 |  | 111 | 235 | 21:18:38 | 21:20:42 |
| 5 | 20081024 | 901000501 |  | 253 | 380 | 21:21:00 | 21:23:07 |
| 5 | 20081024 | 902000501 |  | 406 | 536 | 21:23:33 | 21:25:43 |
| 5 | 20081024 | 903000501 |  | 541 | 591 | 21:25:48 | 21:26:38 |
| 5 | 20081024 | 5000101 | W | 1972 | 2412 | 21:49:39 | 21:56:59 |
| 5 | 20081024 | 5000401 | E | 6687 | 7574 | 23:08:14 | 23:23:01 |
| 5 | 20081024 | 5000601 | W | 5723 | 6577 | 22:52:10 | 23:06:24 |
| 5 | 20081024 | 5000801 | E | 8766 | 9665 | 23:42:53 | 23:57:52 |
| 5 | 20081024 | 5100401 | E | 3199 | 3607 | 22:10:06 | 22:16:54 |
| 5 | 20081024 | 5100601 | W | 7679 | 8519 | 23:24:46 | 23:38:46 |
| 5 | 20081024 | 5101001 | W | 3825 | 4686 | 22:20:32 | 22:34:53 |
| 5 | 20081024 | 5101201 | E | 4734 | 5610 | 22:35:41 | 22:50:17 |
| 5 | 20081024 | 6100901 | E | 2519 | 2919 | 21:58:46 | 22:05:26 |
| 5 | 20081024 | 904000501 |  | 10255 | 10387 | 00:07:42 | 00:09:54 |
| 5 | 20081024 | 905000501 |  | 10395 | 10519 | 00:10:02 | 00:12:06 |
| 5 | 20081024 | 906000501 |  | 10588 | 10713 | 00:13:15 | 00:15:20 |
| 5 | 20081024 | 605000501 |  | 11937 | 12059 | 00:35:44 | 00:37:46 |
| 6 | 20081025 | 5101601 | E | 3643 | 4510 | 21:41:00 | 21:55:27 |
| 6 | 20081025 | 5101801 | W | 2698 | 3562 | 21:25:15 | 21:39:39 |
| 6 | 20081025 | 6103302 | E | 6602 | 6981 | 22:30:19 | 22:36:38 |
| 6 | 20081025 | 6103601 | W | 6159 | 6537 | 22:22:56 | 22:29:14 |
| 6 | 20081025 | 6103701 | W | 7574 | 7951 | 22:46:31 | 22:52:48 |
| 6 | 20081025 | 6103901 | E | 8475 | 8856 | 23:01:32 | 23:07:53 |
| 6 | 20081025 | 6104001 | W | 7101 | 7480 | 22:38:38 | 22:44:57 |
| 6 | 20081025 | 6104201 | W | 8045 | 8385 | 22:54:22 | 23:00:02 |
| 6 | 20081025 | 6104301 | E | 9371 | 9707 | 23:16:28 | 23:22:04 |
| 6 | 20081025 | 6104501 | E | 10188 | 10526 | 23:30:05 | 23:35:43 |
| 6 | 20081025 | 6104601 | W | 8965 | 9300 | 23:09:42 | 23:15:17 |
| 6 | 20081025 | 6104801 | W | 9783 | 9997 | 23:23:20 | 23:26:54 |
| 6 | 20081025 | 6104901 | E | 10939 | 11154 | 23:42:36 | 23:46:11 |
| 6 | 20081025 | 6105001 | W | 10616 | 10828 | 23:37:13 | 23:40:45 |
| 6 | 20081025 | 6105201 | W | 11290 | 11503 | 23:48:27 | 23:52:00 |
| 6 | 20081025 | 6202701 | W | 4650 | 4822 | 21:57:47 | 22:00:39 |
| 6 | 20081025 | 6300101 | E | 5730 | 5860 | 22:15:47 | 22:17:57 |
| 6 | 20081025 | 6300301 | E | 5321 | 5449 | 22:08:58 | 22:11:06 |
| 6 | 20081025 | 6300401 | W | 5065 | 5192 | 22:04:42 | 22:06:49 |
| 7 | 20081026 | 505000701 |  | 123 | 265 | 20:30:23 | 20:32:45 |
| 7 | 20081026 | 900000701 |  | 1049 | 1174 | 20:45:49 | 20:47:54 |
| 7 | 20081026 | 901000701 |  | 1191 | 1314 | 20:48:11 | 20:50:14 |
| 7 | 20081026 | 902000701 |  | 1321 | 1433 | 20:50:21 | 20:52:13 |
| 7 | 20081026 | 903000701 |  | 1468 | 1523 | 20:52:48 | 20:53:43 |
| 7 | 20081026 | 5000201 | W | 9036 | 9388 | 22:58:56 | 23:04:48 |
| 7 | 20081026 | 5100301 | E | 8034 | 8467 | 22:42:14 | 22:49:27 |
| 7 | 20081026 | 6103702 | E | 4405 | 4780 | 21:41:45 | 21:48:00 |
| 7 | 20081026 | 6105101 | E | 3509 | 3719 | 21:26:49 | 21:30:19 |
| 7 | 20081026 | 6105301 | E | 2901 | 3112 | 21:16:41 | 21:20:12 |
| 7 | 20081026 | 6105401 | W | 2504 | 2727 | 21:10:04 | 21:13:47 |
| 7 | 20081026 | 6200201 | E | 8733 | 8912 | 22:53:53 | 22:56:52 |
| 7 | 20081026 | 6200301 | W | 9991 | 10164 | 23:14:51 | 23:17:44 |
| 7 | 20081026 | 6200601 | E | 9676 | 9854 | 23:09:36 | 23:12:34 |
| 7 | 20081026 | 6400101 | E | 6864 | 6993 | 22:22:44 | 22:24:53 |
| 7 | 20081026 | 6400201 | W | 7120 | 7252 | 22:27:00 | 22:29:12 |
| 7 | 20081026 | 6400301 | E | 6400 | 6528 | 22:15:00 | 22:17:08 |
| 7 | 20081026 | 6400401 | W | 6620 | 6751 | 22:18:40 | 22:20:51 |
| 7 | 20081026 | 6400601 | W | 6176 | 6307 | 22:11:16 | 22:13:27 |
| 7 | 20081026 | 6400701 | E | 5935 | 6063 | 22:07:15 | 22:09:23 |
| 7 | 20081026 | 6400801 | W | 5702 | 5836 | 22:03:22 | 22:05:36 |
| 7 | 20081026 | 6400901 | E | 5458 | 5587 | 21:59:18 | 22:01:27 |
| 7 | 20081026 | 6401101 | E | 4936 | 5064 | 21:50:36 | 21:52:44 |
| 7 | 20081026 | 6401201 | W | 5201 | 5335 | 21:55:01 | 21:57:15 |
| 7 | 20081026 | 904000701 |  | 10553 | 10557 | 23:24:13 | 23:24:17 |
| 7 | 20081026 | 904000702 |  | 10566 | 10709 | 23:24:26 | 23:26:49 |
| 7 | 20081026 | 905000701 |  | 10724 | 10850 | 23:27:04 | 23:29:10 |
| 7 | 20081026 | 906000701 |  | 10907 | 11037 | 23:30:07 | 23:32:17 |
| 7 | 20081026 | 605000701 |  | 12559 | 12680 | 23:57:39 | 23:59:40 |
| 8 | 20081027 | 505000801 |  | 123 | 247 | 20:32:42 | 20:34:46 |
| 8 | 20081027 | 900000801 |  | 1525 | 1649 | 20:56:04 | 20:58:08 |
| 8 | 20081027 | 901000801 |  | 1669 | 1793 | 20:58:28 | 21:00:32 |
| 8 | 20081027 | 902000801 |  | 1805 | 1929 | 21:00:44 | 21:02:48 |
| 8 | 20081027 | 903000801 |  | 1936 | 2005 | 21:02:55 | 21:04:04 |
| 8 | 20081027 | 6200501 | W | 3539 | 3705 | 21:29:38 | 21:32:24 |
| 8 | 20081027 | 6200801 | E | 3821 | 4006 | 21:34:20 | 21:37:25 |
| 8 | 20081027 | 6200901 | W | 4116 | 4281 | 21:39:15 | 21:42:00 |
| 8 | 20081027 | 6201101 | W | 4666 | 4831 | 21:48:25 | 21:51:10 |
| 8 | 20081027 | 6201201 | E | 4383 | 4570 | 21:43:42 | 21:46:49 |
| 8 | 20081027 | 6201401 | E | 4918 | 5104 | 21:52:37 | 21:55:43 |
| 8 | 20081027 | 6201501 | W | 5215 | 5379 | 21:57:34 | 22:00:18 |
| 8 | 20081027 | 6201701 | W | 5781 | 5952 | 22:07:00 | 22:09:51 |
| 8 | 20081027 | 6201801 | E | 5479 | 5664 | 22:01:58 | 22:05:03 |
| 8 | 20081027 | 6202001 | E | 6052 | 6242 | 22:11:31 | 22:14:41 |
| 8 | 20081027 | 6202101 | W | 6359 | 6528 | 22:16:38 | 22:19:27 |
| 8 | 20081027 | 6202301 | W | 6933 | 7104 | 22:26:12 | 22:29:03 |
| 8 | 20081027 | 6202401 | E | 6627 | 6817 | 22:21:06 | 22:24:16 |
| 8 | 20081027 | 6202601 | E | 7198 | 7390 | 22:30:37 | 22:33:49 |
| 8 | 20081027 | 6202901 | W | 7481 | 7651 | 22:35:20 | 22:38:10 |
| 8 | 20081027 | 6203001 | E | 7775 | 7963 | 22:40:14 | 22:43:22 |
| 8 | 20081027 | 6203201 | E | 8338 | 8523 | 22:49:37 | 22:52:43 |
| 8 | 20081027 | 6203301 | W | 8061 | 8231 | 22:45:00 | 22:47:50 |
| 8 | 20081027 | 6203501 | W | 8721 | 8992 | 22:56;00 | 23:00:31 |
| 8 | 20081027 | 6203601 | E | 9103 | 8400 | 23:02:22 | 23:07:19 |
| 8 | 20081027 | 6203801 | E | 9882 | 10175 | 23:15:21 | 23:20:14 |
| 8 | 20081027 | 6203901 | W | 9510 | 9780 | 23:09:09 | 23:13:39 |
| 8 | 20081027 | 6204101 | W | 10268 | 10540 | 23:21:47 | 23:26:19 |
| 8 | 20081027 | 6204201 | E | 10665 | 10960 | 23:28:24 | 23:33:19 |
| 8 | 20081027 | 6204401 | E | 11393 | 11626 | 23:40:32 | 23:44:25 |
| 8 | 20081027 | 6204501 | W | 11046 | 11260 | 23:34:45 | 23:38:19 |
| 8 | 20081027 | 904000801 |  | 12037 | 12159 | 23:51:16 | 23:53:18 |
| 8 | 20081027 | 905000801 |  | 12228 | 12352 | 23:54:27 | 23:56:31 |
| 8 | 20081027 | 906000801 |  | 12368 | 12494 | 23:56:47 | 23:58:53 |
| 9 | 20081028 | 505000901 |  | 114 | 258 | 20:36:52 | 20:39:16 |
| 9 | 20081028 | 900000901 |  | 1047 | 1171 | 20:52:25 | 20:54:29 |
| 9 | 20081028 | 901000901 |  | 1195 | 1326 | 20:54:53 | 20:57:04 |
| 9 | 20081028 | 902000901 |  | 1349 | 1471 | 20:57:27 | 20:59:29 |
| 9 | 20081028 | 903000901 |  | 1484 | 1549 | 20:59:42 | 21:00:47 |
| 9 | 20081028 | 910000901 |  | 2784 | 2908 | 21:21:22 | 21:23:26 |
| 9 | 20081028 | 5000301 | W | 4178 | 4774 | 21:44:36 | 21:54:32 |
| 9 | 20081028 | 5000501 | E | 8303 | 8950 | 22:53:21 | 23:04:08 |
| 9 | 20081028 | 5000701 | W | 9082 | 9698 | 23:06:20 | 23:16:36 |
| 9 | 20081028 | 5000901 | E | 9876 | 10525 | 23:19:34 | 23:30:23 |
| 9 | 20081028 | 5100501 | E | 3345 | 4017 | 21:30:43 | 21:41:55 |
| 9 | 20081028 | 5100701 | W | 5901 | 6504 | 22:13:19 | 22:23:22 |
| 9 | 20081028 | 5101101 | W | 7479 | 8094 | 22:39:37 | 22:49:52 |
| 9 | 20081028 | 5101301 | E | 5085 | 5758 | 21:59:43 | 22:10:56 |
| 9 | 20081028 | 5101701 | E | 6695 | 7359 | 22:26:33 | 22:37:37 |
| 9 | 20081028 | 5101901 | W | 10605 | 11215 | 23:31:43 | 23:41:53 |
| 9 | 20081028 | 904000901 |  | 11815 | 11952 | 23:51:53 | 23:54:10 |
| 9 | 20081028 | 905000901 |  | 11961 | 12095 | 23:54:19 | 23:56:33 |
| 9 | 20081028 | 906000901 |  | 12147 | 12274 | 23:57:25 | 23:59:32 |
| 9 | 20081028 | 605000901 |  | 13579 | 13704 | 00:21:17 | 00:23:22 |
| 10 | 20081030 | 505001001 |  | 136 | 260 | 20:35:05 | 20:37:09 |
| 10 | 20081030 | 900001001 |  | 1071 | 1193 | 20:50:40 | 20:52:42 |
| 10 | 20081030 | 901001001 |  | 1237 | 1369 | 20:53:26 | 20:55:38 |
| 10 | 20081030 | 902001001 |  | 1377 | 1501 | 20:55:46 | 20:57:50 |
| 10 | 20081030 | 903001001 |  | 1509 | 1576 | 20:57:58 | 20:59:05 |
| 10 | 20081030 | 5001101 | W | 3232 | 4723 | 21:26:41 | 21:51:32 |
| 10 | 20081030 | 5001301 | E | 8290 | 8516 | 22:50:59 | 22:54:45 |
| 10 | 20081030 | 5101902 | W | 6454 | 7082 | 22:20:23 | 22:30:51 |
| 10 | 20081030 | 5102201 | W | 7268 | 8021 | 22:33:57 | 22:46:30 |
| 10 | 20081030 | 5102401 | E | 4817 | 6263 | 21:53:06 | 22:17:12 |
| 10 | 20081030 | 6204701 | W | 9091 | 9317 | 23:04:20 | 23:08:06 |
| 10 | 20081030 | 6204801 | E | 8743 | 8963 | 22:58:32 | 23:02:12 |
| 10 | 20081030 | 6204901 | W | 9803 | 10027 | 23:16:12 | 23:19:56 |
| 10 | 20081030 | 6205001 | E | 9418 | 9639 | 23:09:47 | 23:13:28 |
| 10 | 20081030 | 904001001 |  | 10449 | 10574 | 23:26:58 | 23:29:03 |
| 10 | 20081030 | 605001001 |  | 10585 | 10708 | 23:29:14 | 23:31:17 |
| 10 | 20081030 | 906001001 |  | 10787 | 10926 | 23:32:36 | 23:34::55 |
| 11 | 20081031 | 505001101 |  | 120 | 258 | 20:51:00 | 20:53:18 |
| 11 | 20081031 | 900001101 |  | 1002 | 1138 | 21:05:42 | 21:07:58 |
| 11 | 20081031 | 901001101 |  | 1165 | 1295 | 21:08:25 | 21:10:35 |
| 11 | 20081031 | 902001101 |  | 1317 | 1446 | 21:10:57 | 21:13:06 |
| 11 | 20081031 | 903001101 |  | 1452 | 1501 | 21:13:12 | 21:14:01 |
| 11 | 20081031 | 5001401 | E | 6315 | 6973 | 22:34:15 | 22:45:13 |
| 11 | 20081031 | 5001701 | E | 5501 | 6018 | 22:20:10 | 22:29:18 |
| 11 | 20081031 | 5102601 | E | 2374 | 3729 | 21:28:34 | 21:51:09 |
| 11 | 20081031 | 5102301 | W | 7316 | 7899 | 22:50:56 | 23:00:39 |
| 11 | 20081031 | 5102701 | W | 3871 | 5177 | 21:53:31 | 22:15:17 |
| 11 | 20081031 | 6205101 | W | 8257 | 8479 | 23:06:37 | 23:10:19 |
| 11 | 20081031 | 6205201 | E | 8619 | 8845 | 23:12:39 | 23:16:25 |
| 11 | 20081031 | 6205301 | W | 8984 | 9204 | 23:18:44 | 23:22:24 |
| 11 | 20081031 | 6205401 | E | 9344 | 9571 | 23:24:44 | 23:28:31 |
| 11 | 20081031 | 6205701 | W | 9682 | 9902 | 23:30:22 | 23:34:02 |
| 11 | 20081031 | 904001101 |  | 10261 | 10404 | 23:40:01 | 23:42:24 |
| 11 | 20081031 | 905001101 |  | 10412 | 10534 | 23:42:32 | 23:44:34 |
| 11 | 20081031 | 906001101 |  | 10637 | 10760 | 23:46:17 | 23:48:20 |
| 11 | 20081031 | 605001101 |  | 11882 | 12009 | 00:07:02 | 00:09:09 |
| 12 | 20081101 | 5001501 | W | 4443 | 4998 | 21:48:46 | 21:58:01 |
| 12 | 20081101 | 5001601 | W | 3555 | 4201 | 21:33:58 | 21:44:44 |
| 13 | 20081102 | 5001801 | E | 3854 | 4542 | 22:18:55 | 22:30:23 |
| 13 | 20081102 | 5002101 | W | 5314 | 5542 | 22:43:15 | 22:47:03 |
| 13 | 20081102 | 5002201 | E | 6262 | 6455 | 22:59:03 | 23:02:16 |
| 13 | 20081102 | 5002301 | W | 7146 | 7358 | 23:13:47 | 23:17:19 |
| 13 | 20081102 | 6205601 | E | 317 | 532 | 21:19:58 | 21:23:33 |
| 13 | 20081102 | 6205801 | E | 1071 | 1289 | 21:32:32 | 21:36:10 |
| 13 | 20081102 | 6205901 | W | 722 | 954 | 21:26:43 | 21:30:35 |
| 13 | 20081102 | 6206001 | E | 1759 | 1977 | 21:44:00 | 21:47:38 |
| 13 | 20081102 | 6206101 | W | 1386 | 1615 | 21:37:47 | 21:41:36 |
| 13 | 20081102 | 6206201 | E | 2486 | 2707 | 21:56:07 | 21:59:48 |
| 13 | 20081102 | 6206301 | W | 2100 | 2330 | 21:49:41 | 21:53:31 |
| 13 | 20081102 | 6206501 | W | 2805 | 3032 | 22:01:26 | 22:05:13 |
| 13 | 20081102 | 6206601 | E | 3190 | 3411 | 22:07:51 | 22:11:32 |
| 13 | 20081102 | 6206701 | W | 3526 | 3752 | 22:13:27 | 22:17:13 |
| 13 | 20081102 | 6600101 | W | 4642 | 4893 | 22:32:03 | 22:36:14 |
| 13 | 20081102 | 6600201 | E | 4993 | 5236 | 22:37:54 | 22:41:57 |
| 13 | 20081102 | 6600401 | E | 5644 | 5862 | 22:48:45 | 22:52:23 |
| 13 | 20081102 | 6600501 | W | 5963 | 6164 | 22:54:04 | 22:57:25 |
| 13 | 20081102 | 6600701 | W | 6550 | 6745 | 23:03:51 | 23:07:06 |
| 13 | 20081102 | 6600801 | E | 6835 | 7043 | 23:08:36 | 23:12:04 |
| 13 | 20081102 | 6601001 | E | 7608 | 8038 | 23:21:29 | 23:28:39 |
| 14 | 20081103 | 505001401 |  | 117 | 247 | 20:43:46 | 20:45:56 |
| 14 | 20081103 | 900001401 |  | 851 | 980 | 21:09:59 | 21:12:08 |
| 14 | 20081103 | 901001401 |  | 1016 | 1183 | 21:12:44 | 21:15:31 |
| 14 | 20081103 | 902001401 |  | 1194 | 1339 | 21:15:42 | 21:18:07 |
| 14 | 20081103 | 903001401 |  | 1344 | 1411 | 21:18:12 | 21:19:19 |
| 14 | 20081103 | 1000201 | W | 7622 | 7871 | 23:02:50 | 23:06:59 |
| 14 | 20081103 | 2800101 | W | 8071 | 8198 | 23:10:19 | 23:12:26 |
| 14 | 20081103 | 2800201 | E | 8304 | 8436 | 23:14:12 | 23:16:24 |
| 14 | 20081103 | 2800301 | W | 8517 | 8647 | 23:17:45 | 23:19:55 |
| 14 | 20081103 | 2800401 | E | 8741 | 8889 | 23:21:29 | 23:23:57 |
| 14 | 20081103 | 2800501 | W | 8965 | 9116 | 23:25:13 | 23:27:44 |
| 14 | 20081103 | 5002401 | E | 2076 | 2599 | 21:30:24 | 21:39:07 |
| 14 | 20081103 | 5002501 | W | 5021 | 5150 | 22:19:29 | 22:21:38 |
| 14 | 20081103 | 5102302 | W | 6266 | 6844 | 22:40:14 | 22:49:52 |
| 14 | 20081103 | 6601101 | W | 2686 | 3169 | 21:40:34 | 21:48:37 |
| 14 | 20081103 | 6601301 | W | 3983 | 4483 | 22:02:11 | 22:10:31 |
| 14 | 20081103 | 6601401 | E | 3379 | 3863 | 21:52:07 | 22:00:11 |
| 14 | 20081103 | 904001401 |  | 9768 | 9901 | 23:38:36 | 23:40:49 |
| 14 | 20081103 | 905001401 |  | 9918 | 10041 | 23:41:06 | 23:43:09 |
| 14 | 20081103 | 906001401 |  | 10056 | 10179 | 23:43:24 | 23:45:27 |
| 14 | 20081103 | 605001401 |  | 11218 | 11344 | 00:02:46 | 00:04:52 |
| 15 | 20081104 | 505001501 |  | 74 | 199 | 20:48:55 | 20:51:00 |
| 15 | 20081104 | 900001501 |  | 914 | 1037 | 21:02:55 | 21:04:58 |
| 15 | 20081104 | 901001501 |  | 1075 | 1198 | 21:05:36 | 21:07:39 |
| 15 | 20081104 | 902001501 |  | 1231 | 1390 | 21:08:12 | 21:10:51 |
| 15 | 20081104 | 903001501 |  | 1406 | 1472 | 21:11:07 | 21:12:13 |
| 15 | 20081104 | 911001501 |  | 2058 | 2206 | 21:21:59 | 21:24:27 |
| 15 | 20081104 | 1000101 | W | 2783 | 3610 | 21:34:04 | 21:47:51 |
| 15 | 20081104 | 1000401 | E | 4031 | 4896 | 21:54:52 | 22:09:17 |
| 15 | 20081104 | 904001501 |  | 1075 | 1198 | 21:05:36 | 21:07:39 |
| 15 | 20081104 | 905001501 |  | 1231 | 1390 | 21:08:12 | 21:10:51 |
| 15 | 20081104 | 605001501 |  | 6423 | 6544 | 22:34:44 | 22:36:45 |
| 16 | 20081106 | 505001601 |  | 123 | 251 | 20:32:51 | 20:34:59 |
| 16 | 20081106 | 900001601 |  | 1047 | 1169 | 20:48:15 | 20:50:17 |
| 16 | 20081106 | 901001601 |  | 1188 | 1311 | 20:50:36 | 20:52:39 |
| 16 | 20081106 | 902001601 |  | 1329 | 1477 | 20:52:57 | 20:55:25 |
| 16 | 20081106 | 903001601 |  | 1489 | 1552 | 20:55:37 | 20:56:40 |
| 16 | 20081106 | 1000701 | W | 2670 | 3931 | 21:15:18 | 21:36:19 |
| 16 | 20081106 | 1000901 | E | 4079 | 5256 | 21:38:47 | 21:58:24 |
| 16 | 20081106 | 1000501 | E | 5738 | 5967 | 22:06:26 | 22:10:15 |
| 16 | 20081106 | 2802101 | W | 6171 | 6692 | 22:13:39 | 22:22:20 |
| 16 | 20081106 | 2800601 | E | 6837 | 7001 | 22:24:45 | 22:27:29 |
| 16 | 20081106 | 2800701 | W | 7076 | 7252 | 22:28:44 | 22:31:40 |
| 16 | 20081106 | 2800801 | E | 7353 | 7540 | 22:33:21 | 22:36:28 |
| 16 | 20081106 | 2800901 | W | 7628 | 7827 | 22:37:56 | 22:41:15 |
| 16 | 20081106 | 2801001 | E | 7926 | 8130 | 22:42:54 | 22:46:18 |
| 16 | 20081106 | 2801101 | W | 8573 | 8783 | 22:53:41 | 22:57:11 |
| 16 | 20081106 | 904001601 |  | 9314 | 9437 | 23:06:02 | 23:08:05 |
| 16 | 20081106 | 905001601 |  | 9447 | 9569 | 23:08:15 | 23:10:17 |
| 16 | 20081106 | 906001601 |  | 9636 | 9759 | 23:11:24 | 23:13:27 |
| 16 | 20081106 | 605001601 |  | 10827 | 10949 | 23:31:15 | 23:33:17 |
| 17 | 20081107 | 505001701 |  | 142 | 267 | 20:24:44 | 20:26:49 |
| 17 | 20081107 | 900001701 |  | 1045 | 1184 | 20:39:47 | 20:42:06 |
| 17 | 20081107 | 901001701 |  | 1213 | 1347 | 20:42:35 | 20:44:49 |
| 17 | 20081107 | 902001701 |  | 1365 | 1487 | 20:45:07 | 20:47:09 |
| 17 | 20081107 | 903001701 |  | 1495 | 1559 | 20:47:17 | 20:48:21 |
| 17 | 20081107 | 1001101 | W | 2457 | 3657 | 21:03:19 | 21:23:19 |
| 17 | 20081107 | 1100101 | E | 3981 | 4474 | 21:28:43 | 21:36:56 |
| 17 | 20081107 | 2801401 | E | 4639 | 4854 | 21:39:41 | 21:43:16 |
| 17 | 20081107 | 2801301 | W | 4982 | 5201 | 21:45:24 | 21:49:03 |
| 17 | 20081107 | 2801601 | E | 5325 | 5540 | 21:51:07 | 21:54:42 |
| 17 | 20081107 | 2801501 | W | 5675 | 5893 | 21:56:57 | 22:00:35 |
| 17 | 20081107 | 2801801 | E | 6024 | 6239 | 22:02:46 | 22:06:21 |
| 17 | 20081107 | 2801701 | W | 6373 | 6589 | 22:08:35 | 22:12:11 |
| 17 | 20081107 | 2802001 | E | 6743 | 7186 | 22:14:45 | 22:22:08 |
| 17 | 20081107 | 2802301 | W | 7422 | 7944 | 22:26:04 | 22:34:46 |
| 17 | 20081107 | 2802201 | E | 8028 | 8543 | 22:36:10 | 22:44:45 |
| 17 | 20081107 | 2802501 | W | 8643 | 9168 | 22:46:25 | 22:55:10 |
| 17 | 20081107 | 2802401 | E | 9292 | 9799 | 22:57:14 | 23:05:41 |
| 17 | 20081107 | 904001701 |  | 10241 | 10363 | 23:13:03 | 23:15:05 |
| 17 | 20081107 | 905001701 |  | 10371 | 10494 | 23:15:13 | 23:17:16 |
| 17 | 20081107 | 906001701 |  | 10506 | 10629 | 23:17:28 | 23:19:31 |
| 17 | 20081107 | 605001701 |  | 11777 | 11899 | 23:38:39 | 23:40:41 |
| 18 | 20081108 | 505001801 |  | 108 | 238 | 20:17:42 | 20:19:52 |
| 18 | 20081108 | 900001801 |  | 1149 | 1277 | 20:35:03 | 20:37:11 |
| 18 | 20081108 | 901001801 |  | 1347 | 1473 | 20:38:21 | 20:40:27 |
| 18 | 20081108 | 902001801 |  | 1518 | 1657 | 20:41:12 | 20:43:31 |
| 18 | 20081108 | 903001801 |  | 1697 | 1754 | 20:44:11 | 20:45:08 |
| 18 | 20081108 | 911001801 |  | 2146 | 2299 | 20:51:40 | 20:54:13 |
| 18 | 20081108 | 1001401 | W | 2796 | 5115 | 21:02:30 | 21:41:09 |
| 18 | 20081108 | 1001201 | E | 5330 | 7388 | 21:44:44 | 22:19:02 |
| 18 | 20081108 | 2801901 | W | 8106 | 8326 | 22:31:00 | 22:34:40 |
| 18 | 20081108 | 2802601 | E | 8687 | 8942 | 22:40:41 | 22:44:56 |
| 18 | 20081108 | 2802901 | W | 9025 | 9282 | 22:46:19 | 22:50:36 |
| 18 | 20081108 | 2802801 | E | 9405 | 9657 | 22:52:39 | 22:56:51 |
| 18 | 20081108 | 2900201 | W | 9827 | 10030 | 22:59:41 | 23:03:04 |
| 18 | 20081108 | 2900101 | E | 10143 | 10338 | 23:04:57 | 23:08:12 |
| 18 | 20081108 | 2900401 | W | 10453 | 10656 | 23:10:07 | 23:13:30 |
| 18 | 20081108 | 2900501 | E | 10781 | 10976 | 23:15:35 | 23:18:50 |
| 18 | 20081108 | 2900601 | W | 11100 | 11306 | 23:20:54 | 23:24:20 |
| 18 | 20081108 | 904001801 |  | 11602 | 11781 | 23:29:16 | 23:32:15 |
| 18 | 20081108 | 905001801 |  | 11781 | 11905 | 23:32:15 | 23:34:19 |
| 18 | 20081108 | 906001801 |  | 11943 | 12067 | 23:34:57 | 23:37:01 |
| 18 | 20081108 | 605001801 |  | 13178 | 13304 | 23:55:32 | 23:57:38 |
| 19 | 20081109 | 505001901 |  | 134 | 294 | 20:24:03 | 20:26:43 |
| 19 | 20081109 | 900001901 |  | 239 | 362 | 20:44:25 | 20:46:28 |
| 19 | 20081109 | 901001901 |  | 413 | 537 | 20:47:19 | 20:49:23 |
| 19 | 20081109 | 902001901 |  | 612 | 736 | 20:50:38 | 20:52:42 |
| 19 | 20081109 | 903001901 |  | 777 | 838 | 20:53:23 | 20:54:24 |
| 19 | 20081109 | 2900801 | W | 2075 | 2275 | 21:15:01 | 21:18:21 |
| 19 | 20081109 | 2900901 | E | 2409 | 2610 | 21:20:35 | 21:23:56 |
| 19 | 20081109 | 2901001 | W | 2719 | 2918 | 21:25:45 | 21:29:04 |
| 19 | 20081109 | 2901101 | E | 3045 | 3248 | 21:31:11 | 21:34:34 |
| 19 | 20081109 | 2901201 | W | 3334 | 3531 | 21:36:00 | 21:39:17 |
| 19 | 20081109 | 2901401 | W | 3945 | 4143 | 21:46:11 | 21:49:29 |
| 19 | 20081109 | 904001901 |  | 4748 | 4897 | 21:59:34 | 22:02:03 |
| 19 | 20081109 | 905001901 |  | 4945 | 5068 | 22:02:51 | 22:04:54 |
| 19 | 20081109 | 906001901 |  | 5143 | 5261 | 22:06:09 | 22:08:07 |
| 19 | 20081109 | 605001901 |  | 6305 | 6404 | 22:25:31 | 22:27:10 |
| 20 | 20081110 | 505002001 |  | 151 | 300 | 20:30:46 | 20:33:15 |
| 20 | 20081110 | 900002001 |  | 1197 | 1322 | 20:48:12 | 20:50:17 |
| 20 | 20081110 | 901002001 |  | 1405 | 1532 | 20:51:40 | 20:53:47 |
| 20 | 20081110 | 902002001 |  | 1577 | 1700 | 20:54:32 | 20:56:35 |
| 20 | 20081110 | 903002001 |  | 1791 | 1856 | 20:58:06 | 20:59:11 |
| 20 | 20081110 | 904002001 |  | 10905 | 11029 | 23:30:00 | 23:32:04 |
| 20 | 20081110 | 905002001 |  | 11074 | 11197 | 23:32:49 | 23:34:52 |
| 20 | 20081110 | 906002001 |  | 11262 | 11387 | 23:35:57 | 23:38:02 |
| 20 | 20081110 | 605002001 |  | 12156 | 12278 | 23:50:51 | 23:52:53 |
| 21 | 20081111 | 505002101 |  | 169 | 293 | 20:27:26 | 20:29:30 |
| 21 | 20081111 | 900002101 |  | 1206 | 1329 | 20:44:43 | 20:46:46 |
| 21 | 20081111 | 901002101 |  | 1374 | 1497 | 20:47:31 | 20:49:34 |
| 21 | 20081111 | 902002101 |  | 1508 | 1633 | 20:49:45 | 20:51:50 |
| 21 | 20081111 | 903002101 |  | 1638 | 1691 | 20:51:55 | 20:52:48 |
| 21 | 20081111 | 911002101 |  | 2130 | 2282 | 21:00:07 | 21:02:39 |
| 21 | 20081111 | 904002101 |  | 5312 | 5436 | 21:53:09 | 21:55:13 |
| 21 | 20081111 | 905002101 |  | 5446 | 5570 | 21:55:23 | 21:57:27 |
| 21 | 20081111 | 906002101 |  | 5584 | 5696 | 21:57:41 | 21:59:33 |
| 21 | 20081111 | 605002101 |  | 6548 | 6669 | 22:13:45 | 22:15:46 |
| 22 | 20081114 | 50502201 |  | 153 | 277 | 20:21:58 | 20:24:02 |
| 22 | 20081114 | 90002201 |  | 1150 | 1275 | 20:38:35 | 20:40:40 |
| 22 | 20081114 | 90102201 |  | 1315 | 1439 | 20:41:20 | 20:43:24 |
| 22 | 20081114 | 90202201 |  | 1476 | 1598 | 20:44:01 | 20:46:03 |
| 22 | 20081114 | 90302201 |  | 1622 | 1699 | 20:46:27 | 20:47:44 |
| 22 | 20081114 | 1100402 | W | 2391 | 2887 | 20:59:16 | 21:07:32 |
| 22 | 20081114 | 1100302 | E | 3170 | 3675 | 21:12:15 | 21:20:40 |
| 22 | 20081114 | 1100602 | W | 3846 | 4343 | 21:23:31 | 21:31:48 |
| 22 | 20081114 | 1101001 | W | 5167 | 5665 | 21:45:32 | 21:53:50 |
| 22 | 20081114 | 1100901 | E | 5767 | 6281 | 21:55:32 | 22:04:06 |
| 22 | 20081114 | 1101201 | W | 6411 | 6909 | 22:06:16 | 22:14:34 |
| 22 | 20081114 | 1101301 | E | 7059 | 7569 | 22:17:04 | 22:25:34 |
| 22 | 20081114 | 1101701 | W | 7689 | 8107 | 22:27:34 | 22:34:32 |
| 22 | 20081114 | 1101501 | E | 8264 | 8773 | 22:37:09 | 22:45:38 |
| 22 | 20081114 | 1101901 | W | 8896 | 9392 | 22:47:41 | 22:55:57 |
| 22 | 20081114 | 1102001 | E | 9537 | 10030 | 22:58:12 | 23:06:35 |
| 22 | 20081114 | 1102301 | W | 10151 | 10582 | 23:08:36 | 23:15:47 |
| 22 | 20081114 | 1102201 | E | 10689 | 11124 | 23:17:34 | 23:24:49 |
| 22 | 20081114 | 90402201 |  | 11357 | 11478 | 23:28:42 | 23:30:43 |
| 22 | 20081114 | 90502201 |  | 11505 | 11627 | 23:31:10 | 23:33:12 |
| 22 | 20081114 | 90602201 |  | 11653 | 11773 | 23:33:38 | 23:35:38 |
| 22 | 20081114 | 60502201 |  | 12550 | 12672 | 23:48:35 | 23:50:37 |
| 23 | 20081115 | 50502301 |  | 530 | 687 | 20:32:14 | 20:34:51 |
| 23 | 20081115 | 90002301 |  | 1397 | 1524 | 20:46:41 | 20:48:48 |
| 23 | 20081115 | 90102301 |  | 1623 | 1746 | 20:50:27 | 20:52:30 |
| 23 | 20081115 | 90202301 |  | 1858 | 1986 | 20:54:22 | 20:56:30 |
| 23 | 20081115 | 90302301 |  | 2060 | 2123 | 20:57:44 | 20:58:47 |
| 23 | 20081115 | 90402301 |  | 4286 | 4408 | 21:34:50 | 21:36:52 |
| 23 | 20081115 | 90502301 |  | 4436 | 4558 | 21:37:20 | 21:39:22 |
| 23 | 20081115 | 90602301 |  | 4601 | 4724 | 21:40:05 | 21:42:08 |
| 23 | 20081115 | 60502301 |  | 5618 | 5740 | 21:57:02 | 21:59:04 |
| 24 | 20081116 | 50502401 |  | 216 | 339 | 21:36:19 | 21:38:22 |
| 24 | 20081116 | 90002401 |  | 971 | 1093 | 21:48:54 | 21:50:56 |
| 24 | 20081116 | 90102401 |  | 1139 | 1261 | 21:51:42 | 20:53:44 |
| 24 | 20081116 | 90202401 |  | 1288 | 1411 | 21:54:11 | 21:56:14 |
| 24 | 20081116 | 90302401 |  | 1427 | 1492 | 21:56:30 | 21:57:35 |
| 24 | 20081116 | 91102401 |  | 1802 | 1956 | 22:02:45 | 22:05:19 |
| 24 | 20081116 | 3000103 | W | 3453 | 3684 | 22:30:16 | 22:34:07 |
| 24 | 20081116 | 3000203 | E | 3800 | 4040 | 22:36:03 | 22:40:03 |
| 24 | 20081116 | 3000301 | W | 4255 | 4483 | 22:43:38 | 22:47:26 |
| 24 | 20081116 | 3000701 | W | 5617 | 5846 | 23:06:20 | 23:10:09 |
| 24 | 20081116 | 3001001 | E | 5989 | 6223 | 23:12:32 | 23:16:26 |
| 24 | 20081116 | 90402401 |  | 7423 | 7543 | 23:36:26 | 23:38:26 |
| 24 | 20081116 | 90502401 |  | 7569 | 7691 | 23:38:52 | 23:40:54 |
| 24 | 20081116 | 90602401 |  | 7713 | 7834 | 23:41:16 | 23:43:17 |
| 24 | 20081116 | 60502401 |  | 9002 | 9147 | 00:02:45 | 00:05:10 |
| 25 | 20081119 | 50502501 |  | 671 | 794 | 20:42:43 | 20:44:46 |
| 25 | 20081119 | 90002501 |  | 1092 | 1213 | 21:14:30 | 21:16:31 |
| 25 | 20081119 | 90102501 |  | 1257 | 1378 | 21:17:15 | 21:19:16 |
| 25 | 20081119 | 90202501 |  | 1423 | 1547 | 21:20:01 | 21:22:05 |
| 25 | 20081119 | 90302501 |  | 1561 | 1628 | 21:22:19 | 21:23:26 |
| 25 | 20081119 | 5002303 | W | 2955 | 3163 | 21:45:33 | 21:49:01 |
| 25 | 20081119 | 6600803 | E | 2613 | 2817 | 21:39:51 | 21:43:15 |
| 25 | 20081119 | 3000502 | W | 3471 | 3697 | 21:54:09 | 21:57:55 |
| 25 | 20081119 | 3000402 | E | 3805 | 4050 | 21:59:43 | 22:03:48 |
| 25 | 20081119 | 3000902 | W | 4112 | 4336 | 22:04:50 | 22:08:34 |
| 25 | 20081119 | 3000602 | E | 4458 | 4698 | 22:10:36 | 22:14:36 |
| 25 | 20081119 | 3001102 | W | 4766 | 4991 | 22:15:44 | 22:19:29 |
| 25 | 20081119 | 3001201 | E | 5113 | 5350 | 22:21:31 | 22:25:28 |
| 25 | 20081119 | 3001301 | W | 5456 | 5683 | 22:27:14 | 22:31:01 |
| 25 | 20081119 | 3001401 | E | 5886 | 6122 | 22:34:24 | 22:38:20 |
| 25 | 20081119 | 3001501 | W | 6200 | 6430 | 22:39:38 | 22:43:28 |
| 25 | 20081119 | 3001601 | E | 6560 | 6795 | 22:45:38 | 22:49:33 |
| 25 | 20081119 | 2900702 | E | 8258 | 8458 | 23:13:56 | 23:17:16 |
| 25 | 20081119 | 2901302 | E | 8752 | 8952 | 23:22:10 | 23:25:30 |
| 25 | 20081119 | 2901502 | E | 9282 | 9480 | 23:31:00 | 23:34:18 |
| 25 | 20081119 | 1001301 | E | 9875 | 10111 | 23:40:53 | 23:44:49 |
| 25 | 20081119 | 1100702 | E | 11734 | 12231 | 00:11:52 | 00:20:09 |
| 25 | 20081119 | 90402501 |  | 12448 | 12580 | 00:23:46 | 00:25:58 |
| 25 | 20081119 | 90502501 |  | 12739 | 12862 | 00:28:37 | 00:30:40 |
| 25 | 20081119 | 90602501 |  | 12885 | 13008 | 00:31:03 | 00:33:06 |
| 25 | 20081119 | 60502501 |  | 13829 | 13952 | 00:46:37 | 00:48:49 |
| 26 | 20081120 | 50502601 |  | 172 | 296 | 21:00:31 | 21:02:35 |
| 26 | 20081120 | 90002601 |  | 939 | 1065 | 21:13:18 | 21:15:24 |
| 26 | 20081120 | 90102601 |  | 1107 | 1232 | 21:16:06 | 21:18:11 |
| 26 | 20081120 | 90202601 |  | 1267 | 1388 | 21:18:46 | 21:20:47 |
| 26 | 20081120 | 90302601 |  | 1404 | 1478 | 21:21:03 | 21:22:17 |
| 26 | 20081120 | 3001802 | E | 2052 | 2279 | 21:31:51 | 21:35:38 |
| 26 | 20081120 | 3001702 | W | 2378 | 2613 | 21:37:17 | 21:41:12 |
| 26 | 20081120 | 1000601 | W | 4414 | 4544 | 22:11:13 | 22:13:23 |
| 26 | 20081120 | 1000301 | E | 4666 | 4797 | 22:15:25 | 22:17:36 |
| 26 | 20081120 | 1001001 | W | 5085 | 5448 | 22:22:24 | 22:28:27 |
| 26 | 20081120 | 1000801 | E | 5702 | 5882 | 22:32:41 | 22:35:41 |
| 26 | 20081120 | 4100201 | W | 6077 | 6205 | 22:38:56 | 22:41:04 |
| 26 | 20081120 | 4100301 | E | 6295 | 6426 | 22:42:34 | 22:44:45 |
| 26 | 20081120 | 4100601 | W | 6546 | 6676 | 22:46:45 | 22:48:55 |
| 26 | 20081120 | 4100501 | E | 6777 | 6907 | 22:50:36 | 22:52:46 |
| 26 | 20081120 | 4100801 | W | 7027 | 7235 | 22:54:46 | 22:58:14 |
| 26 | 20081120 | 4100901 | E | 7361 | 7571 | 23:00:20 | 23:03:50 |
| 26 | 20081120 | 4101201 | W | 7687 | 7936 | 23:05:46 | 23:09:55 |
| 26 | 20081120 | 4101101 | E | 8025 | 8277 | 23:11:24 | 23:15:36 |
| 26 | 20081120 | 4200101 | W | 8822 | 8957 | 23:24:41 | 23:26:56 |
| 26 | 20081120 | 4200401 | E | 9209 | 9338 | 23:31:08 | 23:33:17 |
| 26 | 20081120 | 4200301 | W | 9479 | 9613 | 23:35:38 | 23:37:52 |
| 26 | 20081120 | 4300101 | E | 9847 | 9976 | 23:41:46 | 23:43:55 |
| 26 | 20081120 | 90402601 |  | 10250 | 10372 | 23:48:29 | 23:50:31 |
| 26 | 20081120 | 90502601 |  | 10408 | 10532 | 23:51:07 | 23:53:11 |
| 26 | 20081120 | 90602601 |  | 10559 | 10683 | 23:53:38 | 23:55:42 |
| 26 | 20081120 | 60502601 |  | 11473 | 11597 | 00:08:52 | 00:10:56 |
| 27 | 20081121 | 50502701 |  | 191 | 318 | 20:57:15 | 20:59:22 |
| 27 | 20081121 | 90002701 |  | 1052 | 1177 | 21:11:36 | 21:13:41 |
| 27 | 20081121 | 90102701 |  | 1245 | 1374 | 21:14:49 | 21:16:58 |
| 27 | 20081121 | 90202701 |  | 1393 | 1515 | 21:17:17 | 21:19:19 |
| 27 | 20081121 | 90302701 |  | 1552 | 1629 | 21:19:56 | 21:21:13 |
| 27 | 20081121 | 91102701 |  | 2213 | 2362 | 21:30:57 | 21:33:26 |
| 27 | 20081121 | 90402701 |  | 9716 | 9716 | 23:36:00 | 23:38:03 |
| 27 | 20081121 | 90502701 |  | 9858 | 9858 | 23:38:22 | 23:40:25 |
| 27 | 20081121 | 90602701 |  | 10022 | 10022 | 23:41:06 | 23:43:14 |
| 27 | 20081121 | 60502701 |  | 11198 | 11198 | 00:00:42 | 00:02:45 |
| 28 | 20081122 | 50502801 |  | 293 | 415 | 00:03:28 | 00:05:30 |
| 28 | 20081122 | 90002801 |  | 1046 | 1179 | 00:16:01 | 00:18:14 |
| 28 | 20081122 | 90102801 |  | 1228 | 1353 | 00:19:03 | 00:21:08 |
| 28 | 20081122 | 90202801 |  | 1377 | 1502 | 00:21:32 | 00:23:37 |
| 28 | 20081122 | 90302801 |  | 1556 | 1635 | 00:24:31 | 00:25:50 |
| 28 | 20081122 | 90402801 |  | 10890 | 11011 | 03:00:25 | 03:02:06 |
| 28 | 20081122 | 90502801 |  | 11099 | 11229 | 03:03:34 | 03:05:44 |
| 28 | 20081122 | 90602801 |  | 11269 | 11398 | 03:06:24 | 03:08:33 |
| 28 | 20081122 | 60502801 |  | 12395 | 12516 | 03:25:10 | 03:27:11 |
| 29 | 20081126 | 50502901 |  | 322 | 443 | 21:45:14 | 21:47:15 |
| 29 | 20081126 | 90002901 |  | 1248 | 1371 | 22:00:40 | 22:02:43 |
| 29 | 20081126 | 90102901 |  | 1444 | 1565 | 22:03:56 | 22:05:57 |
| 29 | 20081126 | 90202901 |  | 1830 | 1950 | 22:10:22 | 22:12:22 |
| 29 | 20081126 | 90302901 |  | 2097 | 2154 | 22:14:49 | 22:15:46 |
| 29 | 20081126 | 4300203 | W | 2759 | 2887 | 22:25:51 | 22:27:59 |
| 29 | 20081126 | 4300503 | E | 3253 | 3390 | 22:34:05 | 22:36:22 |
| 29 | 20081126 | 4300403 | W | 3496 | 3624 | 22:38:08 | 22:40:16 |
| 29 | 20081126 | 90402901 |  | 3878 | 4000 | 22:44:30 | 22:46:32 |
| 29 | 20081126 | 90502901 |  | 4038 | 4161 | 22:47:10 | 22:49:13 |
| 29 | 20081126 | 90602901 |  | 4195 | 4318 | 22:49:47 | 22:51:50 |
| 29 | 20081126 | 60502901 |  | 5277 | 5399 | 23:07:49 | 23:09:51 |
| 30 | 20081127 | 50503001 |  | 241 | 366 | 21:06:28 | 21:08:33 |
| 30 | 20081127 | 90003001 |  | 1114 | 1235 | 21:21:01 | 21:23:02 |
| 30 | 20081127 | 90103001 |  | 1267 | 1389 | 21:23:34 | 21:25:36 |
| 30 | 20081127 | 90203001 |  | 1413 | 1534 | 21:26:00 | 21:28:01 |
| 30 | 20081127 | 90303001 |  | 1545 | 1406 | 21:28:12 | 21:29:11 |
| 30 | 20081127 | 91103001 |  | 1968 | 2121 | 21:35:15 | 21:37:48 |
| 30 | 20081127 | 3100104 | W | 3133 | 3263 | 21:54:40 | 21:56:50 |
| 30 | 20081127 | 3100204 | E | 3354 | 3485 | 21:58:21 | 22:00:32 |
| 30 | 20081127 | 3100304 | W | 3607 | 3736 | 22:02:34 | 22:04:43 |
| 30 | 20081127 | 3100404 | E | 3878 | 4008 | 22:07:05 | 22:09:15 |
| 30 | 20081127 | 3100704 | W | 4152 | 4280 | 22:11:39 | 22:13:47 |
| 30 | 20081127 | 3100604 | E | 4392 | 4523 | 22:15:39 | 22:17:50 |
| 30 | 20081127 | 3100904 | W | 4688 | 4817 | 22:20:35 | 22:22:44 |
| 30 | 20081127 | 3100804 | E | 4966 | 5097 | 22:25:13 | 22:27:24 |
| 30 | 20081127 | 3101104 | W | 5250 | 5381 | 22:29:57 | 22:32:08 |
| 30 | 20081127 | 3101204 | E | 5561 | 5693 | 22:35:08 | 22:37:20 |
| 30 | 20081127 | 3101304 | W | 5854 | 5985 | 22:40:01 | 22:42:12 |
| 30 | 20081127 | 3101404 | E | 6150 | 6280 | 22:44:57 | 22:47:07 |
| 30 | 20081127 | 3101704 | W | 6403 | 6534 | 22:49:10 | 22:51:21 |
| 30 | 20081127 | 3101604 | E | 6692 | 6822 | 22:53:59 | 22:56:09 |
| 30 | 20081127 | 3102104 | W | 7412 | 7544 | 23:05:59 | 23:08:11 |
| 30 | 20081127 | 3102204 | E | 7708 | 7836 | 23:10:55 | 23:13:03 |
| 30 | 20081127 | 3102303 | W | 7978 | 8111 | 23:15:25 | 23:17:38 |
| 30 | 20081127 | 3102403 | E | 8228 | 8356 | 23:19:35 | 23:21:43 |
| 30 | 20081127 | 3102703 | W | 8489 | 8625 | 23:23:56 | 23:26:12 |
| 30 | 20081127 | 3102603 | E | 8759 | 8887 | 23:28:26 | 23:30:34 |
| 30 | 20081127 | 3102903 | W | 9015 | 9148 | 23:32:42 | 23:34:55 |
| 30 | 20081127 | 3102803 | E | 9298 | 9426 | 23:37:25 | 23:39:33 |
| 30 | 20081127 | 3103103 | W | 9556 | 9689 | 23:41:43 | 23:43:56 |
| 30 | 20081127 | 3103203 | E | 9835 | 9963 | 23:46:22 | 23:48:30 |
| 30 | 20081127 | 3103303 | W | 10091 | 10223 | 23:50:38 | 23:52:50 |
| 30 | 20081127 | 3103403 | E | 10362 | 10489 | 23:55:09 | 23:57:16 |
| 30 | 20081127 | 3300101 | E | 10684 | 10829 | 00:00:31 | 00:02:56 |
| 30 | 20081127 | 3300201 | W | 10944 | 11095 | 00:04:51 | 00:07:22 |
| 30 | 20081127 | 90403001 |  | 11658 | 11781 | 00:16:45 | 00:18:48 |
| 30 | 20081127 | 90503001 |  | 11801 | 11925 | 00:19:08 | 00:21:12 |
| 30 | 20081127 | 90603001 |  | 11947 | 12069 | 00:21:34 | 00:23:36 |
| 30 | 20081127 | 60503001 |  | 13042 | 13168 | 00:39:49 | 00:41:55 |
| 31 | 20081129 | 50503101 |  | 288 | 411 | 20:58:12 | 21:00:15 |
| 31 | 20081129 | 90003101 |  | 964 | 1093 | 21:09:28 | 21:11:37 |
| 31 | 20081129 | 90103101 |  | 1287 | 1411 | 21:14:51 | 21:16:55 |
| 31 | 20081129 | 90203101 |  | 1440 | 1562 | 21:17:24 | 21:19:26 |
| 31 | 20081129 | 90303101 |  | 1577 | 1629 | 21:19:41 | 21:20:33 |
| 31 | 20081129 | 3101805 | E | 3896 | 4021 | 21:58:20 | 22:00:25 |
| 31 | 20081129 | 3101905 | W | 4158 | 4294 | 22:02:42 | 22:04:58 |
| 31 | 20081129 | 3300302 | E | 4742 | 4890 | 22:12:26 | 22:14:54 |
| 31 | 20081129 | 3300601 | W | 5057 | 5210 | 22:17:41 | 22:20:14 |
| 31 | 20081129 | 3300501 | E | 5320 | 5468 | 22:22:04 | 22:24:32 |
| 31 | 20081129 | 3300801 | W | 5617 | 5768 | 22:27:01 | 22:29:32 |
| 31 | 20081129 | 3300701 | E | 5920 | 6070 | 22:32:04 | 22:34:34 |
| 31 | 20081129 | 3301001 | W | 6184 | 6334 | 22:36:28 | 22:38:58 |
| 31 | 20081129 | 3300901 | E | 6449 | 6596 | 22:40:53 | 22:43:20 |
| 31 | 20081129 | 3301301 | W | 6961 | 7107 | 22:49:25 | 22:51:51 |
| 31 | 20081129 | 3301501 | E | 7522 | 7667 | 22:58:46 | 23:01:11 |
| 31 | 20081129 | 90403101 |  | 8201 | 8323 | 23:10:05 | 23:12:07 |
| 31 | 20081129 | 90503101 |  | 8396 | 8518 | 23:13:20 | 23:15:22 |
| 31 | 20081129 | 90603101 |  | 8550 | 8671 | 23:15:54 | 23:17:55 |
| 31 | 20081129 | 60503101 |  | 9569 | 9695 | 23:32:53 | 23:34:59 |
| 32 | 20081201 | 50503201 |  | 169 | 291 | 20:45:36 | 20:47:38 |
| 32 | 20081201 | 90003201 |  | 993 | 1116 | 20:59:20 | 21:01:23 |
| 32 | 20081201 | 90103201 |  | 1166 | 1290 | 21:02:13 | 21:04:17 |
| 32 | 20081201 | 90203201 |  | 1317 | 1439 | 21:04:44 | 21:06:46 |
| 32 | 20081201 | 90303201 |  | 1468 | 1523 | 21:07:15 | 21:08:10 |
| 32 | 20081201 | 1001501 | E | 2048 | 4386 | 21:16:55 | 21:55:53 |
| 32 | 20081201 | 3301202 | W | 5489 | 5675 | 22:14:16 | 22:17:22 |
| 32 | 20081201 | 3301402 | W | 5959 | 6142 | 22:22:06 | 22:25:09 |
| 32 | 20081201 | 3301701 | E | 6229 | 6405 | 22:26:36 | 22:29:32 |
| 32 | 20081201 | 3301601 | W | 6531 | 6716 | 22:31:38 | 22:34:43 |
| 32 | 20081201 | 3301901 | E | 6811 | 6989 | 22:36:18 | 22:39:16 |
| 32 | 20081201 | 3302001 | W | 7080 | 7264 | 22:40:47 | 22:43:51 |
| 32 | 20081201 | 3302101 | E | 7365 | 7543 | 22:45:32 | 22:48:30 |
| 32 | 20081201 | 3302201 | W | 7627 | 7810 | 22:49:54 | 22:52:57 |
| 32 | 20081201 | 3302301 | E | 7932 | 8104 | 22:54:59 | 22:57:51 |
| 32 | 20081201 | 3302401 | W | 8196 | 8376 | 22:59:23 | 23:02:23 |
| 32 | 20081201 | 3302701 | E | 8477 | 8646 | 23:04:04 | 23:06:53 |
| 32 | 20081201 | 3302601 | W | 8741 | 8919 | 23:08:28 | 23:11:26 |
| 32 | 20081201 | 3302901 | E | 9022 | 9193 | 23:13:09 | 23:16:00 |
| 32 | 20081201 | 3302801 | W | 9294 | 9472 | 23:17:41 | 23:20:39 |
| 32 | 20081201 | 3303101 | E | 9575 | 9747 | 23:22:22 | 23:25:14 |
| 32 | 20081201 | 3303001 | W | 9833 | 10013 | 23:26:40 | 23:29:40 |
| 32 | 20081201 | 3200301 | E | 10400 | 10552 | 23:36:07 | 23:38:39 |
| 32 | 20081201 | 3200201 | W | 10677 | 10831 | 23:40:44 | 23:43:18 |
| 32 | 20081201 | 90403201 |  | 11091 | 11216 | 23:47:38 | 23:49:43 |
| 32 | 20081201 | 90503201 |  | 11243 | 11366 | 23:50:10 | 23:52:13 |
| 32 | 20081201 | 90603201 |  | 11454 | 11577 | 23:53:41 | 23:55:44 |
| 32 | 20081201 | 60503201 |  | 12293 | 12414 | 00:07:40 | 00:09:41 |

Note: Lines 1102201, 1102301, 3200201 and 3200301 were not delivered as part of the Woolner final dataset, but will be reflown and delivered with the Rum Jungle final data.

APPENDIX III – Pre and Post-flight Zero Statistics

Pre-zero Additive Noise

- black trace represents the contract maximum standard deviation

- coloured traces represent the standard deviation of individual lines from all production flights

Woolner Flts 1-14

Woolner Flts 14-32

Post-zero Additive Noise

- black trace represents the contract maximum standard deviation

- coloured traces represent the standard deviation of individual lines from all production flights

Woolner Flts 1-14

Woolner Flts 14-32

Pre-zero Window Bias

- black trace represents the contract maximum mean for each line

- coloured traces represent the mean of individual lines from all production flights

Woolner Flts 1-14

Woolner Flts 14-32

Post-zero Window Bias

- black trace represents the contract maximum mean for each line

- coloured traces represent the mean of individual lines from all production flights

Woolner Flts 1-14

Woolner Flts 14-32

APPENDIX IV – Located Data Format

EM Window data – survey lines

COMM GA PROJECT NUMBER                                                 1196

COMM FAS PROJECT NUMBER                                                2017

COMM AREA NUMBER:                                                         1

COMM SURVEY COMPANY:                                 Fugro Airborne Surveys

COMM CLIENT:                                           Geoscience Australia

COMM SURVEY TYPE:                                       25Hz TEMPEST Survey

COMM AREA NAME:                                                     Woolner

COMM STATE:                                                              NT

COMM COUNTRY:                                                     Australia

COMM SURVEY FLOWN:                                 October to November 2008

COMM LOCATED DATA CREATED:                                       April 2009

COMM

COMM DATUM:                                                           GDA94

COMM PROJECTION:                                                        MGA

COMM ZONE:                                                               52

COMM

COMM SURVEY SPECIFICATIONS

COMM

COMM TRAVERSE LINE SPACING:                                      238-5000 m

COMM TRAVERSE LINE DIRECTION:                                   090-270 deg

COMM NOMINAL TERRAIN CLEARANCE:                                       120 m

COMM FINAL LINE KILOMETRES:                                         6862 km

COMM

COMM LINE NUMBERING

COMM

COMM TRAVERSE LINE NUMBERS:                               1000101 - 1102301

COMM                                                      2800101 - 2802901

COMM                                                      2900101 - 2901502

COMM                                                      3000103 - 3303101

COMM                                                      4100201 - 4300503

COMM                                                      5000101 - 5200101

COMM                                                      6100301 - 6601401

COMM

COMM SURVEY EQUIPMENT

COMM

COMM AIRCRAFT:                                 CASA C212 Turbo Prop, VH-TEM

COMM

COMM MAGNETOMETER:                              Scintrex Cs-2 Cesium Vapour

COMM INSTALLATION:                                            stinger mount

COMM RESOLUTION:                                                   0.001 nT

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM ELECTROMAGNETIC SYSTEM:                                   25Hz TEMPEST

COMM INSTALLATION:                 Transmitter loop mounted on the aircraft

COMM                                         Receiver coils in a towed bird

COMM COIL ORIENTATION:                                                  X,Z

COMM RECORDING INTERVAL:                                              0.2 s

COMM SYSTEM GEOMETRY:

COMM RECEIVER DISTANCE BEHIND THE TRANSMITTER:                       -120 m

COMM RECEIVER DISTANCE BELOW THE TRANSMITTER:                         -35 m

COMM

COMM RADAR ALTIMETER:                                         Sperry RT-220

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM LASER ALTIMETER:                                    Optech 501SB (TEM)

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM NAVIGATION:                                 real-time differential GPS

COMM RECORDING INTERVAL:                                              1.0 s

COMM

COMM ACQUISITION SYSTEM:                                             FASDAS

COMM

COMM DATA PROCESSING

COMM

COMM MAGNETIC DATA

COMM DIURNAL BASE VALUE APPLIED                                    47436 nT

COMM PARALLAX CORRECTION APPLIED (during final processing)            1.0 s

COMM IGRF BASE VALUE APPLIED                                       46836 nT

COMM IGRF MODEL 2005 EXTRAPOLATED TO                                 2008.8

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM ELECTROMAGNETIC DATA

COMM SYSTEM PARALLAX REMOVED, AS FOLLOWS

COMM X-COMPONENT EM DATA                                              1.6 s

COMM Z-COMPONENT EM DATA                                              0.2 s

COMM DATA CORRECTED FOR TRANSMITTER HEIGHT, PITCH AND ROLL

COMM DATA CORRECTED FOR TRANSMITTER-RECEIVER GEOMETRY VARIATIONS

COMM DATA HAVE BEEN MICROLEVELLED

COMM CONDUCTIVITY DEPTH INVERSION CALCULATED                   EMFlow V5.10

COMM CONDUCTIVITIES CALCULATED USING HPRG CORRECTED EMX & EMZ DATA

COMM

COMM DIGITAL TERRAIN DATA

COMM PARALLAX CORRECTION APPLIED TO LIDAR ALIMETER DATA               0.0 s

COMM PARALLAX CORRECTION APPLIED TO GPS ALIMETER DATA                 0.0 s

COMM DTM CALCULATED [DTM = GPS ALTITUDE - LIDAR ALTITUDE - GPS/LIDAR DIST]

COMM N-VALUE APPLIED TO CORRECT DTM TO AUSTRALIAN HEIGHT DATUM (AHD)

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM ----------------------------------------------------------------------

COMM DISCLAIMER

COMM ----------------------------------------------------------------------

COMM It is Fugro Airborne Survey’s understanding that the data provided to

COMM the client is to be used for the purpose agreed between the parties.

COMM That purpose was a significant factor in determining the scope and

COMM level of the Services being offered to the Client. Should the purpose

COMM for which the data is used change, the data may no longer be valid or

COMM appropriate and any further use of, or reliance upon, the data in

COMM those circumstances by the Client without Fugro Airborne Survey's

COMM review and advice shall be at the Client's own or sole risk.

COMM

COMM The Services were performed by Fugro Airborne Survey exclusively for

COMM the purposes of the Client. Should the data be made available in whole

COMM or part to any third party, and such party relies thereon, that party

COMM does so wholly at its own and sole risk and Fugro Airborne Survey

COMM disclaims any liability to such party.

COMM

COMM Where the Services have involved Fugro Airborne Survey's use of any

COMM information provided by the Client or third parties, upon which

COMM Fugro Airborne Survey was reasonably entitled to rely, then the

COMM Services are limited by the accuracy of such information. Fugro

COMM Airborne Survey is not liable for any inaccuracies (including any

COMM incompleteness) in the said information, save as otherwise provided

COMM in the terms of the contract between the Client and Fugro Airborne

COMM Survey.

COMM

COMM With regard to DIGITAL TERRAIN DATA, the accuracy of the elevation

COMM calculation is directly dependent on the accuracy of the two input

COMM parameters lidar altitude and GPS altitude. The radar altitude value may be

COMM erroneous in areas of heavy tree cover, where the altimeter reflects the

COMM distance to the tree canopy rather than the ground. The GPS altitude value

COMM is primarily dependent on the number of available satellites. Although

COMM post-processing of GPS data will yield X and Y accuracies in the

COMM order of 1-2 metres, the accuracy of the altitude value is usually

COMM much less, sometimes in the ±5 metre range.  Further inaccuracies

COMM may be introduced during the interpolation and gridding process.

COMM Because of the inherent inaccuracies of this method, no guarantee is

COMM made or implied that the information displayed is a true

COMM representation of the height above sea level.  Although this product

COMM may be of some use as a general reference,

COMM THIS PRODUCT MUST NOT BE USED FOR NAVIGATION PURPOSES.

COMM ----------------------------------------------------------------------

COMM

COMM ELECTROMAGNETIC SYSTEM

COMM

COMM TEMPEST IS A TIME-DOMAIN SQUARE-WAVE SYSTEM,

COMM TRANSMITTING AT A BASE FREQUENCY OF 25Hz,

COMM WITH 2 ORTHOGONAL-AXIS RECEIVER COILS IN A TOWED BIRD.

COMM FINAL EM OUTPUT IS RECORDED 5 TIMES PER SECOND.

COMM THE TIMES (IN MILLISECONDS) FOR THE 15 WINDOWS ARE:

COMM

COMM WINDOW      START       END      CENTRE

COMM    1        0.007      0.020      0.013

COMM    2        0.033      0.047      0.040

COMM    3        0.060      0.073      0.067

COMM    4        0.087      0.127      0.107

COMM    5        0.140      0.207      0.173

COMM    6        0.220      0.340      0.280

COMM    7        0.353      0.553      0.453

COMM    8        0.567      0.873      0.720

COMM    9        0.887      1.353      1.120

COMM   10        1.367      2.100      1.733

COMM   11        2.113      3.273      2.693

COMM   12        3.287      5.113      4.200

COMM   13        5.127      7.993      6.560

COMM   14        8.007     12.393     10.200

COMM   15       12.407     19.993     16.200

COMM

COMM PULSE WIDTH: 10 ms

COMM

COMM TEMPEST EM data are transformed to the response that would be

COMM obtained with a B-field sensor for a 100% duty cycle square

COMM waveform at the base frequency, involving a 1A change in

COMM current (from -0.5A to +0.5A to -0.5A) in a 1sq.m transmitter.

COMM It is this configuration, rather than the actual acquisition

COMM configuration, which must be specified when modelling TEMPEST data.

COMM

COMM

COMM

COMM LOCATED DATA FORMAT

COMM

COMM  Output field format : DOS  - Flat ascii

COMM  Number of fields    : 102

COMM

COMM Field  Channel         Description                       Units  Undefined    Format

COMM -----  -------         -----------                       -----  ---------    ------

COMM

COMM     1  LINE            Line                                      -99999999      i10

COMM     2  FLIGHT          Flight                                    -99             i4

COMM     3  FID             Fiducial                            (s)   -999999.9     f8.1

COMM     4  PROJECT_FAS     FAS Project Number                        -9999           i6

COMM     5  PROJECT_GA      GA Project Number                         -9999           i6

COMM     6  AIRCRAFT        System Number                             -9              i3

COMM     7  DATE            Date                           ddmmyyyy   -9999999        i9

COMM     8  TIME            Time - local midnight               (s)   -9999.9       f8.1

COMM     9  BEARING         Line Bearing                      (deg)   -99             i4

COMM    10  LATITUDE        Latitude  GDA94                   (deg)   -99.9999999  f12.7

COMM    11  LONGITUDE       Longitude GDA94                   (deg)   -999.9999999 f13.7

COMM    12  EASTING         Easting   MGA51                     (m)   -99999.99    f10.2

COMM    13  NORTHING        Northing  MGA51                     (m)   -999999.99   f11.2

COMM    14  LIDAR           Final Lidar altimeter               (m)   -999.99       f8.2

COMM    15  RADALT          Final Radar altimeter               (m)   -999.99       f8.2

COMM    16  TX_ELEVATION    Final Transmitter Elevation - AHD   (m)   -999.99       f8.2

COMM    17  DTM             Final Ground Elevation - AHD        (m)   -999.99       f8.2

COMM    18  MAG             Final TMI                          (nT)   -99999.999   f11.3

COMM    19  TX_PITCH        Transmitter loop pitch            (deg)   -999.99       f8.2

COMM    20  TX_ROLL         Transmitter loop roll             (deg)   -999.99       f8.2

COMM    21  TX_HEIGHT       Transmitter terrain clearance       (m)   -999.99       f8.2

COMM    22  HSep_Raw        Tx-Rx horizontal separation         (m)   -999.99       f8.2

COMM    23  VSep_Raw        Tx-Rx vertical separation           (m)   -999.99       f8.2

COMM    24  TX_HEIGHT_std   Tx HPRG terrain clearance           (m)   -999.99       f8.2

COMM    25  HSep_std        Tx-Rx HPRG horizontal separation    (m)   -999.99       f8.2

COMM    26  VSep_std        Tx-Rx HPRG vertical separation      (m)   -999.99       f8.2

COMM    27  EMX_nonhprg[1]  Raw (non-HPRG) EMX01 Window        (fT)   -999.999999  f12.6

COMM    28  EMX_nonhprg[2]  Raw (non-HPRG) EMX02 Window        (fT)   -999.999999  f12.6

COMM    29  EMX_nonhprg[3]  Raw (non-HPRG) EMX03 Window        (fT)   -999.999999  f12.6

COMM    30  EMX_nonhprg[4]  Raw (non-HPRG) EMX04 Window        (fT)   -999.999999  f12.6

COMM    31  EMX_nonhprg[5]  Raw (non-HPRG) EMX05 Window        (fT)   -999.999999  f12.6

COMM    32  EMX_nonhprg[6]  Raw (non-HPRG) EMX06 Window        (fT)   -999.999999  f12.6

COMM    33  EMX_nonhprg[7]  Raw (non-HPRG) EMX07 Window        (fT)   -999.999999  f12.6

COMM    34  EMX_nonhprg[8]  Raw (non-HPRG) EMX08 Window        (fT)   -999.999999  f12.6

COMM    35  EMX_nonhprg[9]  Raw (non-HPRG) EMX09 Window        (fT)   -999.999999  f12.6

COMM    36  EMX_nonhprg[10] Raw (non-HPRG) EMX10 Window        (fT)   -999.999999  f12.6

COMM    37  EMX_nonhprg[11] Raw (non-HPRG) EMX11 Window        (fT)   -999.999999  f12.6

COMM    38  EMX_nonhprg[12] Raw (non-HPRG) EMX12 Window        (fT)   -999.999999  f12.6

COMM    39  EMX_nonhprg[13] Raw (non-HPRG) EMX13 Window        (fT)   -999.999999  f12.6

COMM    40  EMX_nonhprg[14] Raw (non-HPRG) EMX14 Window        (fT)   -999.999999  f12.6

COMM    41  EMX_nonhprg[15] Raw (non-HPRG) EMX15 Window        (fT)   -999.999999  f12.6

COMM    42  EMX_hprg[1]     Final HPRG EMX01 Window            (fT)   -999.999999  f12.6

COMM    43  EMX_hprg[2]     Final HPRG EMX02 Window            (fT)   -999.999999  f12.6

COMM    44  EMX_hprg[3]     Final HPRG EMX03 Window            (fT)   -999.999999  f12.6

COMM    45  EMX_hprg[4]     Final HPRG EMX04 Window            (fT)   -999.999999  f12.6

COMM    46  EMX_hprg[5]     Final HPRG EMX05 Window            (fT)   -999.999999  f12.6

COMM    47  EMX_hprg[6]     Final HPRG EMX06 Window            (fT)   -999.999999  f12.6

COMM    48  EMX_hprg[7]     Final HPRG EMX07 Window            (fT)   -999.999999  f12.6

COMM    49  EMX_hprg[8]     Final HPRG EMX08 Window            (fT)   -999.999999  f12.6

COMM    50  EMX_hprg[9]     Final HPRG EMX09 Window            (fT)   -999.999999  f12.6

COMM    51  EMX_hprg[10]    Final HPRG EMX10 Window            (fT)   -999.999999  f12.6

COMM    52  EMX_hprg[11]    Final HPRG EMX11 Window            (fT)   -999.999999  f12.6

COMM    53  EMX_hprg[12]    Final HPRG EMX12 Window            (fT)   -999.999999  f12.6

COMM    54  EMX_hprg[13]    Final HPRG EMX13 Window            (fT)   -999.999999  f12.6

COMM    55  EMX_hprg[14]    Final HPRG EMX14 Window            (fT)   -999.999999  f12.6

COMM    56  EMX_hprg[15]    Final HPRG EMX15 Window            (fT)   -999.999999  f12.6

COMM    57  X_Sferics       X_Sferics                                 -9999.999    f10.3

COMM    58  X_Lowfreq       X_Lowfreq                                 -9999.999    f10.3

COMM    59  X_Powerline     X_Powerline                               -9999.999    f10.3

COMM    60  X_VLF1          X_18.2kHz                                 -9999.999    f10.3

COMM    61  X_VLF2          X_19.8kHz                                 -9999.999    f10.3

COMM    62  X_VLF3          X_21.4kHz                                 -9999.999    f10.3

COMM    63  X_VLF4          X_22.2kHz                                 -9999.999    f10.3

COMM    64  X_Geofact       X_Geometric factor                        -9999.999    f10.3

COMM    65  EMZ_nonhprg[1]  Raw (non-HPRG) EMZ01 Window        (fT)   -999.999999  f12.6

COMM    66  EMZ_nonhprg[2]  Raw (non-HPRG) EMZ02 Window        (fT)   -999.999999  f12.6

COMM    67  EMZ_nonhprg[3]  Raw (non-HPRG) EMZ03 Window        (fT)   -999.999999  f12.6

COMM    68  EMZ_nonhprg[4]  Raw (non-HPRG) EMZ04 Window        (fT)   -999.999999  f12.6

COMM    69  EMZ_nonhprg[5]  Raw (non-HPRG) EMZ05 Window        (fT)   -999.999999  f12.6

COMM    70  EMZ_nonhprg[6]  Raw (non-HPRG) EMZ06 Window        (fT)   -999.999999  f12.6

COMM    71  EMZ_nonhprg[7]  Raw (non-HPRG) EMZ07 Window        (fT)   -999.999999  f12.6

COMM    72  EMZ_nonhprg[8]  Raw (non-HPRG) EMZ08 Window        (fT)   -999.999999  f12.6

COMM    73  EMZ_nonhprg[9]  Raw (non-HPRG) EMZ09 Window        (fT)   -999.999999  f12.6

COMM    74  EMZ_nonhprg[10] Raw (non-HPRG) EMZ10 Window        (fT)   -999.999999  f12.6

COMM    75  EMZ_nonhprg[11] Raw (non-HPRG) EMZ11 Window        (fT)   -999.999999  f12.6

COMM    76  EMZ_nonhprg[12] Raw (non-HPRG) EMZ12 Window        (fT)   -999.999999  f12.6

COMM    77  EMZ_nonhprg[13] Raw (non-HPRG) EMZ13 Window        (fT)   -999.999999  f12.6

COMM    78  EMZ_nonhprg[14] Raw (non-HPRG) EMZ14 Window        (fT)   -999.999999  f12.6

COMM    79  EMZ_nonhprg[15] Raw (non-HPRG) EMZ15 Window        (fT)   -999.999999  f12.6

COMM    80  EMZ_hprg[1]     Final HPRG EMZ01 Window            (fT)   -999.999999  f12.6

COMM    81  EMZ_hprg[2]     Final HPRG EMZ02 Window            (fT)   -999.999999  f12.6

COMM    82  EMZ_hprg[3]     Final HPRG EMZ03 Window            (fT)   -999.999999  f12.6

COMM    83  EMZ_hprg[4]     Final HPRG EMZ04 Window            (fT)   -999.999999  f12.6

COMM    84  EMZ_hprg[5]     Final HPRG EMZ05 Window            (fT)   -999.999999  f12.6

COMM    85  EMZ_hprg[6]     Final HPRG EMZ06 Window            (fT)   -999.999999  f12.6

COMM    86  EMZ_hprg[7]     Final HPRG EMZ07 Window            (fT)   -999.999999  f12.6

COMM    87  EMZ_hprg[8]     Final HPRG EMZ08 Window            (fT)   -999.999999  f12.6

COMM    88  EMZ_hprg[9]     Final HPRG EMZ09 Window            (fT)   -999.999999  f12.6

COMM    89  EMZ_hprg[10]    Final HPRG EMZ10 Window            (fT)   -999.999999  f12.6

COMM    90  EMZ_hprg[11]    Final HPRG EMZ11 Window            (fT)   -999.999999  f12.6

COMM    91  EMZ_hprg[12]    Final HPRG EMZ12 Window            (fT)   -999.999999  f12.6

COMM    92  EMZ_hprg[13]    Final HPRG EMZ13 Window            (fT)   -999.999999  f12.6

COMM    93  EMZ_hprg[14]    Final HPRG EMZ14 Window            (fT)   -999.999999  f12.6

COMM    94  EMZ_hprg[15]    Final HPRG EMZ15 Window            (fT)   -999.999999  f12.6

COMM    95  Z_Sferics       Z_Sferics                                 -9999.999    f10.3

COMM    96  Z_Lowfreq       Z_Lowfreq                                 -9999.999    f10.3

COMM    97  Z_Powerline     Z_Powerline                               -9999.999    f10.3

COMM    98  Z_VLF1          Z_18.2kHz                                 -9999.999    f10.3

COMM    99  Z_VLF2          Z_19.8kHz                                 -9999.999    f10.3

COMM   100  Z_VLF3          Z_21.4kHz                                 -9999.999    f10.3

COMM   101  Z_VLF4          Z_22.2kHz                                 -9999.999    f10.3

COMM   102  Z_Geofact       Z_Geometric factor                        -9999.999    f10.3

COMM

COMM Line Number    X-minimum      X-maximum      Y-minimum      Y-maximum      # of points Total distance

COMM -----------    ---------      ---------      ---------      --------       ----------- --------------

COMM L1000101:15    701699.649     755735.871     8598930.412    8598968.032    4067        54039.886185

COMM L1000201:14    772068.207     788661.980     8598929.413    8598948.284    1187        16594.68325671

COMM L1000301:26    665068.950     673106.890     8593932.710    8593951.015    587         8038.312239841

COMM L1000401:15    701668.710     762773.967     8593915.687    8593969.139    4255        61112.24761159

COMM L1000501:16    771902.492     787461.491     8593926.073    8593956.247    1082        15560.01208714

COMM L1000601:26    665028.782     673114.007     8588929.863    8588946.515    581         8085.413592158

COMM L1000701:16    701630.042     786326.716     8588753.658    8589014.670    6250        84750.80188417

COMM L1000801:26    661356.010     672870.726     8583927.396    8583954.635    834         11515.68305178

COMM L1000901:16    701604.008     783214.729     8583888.921    8583974.947    5812        81630.50498423

COMM L1001001:26    648784.908     672810.212     8578927.246    8578943.893    1744        24025.86922623

COMM L1001101:17    701573.665     781185.843     8578796.729    8579045.528    5934        79649.63924522

COMM L1001201:18    640131.229     783179.249     8573894.275    8574072.448    10225       143077.5668749

COMM L1001301:25    786265.992     801716.058     8573912.754    8573960.395    1114        15455.56091809

COMM L1001401:18    632030.903     787862.563     8568924.538    8568980.949    11527       155839.3222702

COMM L1001501:32    626097.562     787965.201     8563895.226    8563966.772    11471       161891.7224076

COMM L1100101:17    703412.927     737089.658     8590392.827    8590679.382    2396        33730.55523818

COMM L1100302:22    703425.988     737010.183     8587191.670    8587440.317    2455        33619.86523188

COMM L1100402:22    703426.527     736983.536     8585595.994    8585735.860    2411        33569.35971734

COMM L1100602:22    703368.660     736954.479     8582250.991    8582310.062    2416        33592.61775769

COMM L1100702:25    703367.811     736950.876     8580582.247    8580735.818    2410        33600.81996603

COMM L1100901:22    703402.699     736886.226     8577251.793    8577296.300    2494        33487.54320863

COMM L1101001:22    703336.282     736887.723     8575599.873    8575631.461    2415        33553.13927627

COMM L1101201:22    703307.540     736910.005     8572269.610    8572292.654    2422        33603.76590959

COMM L1101301:22    703307.635     736880.380     8570576.829    8570642.146    2479        33579.25229863

COMM L1101501:22    703289.497     736817.273     8567221.157    8567292.258    2455        33535.03962871

COMM L1101701:22    708971.133     736823.359     8565592.437    8565629.569    2009        27854.76621012

COMM L1101901:22    703249.975     736789.062     8562269.461    8562307.967    2408        33542.60500707

COMM L1102001:22    703258.349     736782.540     8560587.675    8560630.890    2445        33527.65579076

COMM L2800101:14    749660.766     757720.612     8597589.903    8597608.604    567         8060.349044366

COMM L2800201:14    750100.598     758179.274     8597265.369    8597293.321    593         8079.132004985

COMM L2800301:14    750545.428     758635.236     8596921.488    8596951.180    573         8091.400555112

COMM L2800401:14    750219.062     759115.658     8596606.353    8596617.980    647         8896.842526245

COMM L2800501:14    749889.823     759575.187     8596254.182    8596283.871    686         9687.397935591

COMM L2800601:16    749550.744     760049.664     8595914.663    8595950.719    752         10501.32977696

COMM L2800701:16    749214.182     760580.571     8595566.481    8595612.925    817         11367.48935581

COMM L2800801:16    748882.338     760983.338     8595255.627    8595283.130    862         12101.51327241

COMM L2800901:16    748620.047     761486.339     8594919.224    8594948.157    924         12867.46105177

COMM L2801001:16    748610.876     761909.183     8594586.586    8594620.862    948         13299.45075511

COMM L2801101:16    748622.235     762511.806     8594203.325    8594282.189    985         13894.35570009

COMM L2801301:17    748597.525     762491.196     8593595.484    8593620.179    1018        13894.94865052

COMM L2801401:17    748602.015     762487.953     8593257.247    8593284.763    1002        13886.89549317

COMM L2801501:17    748603.596     762499.052     8592917.288    8592950.280    1013        13896.69468186

COMM L2801601:17    748606.005     762506.208     8592590.852    8592614.001    1003        13900.83671288

COMM L2801701:17    748613.290     762514.983     8592237.433    8592288.741    1005        13903.98846913

COMM L2801801:17    748618.723     762524.356     8591926.917    8591947.651    1007        13906.7287543

COMM L2801901:18    750610.439     765090.777     8591595.852    8591663.129    1028        14484.80912762

COMM L2802001:17    751533.885     780986.610     8591252.400    8591289.853    2129        29455.95804262

COMM L2802101:16    751515.189     786339.274     8590901.082    8590974.524    2540        34833.64237222

COMM L2802201:17    751491.663     786348.064     8590515.327    8590683.986    2500        34878.81511054

COMM L2802301:17    751468.119     786339.912     8590154.695    8590364.768    2536        34891.41084106

COMM L2802401:17    751438.656     786326.017     8589885.824    8589971.175    2468        34899.06015242

COMM L2802501:17    751460.799     786325.177     8589487.824    8589754.662    2556        34928.2904017

COMM L2802601:18    769553.537     786318.467     8589263.139    8589281.188    1202        16765.98970967

COMM L2802801:18    769499.129     786315.673     8588562.346    8588621.292    1193        16819.46377093

COMM L2802901:18    769494.754     786315.928     8588247.895    8588419.128    1213        16843.79103514

COMM L2900101:18    770261.157     783209.886     8584595.863    8584619.326    911         12949.6503976

COMM L2900201:18    770248.364     783209.653     8584253.371    8584284.195    948         12962.48091359

COMM L2900401:18    770239.851     783216.150     8583599.599    8583627.844    951         12977.34186883

COMM L2900501:18    770248.500     783220.493     8583260.572    8583287.920    906         12972.96892787

COMM L2900601:18    770244.726     783212.026     8582926.479    8582970.154    959         12970.86049936

COMM L2900702:25    770245.835     783219.791     8582588.940    8582620.408    934         12974.61457866

COMM L2900801:19    770248.626     783228.780     8582259.971    8582282.029    929         12981.1348468

COMM L2900901:19    770252.414     783225.772     8581930.006    8581954.285    939         12974.93311861

COMM L2901001:19    770248.482     783228.792     8581583.802    8581635.602    923         12983.81744507

COMM L2901101:19    770243.960     783226.859     8581261.430    8581287.458    943         12983.44570371

COMM L2901201:19    770250.908     783238.731     8580931.425    8580956.444    920         12988.51390562

COMM L2901302:25    770242.716     783237.467     8580585.419    8580616.428    935         12996.45135201

COMM L2901401:19    770251.143     783231.018     8580267.231    8580293.535    918         12980.81610607

COMM L2901502:25    770247.703     783240.074     8579927.863    8579945.300    928         12993.10191638

COMM L3000103:24    786408.450     801684.850     8576234.665    8576279.519    1084        15278.16275695

COMM L3000203:24    786421.824     801697.732     8575916.200    8575949.711    1128        15277.66903479

COMM L3000301:24    786430.176     801700.961     8575587.958    8575613.207    1068        15271.81633116

COMM L3000402:25    786445.815     801696.705     8575265.584    8575280.212    1154        15251.62425591

COMM L3000502:25    786449.578     801701.169     8574932.701    8574945.292    1060        15251.95929669

COMM L3000602:25    786464.388     801715.930     8574594.771    8574627.003    1134        15253.30556705

COMM L3000701:24    786469.302     801713.745     8574262.995    8574307.416    1081        15246.54643568

COMM L3000902:25    786519.508     801711.613     8573595.864    8573625.082    1057        15193.45883508

COMM L3001001:24    786527.079     801692.705     8573266.941    8573281.925    1105        15166.2016407

COMM L3001102:25    786538.073     801674.460     8572927.942    8572951.859    1062        15137.59722351

COMM L3001202:25    786540.063     801659.030     8572592.781    8572627.351    1119        15120.98332133

COMM L3001301:25    786550.665     801643.771     8572260.253    8572280.334    1066        15094.11340808

COMM L3001401:25    786563.375     801619.361     8571928.547    8571949.563    1112        15057.41724108

COMM L3001501:25    786567.828     801601.628     8571447.308    8571615.362    1080        15058.03627977

COMM L3001601:25    786585.932     801588.716     8571260.580    8571286.453    1102        15004.04478285

COMM L3001702:26    786586.543     801570.547     8570933.117    8571025.129    1105        14993.39843432

COMM L3001802:26    786591.991     801548.383     8570582.757    8570614.399    1062        14956.89633035

COMM L3100104:30    703383.520     711436.565     8580244.636    8580304.395    583         8058.28046207

COMM L3100204:30    703382.487     711444.624     8579938.376    8580011.075    584         8066.646199768

COMM L3100304:30    703385.863     711454.785     8579596.434    8579627.490    580         8070.732445964

COMM L3100404:30    703396.590     711450.836     8579244.820    8579353.629    580         8062.040269139

COMM L3100604:30    703358.083     711460.894     8578593.113    8578631.216    589         8103.901442525

COMM L3100704:30    703371.465     711461.260     8578266.648    8578284.670    576         8090.413566565

COMM L3100804:30    703383.207     711479.035     8577928.982    8577960.639    588         8096.68820777

COMM L3100904:30    703387.829     711474.691     8577564.961    8577615.182    577         8089.805879697

COMM L3101104:30    703408.664     711458.673     8576925.527    8576961.352    583         8051.577068533

COMM L3101204:30    703422.494     711466.310     8576596.499    8576620.684    589         8044.707766341

COMM L3101304:30    703438.997     711475.924     8576269.517    8576285.492    582         8037.38240057

COMM L3101404:30    703442.543     711478.985     8575932.158    8575965.130    578         8036.989174333

COMM L3101604:30    703470.368     711509.821     8575260.708    8575305.479    581         8040.433109175

COMM L3101704:30    703468.205     711507.061     8574926.816    8574952.562    585         8039.482734948

COMM L3101805:31    703465.101     711505.002     8574599.809    8574622.763    557         8040.638211187

COMM L3101905:31    703457.958     711514.064     8574264.192    8574287.318    610         8056.599354282

COMM L3102104:30    703484.058     711539.229     8573592.011    8573620.267    591         8056.039267879

COMM L3102204:30    703485.558     711548.135     8573262.859    8573279.923    574         8062.94885347

COMM L3102303:30    703504.153     711547.545     8572926.164    8572955.694    595         8044.784314324

COMM L3102403:30    703507.084     711557.529     8572602.002    8572615.347    574         8050.734155665

COMM L3102603:30    703473.794     711557.499     8571932.007    8571950.003    576         8084.185212064

COMM L3102703:30    703488.245     711561.476     8571595.320    8571617.024    602         8074.584270376

COMM L3102803:30    703510.580     711569.518     8571269.304    8571281.265    573         8059.388638711

COMM L3102903:30    703536.742     711579.866     8570934.549    8570948.045    594         8043.488985628

COMM L3103103:30    703523.968     711581.637     8570267.455    8570285.557    596         8058.429525366

COMM L3103203:30    703523.008     711576.313     8569933.250    8569949.418    571         8053.658415056

COMM L3103303:30    703552.952     711592.147     8569597.776    8569617.024    592         8039.851255946

COMM L3103403:30    703520.806     711567.077     8569262.352    8569282.747    568         8046.765768756

COMM L3300101:30    710253.621     719476.592     8569648.489    8569665.264    652         9223.239090885

COMM L3300201:30    710216.955     719480.191     8569415.736    8569436.432    688         9263.767953628

COMM L3300302:31    710199.132     719475.026     8569158.713    8569192.967    669         9277.200142738

COMM L3300501:31    710192.515     719469.237     8568689.204    8568717.348    675         9278.140372028

COMM L3300601:31    710207.317     719480.081     8568458.646    8568488.249    694         9274.450506544

COMM L3300701:31    710206.350     719480.591     8568196.508    8568243.332    681         9277.582894599

COMM L3300801:31    710204.106     719484.809     8567992.630    8568006.886    686         9281.265661583

COMM L3300901:31    710209.255     719483.064     8567738.591    8567767.556    666         9275.144666803

COMM L3301001:31    710211.505     719490.194     8567494.747    8567554.793    680         9287.646861308

COMM L3301202:32    710200.554     719509.662     8567037.265    8567049.900    710         9309.408894302

COMM L3301301:31    710199.409     719511.619     8566793.806    8566818.609    663         9313.464914218

COMM L3301402:32    710200.977     719515.314     8566554.073    8566576.514    704         9315.013903744

COMM L3301501:31    710206.628     719521.471     8566301.878    8566341.170    659         9318.06965651

COMM L3301601:32    710207.393     719531.450     8566080.027    8566100.197    710         9325.391505681

COMM L3301701:32    710214.342     719524.676     8565800.461    8565858.551    680         9313.560372921

COMM L3301901:32    710238.705     719522.521     8565359.176    8565394.139    685         9287.6503423

COMM L3302001:32    710240.805     719527.625     8565114.105    8565162.477    708         9292.89185908

COMM L3302101:32    710239.602     719527.331     8564875.959    8564932.100    684         9292.620581261

COMM L3302201:32    710243.016     719525.035     8564641.891    8564687.077    705         9287.240271192

COMM L3302301:32    710241.894     719531.790     8564416.477    8564431.334    671         9290.400903946

COMM L3302401:32    710248.399     719535.236     8564174.254    8564274.823    699         9297.606722386

COMM L3302601:32    710216.609     719545.363     8563703.105    8563720.186    702         9329.235370491

COMM L3302701:32    710210.803     719537.240     8563458.180    8563480.504    668         9327.575365182

COMM L3302801:32    710220.765     719551.865     8563229.039    8563245.707    701         9331.444380319

COMM L3302901:32    710222.787     719550.933     8562980.561    8563006.904    668         9329.076758445

COMM L3303001:32    710237.528     719554.336     8562747.284    8562770.027    702         9317.174645236

COMM L3303101:32    710235.463     719557.479     8562341.062    8562534.013    673         9353.149132624

COMM L4100201:26    665061.537     673115.507     8592264.147    8592298.725    575         8055.053220691

COMM L4100301:26    665041.411     673108.959     8590595.564    8590612.075    583         8067.856868137

COMM L4100501:26    665012.377     673113.954     8587267.276    8587287.716    585         8102.13035629

COMM L4100601:26    664997.331     673117.098     8585599.998    8585618.176    586         8120.179530468

COMM L4100801:26    659158.244     672773.542     8582254.955    8582301.002    970         13618.02155689

COMM L4100901:26    659100.918     672715.167     8580598.642    8580615.757    977         13614.83625323

COMM L4101101:26    656096.824     672763.232     8577251.107    8577303.359    1188        16670.23551805

COMM L4101201:26    656106.587     672716.142     8575599.038    8575616.724    1175        16610.23961642

COMM L4200101:26    686161.426     694399.351     8575587.208    8575619.457    604         8239.101499004

COMM L4200301:26    686145.346     694332.832     8572266.100    8572285.846    602         8187.982072498

COMM L4200401:26    686151.207     694337.005     8570589.299    8570613.415    576         8186.256864921

COMM L4300101:26    678499.733     686572.543     8567264.762    8567281.550    575         8073.132143029

COMM L4300203:29    678491.786     686552.846     8565592.971    8565618.822    571         8062.054728129

COMM L4300403:29    678386.909     686438.624     8562259.765    8562288.817    572         8052.609519281

COMM L4300503:29    678372.874     686427.787     8560592.700    8560628.299    617         8056.621934125

COMM L5000101:5     720107.187     750477.663     8638932.608    8638949.648    2131        30371.22105519

COMM L5000201:7     755595.385     779510.480     8638921.919    8638959.683    1689        23916.92980451

COMM L5000301:9     791801.502     833794.852     8638923.696    8638952.174    2911        41994.4787011

COMM L5000401:5     720088.298     779522.655     8633916.259    8633960.822    4366        59441.02660505

COMM L5000501:9     791765.519     834201.055     8633924.623    8633954.300    3160        42438.39642382

COMM L5000601:5     720038.993     779521.084     8628923.242    8628955.397    4202        59485.34164304

COMM L5000701:9     791705.967     834643.276     8628926.523    8628950.669    3012        42939.40164633

COMM L5000801:5     720004.158     779507.946     8623925.114    8623961.219    4425        59509.58544055

COMM L5000901:9     791654.121     833103.237     8623915.325    8623966.872    3079        41455.81916757

COMM L5001101:10    729468.284     829159.243     8618926.246    8618984.176    7388        99696.36127847

COMM L5001301:10    740523.693     755463.667     8613929.191    8613950.585    1061        14941.07570926

COMM L5001401:11    772159.893     815392.936     8613931.109    8613958.167    3224        43234.81794223

COMM L5001501:12    719861.814     755424.710     8608735.868    8609007.057    2707        35614.71717722

COMM L5001601:12    772119.377     814175.053     8608927.256    8608965.415    3163        42057.69236334

COMM L5001701:11    719867.177     755462.339     8603922.281    8604001.521    2519        35601.39600482

COMM L5001801:13    772018.891     818692.749     8603926.062    8603962.928    3376        46676.76210147

COMM L5002101:13    803599.282     818581.398     8598927.877    8598950.520    1071        14983.39539932

COMM L5002201:13    806150.179     818477.940     8593930.020    8593951.468    894         12328.6441434

COMM L5002303:25    804445.656     818369.080     8588933.546    8588949.537    970         13923.84854111

COMM L5002401:14    789366.529     823049.215     8583901.490    8583955.008    2542        33687.29354654

COMM L5002501:14    791966.769     799955.332     8578930.868    8578957.091    573         7989.772704591

COMM L5100301:7     720125.566     749173.845     8637242.447    8637291.555    2093        29051.01544539

COMM L5100401:5     752835.464     779510.211     8637260.142    8637298.920    1966        26677.57650386

COMM L5100501:9     791795.274     833917.906     8637252.353    8637295.886    3286        42125.56832445

COMM L5100601:5     720086.641     779518.249     8635594.839    8635627.971    4129        59435.83723522

COMM L5100701:9     791772.463     834059.573     8635589.055    8635633.797    2945        42290.9510729

COMM L5101001:5     720068.133     779513.032     8632250.496    8632290.023    4235        59447.87420924

COMM L5101101:9     791741.717     834257.563     8632256.762    8632280.647    3002        42517.37932542

COMM L5101201:5     720035.029     779517.853     8630590.028    8630691.648    4315        59491.79376936

COMM L5101301:9     791730.192     834530.292     8630585.077    8630624.858    3288        42803.22504307

COMM L5101601:6     720024.825     779517.102     8627256.988    8627292.417    4266        59494.48030395

COMM L5101701:9     791695.928     834809.037     8627249.468    8627301.377    3251        43119.68969402

COMM L5101801:6     720012.841     779510.268     8625591.981    8625617.957    4253        59499.27419609

COMM L5101902:10    791671.101     834124.861     8625585.858    8625618.330    3072        42456.63823381

COMM L5102201:10    729472.928     779489.198     8622258.655    8622288.773    3694        50018.78634031

COMM L5102302:14    791638.151     831535.254     8622251.213    8622293.165    2822        39899.94576484

COMM L5102401:10    729470.484     830226.489     8620577.087    8620643.864    7162        100762.4624626

COMM L5102601:11    729480.709     823473.426     8617233.363    8617458.435    6706        94020.72460431

COMM L5102701:11    732470.716     820684.363     8615584.152    8615622.037    6459        88219.02821431

COMM L5200101:1     717915.580     725948.096     8655600.052    8655611.742    546         8032.740474386

COMM L5200201:1     716770.857     730683.787     8653928.920    8653952.112    1055        13914.33333755

COMM L5200301:1     716736.102     737247.679     8652243.054    8652317.242    1397        20515.76079584

COMM L5200401:1     717346.590     738885.393     8650594.327    8650643.454    1622        21542.07568177

COMM L5200501:1     718776.882     740500.034     8648909.544    8648955.444    1491        21725.67049193

COMM L5200601:1     720121.005     742104.219     8647260.383    8647307.699    1642        21986.63814852

COMM L5200701:1     720123.156     747233.819     8645596.900    8645644.553    1869        27113.23970991

COMM L5200801:1     720125.528     748092.923     8643923.337    8643963.101    2103        27970.51270417

COMM L5200901:2     720112.534     748933.591     8642253.380    8642287.444    1953        28822.51095423

COMM L5201001:2     720106.457     749767.787     8640583.867    8640622.679    2212        29663.44316136

COMM L6100301:2     720234.610     748851.005     8638368.335    8638404.103    2211        28619.5251476

COMM L6100401:2     720224.936     748492.639     8637806.351    8637836.615    1913        28269.47175788

COMM L6100601:2     720214.052     747829.080     8636698.396    8636731.316    1957        27617.45261031

COMM L6100701:2     720234.027     747499.437     8636149.908    8636179.482    2086        27267.80760209

COMM L6100901:5     720211.701     746833.217     8635036.439    8635069.071    1933        26623.83004493

COMM L6101001:3     720232.829     754611.191     8634473.745    8634512.471    2244        34380.78267573

COMM L6101201:3     720189.249     754643.143     8633368.013    8633400.389    2273        34455.98478564

COMM L6101301:3     720172.634     754643.529     8632814.110    8632842.082    2729        34473.50816128

COMM L6101501:3     720177.871     754648.782     8631702.845    8631740.161    2678        34474.77264267

COMM L6101601:3     720189.605     754664.323     8631146.540    8631171.073    2292        34476.04407059

COMM L6101801:4     720160.343     754616.306     8630024.940    8630057.621    2424        34458.28865371

COMM L6101901:4     720176.515     754615.555     8629295.844    8629508.714    2512        34463.5884103

COMM L6102101:4     720154.811     754609.951     8628313.000    8628489.597    2488        34477.02925688

COMM L6102201:4     720162.275     754621.612     8627801.843    8627851.367    2502        34462.42959196

COMM L6102401:4     720132.923     754594.045     8626700.831    8626735.089    2513        34464.10987448

COMM L6102501:4     720143.340     754597.411     8626144.653    8626187.273    2464        34457.06339234

COMM L6102701:4     720126.684     754613.886     8625037.882    8625069.729    2474        34489.70018707

COMM L6102801:4     720127.952     754594.641     8624484.178    8624517.823    2513        34468.4574873

COMM L6103001:4     720092.756     754582.178     8623367.182    8623395.893    2487        34492.26146985

COMM L6103101:4     720121.032     754575.055     8622793.856    8623483.225    2491        34671.10941824

COMM L6103302:6     729471.099     754559.735     8621703.005    8621732.309    1822        25089.83504492

COMM L6103401:4     729476.869     754541.483     8621116.450    8621176.996    1792        25068.18035014

COMM L6103601:6     729474.739     754535.743     8620031.570    8620063.899    1820        25062.30064502

COMM L6103702:7     729475.966     754531.943     8619467.565    8619518.162    1805        25060.51261528

COMM L6103901:6     729473.648     754527.264     8618360.925    8618400.977    1827        25057.30104748

COMM L6104001:6     729471.987     754515.868     8617814.885    8617844.216    1823        25045.89264026

COMM L6104201:6     732461.947     754521.348     8616704.671    8616727.304    1602        22060.72103978

COMM L6104301:6     732470.170     754498.473     8616150.041    8616178.910    1607        22029.29196162

COMM L6104501:6     732479.579     754494.368     8615038.273    8615071.188    1610        22016.91311817

COMM L6104601:6     732352.558     754518.375     8614476.949    8614514.876    1608        22168.26320311

COMM L6104801:6     740675.357     754423.587     8613361.965    8613408.414    986         13750.46529065

COMM L6104901:6     740677.206     754437.120     8612821.053    8612897.804    1002        13762.85249556

COMM L6105001:6     740670.143     754443.667     8612256.439    8612287.328    988         13774.60505259

COMM L6105101:7     740686.458     754449.670     8611700.448    8611737.288    980         13764.34326173

COMM L6105201:6     740685.879     754461.161     8611135.023    8611179.319    993         13778.1996528

COMM L6105301:7     740691.502     754462.016     8610599.816    8610815.748    984         13792.74064857

COMM L6105401:7     740555.708     754464.535     8609842.196    8610073.339    1045        13946.86004767

COMM L6200201:7     768233.810     779513.612     8638377.189    8638404.078    826         11281.03835174

COMM L6200301:7     768237.667     779515.541     8637818.505    8637834.686    798         11278.14689954

COMM L6200501:8     768228.322     779510.914     8636707.166    8636728.786    766         11283.23776866

COMM L6200601:7     768239.811     779517.330     8636156.183    8636184.651    823         11278.66748014

COMM L6200801:8     768251.431     779510.687     8635029.768    8635060.002    855         11260.65792975

COMM L6200901:8     768274.769     779516.368     8634482.046    8634509.136    757         11242.43000695

COMM L6201101:8     768224.566     779508.957     8633358.979    8633390.952    760         11285.1463792

COMM L6201201:8     768221.876     779522.369     8632793.875    8633050.220    865         11350.42096331

COMM L6201401:8     768196.615     779520.699     8631654.029    8631946.103    863         11363.96527957

COMM L6201501:8     768220.253     779514.416     8631142.558    8631174.529    761         11295.07067942

COMM L6201701:8     768189.292     779514.332     8630024.933    8630063.011    791         11326.76367785

COMM L6201801:8     768211.558     779516.913     8629490.885    8629511.321    854         11305.91254559

COMM L6202001:8     768201.894     779516.791     8628367.351    8628400.015    885         11316.92503204

COMM L6202101:8     768223.926     779510.387     8627820.858    8627846.948    776         11287.17484631

COMM L6202301:8     768197.141     779513.179     8626709.185    8626731.144    787         11316.64742445

COMM L6202401:8     768194.932     779513.065     8626150.842    8626182.901    881         11319.74194937

COMM L6202601:8     768173.719     779517.557     8625035.270    8625063.529    879         11344.669094

COMM L6202701:6     768195.575     779513.842     8624479.346    8624502.432    793         11318.79292839

COMM L6202901:8     768202.162     779495.565     8623368.904    8623390.555    783         11294.25756658

COMM L6203001:8     768193.283     779497.510     8622809.879    8622840.285    867         11304.93008761

COMM L6203201:8     768189.363     779474.232     8621712.684    8621730.730    862         11285.68224547

COMM L6203301:8     768215.169     779466.765     8621156.794    8621176.118    783         11252.08377397

COMM L6203501:8     768196.247     786603.082     8620045.821    8620060.423    1277        18407.44138959

COMM L6203601:8     768198.013     786605.243     8619488.083    8619515.533    1410        18408.68834663

COMM L6203801:8     768188.383     786620.994     8618370.492    8618394.210    1391        18433.76172573

COMM L6203901:8     768198.700     786650.445     8617805.845    8617836.456    1286        18453.51605909

COMM L6204101:8     768154.391     786613.448     8616692.517    8616733.991    1290        18460.56810286

COMM L6204201:8     768169.993     786606.934     8616141.837    8616182.349    1401        18440.50247273

COMM L6204401:8     772160.354     786582.711     8615035.595    8615072.115    1093        14424.03168719

COMM L6204501:8     772162.139     786596.023     8614481.595    8614521.483    999         14436.83705216

COMM L6204701:10    772157.233     786606.816     8613339.361    8613403.776    1058        14451.83147161

COMM L6204801:10    772153.287     786596.881     8612822.885    8612857.814    1031        14444.754359

COMM L6204901:10    772144.455     786594.483     8612265.681    8612288.845    1051        14451.32984948

COMM L6205001:10    772140.339     786582.994     8611704.874    8611740.780    1038        14444.71478274

COMM L6205101:11    772147.241     786575.112     8611140.764    8611167.735    1039        14429.03503471

COMM L6205201:11    772133.120     786578.886     8610587.515    8610621.620    1065        14447.57304443

COMM L6205301:11    772127.450     786564.459     8610045.173    8610071.599    1027        14438.36160167

COMM L6205401:11    772131.476     786557.500     8609485.413    8609509.759    1063        14426.9372661

COMM L6205601:13    772120.895     786604.943     8608372.940    8608393.357    1005        14484.61658125

COMM L6205701:11    772121.243     786603.136     8607822.838    8607843.429    1028        14482.73295754

COMM L6205801:13    772111.914     786590.240     8607263.908    8607283.057    1021        14478.84789557

COMM L6205901:13    772110.668     786585.889     8606704.837    8606724.427    1089        14475.65383957

COMM L6206001:13    772100.047     786579.343     8606154.683    8606191.159    1025        14480.48368259

COMM L6206101:13    772103.906     786571.135     8605599.792    8605617.386    1076        14467.88175451

COMM L6206201:13    771987.563     786569.948     8605043.669    8605076.226    1036        14583.4707097

COMM L6206301:13    771996.718     786566.000     8604475.809    8604550.019    1081        14575.0787916

COMM L6206501:13    772029.847     786566.559     8603367.871    8603405.140    1066        14537.83768018

COMM L6206601:13    772047.787     786566.010     8602820.776    8602840.788    1039        14518.72706752

COMM L6206701:13    772050.884     786558.969     8602256.664    8602285.172    1060        14509.0701642

COMM L6300101:6     758578.520     766626.039     8625929.057    8625946.405    584         8047.651841442

COMM L6300301:6     758574.694     766624.127     8625241.174    8625285.370    573         8050.699283664

COMM L6300401:6     758579.114     766638.714     8624932.489    8624962.395    566         8060.846204185

COMM L6400101:7     756861.728     764923.977     8621931.655    8621945.675    576         8062.566586962

COMM L6400201:7     756857.843     764915.320     8621597.931    8621613.841    589         8058.070356334

COMM L6400301:7     756857.002     764898.030     8621260.883    8621310.804    572         8043.357735596

COMM L6400401:7     756857.379     764881.953     8620933.166    8620957.177    584         8025.653506142

COMM L6400601:7     756839.334     764861.664     8620256.936    8620287.186    588         8023.340574569

COMM L6400701:7     756833.304     764869.922     8619932.269    8619946.810    569         8036.809943374

COMM L6400801:7     756839.415     764874.219     8619599.331    8619616.712    597         8035.122685334

COMM L6400901:7     756833.999     764876.028     8619261.549    8619292.282    571         8042.486019439

COMM L6401101:7     756818.023     764887.351     8618591.071    8618615.203    572         8070.658829634

COMM L6401201:7     756827.914     764890.186     8618264.311    8618295.121    601         8063.039484618

COMM L6600101:13    801986.664     818651.691     8602258.639    8602286.085    1191        16666.41277028

COMM L6600201:13    802796.934     818614.537     8600568.040    8600620.293    1150        15820.77850865

COMM L6600401:13    804502.103     818550.345     8597263.761    8597282.307    1024        14048.89687954

COMM L6600501:13    805413.504     818513.200     8595593.549    8595617.246    932         13100.52516238

COMM L6600701:13    806173.019     818437.287     8592251.273    8592287.204    884         12265.29772811

COMM L6600803:25    805342.113     818404.945     8590588.086    8590612.968    956         13063.39454015

COMM L6601001:13    790754.445     818903.542     8587254.756    8587286.946    2054        28151.10247592

COMM L6601101:14    790130.639     823035.689     8585591.827    8585626.846    2345        32907.41663958

COMM L6601301:14    788730.375     823063.330     8582253.913    8582293.856    2431        34335.50943019

COMM L6601401:14    791849.020     823062.897     8580594.587    8580622.852    2350        31215.14638057

COMM ALL            626097.562     834809.037     8560587.675    8655611.742    496536      6861933.602307

EM Window data – repeat lines

COMM GA PROJECT NUMBER                                                 1196

COMM FAS PROJECT NUMBER                                                2017

COMM AREA NUMBER:                                                         1

COMM SURVEY COMPANY:                                 Fugro Airborne Surveys

COMM CLIENT:                                           Geoscience Australia

COMM SURVEY TYPE:                                       25Hz TEMPEST Survey

COMM AREA NAME:                                                     Woolner

COMM STATE:                                                              NT

COMM COUNTRY:                                                     Australia

COMM SURVEY FLOWN:                                 October to November 2008

COMM LOCATED DATA CREATED:                                       April 2009

COMM

COMM DATUM:                                                           GDA94

COMM PROJECTION:                                                        MGA

COMM ZONE:                                                               52

COMM

COMM SURVEY SPECIFICATIONS

COMM

COMM NOMINAL TERRAIN CLEARANCE:                                       120 m

COMM FINAL REPEAT LINE KILOMETRES:                                   104 km

COMM

COMM LINE NUMBERING

COMM

COMM REPEAT LINE NUMBERS:                             910000201 - 910001201

COMM                                                  911001501 - 911003001

COMM

COMM SURVEY EQUIPMENT

COMM

COMM AIRCRAFT:                                 CASA C212 Turbo Prop, VH-TEM

COMM

COMM MAGNETOMETER:                              Scintrex Cs-2 Cesium Vapour

COMM INSTALLATION:                                            stinger mount

COMM RESOLUTION:                                                   0.001 nT

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM ELECTROMAGNETIC SYSTEM:                                   25Hz TEMPEST

COMM INSTALLATION:                 Transmitter loop mounted on the aircraft

COMM                                         Receiver coils in a towed bird

COMM COIL ORIENTATION:                                                  X,Z

COMM RECORDING INTERVAL:                                              0.2 s

COMM SYSTEM GEOMETRY:

COMM RECEIVER DISTANCE BEHIND THE TRANSMITTER:                       -120 m

COMM RECEIVER DISTANCE BELOW THE TRANSMITTER:                         -35 m

COMM

COMM RADAR ALTIMETER:                                         Sperry RT-220

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM LASER ALTIMETER:                                    Optech 501SB (TEM)

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM NAVIGATION:                                 real-time differential GPS

COMM RECORDING INTERVAL:                                              1.0 s

COMM

COMM ACQUISITION SYSTEM:                                             FASDAS

COMM

COMM DATA PROCESSING

COMM

COMM MAGNETIC DATA

COMM DIURNAL BASE VALUE APPLIED                                    47436 nT

COMM PARALLAX CORRECTION APPLIED (during final processing)            1.0 s

COMM IGRF BASE VALUE APPLIED                                       46836 nT

COMM IGRF MODEL 2005 EXTRAPOLATED TO                                 2008.8

COMM

COMM ELECTROMAGNETIC DATA

COMM SYSTEM PARALLAX REMOVED, AS FOLLOWS

COMM X-COMPONENT EM DATA                                              1.6 s

COMM Z-COMPONENT EM DATA                                              0.2 s

COMM DATA CORRECTED FOR TRANSMITTER HEIGHT, PITCH AND ROLL

COMM DATA CORRECTED FOR TRANSMITTER-RECEIVER GEOMETRY VARIATIONS

COMM CONDUCTIVITY DEPTH INVERSION CALCULATED                   EMFlow V5.10

COMM CONDUCTIVITIES CALCULATED USING HPRG CORRECTED EMX & EMZ DATA

COMM

COMM DIGITAL TERRAIN DATA

COMM PARALLAX CORRECTION APPLIED TO LIDAR ALIMETER DATA               0.0 s

COMM PARALLAX CORRECTION APPLIED TO GPS ALIMETER DATA                 0.0 s

COMM DTM CALCULATED [DTM = GPS ALTITUDE - LIDAR ALTITUDE - GPS/LIDAR DIST]

COMM N-VALUE APPLIED TO CORRECT DTM TO AUSTRALIAN HEIGHT DATUM (AHD)

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM ----------------------------------------------------------------------

COMM DISCLAIMER

COMM ----------------------------------------------------------------------

COMM It is Fugro Airborne Survey’s understanding that the data provided to

COMM the client is to be used for the purpose agreed between the parties.

COMM That purpose was a significant factor in determining the scope and

COMM level of the Services being offered to the Client. Should the purpose

COMM for which the data is used change, the data may no longer be valid or

COMM appropriate and any further use of, or reliance upon, the data in

COMM those circumstances by the Client without Fugro Airborne Survey's

COMM review and advice shall be at the Client's own or sole risk.

COMM

COMM The Services were performed by Fugro Airborne Survey exclusively for

COMM the purposes of the Client. Should the data be made available in whole

COMM or part to any third party, and such party relies thereon, that party

COMM does so wholly at its own and sole risk and Fugro Airborne Survey

COMM disclaims any liability to such party.

COMM

COMM Where the Services have involved Fugro Airborne Survey's use of any

COMM information provided by the Client or third parties, upon which

COMM Fugro Airborne Survey was reasonably entitled to rely, then the

COMM Services are limited by the accuracy of such information. Fugro

COMM Airborne Survey is not liable for any inaccuracies (including any

COMM incompleteness) in the said information, save as otherwise provided

COMM in the terms of the contract between the Client and Fugro Airborne

COMM Survey.

COMM

COMM With regard to DIGITAL TERRAIN DATA, the accuracy of the elevation

COMM calculation is directly dependent on the accuracy of the two input

COMM parameters lidar altitude and GPS altitude. The radar altitude value may be

COMM erroneous in areas of heavy tree cover, where the altimeter reflects the

COMM distance to the tree canopy rather than the ground. The GPS altitude value

COMM is primarily dependent on the number of available satellites. Although

COMM post-processing of GPS data will yield X and Y accuracies in the

COMM order of 1-2 metres, the accuracy of the altitude value is usually

COMM much less, sometimes in the ±5 metre range.  Further inaccuracies

COMM may be introduced during the interpolation and gridding process.

COMM Because of the inherent inaccuracies of this method, no guarantee is

COMM made or implied that the information displayed is a true

COMM representation of the height above sea level.  Although this product

COMM may be of some use as a general reference,

COMM THIS PRODUCT MUST NOT BE USED FOR NAVIGATION PURPOSES.

COMM ----------------------------------------------------------------------

COMM

COMM ELECTROMAGNETIC SYSTEM

COMM

COMM TEMPEST IS A TIME-DOMAIN SQUARE-WAVE SYSTEM,

COMM TRANSMITTING AT A BASE FREQUENCY OF 25Hz,

COMM WITH 2 ORTHOGONAL-AXIS RECEIVER COILS IN A TOWED BIRD.

COMM FINAL EM OUTPUT IS RECORDED 5 TIMES PER SECOND.

COMM THE TIMES (IN MILLISECONDS) FOR THE 15 WINDOWS ARE:

COMM

COMM WINDOW      START       END      CENTRE

COMM    1        0.007      0.020      0.013

COMM    2        0.033      0.047      0.040

COMM    3        0.060      0.073      0.067

COMM    4        0.087      0.127      0.107

COMM    5        0.140      0.207      0.173

COMM    6        0.220      0.340      0.280

COMM    7        0.353      0.553      0.453

COMM    8        0.567      0.873      0.720

COMM    9        0.887      1.353      1.120

COMM   10        1.367      2.100      1.733

COMM   11        2.113      3.273      2.693

COMM   12        3.287      5.113      4.200

COMM   13        5.127      7.993      6.560

COMM   14        8.007     12.393     10.200

COMM   15       12.407     19.993     16.200

COMM

COMM PULSE WIDTH: 10 ms

COMM

COMM TEMPEST EM data are transformed to the response that would be

COMM obtained with a B-field sensor for a 100% duty cycle square

COMM waveform at the base frequency, involving a 1A change in

COMM current (from -0.5A to +0.5A to -0.5A) in a 1sq.m transmitter.

COMM It is this configuration, rather than the actual acquisition

COMM configuration, which must be specified when modelling TEMPEST data.

COMM

COMM

COMM

COMM LOCATED DATA FORMAT

COMM

COMM  Output field format : DOS  - Flat ascii

COMM  Number of fields    : 102

COMM

COMM Field  Channel         Description                       Units  Undefined    Format

COMM -----  -------         -----------                       -----  ---------    ------

COMM

COMM     1  LINE            Line                                      -99999999      i10

COMM     2  FLIGHT          Flight                                    -99             i4

COMM     3  FID             Fiducial                            (s)   -999999.9     f8.1

COMM     4  PROJECT_FAS     FAS Project Number                        -9999           i6

COMM     5  PROJECT_GA      GA Project Number                         -9999           i6

COMM     6  AIRCRAFT        System Number                             -9              i3

COMM     7  DATE            Date                           ddmmyyyy   -9999999        i9

COMM     8  TIME            Time - local midnight               (s)   -9999.9       f8.1

COMM     9  BEARING         Line Bearing                      (deg)   -99             i4

COMM    10  LATITUDE        Latitude  GDA94                   (deg)   -99.9999999  f12.7

COMM    11  LONGITUDE       Longitude GDA94                   (deg)   -999.9999999 f13.7

COMM    12  EASTING         Easting   MGA51                     (m)   -99999.99    f10.2

COMM    13  NORTHING        Northing  MGA51                     (m)   -999999.99   f11.2

COMM    14  LIDAR           Final Lidar altimeter               (m)   -999.99       f8.2

COMM    15  RADALT          Final Radar altimeter               (m)   -999.99       f8.2

COMM    16  TX_ELEVATION    Final Transmitter Elevation - AHD   (m)   -999.99       f8.2

COMM    17  DTM             Final Ground Elevation - AHD        (m)   -999.99       f8.2

COMM    18  MAG             Final TMI                          (nT)   -99999.999   f11.3

COMM    19  TX_PITCH        Transmitter loop pitch            (deg)   -999.99       f8.2

COMM    20  TX_ROLL         Transmitter loop roll             (deg)   -999.99       f8.2

COMM    21  TX_HEIGHT       Transmitter terrain clearance       (m)   -999.99       f8.2

COMM    22  HSep_Raw        Tx-Rx horizontal separation         (m)   -999.99       f8.2

COMM    23  VSep_Raw        Tx-Rx vertical separation           (m)   -999.99       f8.2

COMM    24  TX_HEIGHT_std   Tx HPRG terrain clearance           (m)   -999.99       f8.2

COMM    25  HSep_std        Tx-Rx HPRG horizontal separation    (m)   -999.99       f8.2

COMM    26  VSep_std        Tx-Rx HPRG vertical separation      (m)   -999.99       f8.2

COMM    27  EMX_nonhprg[1]  Raw (non-HPRG) EMX01 Window        (fT)   -999.999999  f12.6

COMM    28  EMX_nonhprg[2]  Raw (non-HPRG) EMX02 Window        (fT)   -999.999999  f12.6

COMM    29  EMX_nonhprg[3]  Raw (non-HPRG) EMX03 Window        (fT)   -999.999999  f12.6

COMM    30  EMX_nonhprg[4]  Raw (non-HPRG) EMX04 Window        (fT)   -999.999999  f12.6

COMM    31  EMX_nonhprg[5]  Raw (non-HPRG) EMX05 Window        (fT)   -999.999999  f12.6

COMM    32  EMX_nonhprg[6]  Raw (non-HPRG) EMX06 Window        (fT)   -999.999999  f12.6

COMM    33  EMX_nonhprg[7]  Raw (non-HPRG) EMX07 Window        (fT)   -999.999999  f12.6

COMM    34  EMX_nonhprg[8]  Raw (non-HPRG) EMX08 Window        (fT)   -999.999999  f12.6

COMM    35  EMX_nonhprg[9]  Raw (non-HPRG) EMX09 Window        (fT)   -999.999999  f12.6

COMM    36  EMX_nonhprg[10] Raw (non-HPRG) EMX10 Window        (fT)   -999.999999  f12.6

COMM    37  EMX_nonhprg[11] Raw (non-HPRG) EMX11 Window        (fT)   -999.999999  f12.6

COMM    38  EMX_nonhprg[12] Raw (non-HPRG) EMX12 Window        (fT)   -999.999999  f12.6

COMM    39  EMX_nonhprg[13] Raw (non-HPRG) EMX13 Window        (fT)   -999.999999  f12.6

COMM    40  EMX_nonhprg[14] Raw (non-HPRG) EMX14 Window        (fT)   -999.999999  f12.6

COMM    41  EMX_nonhprg[15] Raw (non-HPRG) EMX15 Window        (fT)   -999.999999  f12.6

COMM    42  EMX_hprg[1]     HPRG Corrected EMX01 Window        (fT)   -999.999999  f12.6

COMM    43  EMX_hprg[2]     HPRG Corrected EMX02 Window        (fT)   -999.999999  f12.6

COMM    44  EMX_hprg[3]     HPRG Corrected EMX03 Window        (fT)   -999.999999  f12.6

COMM    45  EMX_hprg[4]     HPRG Corrected EMX04 Window        (fT)   -999.999999  f12.6

COMM    46  EMX_hprg[5]     HPRG Corrected EMX05 Window        (fT)   -999.999999  f12.6

COMM    47  EMX_hprg[6]     HPRG Corrected EMX06 Window        (fT)   -999.999999  f12.6

COMM    48  EMX_hprg[7]     HPRG Corrected EMX07 Window        (fT)   -999.999999  f12.6

COMM    49  EMX_hprg[8]     HPRG Corrected EMX08 Window        (fT)   -999.999999  f12.6

COMM    50  EMX_hprg[9]     HPRG Corrected EMX09 Window        (fT)   -999.999999  f12.6

COMM    51  EMX_hprg[10]    HPRG Corrected EMX10 Window        (fT)   -999.999999  f12.6

COMM    52  EMX_hprg[11]    HPRG Corrected EMX11 Window        (fT)   -999.999999  f12.6

COMM    53  EMX_hprg[12]    HPRG Corrected EMX12 Window        (fT)   -999.999999  f12.6

COMM    54  EMX_hprg[13]    HPRG Corrected EMX13 Window        (fT)   -999.999999  f12.6

COMM    55  EMX_hprg[14]    HPRG Corrected EMX14 Window        (fT)   -999.999999  f12.6

COMM    56  EMX_hprg[15]    HPRG Corrected EMX15 Window        (fT)   -999.999999  f12.6

COMM    57  X_Sferics       X_Sferics                                 -9999.999    f10.3

COMM    58  X_Lowfreq       X_Lowfreq                                 -9999.999    f10.3

COMM    59  X_Powerline     X_Powerline                               -9999.999    f10.3

COMM    60  X_VLF1          X_18.2kHz                                 -9999.999    f10.3

COMM    61  X_VLF2          X_19.8kHz                                 -9999.999    f10.3

COMM    62  X_VLF3          X_21.4kHz                                 -9999.999    f10.3

COMM    63  X_VLF4          X_22.2kHz                                 -9999.999    f10.3

COMM    64  X_Geofact       X_Geometric factor                        -9999.999    f10.3

COMM    65  EMZ_nonhprg[1]  Raw (non-HPRG) EMZ01 Window        (fT)   -999.999999  f12.6

COMM    66  EMZ_nonhprg[2]  Raw (non-HPRG) EMZ02 Window        (fT)   -999.999999  f12.6

COMM    67  EMZ_nonhprg[3]  Raw (non-HPRG) EMZ03 Window        (fT)   -999.999999  f12.6

COMM    68  EMZ_nonhprg[4]  Raw (non-HPRG) EMZ04 Window        (fT)   -999.999999  f12.6

COMM    69  EMZ_nonhprg[5]  Raw (non-HPRG) EMZ05 Window        (fT)   -999.999999  f12.6

COMM    70  EMZ_nonhprg[6]  Raw (non-HPRG) EMZ06 Window        (fT)   -999.999999  f12.6

COMM    71  EMZ_nonhprg[7]  Raw (non-HPRG) EMZ07 Window        (fT)   -999.999999  f12.6

COMM    72  EMZ_nonhprg[8]  Raw (non-HPRG) EMZ08 Window        (fT)   -999.999999  f12.6

COMM    73  EMZ_nonhprg[9]  Raw (non-HPRG) EMZ09 Window        (fT)   -999.999999  f12.6

COMM    74  EMZ_nonhprg[10] Raw (non-HPRG) EMZ10 Window        (fT)   -999.999999  f12.6

COMM    75  EMZ_nonhprg[11] Raw (non-HPRG) EMZ11 Window        (fT)   -999.999999  f12.6

COMM    76  EMZ_nonhprg[12] Raw (non-HPRG) EMZ12 Window        (fT)   -999.999999  f12.6

COMM    77  EMZ_nonhprg[13] Raw (non-HPRG) EMZ13 Window        (fT)   -999.999999  f12.6

COMM    78  EMZ_nonhprg[14] Raw (non-HPRG) EMZ14 Window        (fT)   -999.999999  f12.6

COMM    79  EMZ_nonhprg[15] Raw (non-HPRG) EMZ15 Window        (fT)   -999.999999  f12.6

COMM    80  EMZ_hprg[1]     HPRG Corrected EMZ01 Window        (fT)   -999.999999  f12.6

COMM    81  EMZ_hprg[2]     HPRG Corrected EMZ02 Window        (fT)   -999.999999  f12.6

COMM    82  EMZ_hprg[3]     HPRG Corrected EMZ03 Window        (fT)   -999.999999  f12.6

COMM    83  EMZ_hprg[4]     HPRG Corrected EMZ04 Window        (fT)   -999.999999  f12.6

COMM    84  EMZ_hprg[5]     HPRG Corrected EMZ05 Window        (fT)   -999.999999  f12.6

COMM    85  EMZ_hprg[6]     HPRG Corrected EMZ06 Window        (fT)   -999.999999  f12.6

COMM    86  EMZ_hprg[7]     HPRG Corrected EMZ07 Window        (fT)   -999.999999  f12.6

COMM    87  EMZ_hprg[8]     HPRG Corrected EMZ08 Window        (fT)   -999.999999  f12.6

COMM    88  EMZ_hprg[9]     HPRG Corrected EMZ09 Window        (fT)   -999.999999  f12.6

COMM    89  EMZ_hprg[10]    HPRG Corrected EMZ10 Window        (fT)   -999.999999  f12.6

COMM    90  EMZ_hprg[11]    HPRG Corrected EMZ11 Window        (fT)   -999.999999  f12.6

COMM    91  EMZ_hprg[12]    HPRG Corrected EMZ12 Window        (fT)   -999.999999  f12.6

COMM    92  EMZ_hprg[13]    HPRG Corrected EMZ13 Window        (fT)   -999.999999  f12.6

COMM    93  EMZ_hprg[14]    HPRG Corrected EMZ14 Window        (fT)   -999.999999  f12.6

COMM    94  EMZ_hprg[15]    HPRG Corrected EMZ15 Window        (fT)   -999.999999  f12.6

COMM    95  Z_Sferics       Z_Sferics                                 -9999.999    f10.3

COMM    96  Z_Lowfreq       Z_Lowfreq                                 -9999.999    f10.3

COMM    97  Z_Powerline     Z_Powerline                               -9999.999    f10.3

COMM    98  Z_VLF1          Z_18.2kHz                                 -9999.999    f10.3

COMM    99  Z_VLF2          Z_19.8kHz                                 -9999.999    f10.3

COMM   100  Z_VLF3          Z_21.4kHz                                 -9999.999    f10.3

COMM   101  Z_VLF4          Z_22.2kHz                                 -9999.999    f10.3

COMM   102  Z_Geofact       Z_Geometric factor                        -9999.999    f10.3

COMM

COMM Line Number    X-minimum      X-maximum      Y-minimum      Y-maximum      # of points Total distance

-----------    ---------      ---------      ---------      --------       ----------- --------------

D910000101:1   758617.973    766595.117     8625929.214    8625951.529    593      7977.960349781

D910000201:2   758614.627    766605.068     8625930.235    8625948.988    611      7991.157895713

D910000301:3   758617.752    766602.559     8625930.117    8625948.046    630      7985.071977546

D910000401:4   758611.606    766594.805     8625921.834    8625947.767    576      7984.167942048

D910000601:6   758606.464    766598.474     8625929.259    8625946.404    580      7992.141857788

D910000901:9   758613.838    766605.406     8625925.485    8625954.563    573      7992.126408156

D910001201:12  758607.447    766606.442     8625928.823    8625945.733    546      7999.319002098

D910001201:12  758622.956    766606.442     8625928.823    8625945.733    545      7983.803057622

D911001501:15  751706.078    759788.070     8568934.638    8568950.168    559      8082.2088958

D911001801:18  751701.721    759789.825     8568922.931    8568945.713    582      8088.483041208

D911002101:21  751710.376    759788.164     8568938.295    8568948.949    575      8078.168517275

D911002401:24  751703.730    759783.265     8568936.783    8568946.489    582      8079.688873307

D911002701:27  751705.815    759787.134     8568932.214    8568950.259    568      8081.686572674

D911003001:30  751700.154    759780.395     8568936.550    8568950.734    581      8080.582651933

ALL            751700.154    766608.622     8568922.931    8625954.563    7556     104410.5028871

Conductivity data – survey lines

COMM GA PROJECT NUMBER                                                 1196

COMM FAS PROJECT NUMBER                                                2017

COMM AREA NUMBER:                                                         1

COMM SURVEY COMPANY:                                 Fugro Airborne Surveys

COMM CLIENT:                                           Geoscience Australia

COMM SURVEY TYPE:                                       25Hz TEMPEST Survey

COMM AREA NAME:                                                     Woolner

COMM STATE:                                                              NT

COMM COUNTRY:                                                     Australia

COMM SURVEY FLOWN:                                 October to November 2008

COMM LOCATED DATA CREATED:                                       April 2009

COMM

COMM DATUM:                                                           GDA94

COMM PROJECTION:                                                        MGA

COMM ZONE:                                                               52

COMM

COMM SURVEY SPECIFICATIONS

COMM

COMM TRAVERSE LINE SPACING:                                      238-5000 m

COMM TRAVERSE LINE DIRECTION:                                   090-270 deg

COMM NOMINAL TERRAIN CLEARANCE:                                       120 m

COMM FINAL LINE KILOMETRES:                                         6862 km

COMM

COMM LINE NUMBERING

COMM

COMM TRAVERSE LINE NUMBERS:                               1000101 - 1102301

COMM                                                      2800101 - 2802901

COMM                                                      2900101 - 2901502

COMM                                                      3000103 - 3303101

COMM                                                      4100201 - 4300503

COMM                                                      5000101 - 5200101

COMM                                                      6100301 - 6601401

COMM

COMM SURVEY EQUIPMENT

COMM

COMM AIRCRAFT:                                 CASA C212 Turbo Prop, VH-TEM

COMM

COMM MAGNETOMETER:                              Scintrex Cs-2 Cesium Vapour

COMM INSTALLATION:                                            stinger mount

COMM RESOLUTION:                                                   0.001 nT

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM ELECTROMAGNETIC SYSTEM:                                   25Hz TEMPEST

COMM INSTALLATION:                 Transmitter loop mounted on the aircraft

COMM                                         Receiver coils in a towed bird

COMM COIL ORIENTATION:                                                  X,Z

COMM RECORDING INTERVAL:                                              0.2 s

COMM SYSTEM GEOMETRY:

COMM RECEIVER DISTANCE BEHIND THE TRANSMITTER:                       -120 m

COMM RECEIVER DISTANCE BELOW THE TRANSMITTER:                         -35 m

COMM

COMM RADAR ALTIMETER:                                         Sperry RT-220

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM LASER ALTIMETER:                                    Optech 501SB (TEM)

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM NAVIGATION:                                 real-time differential GPS

COMM RECORDING INTERVAL:                                              1.0 s

COMM

COMM ACQUISITION SYSTEM:                                             FASDAS

COMM

COMM DATA PROCESSING

COMM

COMM MAGNETIC DATA

COMM DIURNAL BASE VALUE APPLIED                                    47436 nT

COMM PARALLAX CORRECTION APPLIED (during final processing)            1.0 s

COMM IGRF BASE VALUE APPLIED                                       46836 nT

COMM IGRF MODEL 2005 EXTRAPOLATED TO                                 2008.8

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM ELECTROMAGNETIC DATA

COMM SYSTEM PARALLAX REMOVED, AS FOLLOWS

COMM X-COMPONENT EM DATA                                              1.6 s

COMM Z-COMPONENT EM DATA                                              0.2 s

COMM DATA CORRECTED FOR TRANSMITTER HEIGHT, PITCH AND ROLL

COMM DATA CORRECTED FOR TRANSMITTER-RECEIVER GEOMETRY VARIATIONS

COMM DATA HAVE BEEN MICROLEVELLED

COMM CONDUCTIVITY DEPTH INVERSION CALCULATED                   EMFlow V5.10

COMM CONDUCTIVITIES CALCULATED USING HPRG CORRECTED EMX & EMZ DATA

COMM

COMM DIGITAL TERRAIN DATA

COMM PARALLAX CORRECTION APPLIED TO LIDAR ALIMETER DATA               0.0 s

COMM PARALLAX CORRECTION APPLIED TO GPS ALIMETER DATA                 0.0 s

COMM DTM CALCULATED [DTM = GPS ALTITUDE - LIDAR ALTITUDE - GPS/LIDAR DIST]

COMM N-VALUE APPLIED TO CORRECT DTM TO AUSTRALIAN HEIGHT DATUM (AHD)

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM ----------------------------------------------------------------------

COMM DISCLAIMER

COMM ----------------------------------------------------------------------

COMM It is Fugro Airborne Survey’s understanding that the data provided to

COMM the client is to be used for the purpose agreed between the parties.

COMM That purpose was a significant factor in determining the scope and

COMM level of the Services being offered to the Client. Should the purpose

COMM for which the data is used change, the data may no longer be valid or

COMM appropriate and any further use of, or reliance upon, the data in

COMM those circumstances by the Client without Fugro Airborne Survey's

COMM review and advice shall be at the Client's own or sole risk.

COMM

COMM The Services were performed by Fugro Airborne Survey exclusively for

COMM the purposes of the Client. Should the data be made available in whole

COMM or part to any third party, and such party relies thereon, that party

COMM does so wholly at its own and sole risk and Fugro Airborne Survey

COMM disclaims any liability to such party.

COMM

COMM Where the Services have involved Fugro Airborne Survey's use of any

COMM information provided by the Client or third parties, upon which

COMM Fugro Airborne Survey was reasonably entitled to rely, then the

COMM Services are limited by the accuracy of such information. Fugro

COMM Airborne Survey is not liable for any inaccuracies (including any

COMM incompleteness) in the said information, save as otherwise provided

COMM in the terms of the contract between the Client and Fugro Airborne

COMM Survey.

COMM

COMM With regard to DIGITAL TERRAIN DATA, the accuracy of the elevation

COMM calculation is directly dependent on the accuracy of the two input

COMM parameters lidar altitude and GPS altitude. The radar altitude value may be

COMM erroneous in areas of heavy tree cover, where the altimeter reflects the

COMM distance to the tree canopy rather than the ground. The GPS altitude value

COMM is primarily dependent on the number of available satellites. Although

COMM post-processing of GPS data will yield X and Y accuracies in the

COMM order of 1-2 metres, the accuracy of the altitude value is usually

COMM much less, sometimes in the ±5 metre range.  Further inaccuracies

COMM may be introduced during the interpolation and gridding process.

COMM Because of the inherent inaccuracies of this method, no guarantee is

COMM made or implied that the information displayed is a true

COMM representation of the height above sea level.  Although this product

COMM may be of some use as a general reference,

COMM THIS PRODUCT MUST NOT BE USED FOR NAVIGATION PURPOSES.

COMM ----------------------------------------------------------------------

COMM

COMM ELECTROMAGNETIC SYSTEM

COMM

COMM TEMPEST IS A TIME-DOMAIN SQUARE-WAVE SYSTEM,

COMM TRANSMITTING AT A BASE FREQUENCY OF 25Hz,

COMM WITH 2 ORTHOGONAL-AXIS RECEIVER COILS IN A TOWED BIRD.

COMM FINAL EM OUTPUT IS RECORDED 5 TIMES PER SECOND.

COMM THE TIMES (IN MILLISECONDS) FOR THE 15 WINDOWS ARE:

COMM

COMM WINDOW      START       END      CENTRE

COMM    1        0.007      0.020      0.013

COMM    2        0.033      0.047      0.040

COMM    3        0.060      0.073      0.067

COMM    4        0.087      0.127      0.107

COMM    5        0.140      0.207      0.173

COMM    6        0.220      0.340      0.280

COMM    7        0.353      0.553      0.453

COMM    8        0.567      0.873      0.720

COMM    9        0.887      1.353      1.120

COMM   10        1.367      2.100      1.733

COMM   11        2.113      3.273      2.693

COMM   12        3.287      5.113      4.200

COMM   13        5.127      7.993      6.560

COMM   14        8.007     12.393     10.200

COMM   15       12.407     19.993     16.200

COMM

COMM PULSE WIDTH: 10 ms

COMM

COMM TEMPEST EM data are transformed to the response that would be

COMM obtained with a B-field sensor for a 100% duty cycle square

COMM waveform at the base frequency, involving a 1A change in

COMM current (from -0.5A to +0.5A to -0.5A) in a 1sq.m transmitter.

COMM It is this configuration, rather than the actual acquisition

COMM configuration, which must be specified when modelling TEMPEST data.

COMM

COMM

COMM

COMM LOCATED DATA FORMAT

COMM

COMM  Output field format : DOS  - Flat ascii

COMM  Number of fields    : 68

COMM

COMM Field  Channel         Description                       Units  Undefined    Format

COMM -----  -------         -----------                       -----  ---------    ------

COMM

COMM     1  LINE            Line                                      -99999999      i10

COMM     2  FLIGHT          Flight                                    -99             i4

COMM     3  FID             Fiducial                            (s)   -999999.9     f8.1

COMM     4  PROJECT_FAS     FAS Project Number                        -9999           i6

COMM     5  PROJECT_GA      GA Project Number                         -9999           i6

COMM     6  AIRCRAFT        System Number                             -9              i3

COMM     7  DATE            Date                           ddmmyyyy   -9999999        i9

COMM     8  TIME            Time - local midnight               (s)   -9999.9       f8.1

COMM     9  BEARING         Line Bearing                      (deg)   -99             i4

COMM    10  LATITUDE        Latitude  GDA94                   (deg)   -99.9999999  f12.7

COMM    11  LONGITUDE       Longitude GDA94                   (deg)   -999.9999999 f13.7

COMM    12  EASTING         Easting   MGA51                     (m)   -99999.99    f10.2

COMM    13  NORTHING        Northing  MGA51                     (m)   -999999.99   f11.2

COMM    14  LIDAR           Final Lidar altimeter               (m)   -999.99       f8.2

COMM    15  RADALT          Final Radar altimeter               (m)   -999.99       f8.2

COMM    16  TX_ELEVATION    Final Transmitter Elevation - AHD   (m)   -999.99       f8.2

COMM    17  DTM             Final Ground Elevation - AHD        (m)   -999.99       f8.2

COMM    18  MAG             Final TMI                          (nT)   -99999.999   f11.3

COMM    19  CND_DS01        CDI_depth_slice_01   0-    5 m   (mS/m)   -9999.999    f10.3

COMM    20  CND_DS02        CDI_depth_slice_02   5-   10 m   (mS/m)   -9999.999    f10.3

COMM    21  CND_DS03        CDI_depth_slice_03   10-  15 m   (mS/m)   -9999.999    f10.3

COMM    22  CND_DS04        CDI_depth_slice_04   15-  20 m   (mS/m)   -9999.999    f10.3

COMM    23  CND_DS05        CDI_depth_slice_05   20-  30 m   (mS/m)   -9999.999    f10.3

COMM    24  CND_DS06        CDI_depth_slice_06   30-  40 m   (mS/m)   -9999.999    f10.3

COMM    25  CND_DS07        CDI_depth_slice_07   40-  60 m   (mS/m)   -9999.999    f10.3

COMM    26  CND_DS08        CDI_depth_slice_08   60- 100 m   (mS/m)   -9999.999    f10.3

COMM    27  CND_DS09        CDI_depth_slice_09   100-150 m   (mS/m)   -9999.999    f10.3

COMM    28  CND_DS10        CDI_depth_slice_10   150-200 m   (mS/m)   -9999.999    f10.3

COMM    29  CND[1]          Conductivity_001       0-  5 m   (mS/m)   -9999.999    f10.3

COMM    30  CND[2]          Conductivity_002       5- 10 m   (mS/m)   -9999.999    f10.3

COMM    31  CND[3]          Conductivity_003      10- 15 m   (mS/m)   -9999.999    f10.3

COMM    32  CND[4]          Conductivity_004      15- 20 m   (mS/m)   -9999.999    f10.3

COMM    33  CND[5]          Conductivity_005      20- 25 m   (mS/m)   -9999.999    f10.3

COMM    34  CND[6]          Conductivity_006      25- 30 m   (mS/m)   -9999.999    f10.3

COMM    35  CND[7]          Conductivity_007      30- 35 m   (mS/m)   -9999.999    f10.3

COMM    36  CND[8]          Conductivity_008      35- 40 m   (mS/m)   -9999.999    f10.3

COMM    37  CND[9]          Conductivity_009      40- 45 m   (mS/m)   -9999.999    f10.3

COMM    38  CND[10]         Conductivity_010      45- 50 m   (mS/m)   -9999.999    f10.3

COMM    39  CND[11]         Conductivity_011      50- 55 m   (mS/m)   -9999.999    f10.3

COMM    40  CND[12]         Conductivity_012      55- 60 m   (mS/m)   -9999.999    f10.3

COMM    41  CND[13]         Conductivity_013      60- 65 m   (mS/m)   -9999.999    f10.3

COMM    42  CND[14]         Conductivity_014      65- 70 m   (mS/m)   -9999.999    f10.3

COMM    43  CND[15]         Conductivity_015      70- 75 m   (mS/m)   -9999.999    f10.3

COMM    44  CND[16]         Conductivity_016      75- 80 m   (mS/m)   -9999.999    f10.3

COMM    45  CND[17]         Conductivity_017      80- 85 m   (mS/m)   -9999.999    f10.3

COMM    46  CND[18]         Conductivity_018      85- 90 m   (mS/m)   -9999.999    f10.3

COMM    47  CND[19]         Conductivity_019      90- 95 m   (mS/m)   -9999.999    f10.3

COMM    48  CND[20]         Conductivity_020      95-100 m   (mS/m)   -9999.999    f10.3

COMM    49  CND[21]         Conductivity_021     100-105 m   (mS/m)   -9999.999    f10.3

COMM    50  CND[22]         Conductivity_022     105-110 m   (mS/m)   -9999.999    f10.3

COMM    51  CND[23]         Conductivity_023     110-115 m   (mS/m)   -9999.999    f10.3

COMM    52  CND[24]         Conductivity_024     115-120 m   (mS/m)   -9999.999    f10.3

COMM    53  CND[25]         Conductivity_025     120-125 m   (mS/m)   -9999.999    f10.3

COMM    54  CND[26]         Conductivity_026     125-130 m   (mS/m)   -9999.999    f10.3

COMM    55  CND[27]         Conductivity_027     130-135 m   (mS/m)   -9999.999    f10.3

COMM    56  CND[28]         Conductivity_028     135-140 m   (mS/m)   -9999.999    f10.3

COMM    57  CND[29]         Conductivity_029     140-145 m   (mS/m)   -9999.999    f10.3

COMM    58  CND[30]         Conductivity_030     145-150 m   (mS/m)   -9999.999    f10.3

COMM    59  CND[31]         Conductivity_031     150-155 m   (mS/m)   -9999.999    f10.3

COMM    60  CND[32]         Conductivity_032     155-160 m   (mS/m)   -9999.999    f10.3

COMM    61  CND[33]         Conductivity_033     160-165 m   (mS/m)   -9999.999    f10.3

COMM    62  CND[34]         Conductivity_034     165-170 m   (mS/m)   -9999.999    f10.3

COMM    63  CND[35]         Conductivity_035     170-175 m   (mS/m)   -9999.999    f10.3

COMM    64  CND[36]         Conductivity_036     175-180 m   (mS/m)   -9999.999    f10.3

COMM    65  CND[37]         Conductivity_037     180-185 m   (mS/m)   -9999.999    f10.3

COMM    66  CND[38]         Conductivity_038     185-190 m   (mS/m)   -9999.999    f10.3

COMM    67  CND[39]         Conductivity_039     190-195 m   (mS/m)   -9999.999    f10.3

COMM    68  CND[40]         Conductivity_040     195-200 m   (mS/m)   -9999.999    f10.3

COMM

COMM Line Number    X-minimum      X-maximum      Y-minimum      Y-maximum      # of points Total distance

COMM -----------    ---------      ---------      ---------      --------       ----------- --------------

COMM L1000101:15    701699.649     755735.871     8598930.412    8598968.032    4067        54039.886185

COMM L1000201:14    772068.207     788661.980     8598929.413    8598948.284    1187        16594.68325671

COMM L1000301:26    665068.950     673106.890     8593932.710    8593951.015    587         8038.312239841

COMM L1000401:15    701668.710     762773.967     8593915.687    8593969.139    4255        61112.24761159

COMM L1000501:16    771902.492     787461.491     8593926.073    8593956.247    1082        15560.01208714

COMM L1000601:26    665028.782     673114.007     8588929.863    8588946.515    581         8085.413592158

COMM L1000701:16    701630.042     786326.716     8588753.658    8589014.670    6250        84750.80188417

COMM L1000801:26    661356.010     672870.726     8583927.396    8583954.635    834         11515.68305178

COMM L1000901:16    701604.008     783214.729     8583888.921    8583974.947    5812        81630.50498423

COMM L1001001:26    648784.908     672810.212     8578927.246    8578943.893    1744        24025.86922623

COMM L1001101:17    701573.665     781185.843     8578796.729    8579045.528    5934        79649.63924522

COMM L1001201:18    640131.229     783179.249     8573894.275    8574072.448    10225       143077.5668749

COMM L1001301:25    786265.992     801716.058     8573912.754    8573960.395    1114        15455.56091809

COMM L1001401:18    632030.903     787862.563     8568924.538    8568980.949    11527       155839.3222702

COMM L1001501:32    626097.562     787965.201     8563895.226    8563966.772    11471       161891.7224076

COMM L1100101:17    703412.927     737089.658     8590392.827    8590679.382    2396        33730.55523818

COMM L1100302:22    703425.988     737010.183     8587191.670    8587440.317    2455        33619.86523188

COMM L1100402:22    703426.527     736983.536     8585595.994    8585735.860    2411        33569.35971734

COMM L1100602:22    703368.660     736954.479     8582250.991    8582310.062    2416        33592.61775769

COMM L1100702:25    703367.811     736950.876     8580582.247    8580735.818    2410        33600.81996603

COMM L1100901:22    703402.699     736886.226     8577251.793    8577296.300    2494        33487.54320863

COMM L1101001:22    703336.282     736887.723     8575599.873    8575631.461    2415        33553.13927627

COMM L1101201:22    703307.540     736910.005     8572269.610    8572292.654    2422        33603.76590959

COMM L1101301:22    703307.635     736880.380     8570576.829    8570642.146    2479        33579.25229863

COMM L1101501:22    703289.497     736817.273     8567221.157    8567292.258    2455        33535.03962871

COMM L1101701:22    708971.133     736823.359     8565592.437    8565629.569    2009        27854.76621012

COMM L1101901:22    703249.975     736789.062     8562269.461    8562307.967    2408        33542.60500707

COMM L1102001:22    703258.349     736782.540     8560587.675    8560630.890    2445        33527.65579076

COMM L2800101:14    749660.766     757720.612     8597589.903    8597608.604    567         8060.349044366

COMM L2800201:14    750100.598     758179.274     8597265.369    8597293.321    593         8079.132004985

COMM L2800301:14    750545.428     758635.236     8596921.488    8596951.180    573         8091.400555112

COMM L2800401:14    750219.062     759115.658     8596606.353    8596617.980    647         8896.842526245

COMM L2800501:14    749889.823     759575.187     8596254.182    8596283.871    686         9687.397935591

COMM L2800601:16    749550.744     760049.664     8595914.663    8595950.719    752         10501.32977696

COMM L2800701:16    749214.182     760580.571     8595566.481    8595612.925    817         11367.48935581

COMM L2800801:16    748882.338     760983.338     8595255.627    8595283.130    862         12101.51327241

COMM L2800901:16    748620.047     761486.339     8594919.224    8594948.157    924         12867.46105177

COMM L2801001:16    748610.876     761909.183     8594586.586    8594620.862    948         13299.45075511

COMM L2801101:16    748622.235     762511.806     8594203.325    8594282.189    985         13894.35570009

COMM L2801301:17    748597.525     762491.196     8593595.484    8593620.179    1018        13894.94865052

COMM L2801401:17    748602.015     762487.953     8593257.247    8593284.763    1002        13886.89549317

COMM L2801501:17    748603.596     762499.052     8592917.288    8592950.280    1013        13896.69468186

COMM L2801601:17    748606.005     762506.208     8592590.852    8592614.001    1003        13900.83671288

COMM L2801701:17    748613.290     762514.983     8592237.433    8592288.741    1005        13903.98846913

COMM L2801801:17    748618.723     762524.356     8591926.917    8591947.651    1007        13906.7287543

COMM L2801901:18    750610.439     765090.777     8591595.852    8591663.129    1028        14484.80912762

COMM L2802001:17    751533.885     780986.610     8591252.400    8591289.853    2129        29455.95804262

COMM L2802101:16    751515.189     786339.274     8590901.082    8590974.524    2540        34833.64237222

COMM L2802201:17    751491.663     786348.064     8590515.327    8590683.986    2500        34878.81511054

COMM L2802301:17    751468.119     786339.912     8590154.695    8590364.768    2536        34891.41084106

COMM L2802401:17    751438.656     786326.017     8589885.824    8589971.175    2468        34899.06015242

COMM L2802501:17    751460.799     786325.177     8589487.824    8589754.662    2556        34928.2904017

COMM L2802601:18    769553.537     786318.467     8589263.139    8589281.188    1202        16765.98970967

COMM L2802801:18    769499.129     786315.673     8588562.346    8588621.292    1193        16819.46377093

COMM L2802901:18    769494.754     786315.928     8588247.895    8588419.128    1213        16843.79103514

COMM L2900101:18    770261.157     783209.886     8584595.863    8584619.326    911         12949.6503976

COMM L2900201:18    770248.364     783209.653     8584253.371    8584284.195    948         12962.48091359

COMM L2900401:18    770239.851     783216.150     8583599.599    8583627.844    951         12977.34186883

COMM L2900501:18    770248.500     783220.493     8583260.572    8583287.920    906         12972.96892787

COMM L2900601:18    770244.726     783212.026     8582926.479    8582970.154    959         12970.86049936

COMM L2900702:25    770245.835     783219.791     8582588.940    8582620.408    934         12974.61457866

COMM L2900801:19    770248.626     783228.780     8582259.971    8582282.029    929         12981.1348468

COMM L2900901:19    770252.414     783225.772     8581930.006    8581954.285    939         12974.93311861

COMM L2901001:19    770248.482     783228.792     8581583.802    8581635.602    923         12983.81744507

COMM L2901101:19    770243.960     783226.859     8581261.430    8581287.458    943         12983.44570371

COMM L2901201:19    770250.908     783238.731     8580931.425    8580956.444    920         12988.51390562

COMM L2901302:25    770242.716     783237.467     8580585.419    8580616.428    935         12996.45135201

COMM L2901401:19    770251.143     783231.018     8580267.231    8580293.535    918         12980.81610607

COMM L2901502:25    770247.703     783240.074     8579927.863    8579945.300    928         12993.10191638

COMM L3000103:24    786408.450     801684.850     8576234.665    8576279.519    1084        15278.16275695

COMM L3000203:24    786421.824     801697.732     8575916.200    8575949.711    1128        15277.66903479

COMM L3000301:24    786430.176     801700.961     8575587.958    8575613.207    1068        15271.81633116

COMM L3000402:25    786445.815     801696.705     8575265.584    8575280.212    1154        15251.62425591

COMM L3000502:25    786449.578     801701.169     8574932.701    8574945.292    1060        15251.95929669

COMM L3000602:25    786464.388     801715.930     8574594.771    8574627.003    1134        15253.30556705

COMM L3000701:24    786469.302     801713.745     8574262.995    8574307.416    1081        15246.54643568

COMM L3000902:25    786519.508     801711.613     8573595.864    8573625.082    1057        15193.45883508

COMM L3001001:24    786527.079     801692.705     8573266.941    8573281.925    1105        15166.2016407

COMM L3001102:25    786538.073     801674.460     8572927.942    8572951.859    1062        15137.59722351

COMM L3001202:25    786540.063     801659.030     8572592.781    8572627.351    1119        15120.98332133

COMM L3001301:25    786550.665     801643.771     8572260.253    8572280.334    1066        15094.11340808

COMM L3001401:25    786563.375     801619.361     8571928.547    8571949.563    1112        15057.41724108

COMM L3001501:25    786567.828     801601.628     8571447.308    8571615.362    1080        15058.03627977

COMM L3001601:25    786585.932     801588.716     8571260.580    8571286.453    1102        15004.04478285

COMM L3001702:26    786586.543     801570.547     8570933.117    8571025.129    1105        14993.39843432

COMM L3001802:26    786591.991     801548.383     8570582.757    8570614.399    1062        14956.89633035

COMM L3100104:30    703383.520     711436.565     8580244.636    8580304.395    583         8058.28046207

COMM L3100204:30    703382.487     711444.624     8579938.376    8580011.075    584         8066.646199768

COMM L3100304:30    703385.863     711454.785     8579596.434    8579627.490    580         8070.732445964

COMM L3100404:30    703396.590     711450.836     8579244.820    8579353.629    580         8062.040269139

COMM L3100604:30    703358.083     711460.894     8578593.113    8578631.216    589         8103.901442525

COMM L3100704:30    703371.465     711461.260     8578266.648    8578284.670    576         8090.413566565

COMM L3100804:30    703383.207     711479.035     8577928.982    8577960.639    588         8096.68820777

COMM L3100904:30    703387.829     711474.691     8577564.961    8577615.182    577         8089.805879697

COMM L3101104:30    703408.664     711458.673     8576925.527    8576961.352    583         8051.577068533

COMM L3101204:30    703422.494     711466.310     8576596.499    8576620.684    589         8044.707766341

COMM L3101304:30    703438.997     711475.924     8576269.517    8576285.492    582         8037.38240057

COMM L3101404:30    703442.543     711478.985     8575932.158    8575965.130    578         8036.989174333

COMM L3101604:30    703470.368     711509.821     8575260.708    8575305.479    581         8040.433109175

COMM L3101704:30    703468.205     711507.061     8574926.816    8574952.562    585         8039.482734948

COMM L3101805:31    703465.101     711505.002     8574599.809    8574622.763    557         8040.638211187

COMM L3101905:31    703457.958     711514.064     8574264.192    8574287.318    610         8056.599354282

COMM L3102104:30    703484.058     711539.229     8573592.011    8573620.267    591         8056.039267879

COMM L3102204:30    703485.558     711548.135     8573262.859    8573279.923    574         8062.94885347

COMM L3102303:30    703504.153     711547.545     8572926.164    8572955.694    595         8044.784314324

COMM L3102403:30    703507.084     711557.529     8572602.002    8572615.347    574         8050.734155665

COMM L3102603:30    703473.794     711557.499     8571932.007    8571950.003    576         8084.185212064

COMM L3102703:30    703488.245     711561.476     8571595.320    8571617.024    602         8074.584270376

COMM L3102803:30    703510.580     711569.518     8571269.304    8571281.265    573         8059.388638711

COMM L3102903:30    703536.742     711579.866     8570934.549    8570948.045    594         8043.488985628

COMM L3103103:30    703523.968     711581.637     8570267.455    8570285.557    596         8058.429525366

COMM L3103203:30    703523.008     711576.313     8569933.250    8569949.418    571         8053.658415056

COMM L3103303:30    703552.952     711592.147     8569597.776    8569617.024    592         8039.851255946

COMM L3103403:30    703520.806     711567.077     8569262.352    8569282.747    568         8046.765768756

COMM L3300101:30    710253.621     719476.592     8569648.489    8569665.264    652         9223.239090885

COMM L3300201:30    710216.955     719480.191     8569415.736    8569436.432    688         9263.767953628

COMM L3300302:31    710199.132     719475.026     8569158.713    8569192.967    669         9277.200142738

COMM L3300501:31    710192.515     719469.237     8568689.204    8568717.348    675         9278.140372028

COMM L3300601:31    710207.317     719480.081     8568458.646    8568488.249    694         9274.450506544

COMM L3300701:31    710206.350     719480.591     8568196.508    8568243.332    681         9277.582894599

COMM L3300801:31    710204.106     719484.809     8567992.630    8568006.886    686         9281.265661583

COMM L3300901:31    710209.255     719483.064     8567738.591    8567767.556    666         9275.144666803

COMM L3301001:31    710211.505     719490.194     8567494.747    8567554.793    680         9287.646861308

COMM L3301202:32    710200.554     719509.662     8567037.265    8567049.900    710         9309.408894302

COMM L3301301:31    710199.409     719511.619     8566793.806    8566818.609    663         9313.464914218

COMM L3301402:32    710200.977     719515.314     8566554.073    8566576.514    704         9315.013903744

COMM L3301501:31    710206.628     719521.471     8566301.878    8566341.170    659         9318.06965651

COMM L3301601:32    710207.393     719531.450     8566080.027    8566100.197    710         9325.391505681

COMM L3301701:32    710214.342     719524.676     8565800.461    8565858.551    680         9313.560372921

COMM L3301901:32    710238.705     719522.521     8565359.176    8565394.139    685         9287.6503423

COMM L3302001:32    710240.805     719527.625     8565114.105    8565162.477    708         9292.89185908

COMM L3302101:32    710239.602     719527.331     8564875.959    8564932.100    684         9292.620581261

COMM L3302201:32    710243.016     719525.035     8564641.891    8564687.077    705         9287.240271192

COMM L3302301:32    710241.894     719531.790     8564416.477    8564431.334    671         9290.400903946

COMM L3302401:32    710248.399     719535.236     8564174.254    8564274.823    699         9297.606722386

COMM L3302601:32    710216.609     719545.363     8563703.105    8563720.186    702         9329.235370491

COMM L3302701:32    710210.803     719537.240     8563458.180    8563480.504    668         9327.575365182

COMM L3302801:32    710220.765     719551.865     8563229.039    8563245.707    701         9331.444380319

COMM L3302901:32    710222.787     719550.933     8562980.561    8563006.904    668         9329.076758445

COMM L3303001:32    710237.528     719554.336     8562747.284    8562770.027    702         9317.174645236

COMM L3303101:32    710235.463     719557.479     8562341.062    8562534.013    673         9353.149132624

COMM L4100201:26    665061.537     673115.507     8592264.147    8592298.725    575         8055.053220691

COMM L4100301:26    665041.411     673108.959     8590595.564    8590612.075    583         8067.856868137

COMM L4100501:26    665012.377     673113.954     8587267.276    8587287.716    585         8102.13035629

COMM L4100601:26    664997.331     673117.098     8585599.998    8585618.176    586         8120.179530468

COMM L4100801:26    659158.244     672773.542     8582254.955    8582301.002    970         13618.02155689

COMM L4100901:26    659100.918     672715.167     8580598.642    8580615.757    977         13614.83625323

COMM L4101101:26    656096.824     672763.232     8577251.107    8577303.359    1188        16670.23551805

COMM L4101201:26    656106.587     672716.142     8575599.038    8575616.724    1175        16610.23961642

COMM L4200101:26    686161.426     694399.351     8575587.208    8575619.457    604         8239.101499004

COMM L4200301:26    686145.346     694332.832     8572266.100    8572285.846    602         8187.982072498

COMM L4200401:26    686151.207     694337.005     8570589.299    8570613.415    576         8186.256864921

COMM L4300101:26    678499.733     686572.543     8567264.762    8567281.550    575         8073.132143029

COMM L4300203:29    678491.786     686552.846     8565592.971    8565618.822    571         8062.054728129

COMM L4300403:29    678386.909     686438.624     8562259.765    8562288.817    572         8052.609519281

COMM L4300503:29    678372.874     686427.787     8560592.700    8560628.299    617         8056.621934125

COMM L5000101:5     720107.187     750477.663     8638932.608    8638949.648    2131        30371.22105519

COMM L5000201:7     755595.385     779510.480     8638921.919    8638959.683    1689        23916.92980451

COMM L5000301:9     791801.502     833794.852     8638923.696    8638952.174    2911        41994.4787011

COMM L5000401:5     720088.298     779522.655     8633916.259    8633960.822    4366        59441.02660505

COMM L5000501:9     791765.519     834201.055     8633924.623    8633954.300    3160        42438.39642382

COMM L5000601:5     720038.993     779521.084     8628923.242    8628955.397    4202        59485.34164304

COMM L5000701:9     791705.967     834643.276     8628926.523    8628950.669    3012        42939.40164633

COMM L5000801:5     720004.158     779507.946     8623925.114    8623961.219    4425        59509.58544055

COMM L5000901:9     791654.121     833103.237     8623915.325    8623966.872    3079        41455.81916757

COMM L5001101:10    729468.284     829159.243     8618926.246    8618984.176    7388        99696.36127847

COMM L5001301:10    740523.693     755463.667     8613929.191    8613950.585    1061        14941.07570926

COMM L5001401:11    772159.893     815392.936     8613931.109    8613958.167    3224        43234.81794223

COMM L5001501:12    719861.814     755424.710     8608735.868    8609007.057    2707        35614.71717722

COMM L5001601:12    772119.377     814175.053     8608927.256    8608965.415    3163        42057.69236334

COMM L5001701:11    719867.177     755462.339     8603922.281    8604001.521    2519        35601.39600482

COMM L5001801:13    772018.891     818692.749     8603926.062    8603962.928    3376        46676.76210147

COMM L5002101:13    803599.282     818581.398     8598927.877    8598950.520    1071        14983.39539932

COMM L5002201:13    806150.179     818477.940     8593930.020    8593951.468    894         12328.6441434

COMM L5002303:25    804445.656     818369.080     8588933.546    8588949.537    970         13923.84854111

COMM L5002401:14    789366.529     823049.215     8583901.490    8583955.008    2542        33687.29354654

COMM L5002501:14    791966.769     799955.332     8578930.868    8578957.091    573         7989.772704591

COMM L5100301:7     720125.566     749173.845     8637242.447    8637291.555    2093        29051.01544539

COMM L5100401:5     752835.464     779510.211     8637260.142    8637298.920    1966        26677.57650386

COMM L5100501:9     791795.274     833917.906     8637252.353    8637295.886    3286        42125.56832445

COMM L5100601:5     720086.641     779518.249     8635594.839    8635627.971    4129        59435.83723522

COMM L5100701:9     791772.463     834059.573     8635589.055    8635633.797    2945        42290.9510729

COMM L5101001:5     720068.133     779513.032     8632250.496    8632290.023    4235        59447.87420924

COMM L5101101:9     791741.717     834257.563     8632256.762    8632280.647    3002        42517.37932542

COMM L5101201:5     720035.029     779517.853     8630590.028    8630691.648    4315        59491.79376936

COMM L5101301:9     791730.192     834530.292     8630585.077    8630624.858    3288        42803.22504307

COMM L5101601:6     720024.825     779517.102     8627256.988    8627292.417    4266        59494.48030395

COMM L5101701:9     791695.928     834809.037     8627249.468    8627301.377    3251        43119.68969402

COMM L5101801:6     720012.841     779510.268     8625591.981    8625617.957    4253        59499.27419609

COMM L5101902:10    791671.101     834124.861     8625585.858    8625618.330    3072        42456.63823381

COMM L5102201:10    729472.928     779489.198     8622258.655    8622288.773    3694        50018.78634031

COMM L5102302:14    791638.151     831535.254     8622251.213    8622293.165    2822        39899.94576484

COMM L5102401:10    729470.484     830226.489     8620577.087    8620643.864    7162        100762.4624626

COMM L5102601:11    729480.709     823473.426     8617233.363    8617458.435    6706        94020.72460431

COMM L5102701:11    732470.716     820684.363     8615584.152    8615622.037    6459        88219.02821431

COMM L5200101:1     717915.580     725948.096     8655600.052    8655611.742    546         8032.740474386

COMM L5200201:1     716770.857     730683.787     8653928.920    8653952.112    1055        13914.33333755

COMM L5200301:1     716736.102     737247.679     8652243.054    8652317.242    1397        20515.76079584

COMM L5200401:1     717346.590     738885.393     8650594.327    8650643.454    1622        21542.07568177

COMM L5200501:1     718776.882     740500.034     8648909.544    8648955.444    1491        21725.67049193

COMM L5200601:1     720121.005     742104.219     8647260.383    8647307.699    1642        21986.63814852

COMM L5200701:1     720123.156     747233.819     8645596.900    8645644.553    1869        27113.23970991

COMM L5200801:1     720125.528     748092.923     8643923.337    8643963.101    2103        27970.51270417

COMM L5200901:2     720112.534     748933.591     8642253.380    8642287.444    1953        28822.51095423

COMM L5201001:2     720106.457     749767.787     8640583.867    8640622.679    2212        29663.44316136

COMM L6100301:2     720234.610     748851.005     8638368.335    8638404.103    2211        28619.5251476

COMM L6100401:2     720224.936     748492.639     8637806.351    8637836.615    1913        28269.47175788

COMM L6100601:2     720214.052     747829.080     8636698.396    8636731.316    1957        27617.45261031

COMM L6100701:2     720234.027     747499.437     8636149.908    8636179.482    2086        27267.80760209

COMM L6100901:5     720211.701     746833.217     8635036.439    8635069.071    1933        26623.83004493

COMM L6101001:3     720232.829     754611.191     8634473.745    8634512.471    2244        34380.78267573

COMM L6101201:3     720189.249     754643.143     8633368.013    8633400.389    2273        34455.98478564

COMM L6101301:3     720172.634     754643.529     8632814.110    8632842.082    2729        34473.50816128

COMM L6101501:3     720177.871     754648.782     8631702.845    8631740.161    2678        34474.77264267

COMM L6101601:3     720189.605     754664.323     8631146.540    8631171.073    2292        34476.04407059

COMM L6101801:4     720160.343     754616.306     8630024.940    8630057.621    2424        34458.28865371

COMM L6101901:4     720176.515     754615.555     8629295.844    8629508.714    2512        34463.5884103

COMM L6102101:4     720154.811     754609.951     8628313.000    8628489.597    2488        34477.02925688

COMM L6102201:4     720162.275     754621.612     8627801.843    8627851.367    2502        34462.42959196

COMM L6102401:4     720132.923     754594.045     8626700.831    8626735.089    2513        34464.10987448

COMM L6102501:4     720143.340     754597.411     8626144.653    8626187.273    2464        34457.06339234

COMM L6102701:4     720126.684     754613.886     8625037.882    8625069.729    2474        34489.70018707

COMM L6102801:4     720127.952     754594.641     8624484.178    8624517.823    2513        34468.4574873

COMM L6103001:4     720092.756     754582.178     8623367.182    8623395.893    2487        34492.26146985

COMM L6103101:4     720121.032     754575.055     8622793.856    8623483.225    2491        34671.10941824

COMM L6103302:6     729471.099     754559.735     8621703.005    8621732.309    1822        25089.83504492

COMM L6103401:4     729476.869     754541.483     8621116.450    8621176.996    1792        25068.18035014

COMM L6103601:6     729474.739     754535.743     8620031.570    8620063.899    1820        25062.30064502

COMM L6103702:7     729475.966     754531.943     8619467.565    8619518.162    1805        25060.51261528

COMM L6103901:6     729473.648     754527.264     8618360.925    8618400.977    1827        25057.30104748

COMM L6104001:6     729471.987     754515.868     8617814.885    8617844.216    1823        25045.89264026

COMM L6104201:6     732461.947     754521.348     8616704.671    8616727.304    1602        22060.72103978

COMM L6104301:6     732470.170     754498.473     8616150.041    8616178.910    1607        22029.29196162

COMM L6104501:6     732479.579     754494.368     8615038.273    8615071.188    1610        22016.91311817

COMM L6104601:6     732352.558     754518.375     8614476.949    8614514.876    1608        22168.26320311

COMM L6104801:6     740675.357     754423.587     8613361.965    8613408.414    986         13750.46529065

COMM L6104901:6     740677.206     754437.120     8612821.053    8612897.804    1002        13762.85249556

COMM L6105001:6     740670.143     754443.667     8612256.439    8612287.328    988         13774.60505259

COMM L6105101:7     740686.458     754449.670     8611700.448    8611737.288    980         13764.34326173

COMM L6105201:6     740685.879     754461.161     8611135.023    8611179.319    993         13778.1996528

COMM L6105301:7     740691.502     754462.016     8610599.816    8610815.748    984         13792.74064857

COMM L6105401:7     740555.708     754464.535     8609842.196    8610073.339    1045        13946.86004767

COMM L6200201:7     768233.810     779513.612     8638377.189    8638404.078    826         11281.03835174

COMM L6200301:7     768237.667     779515.541     8637818.505    8637834.686    798         11278.14689954

COMM L6200501:8     768228.322     779510.914     8636707.166    8636728.786    766         11283.23776866

COMM L6200601:7     768239.811     779517.330     8636156.183    8636184.651    823         11278.66748014

COMM L6200801:8     768251.431     779510.687     8635029.768    8635060.002    855         11260.65792975

COMM L6200901:8     768274.769     779516.368     8634482.046    8634509.136    757         11242.43000695

COMM L6201101:8     768224.566     779508.957     8633358.979    8633390.952    760         11285.1463792

COMM L6201201:8     768221.876     779522.369     8632793.875    8633050.220    865         11350.42096331

COMM L6201401:8     768196.615     779520.699     8631654.029    8631946.103    863         11363.96527957

COMM L6201501:8     768220.253     779514.416     8631142.558    8631174.529    761         11295.07067942

COMM L6201701:8     768189.292     779514.332     8630024.933    8630063.011    791         11326.76367785

COMM L6201801:8     768211.558     779516.913     8629490.885    8629511.321    854         11305.91254559

COMM L6202001:8     768201.894     779516.791     8628367.351    8628400.015    885         11316.92503204

COMM L6202101:8     768223.926     779510.387     8627820.858    8627846.948    776         11287.17484631

COMM L6202301:8     768197.141     779513.179     8626709.185    8626731.144    787         11316.64742445

COMM L6202401:8     768194.932     779513.065     8626150.842    8626182.901    881         11319.74194937

COMM L6202601:8     768173.719     779517.557     8625035.270    8625063.529    879         11344.669094

COMM L6202701:6     768195.575     779513.842     8624479.346    8624502.432    793         11318.79292839

COMM L6202901:8     768202.162     779495.565     8623368.904    8623390.555    783         11294.25756658

COMM L6203001:8     768193.283     779497.510     8622809.879    8622840.285    867         11304.93008761

COMM L6203201:8     768189.363     779474.232     8621712.684    8621730.730    862         11285.68224547

COMM L6203301:8     768215.169     779466.765     8621156.794    8621176.118    783         11252.08377397

COMM L6203501:8     768196.247     786603.082     8620045.821    8620060.423    1277        18407.44138959

COMM L6203601:8     768198.013     786605.243     8619488.083    8619515.533    1410        18408.68834663

COMM L6203801:8     768188.383     786620.994     8618370.492    8618394.210    1391        18433.76172573

COMM L6203901:8     768198.700     786650.445     8617805.845    8617836.456    1286        18453.51605909

COMM L6204101:8     768154.391     786613.448     8616692.517    8616733.991    1290        18460.56810286

COMM L6204201:8     768169.993     786606.934     8616141.837    8616182.349    1401        18440.50247273

COMM L6204401:8     772160.354     786582.711     8615035.595    8615072.115    1093        14424.03168719

COMM L6204501:8     772162.139     786596.023     8614481.595    8614521.483    999         14436.83705216

COMM L6204701:10    772157.233     786606.816     8613339.361    8613403.776    1058        14451.83147161

COMM L6204801:10    772153.287     786596.881     8612822.885    8612857.814    1031        14444.754359

COMM L6204901:10    772144.455     786594.483     8612265.681    8612288.845    1051        14451.32984948

COMM L6205001:10    772140.339     786582.994     8611704.874    8611740.780    1038        14444.71478274

COMM L6205101:11    772147.241     786575.112     8611140.764    8611167.735    1039        14429.03503471

COMM L6205201:11    772133.120     786578.886     8610587.515    8610621.620    1065        14447.57304443

COMM L6205301:11    772127.450     786564.459     8610045.173    8610071.599    1027        14438.36160167

COMM L6205401:11    772131.476     786557.500     8609485.413    8609509.759    1063        14426.9372661

COMM L6205601:13    772120.895     786604.943     8608372.940    8608393.357    1005        14484.61658125

COMM L6205701:11    772121.243     786603.136     8607822.838    8607843.429    1028        14482.73295754

COMM L6205801:13    772111.914     786590.240     8607263.908    8607283.057    1021        14478.84789557

COMM L6205901:13    772110.668     786585.889     8606704.837    8606724.427    1089        14475.65383957

COMM L6206001:13    772100.047     786579.343     8606154.683    8606191.159    1025        14480.48368259

COMM L6206101:13    772103.906     786571.135     8605599.792    8605617.386    1076        14467.88175451

COMM L6206201:13    771987.563     786569.948     8605043.669    8605076.226    1036        14583.4707097

COMM L6206301:13    771996.718     786566.000     8604475.809    8604550.019    1081        14575.0787916

COMM L6206501:13    772029.847     786566.559     8603367.871    8603405.140    1066        14537.83768018

COMM L6206601:13    772047.787     786566.010     8602820.776    8602840.788    1039        14518.72706752

COMM L6206701:13    772050.884     786558.969     8602256.664    8602285.172    1060        14509.0701642

COMM L6300101:6     758578.520     766626.039     8625929.057    8625946.405    584         8047.651841442

COMM L6300301:6     758574.694     766624.127     8625241.174    8625285.370    573         8050.699283664

COMM L6300401:6     758579.114     766638.714     8624932.489    8624962.395    566         8060.846204185

COMM L6400101:7     756861.728     764923.977     8621931.655    8621945.675    576         8062.566586962

COMM L6400201:7     756857.843     764915.320     8621597.931    8621613.841    589         8058.070356334

COMM L6400301:7     756857.002     764898.030     8621260.883    8621310.804    572         8043.357735596

COMM L6400401:7     756857.379     764881.953     8620933.166    8620957.177    584         8025.653506142

COMM L6400601:7     756839.334     764861.664     8620256.936    8620287.186    588         8023.340574569

COMM L6400701:7     756833.304     764869.922     8619932.269    8619946.810    569         8036.809943374

COMM L6400801:7     756839.415     764874.219     8619599.331    8619616.712    597         8035.122685334

COMM L6400901:7     756833.999     764876.028     8619261.549    8619292.282    571         8042.486019439

COMM L6401101:7     756818.023     764887.351     8618591.071    8618615.203    572         8070.658829634

COMM L6401201:7     756827.914     764890.186     8618264.311    8618295.121    601         8063.039484618

COMM L6600101:13    801986.664     818651.691     8602258.639    8602286.085    1191        16666.41277028

COMM L6600201:13    802796.934     818614.537     8600568.040    8600620.293    1150        15820.77850865

COMM L6600401:13    804502.103     818550.345     8597263.761    8597282.307    1024        14048.89687954

COMM L6600501:13    805413.504     818513.200     8595593.549    8595617.246    932         13100.52516238

COMM L6600701:13    806173.019     818437.287     8592251.273    8592287.204    884         12265.29772811

COMM L6600803:25    805342.113     818404.945     8590588.086    8590612.968    956         13063.39454015

COMM L6601001:13    790754.445     818903.542     8587254.756    8587286.946    2054        28151.10247592

COMM L6601101:14    790130.639     823035.689     8585591.827    8585626.846    2345        32907.41663958

COMM L6601301:14    788730.375     823063.330     8582253.913    8582293.856    2431        34335.50943019

COMM L6601401:14    791849.020     823062.897     8580594.587    8580622.852    2350        31215.14638057

COMM ALL            626097.562     834809.037     8560587.675    8655611.742    496536      6861933.602307

Conductivity data – repeat lines

COMM GA PROJECT NUMBER                                                 1196

COMM FAS PROJECT NUMBER                                                2017

COMM AREA NUMBER:                                                         1

COMM SURVEY COMPANY:                                 Fugro Airborne Surveys

COMM CLIENT:                                           Geoscience Australia

COMM SURVEY TYPE:                                       25Hz TEMPEST Survey

COMM AREA NAME:                                                     Woolner

COMM STATE:                                                              NT

COMM COUNTRY:                                                     Australia

COMM SURVEY FLOWN:                                 October to November 2008

COMM LOCATED DATA CREATED:                                       April 2009

COMM

COMM DATUM:                                                           GDA94

COMM PROJECTION:                                                        MGA

COMM ZONE:                                                               52

COMM

COMM SURVEY SPECIFICATIONS

COMM

COMM NOMINAL TERRAIN CLEARANCE:                                       120 m

COMM FINAL REPEAT LINE KILOMETRES:                                   104 km

COMM

COMM LINE NUMBERING

COMM

COMM REPEAT LINE NUMBERS:                             910000201 - 910001201

COMM                                                  911001501 - 911003001

COMM

COMM SURVEY EQUIPMENT

COMM

COMM AIRCRAFT:                                 CASA C212 Turbo Prop, VH-TEM

COMM

COMM MAGNETOMETER:                              Scintrex Cs-2 Cesium Vapour

COMM INSTALLATION:                                            stinger mount

COMM RESOLUTION:                                                   0.001 nT

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM ELECTROMAGNETIC SYSTEM:                                   25Hz TEMPEST

COMM INSTALLATION:                 Transmitter loop mounted on the aircraft

COMM                                         Receiver coils in a towed bird

COMM COIL ORIENTATION:                                                  X,Z

COMM RECORDING INTERVAL:                                              0.2 s

COMM SYSTEM GEOMETRY:

COMM RECEIVER DISTANCE BEHIND THE TRANSMITTER:                       -120 m

COMM RECEIVER DISTANCE BELOW THE TRANSMITTER:                         -35 m

COMM

COMM RADAR ALTIMETER:                                         Sperry RT-220

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM LASER ALTIMETER:                                    Optech 501SB (TEM)

COMM RECORDING INTERVAL:                                              0.2 s

COMM

COMM NAVIGATION:                                 real-time differential GPS

COMM RECORDING INTERVAL:                                              1.0 s

COMM

COMM ACQUISITION SYSTEM:                                             FASDAS

COMM

COMM DATA PROCESSING

COMM

COMM MAGNETIC DATA

COMM DIURNAL BASE VALUE APPLIED                                    47436 nT

COMM PARALLAX CORRECTION APPLIED (during final processing)            1.0 s

COMM IGRF BASE VALUE APPLIED                                       46836 nT

COMM IGRF MODEL 2005 EXTRAPOLATED TO                                 2008.8

COMM

COMM ELECTROMAGNETIC DATA

COMM SYSTEM PARALLAX REMOVED, AS FOLLOWS

COMM X-COMPONENT EM DATA                                              1.6 s

COMM Z-COMPONENT EM DATA                                              0.2 s

COMM DATA CORRECTED FOR TRANSMITTER HEIGHT, PITCH AND ROLL

COMM DATA CORRECTED FOR TRANSMITTER-RECEIVER GEOMETRY VARIATIONS

COMM CONDUCTIVITY DEPTH INVERSION CALCULATED                   EMFlow V5.10

COMM CONDUCTIVITIES CALCULATED USING HPRG CORRECTED EMX & EMZ DATA

COMM

COMM DIGITAL TERRAIN DATA

COMM PARALLAX CORRECTION APPLIED TO LIDAR ALIMETER DATA               0.0 s

COMM PARALLAX CORRECTION APPLIED TO GPS ALIMETER DATA                 0.0 s

COMM DTM CALCULATED [DTM = GPS ALTITUDE - LIDAR ALTITUDE - GPS/LIDAR DIST]

COMM N-VALUE APPLIED TO CORRECT DTM TO AUSTRALIAN HEIGHT DATUM (AHD)

COMM DATA HAVE BEEN MICROLEVELLED

COMM

COMM ----------------------------------------------------------------------

COMM DISCLAIMER

COMM ----------------------------------------------------------------------

COMM It is Fugro Airborne Survey’s understanding that the data provided to

COMM the client is to be used for the purpose agreed between the parties.

COMM That purpose was a significant factor in determining the scope and

COMM level of the Services being offered to the Client. Should the purpose

COMM for which the data is used change, the data may no longer be valid or

COMM appropriate and any further use of, or reliance upon, the data in

COMM those circumstances by the Client without Fugro Airborne Survey's

COMM review and advice shall be at the Client's own or sole risk.

COMM

COMM The Services were performed by Fugro Airborne Survey exclusively for

COMM the purposes of the Client. Should the data be made available in whole

COMM or part to any third party, and such party relies thereon, that party

COMM does so wholly at its own and sole risk and Fugro Airborne Survey

COMM disclaims any liability to such party.

COMM

COMM Where the Services have involved Fugro Airborne Survey's use of any

COMM information provided by the Client or third parties, upon which

COMM Fugro Airborne Survey was reasonably entitled to rely, then the

COMM Services are limited by the accuracy of such information. Fugro

COMM Airborne Survey is not liable for any inaccuracies (including any

COMM incompleteness) in the said information, save as otherwise provided

COMM in the terms of the contract between the Client and Fugro Airborne

COMM Survey.

COMM

COMM With regard to DIGITAL TERRAIN DATA, the accuracy of the elevation

COMM calculation is directly dependent on the accuracy of the two input

COMM parameters lidar altitude and GPS altitude. The radar altitude value may be

COMM erroneous in areas of heavy tree cover, where the altimeter reflects the

COMM distance to the tree canopy rather than the ground. The GPS altitude value

COMM is primarily dependent on the number of available satellites. Although

COMM post-processing of GPS data will yield X and Y accuracies in the

COMM order of 1-2 metres, the accuracy of the altitude value is usually

COMM much less, sometimes in the ±5 metre range.  Further inaccuracies

COMM may be introduced during the interpolation and gridding process.

COMM Because of the inherent inaccuracies of this method, no guarantee is

COMM made or implied that the information displayed is a true

COMM representation of the height above sea level.  Although this product

COMM may be of some use as a general reference,

COMM THIS PRODUCT MUST NOT BE USED FOR NAVIGATION PURPOSES.

COMM ----------------------------------------------------------------------

COMM

COMM ELECTROMAGNETIC SYSTEM

COMM

COMM TEMPEST IS A TIME-DOMAIN SQUARE-WAVE SYSTEM,

COMM TRANSMITTING AT A BASE FREQUENCY OF 25Hz,

COMM WITH 2 ORTHOGONAL-AXIS RECEIVER COILS IN A TOWED BIRD.

COMM FINAL EM OUTPUT IS RECORDED 5 TIMES PER SECOND.

COMM THE TIMES (IN MILLISECONDS) FOR THE 15 WINDOWS ARE:

COMM

COMM WINDOW      START       END      CENTRE

COMM    1        0.007      0.020      0.013

COMM    2        0.033      0.047      0.040

COMM    3        0.060      0.073      0.067

COMM    4        0.087      0.127      0.107

COMM    5        0.140      0.207      0.173

COMM    6        0.220      0.340      0.280

COMM    7        0.353      0.553      0.453

COMM    8        0.567      0.873      0.720

COMM    9        0.887      1.353      1.120

COMM   10        1.367      2.100      1.733

COMM   11        2.113      3.273      2.693

COMM   12        3.287      5.113      4.200

COMM   13        5.127      7.993      6.560

COMM   14        8.007     12.393     10.200

COMM   15       12.407     19.993     16.200

COMM

COMM PULSE WIDTH: 10 ms

COMM

COMM TEMPEST EM data are transformed to the response that would be

COMM obtained with a B-field sensor for a 100% duty cycle square

COMM waveform at the base frequency, involving a 1A change in

COMM current (from -0.5A to +0.5A to -0.5A) in a 1sq.m transmitter.

COMM It is this configuration, rather than the actual acquisition

COMM configuration, which must be specified when modelling TEMPEST data.

COMM

COMM

COMM

COMM LOCATED DATA FORMAT

COMM

COMM  Output field format : DOS  - Flat ascii

COMM  Number of fields    : 68

COMM

COMM Field  Channel         Description                       Units  Undefined    Format

COMM -----  -------         -----------                       -----  ---------    ------

COMM

COMM     1  LINE            Line                                      -99999999      i10

COMM     2  FLIGHT          Flight                                    -99             i4

COMM     3  FID             Fiducial                            (s)   -999999.9     f8.1

COMM     4  PROJECT_FAS     FAS Project Number                        -9999           i6

COMM     5  PROJECT_GA      GA Project Number                         -9999           i6

COMM     6  AIRCRAFT        System Number                             -9              i3

COMM     7  DATE            Date                           ddmmyyyy   -9999999        i9

COMM     8  TIME            Time - local midnight               (s)   -9999.9       f8.1

COMM     9  BEARING         Line Bearing                      (deg)   -99             i4

COMM    10  LATITUDE        Latitude  GDA94                   (deg)   -99.9999999  f12.7

COMM    11  LONGITUDE       Longitude GDA94                   (deg)   -999.9999999 f13.7

COMM    12  EASTING         Easting   MGA51                     (m)   -99999.99    f10.2

COMM    13  NORTHING        Northing  MGA51                     (m)   -999999.99   f11.2

COMM    14  LIDAR           Final Lidar altimeter               (m)   -999.99       f8.2

COMM    15  RADALT          Final Radar altimeter               (m)   -999.99       f8.2

COMM    16  TX_ELEVATION    Final Transmitter Elevation - AHD   (m)   -999.99       f8.2

COMM    17  DTM             Final Ground Elevation - AHD        (m)   -999.99       f8.2

COMM    18  MAG             Final TMI                          (nT)   -99999.999   f11.3

COMM    19  CND_DS01        CDI_depth_slice_01   0-    5 m   (mS/m)   -9999.999    f10.3

COMM    20  CND_DS02        CDI_depth_slice_02   5-   10 m   (mS/m)   -9999.999    f10.3

COMM    21  CND_DS03        CDI_depth_slice_03   10-  15 m   (mS/m)   -9999.999    f10.3

COMM    22  CND_DS04        CDI_depth_slice_04   15-  20 m   (mS/m)   -9999.999    f10.3

COMM    23  CND_DS05        CDI_depth_slice_05   20-  30 m   (mS/m)   -9999.999    f10.3

COMM    24  CND_DS06        CDI_depth_slice_06   30-  40 m   (mS/m)   -9999.999    f10.3

COMM    25  CND_DS07        CDI_depth_slice_07   40-  60 m   (mS/m)   -9999.999    f10.3

COMM    26  CND_DS08        CDI_depth_slice_08   60- 100 m   (mS/m)   -9999.999    f10.3

COMM    27  CND_DS09        CDI_depth_slice_09   100-150 m   (mS/m)   -9999.999    f10.3

COMM    28  CND_DS10        CDI_depth_slice_10   150-200 m   (mS/m)   -9999.999    f10.3

COMM    29  CND[1]          Conductivity_001       0-  5 m   (mS/m)   -9999.999    f10.3

COMM    30  CND[2]          Conductivity_002       5- 10 m   (mS/m)   -9999.999    f10.3

COMM    31  CND[3]          Conductivity_003      10- 15 m   (mS/m)   -9999.999    f10.3

COMM    32  CND[4]          Conductivity_004      15- 20 m   (mS/m)   -9999.999    f10.3

COMM    33  CND[5]          Conductivity_005      20- 25 m   (mS/m)   -9999.999    f10.3

COMM    34  CND[6]          Conductivity_006      25- 30 m   (mS/m)   -9999.999    f10.3

COMM    35  CND[7]          Conductivity_007      30- 35 m   (mS/m)   -9999.999    f10.3

COMM    36  CND[8]          Conductivity_008      35- 40 m   (mS/m)   -9999.999    f10.3

COMM    37  CND[9]          Conductivity_009      40- 45 m   (mS/m)   -9999.999    f10.3

COMM    38  CND[10]         Conductivity_010      45- 50 m   (mS/m)   -9999.999    f10.3

COMM    39  CND[11]         Conductivity_011      50- 55 m   (mS/m)   -9999.999    f10.3

COMM    40  CND[12]         Conductivity_012      55- 60 m   (mS/m)   -9999.999    f10.3

COMM    41  CND[13]         Conductivity_013      60- 65 m   (mS/m)   -9999.999    f10.3

COMM    42  CND[14]         Conductivity_014      65- 70 m   (mS/m)   -9999.999    f10.3

COMM    43  CND[15]         Conductivity_015      70- 75 m   (mS/m)   -9999.999    f10.3

COMM    44  CND[16]         Conductivity_016      75- 80 m   (mS/m)   -9999.999    f10.3

COMM    45  CND[17]         Conductivity_017      80- 85 m   (mS/m)   -9999.999    f10.3

COMM    46  CND[18]         Conductivity_018      85- 90 m   (mS/m)   -9999.999    f10.3

COMM    47  CND[19]         Conductivity_019      90- 95 m   (mS/m)   -9999.999    f10.3

COMM    48  CND[20]         Conductivity_020      95-100 m   (mS/m)   -9999.999    f10.3

COMM    49  CND[21]         Conductivity_021     100-105 m   (mS/m)   -9999.999    f10.3

COMM    50  CND[22]         Conductivity_022     105-110 m   (mS/m)   -9999.999    f10.3

COMM    51  CND[23]         Conductivity_023     110-115 m   (mS/m)   -9999.999    f10.3

COMM    52  CND[24]         Conductivity_024     115-120 m   (mS/m)   -9999.999    f10.3

COMM    53  CND[25]         Conductivity_025     120-125 m   (mS/m)   -9999.999    f10.3

COMM    54  CND[26]         Conductivity_026     125-130 m   (mS/m)   -9999.999    f10.3

COMM    55  CND[27]         Conductivity_027     130-135 m   (mS/m)   -9999.999    f10.3

COMM    56  CND[28]         Conductivity_028     135-140 m   (mS/m)   -9999.999    f10.3

COMM    57  CND[29]         Conductivity_029     140-145 m   (mS/m)   -9999.999    f10.3

COMM    58  CND[30]         Conductivity_030     145-150 m   (mS/m)   -9999.999    f10.3

COMM    59  CND[31]         Conductivity_031     150-155 m   (mS/m)   -9999.999    f10.3

COMM    60  CND[32]         Conductivity_032     155-160 m   (mS/m)   -9999.999    f10.3

COMM    61  CND[33]         Conductivity_033     160-165 m   (mS/m)   -9999.999    f10.3

COMM    62  CND[34]         Conductivity_034     165-170 m   (mS/m)   -9999.999    f10.3

COMM    63  CND[35]         Conductivity_035     170-175 m   (mS/m)   -9999.999    f10.3

COMM    64  CND[36]         Conductivity_036     175-180 m   (mS/m)   -9999.999    f10.3

COMM    65  CND[37]         Conductivity_037     180-185 m   (mS/m)   -9999.999    f10.3

COMM    66  CND[38]         Conductivity_038     185-190 m   (mS/m)   -9999.999    f10.3

COMM    67  CND[39]         Conductivity_039     190-195 m   (mS/m)   -9999.999    f10.3

COMM    68  CND[40]         Conductivity_040     195-200 m   (mS/m)   -9999.999    f10.3

COMM

Line Number    X-minimum      X-maximum      Y-minimum      Y-maximum      # of points Total distance

-----------    ---------      ---------      ---------      --------       ----------- --------------

D910000101:1   758617.973     766595.117     8625929.214    8625951.529    593     7977.960349781

D910000201:2   758614.627     766605.068     8625930.235    8625948.988    611     7991.157895713

D910000301:3   758617.752     766602.559     8625930.117    8625948.046    630     7985.071977546

D910000401:4   758611.606     766594.805     8625921.834    8625947.767    576     7984.167942048

D910000601:6   758606.464     766598.474     8625929.259    8625946.404    580     7992.141857788

D910000901:9   758613.838     766605.406     8625925.485    8625954.563    573     7992.126408156

D910001201:12  758607.447     766606.442     8625928.823    8625945.733    546     7999.319002098

D910001201:12  758622.956     766606.442     8625928.823    8625945.733    545     7983.803057622

D911001501:15  751706.078     759788.070     8568934.638    8568950.168    559     8082.2088958

D911001801:18  751701.721     759789.825     8568922.931    8568945.713    582     8088.483041208

D911002101:21  751710.376     759788.164     8568938.295    8568948.949    575     8078.168517275

D911002401:24  751703.730     759783.265     8568936.783    8568946.489    582     8079.688873307

D911002701:27  751705.815     759787.134     8568932.214    8568950.259    568     8081.686572674

D911003001:30  751700.154     759780.395     8568936.550    8568950.734    581     8080.582651933

ALL            751700.154     766608.622     8568922.931    8625954.563    7556    104410.5028871

APPENDIX V – Standby Days

| Aircraft: VH-TEM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date | Days | Description |  | Kms Flown |  |  |
| 20/10/2008 | 1 | High sferics |  | 0 |  |  |
| 21/10/2008 | 1 | Strong Winds/High Spherics |  | 0 |  |  |

APPENDIX VI – List of all Supplied Data and Products

Streamed EM Data

For each survey flight the raw continuous time series data was delivered as a series of binary (*.raw) files for each traverse and calibration line (each file being a maximum 1Gb in size). An associated text file linking the traverse line to the file name accompanied the data.

Field Located Data

Following completion of acquisition, located data was delivered for all survey lines.

For the filename below, there are 2 files.  Extension ‘.asc’ is the data and ‘.hdr’ is a header describing the data format and survey specifications.

Traverse lines:

Woolner_TEM_field_data_all

High-Altitude (zero) lines:

Woolner_TEM_field_zero_line_data

Repeat lines:

Woolner_TEM_field_repeat_line_data

Final Located Data

For each filename below, there are 2 files.  Extension ‘.asc’ is the data and ‘.hdr’ is a header file describing the data format and survey specifications.

Traverse lines:

Woolner_Final_EM

Woolner_Final_CND

Repeat lines:

Woolner_Final_EM_RptLines

Woolner_Final_CND_RptLines

For each of the suffixes EM and CND, the contents are as follows:

EM	–	data with and without geometry correction as defined in Attachment 1, section 3.3 (b) of the contract.

CND	–	Conductivity-depth data as defined in Attachment 1, section 3.3(c) of the contract.

Final Gridded Products

The following ERMapper format grids were delivered for each regional and infill area (with National Parks excluded), using the optimal grid cell size (1/5th of the line spacing):

- Total Magnetic Intensity

- Digital Elevation Model

- Interval Conductivity grids for the following depth ranges:

- 0-5 metres

- 5-10 metres

- 10-15 metres

- 15-20 metres

- 20-30 metres

- 30-40 metres

- 40-60 metres

- 60-100 metres

- 100-150 metres

- 150-200 metres

CDI Multiplots

PDF format files of the EM multiplots, derived from the final processed and combined EM X and EM Z component data, were delivered for every survey line at 1:50 000 scale. The multiplots were trimmed to the National Park boundaries.

Final Acquisition and Processing Report

Delivered as hardcopy and digitally (6 copies)

APPENDIX VII – EM Repeat line CDIs

Repeat Line 910FFFFPP (758600mE 8625930mN to 766600mE 8625930mN)

Repeat Line 911FFFFPP (749700mE 8568950mN to 760000mE 8568950mN)