































CSA

A.B.N. 67 077 165 532

Geological Consultants

CSA Australia Pty Ltd

Date Created:  6 May 2008

Date Printed:  6 May 2008

Report No. 128.2002

Newera

Assessment of EL’s

24838, 25296, 25674, 26046, 26047, and 26048

And Report on Previous Exploration

Northern Territory

By

Karl Lindsay-Park

Member AusIMM

For:

Newera Uranium

Suite 5/2 Centro Ave

Subiaco WA 6008

By:

CSA Australia Pty Ltd
Level 1, 47 Burswood Road
Burswood
Western Australia 6100

Approved

________________________

Managing Director

Executive Summary

A review of the previous exploration work and an assessment of the public domain geophysical data have been completed. The review has shown that the only significant work in the area of the exploration licences was undertaken by PNC Australia. Their work consisted of large-area airborne magnetic and radiometric surveys followed by ground mapping and rock chip sampling.

Several hundred anomalous areas were defined and several, but not all were visited. The assay results from the rock chip sampling have been digitally captured by the NTGS and are available from their Explorer 3 data base. Anomalous rock chip assay results for base metals, Nickel, copper and particularly for uranium are present.

To advance the exploration effort in the area of the licences all of the PNC data will need to be digitally captured and interrogated. The areas of anomalous rock chip geochemistry will need to be visited and an assessment of their economic potential made.

Contents

Executive Summary	II

Contents	III

1.	Introduction	1

2.	Previous Exploration	1

3.	Previous Exploration Discussion	2

4.	Geology	2

5.	Geophysics	3

6.	Geochemistry	3

7.	Conclusions	3

Tables

Table 1.  License Details	1

## Introduction

The exploration licences form an almost contiguous group which lies approximately 150 km east of Alice Springs in a straight line.  To drive to the licenses requires a drive of nearly double that being 75 km along the Stuart Highway, 170 km east along the well formed but unsealed Plenty Highway followed by 40 km south along station tracks.  Table 1 below displays the relevant tenement details.  The tenements lie on the Illogwa Creek 1:250,000 geological map sheet, (SF5315).

Table 1.  License Details

| TENEMENT | NO OF BLOCKS | AREA (SQKM) | GTRANT DATE |
| --- | --- | --- | --- |
| EL 26048 | 10 | 31.6 | 26/11/07 |
| EL 25647 | 3 | 9 | 24/8/07 |
| EL 26047 | 4 | 12.5 |  |
| EL 25296 | 33 | 104.2 | 2/11/06 |
| EL 24838 | 14 | 44.2 | 6/4/06 |
| EL 26046 | 5 | 15.79 | 26/11/07 |

Whilst access to the licences area is reasonably good travel within the licences is made difficult due to the steep and rocky hillsides.  Several old mica mines occur in the region although there are very few records of this work

The Alice Springs region typically enjoys a long, dry and hot summer and a cold and dry winter.  Rainfall is uncommon and does not represent an impediment to exploration activities.

The area is considered prospective for base metals, Nickel, Rare Earth Elements (REE’s) and uranium.

## Previous Exploration

Rather than describe all of the previous exploration, much of which was for gemstones, diamonds and garnets only the exploration for the commodities listed above will be described.  Because the current tenements are so small relative to their predecessors the licences will be dealt with as a group.  For a description of the previous exploration on a licence by licence basis the reader is referred to the Previous Tenure spreadsheets that accompany this report.

AP 1991, AP 2277: Placer Prospecting explored for REE's , tantalite and Uranium via ground based radiometric surveys and base metals via stream sediment sampling.  No locations or results presented.

AP 2568: was explored by Arcadia Minerals.  They completed a geological assessment and ground based spectrometer and magnetic surveys.  Their work focused on the pegmatite and ultramafic rocks. Some elevated Cu-Ni rock chip samples are described but no locations are provided.

EL 568 was explored for tungsten mineralisation hosted by metamorphosed calc-silicates. An auger was used to collect soil samples at Prospect A which is located in EL 24068.

EL 6604 did not cover much of the combined licence area however Pan Continental did compile all of the historical stream sediment data in their area of interest. 87 anomalies were recognized for gold, base metals and REE’s.  The data pertaining to the current EL’s could be captured.

EL’s 7967, 7990, 7991, 7992, 7993, 7994, 8036, 8148, 8220, 8675, 8901 and 8906 were all part of a large tenement holding explored by PNC Australia. The exploration activities on all of the licences were jointly reported, thus the open file reports are enormous, for example CR1995-0285 is over 1.8GB and CR1996-0285 is 326 MB.

The work done included flying 15000 km of airborne geophysics which led to the identification of hundreds of anomalies of which 217 were pursued. Geochemical sampling, mapping and drilling were all used.  PNC walked away from the project despite some of the good results obtained.  It should be noted at this point that PNC also walked away from the world-class Nolan’s Bore Phosphate-REE-Uranium mineralisation.

Some of the data (rock chip sampling) contained in the reports has been captured and is presented in Harts Range Rock Chips.xls which accompanies this report.  The rest of the data contained in the reports can be captured but it is outside the scope of this report to do so.

## Previous Exploration Discussion

The historical exploration completed on the tenements has covered many commodities including gold, uranium, base metals, REE’s and several industrial products like garnets and gem stones.  Of all the work done, that undertaken by PNC is by far the most exhaustive and of the greatest value.  PNC’s work used the latest instruments available at the time and was managed by skilled and experienced geologists.  The conclusions they reached were correct based on the data they had collected and for the time in which they operated.  Recent changes in commodity prices, infrastructure and mining methods have invalidated their conclusions.

## Geology

The NTGS and GA have recently finished remapping and publishing the Quartz 1:100,000 map sheet.  The new data has been ordered and when available will prove an asset in providing a frame work on which the results of previous work can be understood and new work can be planned.

## Geophysics

Examination of the NTGS 2006 total magnetic intensity imagery reveals two areas of interest. The magnetic anomalies are large (2 to 2.5 km across) and apparently related to ultramafic intrusives.  The anomalies are located in the central part of EL’s 26047 and 24838.  The Vtem image provided by Newera Uranium shows the position of the magnetic anomalies and a number of EM anomalies associated with these bodies.  The old (1963) 1:250k geological map-sheet shows a copper occurrence near the magnetic anomaly in EL 24838.

Elsewhere the airborne magnetic survey data responds to the gross stratigraphy.

The radiometric data available in the area is too course to allow more than the recognition of just the broadest geological elements.  Likewise the uranium alone image fails to provide any illumination.

## Geochemistry

The Explorer 3 geochemical database managed by the NTGS contains the results of the analysis of 373 rock chip samples collected from within and near the Newera Uranium tenements.  Most of the samples were collected by PNC and subject to multi-element analysis. A casual inspection of the data, presented in Harts Range Rock Chips.xls, show there are several highly anomalous areas.

Anomalous responses have been recognized for Cu, Pb, Zn, Ni, Cr, V, P, U, and Th.  The results for uranium are spectacular and range from numerous samples reporting hundreds of ppm, several samples reporting thousands of ppm and some reporting tens of thousands of ppm.  A detailed assessment of the data is warranted to establish the geographic distribution of the better results and determine the significance of the results in terms of what was sampled and any economic potential.

A similar assessment of all of the rock chip data is required to determine its real value.

## Conclusions

The Harts Range group of tenements has the potential to host base metal, nickel, copper, REE and uranium / thorium mineralisation.  The previously completed work has provided some useful information in the area but most of the work done has been of very limited extent.  The work completed by PNC on the other hand has been extensive, well managed and thorough.

PNC’s work identified several areas of anomalism particularly uranium, thorium, and phosphorus.  Importantly, they did not assay for REE’s but results obtained by Arafura Resources at Nolan’s Bore clearly demonstrate the link between uranium, REE and apatite (phosphorus) mineralisation.

It is understood that Newera Uranium has flown a Vtem survey over at least part of the tenement holding.  The anomalies defined by the survey appear to be associated with some large magnetic anomalies related to ultra-mafic intrusions.  The rock chip descriptions and geochemical data need to be examined to see what information can be gained.