MINERALOGICAL REPORT No.  7742

by Alan C. Purvis,  PhD.

December 3rd , 1998

TO :	Mr Paul Melville

PNC Exploration (Aust) Pty Ltd

Unit 5/4 Durand Court

COCONUT GROVE  NT  0810

YOUR REFERENCE :	Your letter dated 29th October 1998

MATERIAL &	RAB cuttings (15),	9603 to 9627

IDENTIFICATION :	Rock Sample (1),	9648

WORK REQUESTED :	Thin section preparation, description and

report with comments and interpretations as specified.

SAMPLES & SECTIONS :	Returned to you with this report.

PONTIFEX & ASSOCIATES PTY. LTD.

SUMMARY COMMENTS

This report discusses twenty-nine samples from Arnhem Land in the Northern Territory, mainly as petrographic descriptions of fifteen samples of drill chips which were mounted in epoxy and made into composite thin sections and fourteen rock and core samples examined in normal thin sections.  Opaque minerals were identified using low angle incident light.

Chip Samples

The chip samples include gneisses, (possibly metasediments), a suite of largely sericitised to silicified granitoids, and two altered dolerites, represented as follows:

- Granitoid:  	9603, 9604, 9609, 9614, 9620, 9625, 9626, 9627

- Dolerite:  	9605, 9606

The granitoids have (inequigranular) granuloblastic fabric, and plagioclase distinctively, totally altered to sericite ± quartz, including local greisen-like quartz.  Some samples have a minor microcline and some have chips large enough to display a tectonic foliation.  Some have low-temperature hydrothermal to supergene quartz and sample 9604 is largely supergene silica and soil.  The inherent quartz in this lithology is generally finer than in the gneissic group above.

The dolerites are weathered but seen to be undeformed and probably post-metamorphic.  In addition to the samples of dolerite referred to above, there are also chips of weathered dolerite in sample 9609.

Rock sample

The single rock sample 9648 is a massive (fresh) anthophyllite-chlorite rock, which objectively (and in isolation) could be interpreted as a metapyroxenite, or a metamorphosed impure dolomite.

INDIVIDUAL DESCRIPTIONS

RAB CHIP SAMPLES

9603	Chips of quartz, sericite apparently after plagioclase also of sericite-limonite-leucoxene after biotite.  Most likely a meta-granitoid.  Also chips which contain or consist of vein quartz, and supergene to low-temperature hydrothermal quartz.

Small chips are seen in this sample and these are dominated by quartz, also sericite apparently after plagioclase, accompanied by sericite-limonite-leucoxene pseudomorphs of biotite flakes, all to 1mm in grainsize.  Rarely the formerly quartzofeldspathic chips have spherulites of chlorite and patches of muscovite. These quartzofeldspathic chips seem mostly likely to represent a granitoid.  Other chips consist of probable vein quartz and some of possibly of low-temperature hydrothermal quartz.  One chip is a breccia of quartz fragments in microcrystalline quartz (?silcrete).

9604	Most chips of supergene silica and soil.  Minor chips of altered granitoid as in the previous sample.

There are rare chips in this sample composed of quartz and sericite, apparently after plagioclase, as in the previous sample.  The majority of the chips however, are dominated by supergene quartz or by limonite-clouded clays with some patches that may represent former biotite flakes.  The clay chips seem to represent possibly residual soils but this sample seems to be too shallow for adequate interpretation of the bedrock.

9605	Weathered massive quartz-bearing dolerite.

Clays, probably smectites, limonite and leucoxene alteration products are the main components in this sample and these seem to represent:

- Plagioclase laths to 3 mm long.	50-55%

- Ophitic mafic silicates, probably pyroxene, to 3mm in grainsize.	40%

- Skeletal opaque oxide to 2 mm in grainsize, now leucoxene.	5%

- Also rare late magmatic quartz 	3%

- Rare residual apatite to 0.5 mm in grainsize	<1%

This interpreted mineralogy and the recognition of ophitic (textured) ex-mafic silicates and the scattered ex-skeletal opaque oxides pseudomorphed by leucoxene, allows the confident interpretation of an original dolerite.

9606	Weathered to silicified quartz dolerite (as in the previous sample), also minor vein quartz.  Includes chips with micro-quench texture.

Most of these chips represent a similarly altered quartz dolerite to that in the previous sample, albeit with less abundant limonite and locally with silicification in addition to primary late magmatic quartz.  Some of the chips represent a finer-grained dolerite however, than that in the previous sample, with plagioclase about 1 mm in grainsize.  Indeed the fine textures in some chips seem to represent quenching.  Rare chips of dog-tooth sparry vein quartz are also evident.

9609	Sericitised and partly silicified granitoid as in 9603-4 with minor microcline, also vein quartz ± clays and brecciated quartz with supergene silica and clays.

Chips similar to the altered granitoids in samples 9603-4 are seen in this sample, albeit with minor microcline to 4 mm in grainsize as well as fine-grained quartz and sericite ± albite-altered plagioclase.  Some of the chips have chlorite and/or clays, partly interlaminated with leucoxene and after biotite flakes.

Rare chips of weathered dolerite occur as in the previous two samples, and there are silicified chips with granular to prismatic quartz as well as brecciated silicified quartz-rich chips with disseminated clays and supergene quartz.

9614	Silicified and sericitised granitoid as in 9603-4 and 9609, also siliceous breccias.

In this sample granular quartz is associated with areas rich in secondary low-temperature hydrothermal quartz with minor to abundant sericite, possibly derived from feldspar.  Very minor chlorite-leucoxene-altered biotite is also evident as well as altered myrmekite patches to 1mm in diameter.  Some of the chips are composed of fine-grained breccias with quartz and quartz-rich to sericite-rich fragments in a quartz to clay-rich matrix.  These breccias seem to be of low-temperature hydrothermal or supergene origin.

These chips seem to represent a silicified granitoid as in 9603-4 and 9609.

9620	Sericitised mostly fine grained granitoid similar to 9603-4, 9609, 9614, including greisen-like quartz enrichment.  Also chips of vein quartz and low temperature hydrothermal quartz.

Chips with granular primary quartz (to 5 mm in grainsize) and albite to sericite to quartz-sericite-altered plagioclase are common in this sample, with quartz as seen in some greisens. Chlorite to clay-leucoxene-altered biotite is widespread and up to 1 mm in grainsize.  Some of the chips have been altered and have low-temperature hydrothermal quartz, or are composed of vein quartz with chloritised biotite and patches of sericite.  The majority of the chips represent granitoid as seen in several previous samples, partly greisenised, with some vein quartz and low-temperature hydrothermal quartz

9625	Very extensively sericitised granitoid as in the several samples of this classification described above, but without silicification.  Minor vein quartz.

Inequigranular granitoid chips with sericite after feldspar and quartz from 0.2 to 4 mm in grainsize are seen in this sample and clearly belong to the same granitoid as that in the previous samples, but without the silicification seen in those described immediately above.  Minor sericite to chlorite-leucoxene-altered biotite is disseminated.  One chip is a crack-seal quartz vein with lamellae of clays separating layers of granular to prismatic quartz.  Other chips of vein quartz occur, locally interlaminated with sericite.

9626	Sericite-clay-chlorite-leucoxene-altered granitoid, also low-temperature hydrothermal quartz ± chlorite.  (Altered) chips of uncertain origin composed of clay-sericite-quartz.

Chips of sericite-altered granitoid occur in this sample, as in the previous samples, but these chips are much richer in sericite to chlorite to clay-leucoxene-altered biotite as flakes to 3 mm long (~7-8%). They also seem to have less abundant quartz (25-30%) and there is little evidence of silification.

There are also chips of low-temperature hydrothermal granular to prismatic quartz ± chlorite and various clay to sericite to quartz-rich chips of uncertain origin.  These latter chips have millimetre-scale domains that are alternately rich in clays and sericite and locally contain minor quartz.

9627 	Foliated biotite granitoid with sericitised plagioclase and clay to sericite-leucoxene-altered biotite.

Larger chips in this sample are altered biotite-beating granitoids, with 20-25% quartz as lobate grains, as well as 65% sericite after plagioclase, 10% clay-leucoxene-altered biotite as well-oriented flakes to 2 mm long and 2-3% microcline to 3 mm in grainsize.  The plagioclase is granular to subhedral and up to 4 mm in grainsize with quartz to 2 mm.  Very narrow veins occur with quartz or with albite.

Smaller chips seem to represent fragments of this same lithology.  The gross mineralogy and texture indicates a granitoid lithology as basically seen in “granitoid” samples described above.

ROCK SAMPLE

9648	Anthophyllite rock with chlorite, possible talc and limonite.  Objectively (and in isolation) this could be interpreted as a metapyroxenite or metamorphosed impure dolomite.

This is an unusual rock with:

- Abundant anthophyllite as unoriented prisms to 4 mm long, partly in low-

- angle fans, fractured at a high angle to the elongation of the crystals,	90%

- Interstitial coarse-grained chlorite, apparently clinochlore.	5-6%

- Minor possible talc.	1%

- Limonite on grain boundaries and interstitial,	3%

It may represent a metamorphosed pyroxenite or a metamorphosed impure dolomite.