- Giants Reef Mining Limited

- Gravity / GPS surveys in NT near Tennant Creek

- June 2001

Report Number 01023

DR Daish

CLIENT:

Giants Reef Mining Limited

1/26 Irvine Street

PO Box 1244

TENNANT CREEK,  NT  0861

- CLIENT CONTACT

Frank Lindeman

Tel:	(03) 94211895

- SURVEY CONTRACTOR

- DAISHSAT PTY. LTD

P.O. Box 766

MURRAY BRIDGE S.A. 5253

Tel:  (08) 8531 0349

Fax: (08) 8531 0684

## CONTENTS

1.   SURVEY OVERVIEW						2

  - Data acquisition procedures				2

1.2	Data acquisition progress				3

2.	PERSONNEL AND EQUIPMENT				4

2.1	Personnel						4

2.2	Survey Equipment					4

2.3 	Vehicles and transport					4

2.4	Camp							4

3.	GPS SURVEYING AND PROCESSING			5

3.1 	Set out of the gravity grid				5

3.2 	Set up of GPS control					5

3.3	Surveying of the position and level data		6

3.4	Post processing of the position and level data	6

4. 	GRAVITY ACQUISITION AND PROCESSING		7

4.1	Gravity data acquisition				7

4.2 	Gravity data processing				8

  - Set up of the gravity control				10

## APPENDICES

Appendix A	Images and maps

Appendix B	DISK of RESULTS

1.	SURVEY OVERVIEW

The Blue Bush gravity survey was carried out between the 1st and 8th June 2001, for Giants Reef Mining Limited.  The job consisted of approximately 650 stations on a 1km by 1km grid and 180 200m infill stations located in open spinifex, mulga and turpentine country, about 40 km to the West of Tennant Creek in the Northern Territory.

Scintrex CG-3 gravity metres were used for the gravity acquisition while position and level data was obtained using Ashtech Z12 GPS units to produce precise kinematic differential GPS locations.

Access was by helicopter (Bell 47 G5 VH-TZW) on the 1km stations and Landcruiser 4wd utility on the open infill areas.

##### The Landcruiser used on the surveying in typical country.

1.1	Data acquisition procedures

The entire survey was carried out by helicopter and vehicle, using the standard loop procedure, with each loop starting and finishing at the Tennant Creek gravity base at the airport.  The time between initial and final occupation of a base was less than 8 hours in most instances.  Longer loops were used when it was not always efficient to come back to base at lunchtime.  This was kept to a minimum, but was unavoidable when working in the remote areas.  In any case, the gravity loop closure and gravity repeat data was sufficiently accurate to exceed project specifications, even with the all day loops.

Navigation to each station utilised Garmin GPS II+’s.  Although care was taken to ensure that the helicopter was generally within 100 metres of the ideal station coordinates, some stations were located further than this from their nominated coordinates due to topographical features such as mulga thickets, dense foliage and other factors.    On the 200m-infill survey, the stations were typically located better than 20m from their nominated station coordinates.

During field operations, there was generally a two-man rover crew.  In the case of the helicopter survey, the crew consisted of the pilot and the gravity surveyor. In the case of the infill survey, two gravity surveyors were used.  Gravity readings were taken as close to the car or helicopter as was practical. The GPS antenna was set up on the LHS of the Landcruiser and on the tail boom of the helicopter. Due to the generally flat topography in the surveyed areas, the gravity meter was deemed to be within a couple of centimetres of the measured ground level.  In order to reduce any noise, the motor of the vehicle was always switched off while readings were being taken.    For the helicopter work, it is not practical to shut down the engine for each reading although the revs and collective pitch are lowered to a level that minimally affects the gravity readings.

###### Scrub bashing through turpentine on the 200m infill survey

1.2 Data acquisition progress

Progress was steady throughout the entire job.  The following chart shows the station occupation rate during the survey.   Typical helicopter production (2/6/01 – 6/6/01) was around 120 – 130 new stations per day while on the 200m infill survey (7/6/01 – 8/6/01), typical production for a full day was just over 100 stations per day

2. 	PERSONNEL AND EQUIPMENT

2.1 	Personnel

The surveyor in charge of the job was David Daish who was responsible for the daily running of the job, processing of the data and data acquisition.  Tim Klau carried out additional acquisition, GPS and gravity control, most of the processing and running around after the helicopter.  Further reduction of the data, and presentation of the results in the required format, was carried out by the company geophysicist Leon Mathews.

The helicopter pilot was Mark Robbins from Helimuster.

2.2 Survey equipment

	Two Scintrex CG-3 gravity meter’s (8810110, 9303204),

	One Leica SR9500 dual frequency GPS receiver,

	One Leica SR530 dual frequency GPS receiver,

- Two Ashtech Z12 GPS receivers,

	Three Garmin GPS II+’s navigation GPS receivers,

- One Bell 47 G5 – VH TZW

	One 4wd Landcruiser tray back vehicle,

	Two Compaq notebooks for data processing.

2.3	Vehicles and transport

Due to the type of terrain to be encountered, a combination of 4wd Landcruiser tray back and helicopter was utilized for the duration of the job. The helicopter was used on the 1 km spacing while the 4wd was used on the 200m infill and for servicing and supporting the helicopter operations.

2.4 Camp

Accommodation was provided by the client in a donga at the Outback caravan park.

3.	GPS SURVEYING AND PROCESSING

3.1	Set out of the gravity grid

The 1km by 1km grid was set out using the Garmin GPS II+’s in autonomous mode.  This was done concurrently with the gravity data acquisition.  Where possible, the readings were taken close to the ideal coordinates. Some stations were offset due to the nature of the terrain, eg salt lakes and other depressions, mulga and thickly vegetated areas. As the Garmin was operating in autonomous mode, the displayed position may have differed from the real coordinates by as much as 5m (with Selective Availability off).

Repeat gravity stations were temporarily marked with a pink pin flag.  Upon each reoccupation of these repeats, the helicopter attempted to land in the skid marks from the previous occupation, although this was not always possible due to prevailing winds and other factors.

3.2	Set up of GPS control

For horizontal and vertical GPS control, a total of 4 GPS receivers, two Leica GPS and two Ashtech Z12’s were used.  One point – station 1, was set up in an accessible location atop a Giants Reef donga. Another station, 2, was a short star picket in the paddock adjacent the airport and Giants Reef compound. Both stations were connected to the National Mapping horizontal marks, SHAMROCK (NMG 2, NTS 233) and GREYS BLUFF (NTS 171).  These stations were close to Tennant Creek.  New calculated values of the bases used are depicted below.

1 (Sy 530 atop Giants Reef donga)

Latitude (GDA94) 	 -19 38 33.7962

Longitude (GDA94)	134 11 03.7507

Ellipsoid height		          411.624

N (Ausgeoid98)		          +33.042

AHD level		          378.582

2 (Star dropper in paddock adjacent Giants Reef office)

Latitude (GDA94) 	 -19 38 32.2953

Longitude (GDA94)	134 11 08.1667

Ellipsoid height		          407.423

N (Ausgeoid98)		          +33.046

AHD level		          374.377

3.3	Surveying of the position and level data

Position and level data were surveyed using the Leica SR9500, Leica SR530 and Ashtech Z12 GPS receivers. The satellite antenna was securely fixed to the helicopter and vehicle allowing reliable and repeatable levels to be obtained.  Using a 10 degrees satellite elevation cut off angle allowed all day GPS acquisition.  From the statistics below, the standard deviation of the GDA ellipsoidal height repeats was 0.03m.

| DAHD |  |
| --- | --- |
| Mean | 0.027 |
| Standard Error | 0.008 |
| Median | 0.013 |
| Mode | 0.013 |
| Standard Deviation | 0.029 |
| Sample Variance | 0.001 |
| Kurtosis | 3.725 |
| Skewness | 2.020 |
| Range | 0.093 |
| Minimum | 0.008 |
| Maximum | 0.101 |
| Sum | 0.322 |
| Count | 12 |

3.4 Post processing of the position and level data

The kinematic GPS data was recorded on 10MB flash cards and internal memory that were downloaded onto computers daily.  Processing of the data was carried out using Waypoint, GrafNav Version 6.02.

Waypoints Software GrafNav V6.02 processing example.

4.		GRAVITY ACQUISITION AND PROCESSING

4.1 Gravity data acquisition

The entire survey was carried out by helicopter and vehicle, using the standard loop procedure, with each loop starting and finishing at a particular base station.  The time between initial and final occupation of a base was less than 8 hours in most instances.  Longer loops were used when it was not always efficient to come back to base at lunchtime.  This was kept to a minimum but was unavoidable when working in the remote areas.  In any case, the gravity loop closure and gravity repeat data was sufficiently accurate to exceed project specifications, even with the all day loops in some instances. The same base was used for the initial and final reading for each loop. Daily drift was monitored by comparing earth tide corrected readings at the base at the start and end of each loop.

Upon arrival at each station, the engine was switched off to eliminate vehicle related noise, and a reading was taken as close as practical to the vehicle and GPS antenna.  For the helicopter work it is not practical to shut down the engine for each reading although the revs and collective pitch are lowered to a level that minimally affects the gravity readings.

A typical gravity observation at a repeat station (photo from a previous survey)

Repeat gravity stations were marked with a pink pin flag.  Upon reoccupation of these repeat stations the gravity meter tripod was placed on the marks left during the initial occupation.  From the statistics below the standard deviation of the observed gravity repeats was 0.024mGal.

| dGo |  |
| --- | --- |
| Mean | 0.029 |
| Standard Error | 0.007 |
| Median | 0.023 |
| Mode | 0.020 |
| Standard Deviation | 0.024 |
| Sample Variance | 0.001 |
| Kurtosis | 0.945 |
| Skewness | 1.262 |
| Range | 0.077 |
| Minimum | 0.003 |
| Maximum | 0.080 |
| Sum | 0.353 |
| Count | 12 |

4.2 Gravity data processing

Raw data was processed using Geosoft GRAVRED gravity processing software.  This combined the position and level for each station and the earth tide corrected gravity value.  The resulting Bouguer gravity anomaly was imaged using a graphical program to check for any anomalous readings that would have to be repeated.

The Gravred processing used the following constants and formulae..

Gravity meter scale factors

| Meter | Meter Serial Number | Instrument Scale Factor |
| --- | --- | --- |
| Scintrex CG3-H | 9303204 | 1.0000000 |
| Scintrex CG3-H | 8810110 | 1.0000000 |

Formulae Used

Astronomic Tide Correction

Too complex to list.

International Gravity Formula 1967

gn = 978031.846(1+0.005278896sin2f-0.000023462sin42f)

where 	f is latitude

gn is Theoretical Gravity in mGals

Free Air Correction

FA = 3.0859h

where	h is in m AHD

FA is correction in mGals

Bouguer Anomaly

BA = 0.419088rh

where	r is rock density (2.67 gm/cc)

h is in m AHD

BA is correction in mGals

Bouguer Gravity

BG2.67 = ObsG- gn + (FA-BA)+TC

where	BG2.67 is in mGals

Software Used

Geosoft Gravred

Waypoint Consulting GrafNet v 6.02 Beta

Leica Ski Pro

ERMapper 6.1

Microsoft Excel 2000

4.3  Set up of the gravity control

At the Tennant Creek Airport, a gravity base station plaque (“6793 0134”) was found just outside the terminal and this was used for all the gravity observations and reductions.   The following information was obtained from the AGSO website.

Station name:TENNANT CREEK A/S TERMINAL

Station number:1967930134

Latitude:-19.64167	 Longitude:134.18167	 Units:DEG	 Datum:AGD66

Elevation:375

Observation date:	Gravity: 9785145.83

Station type:Base station	Station name:TENNANT CREEK A/S TERMINAL

Station number:1967930134

Scintrex CG3 9303204 on

gravity base 6793 0134

A close up photo of the base plaque 6793 0134

Appendix A		Images and maps

Appendix B		Disk of Results