MINERALOGICAL REPORT No.  9455

Ian R. Pontifex  MSc.  and  Alan C. Purvis,  PhD

December 30th,  2008

TO :	Mr Jim McKinnon Matthews

Mithril Resources Ltd

60 King William Road

GOODWOOD  SA  5034

YOUR REFERENCE :	Your email 17/11/08.

MATERIAL &	Rock samples, OKRC-102, 120, 144, 163, 178,

IDENTIFICATION :	181, 183

WORK REQUESTED :	Section preparation, petrographic/mineragraphic description.

SAMPLES & SECTIONS :	Returned to you with final report.

DIGITAL COPY :	This final report emailed 30/12/08 to:

<jimm@mithrilresources.com.au>

PONTIFEX & ASSOCIATES PTY. LTD.

# INTRODUCTION

Seven outcrop rock samples from Mithril’s Black Adder Project area, NT, are described in this report from polished thin sections, particularly to identify rock types, also species and mode of occurrence of minor contained sulphides.

Pontifex provided a preliminary version of this report December 11th, 2008.  This final version follows a review by Dr Alan Purvis.  This incudes several minor amendments to page 2 and petrogenetic comment, notably with suggestion of minor and variable metamorphism of the otherwise primary gabbroic/noritic rock types.  The interpretation of a basic primary mode of occurrence of the minor sulphides remains valid.

BRIEF PETROGRAPHIC DESCRIPTIONS

OKRC-102

Olivine and plagioclase are separated by zoned rims of possible orthopyroxene and amphibole ± spinel representing cooling or metamorphism. Olivine and plagioclase to 3mm are accompanied by orthopyroxene to 2mm, fine-grained clinopyroxene, biotite, and opaque minerals as indicated.  The plagioclase is interstitial and probably postcumulus, suggesting olivine-orthopyroxene cumulus gabbro.

OKRC-120

Plagioclase and less abundant clinopyroxene occur as residual igneous grains (phenocrysts or porphyroclasts?) to 4mm in a recrystallised, apparently metamorphic aggregate of mostly fine-grained hornblende-clinopyroxene ± orthopyroxene.  Some probable orthopyroxene may be ex-olivine, as spinel-bearing symplectites, occur adjacent to this clearly non-primary orthopyroxene.  There is also clinopyroxene that has zoning defined by brown schiller inclusions (oxide?).  This sample has a largely metamorphic fabric but is probably unaltered.

OKRC-144

Olivine occurs as porphyroclasts to 6mm in diameter with less abundant smaller plagioclase porphyroclasts in a recrystallised, apparently metamorphic pyroxene-hornblende matrix.  The olivine has rims of orthopyroxene similar to aggregates in the previous sample interpreted as possibly ex-olivine and also encloses some of the minor chrome spinel in this thin section.  Sparse clay and limonite-filled fractures are evident.  This sample is very mafic and approaching ultramafic composition, but some plagioclase may have reacted with pyroxene and olivine to form hornblende.  Although brown hornblende may be primary, the overall fine-grained fabric suggests recrystallisation and metamorphism.

OKRC-163

Poikilitic or ophitic pyroxene is abundant in this sample as well as plagioclase, with less abundant olivine that has been partly replaced by pyroxene.  There are zones with secondary amphibole and very minor fine-grained carbonate.  This sample is classified as poikilitic or ophitic gabbro.

OKRC-178

Olivine-free, plagioclase-rich gabbro with larger plagioclase grains (to 6mm long) riddled with altered pyroxene inclusions, as well as inclusion-free plagioclase laths.  Uralitic alteration is widespread, with localised carbonate, but residual pyroxene is disseminated with possible exsolution textures.  Biotite is also unusually abundant and coarse-grained (to 3mm long).  Rare zoned tourmaline was seen as well as rare fine-grained apatite.  This seems to be relatively fractionated gabbro and may have more abundant plagioclase (>50%), some of which may represent phenocrysts or xenocrysts.

OKRC-181

Mostly fine-grained granoblastic gabbro with some prismatic orthopyroxene to 3mm (35-40%), but only minor (2-3%) clinopyroxene, 25% plagioclase and abundant hornblende as well as minor biotite.  The orthopyroxene has abundant exsolved clinopyroxene.  The plagioclase and hornblende are largely interstitial.  This sample is best classified as mafic hornblende norite with some possible metamorphism, and is transitional mafic to ultramafic.

OKRC-183

Olivine and some plagioclase occur as porphyroclasts, with coronas of orthopyroxene rimming olivine and inclusions of pyroxene in the larger plagioclase grains.  Some residual zoned clinopyroxene is also disseminated but au but abundant fine-grained pyroxene and hornblende are interpreted as metamorphic, but plagioclase is commonly zoned and igneous.  There is an irregular development of possibly spinel-bearing symplectites against plagioclase, possibly related to nearby olivine and possibly of metamorphic origin.

Summary estimated vol% silicate mineralogy of seven OKRC samples

|  | 102 | 120 | 144 | 163 | 178 | 181 | 183 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Olivine | 7 | - | 10 | 15 | - | - | 10 |
| Tremolite | - | - | 10 | - | - | - | 5 |
| Orthopyroxene | 15 | 10 | 5 | - | - | 35 | 15 |
| Clinopyroxene | 30 | 20 | 15 | 40 | 10 | 23 | 15 |
| Plagioclase | 40 | 40 | 30 | 40 | 40 | 25-30 | 35 |
| Biotite | 8 | - | - | - | 10 | 2 | - |
| Hornblende | - | 25 | 30 | - | - | 35 | 15 |
| Spinel-amphibole | 1 | 5 |  |  |  |  |  |
| Uralitic amphibole alteration | - | - | - | 5 | 35 | - | - |

- The listed tremolite is primary fine crystalline, mostly as coronas around coarse olivine (phenocrysts).  Minor extremely fine spinel-amphibole rims in 102 and 120 and is the result of reaction between olivine and plagioclase.  (This also occurs in ORC-055 previously described in Report 9397.)

- The listed hornblende is (optically) pale brown fine granular, primary, more or less interstitial and forming a groundmass to coarser mafic silicate crystals

- The uralitic amphibole alteration in 163 is mostly within discontinuous veins.  In 178 it is extensive largely replacing inherent primary clinopyroxene ± carbonate, and trace tourmaline also occurs in this rock.

Summary rock type classification of seven OKRC chip samples.  All are primary magmatic and mostly unaltered “gabbroic” rocks, but with minor superimposed metamorphism mentioned in the discussion below.

| OKRC-102 | Biotite, pyroxene gabbro |
| --- | --- |
| OKRC-120 | Hornblende, orthopyroxene and plagioclase-porphyritic gabbro |
| OKRC-144 | Hornblende, olivine-porphyritic gabbro |
| OKRC-163 | Poikilitic augite, olivine gabbro |
| OKRC-178 | Uralitically altered gabbro |
| OKRC-181 | Hornblende norite |
| OKRC-183 | Olivine porphyritic, gabbro-norite |

### DISCUSSION

These samples represent moderately and variably metamorphosed mafic and transitional mafic to ultramafic lithologies with minor essentially primary sulphides and magmatic oxides.  Most samples contain primary olivine and/or orthopyroxene, as well as plagioclase, clinopyroxene, hornblende and biotite.  Most have clinopyroxene > orthopyroxene, except for sample OKRC181, in which almost all of the pyroxene is orthopyroxene.  Olivine is commonly rimmed by orthopyroxene and/or extremely fine spinel-bearing symplectites suggesting metamorphism at possibly 500-700MPa and 600°C and reactions with accompanying plagioclase.

Several samples have abundant fine-grained clinopyroxene-hornblende aggregates interpreted as metamorphic.  Two samples (OKRC163 and 178) contain minor uralitic amphibole ± carbonate, representing lower-grade metamorphism than that indicated by the microcrystalline clinopyroxene-hornblende aggregates.  These two samples may also have compositions least affected by cumulus processes.

Although the sulphides may have been modified by metamorphism, they are interpreted as ultimately of primary magmatic origin within gabbro, olivine gabbro and norite.  Alteration is probably not significant although tourmaline occurs in sample OKRC178.

SUMMARY OF SULPHIDES (AND ILMENITE) IDENTIFIED OPTICALLY IN POLISHED THIN SECTIONS OF SEVEN OKRC ROCK SAMPLES.

| OKRC 102 | Pyrrhotite (commonly partly oxidised) Pentlandite Chalcopyrite Ilmenite > magnetite |
| --- | --- |

Total abundance of these sulphides is ~3% of the polished thin section area, mostly randomly scattered composite grains, size 0.1mm to (rarely) 1.2mm.  Pyrrhotite-pentlandite grains are most common, together with partial goethitic alteration of some pyrrhotite.  Chalcopyrite occurs in about 1/3 of these composite grains.  Minute exsolved flames of pentlandite occur as inclusions in some pyrrhotite.

Lesser smaller (50 micron or less) single monomineralic grains of these same sulphides are disseminated, total <1 vol %.

Ilmenite > magnetite grains of 50 mm to 300 mm are scattered as individuals, overall forming <1% of the section area.

OKRC-120

One composite grain (1mm) of oxidised pyrrhotite/fresh-pyrrhotite/pentlandite/chalcopyrite.  Three smaller (0.3mm) grains of the same oxidised pyrrhotite/fresh-pyrrhotite/pentlandite/ chalcopyrite (<1%).

Also much smaller 50 mm disseminated grains of pyrrhotite-pentlandite and of pyrrhotite only.  Total vol% abundance <<1%.

Ilmenite > magnetite grains, 20 mm to 300 mm disseminated as bleb-like inclusions in silicate minerals.

OKRC-144

Three composite grains of pyrrhotite-pentlandite-chalcopyrite, 100 mm to 300 mm.  Also “abundant” disseminated much finer sulphide grains, 5 mm to 100 mm, variably of composite pyrrhotite-pentlandite and of pyrrhotite only.  Total vol % abundance of sulphides is <1%.

Several single chromite grains about 0.3 (300 mm) size occur independently.

Ilmenite grains, 10 mm to 300 mm, <<1%

OKRC-163

Sparse disseminated sulphide particles size 10 mm to 200 mm, in total forming <1 vol% of the polished section area.  Mostly single phase particles of chalcopyrite > pyrrhotite > pentlandite, but with rare composites of pyrrhotite-pentlandite including microflame inclusions of pentlandite in pyrrhotite.  Extremely fine chalcopyrite locally abundant as inclusions in selected pyroxene crystals.

Magnetite > ilmenite grains (1%) disseminated, size range 20 mm to 500 mm.  Trace chrome-magnetite grains.

OKRC-178

Trace scattered sulphide particles (<<1%) all <50 mm size.  Mostly (altered) pyrrhotite, rarer chalcopyrite.  (no pentlandite)

Magnetite > ilmenite grains also <1%, rarely to 0.5mm, bleb-like and amoeboidal inclusions in silicate.

OKRC-181

Trace (<<1%) sulphide grains, mostly oxidised pyrrhotite, rarely composite with chalcopyrite.

Trace extremely small primary oxide minerals seem to be ilmenite.

Trace very small 10 mm x 100 mm random flakes of tochilinite (nickeliferous interlayered FeSMgOH).

OKRC-183

Sulphides are (macroscopically) visible as small grains to 0.5mm in a local cluster 1mm x 2.5mm in this polished thin section, predominantly of fresh pyrrhotite, about 1/3 composite with chalcopyrite and lesser (rare-trace) pentlandite.  This cluster also includes anomalously coarse granular marcasite composite with several ilmenite grains and chalcopyrite.

More sparsely disseminated (<<1%), smaller (50 mm) grains of pyrrhotite, rare-trace pentlandite are disseminated.

Ilmenite grains (1-2%), size range 100 mm to 500 mm are scattered, some skeletal.