## HISTORY OF TENURES

A portion of the work done by Spectrum Rare earthson EL27151 for this year consists of a literature review and data compilation. The results are presented in the section below. The paragraph below contains the list of previous tenures, and reports from previous tenures. Figure 1 display the location of previous drilling and Figure 2 highlights significant areas from the previous exploration. Previous exploration work done over EL27151 by other companies is presented below by historic tenements from the oldest to the most recent.

AP1682 covered the northeast half of EL27151, but most of AP1682 was outside the tenure, extending to the northeast. IMC Development Corporation explored the AP for the phosphatic potential of the limestones. Samples were qualitatively tested for phosphate in the field with ammonium molybdate solution, with selected samples sent for qualitative analysis. No anomalous radioactivity was associated with the phosphate. The best phosphate assay value of 0.75% P2O5 was obtained from Ooloo Limestone outside of EL25222. They concluded that though the base of the limestone was anomalously phosphatic, no occurrences of phosphate rock were found to encourage further research. (CR1967-0020)

AP1771 covered the southwest half of EL27151, it also covered the south half of Skyfall radiometric anomaly. Although most of the AP is outside of the tenure, it is extending to the west. They had two main exploration targets in the area. One consisted in two radioactive anomalies. The second was possible disseminated copper mineralisation in the Antrim basalts and also a slight possibility to find phosphate in Cretaceous sediments. None of their samples or scintillometer traverses returned any interesting value. (CR1968-0047)

AP2437 covered the southwest corner of EL27151, just below Skyfall radiometric anomaly and extending to the southwest. Sampling done by CRA Exploration in 1969, they were prospecting for bauxite, has shown that the sampled areas are of no interest and they have been relinquished in consequence. (CR1969-0064)

AP2545 covered the central part of EL27151, including Skyfall and Largo radiometric anomalies. Work done by Euralba Mining consisted in a helicopter reconnaissance survey and scintillometer traverses. If no apparent radiometric anomalies were located, some samples returned anomalous iron, lead and zinc values (up to 3% Pb). Mapping campaign on the AP revealed that the Waterbag Creek Formation contained ferruginous and non-ferruginous beds at the time it was deposited and also some syngenetic lead and zinc. However the lead appears to be restricted to a few favourable beds. Syngenetic zinc became concentrated in cavities in the ironstone. Some features of the lead bearing rock appear to show that syngenetic lead was being replaced by concretions of chert. (CR1970-0024)

EL574 covered the southwest corner of EL27151, just below Skyfall radiometric anomaly and extend further south. Euralba Mining exploration target was the possible occurrence of barite and base metals associated. Exploration for barite was unsuccessful but anomalous lead and zinc samples were obtained from samples on the eastern side of Bamboo Creek. (CR1974-0042)

EL1768 covered most EL27151 and extend further north.

- Work by Euralba Mining and Ashton Mining, in 1980, evaluated the base metal potential along the Dorisvale Fault. Rock-chips sampling returned anomalous copper, lead, zinc and uranium values (up to 320ppm U; 800ppm Cu; 19.3% Pb and 1.65% Zn). However most the samples location has been lost due to a helicopter accident (CR1980-0101).

- In 1981, Ashton Mining identified large number of U anomalies (with up to 448 ppm U) in The Bamboo Creek area, near Dorisvale fault and associated to significant Pb-Zn values. The area is considered to hold potential for a McArthur River or Rammelsberg type deposit and possibility for a sandstone/siltstone U mineralization. However research for kimberlite indicator was unsuccessful (CR1981-0089).

- In 1982, Euralba Mining did 10 percussion and diamond holes over Skyfall area (Figure 1). Drilling was prompted by anomalous zinc and lead values associated with the Dorisvale Fault but was initially designed to test unweathered rocks at depth, particularly those representing geochemically active horizons at the surface. The best uranium (only up to 47 ppm), lead (up to 20000 ppm) and zinc (up to 48000 ppm over an interval of approximately 45m thick) were below the oxidized zone in leached shale, mudstone and siltstone with chert beds near base. The stronger anomalous values tend to be associated with nodular, fossiliferous limestone and chert nodule layers. The correlation between drill holes data and surface outcrop has established that the inferred target rocks belong to the lower and possibly middle Tindall Limestone. The limestone contains a very porous matrix component considered to represent a second phase impure carbonate component. The intergranular porosity was interpreted by Euralba Mining as a possible host rock for “Mississippi Valley type” ore deposit. They suggested that more exploration should be done both along the strike and further into the Daly River Basin. (CR1982-0125).

- In 1983, Esso Australia drilled three vertical diamond holes over the Skyfall area (Figure 1). They were exploring for massive sulphide with a focus on NW-SE trend indicated by geochemical data from previous Euralba drilling. They pointed out that sulphides were associated with sediment breccia, mainly as fractured filling and in porous or brecciated zones. The best values were 1000ppm lead, 220ppm copper and 7500ppm zinc. They suggested a carbonate-hosted epigenetic type of deposit. (CR1983-0043).

Figure 1: Euralba Mining and Esso Australia drilling location.

EL3004 only overlap part of the eastern side of EL27151, it covers north and south of Largo anomaly. Nothing was done (CR1986-0124)

EL3507 overlap part of Largo anomaly nothing done (CR1986-0125).

EL4905 is included in EL27151 and overlap part of Skyfall anomaly. Exploration on the tenement was made by Seven State Mine, they conducted a study of past exploration work and of reconnaissance auger drilling. The auger drilling aim was to examine gold content in soil but no anomalous values was found (CR1987-0278).

EL4821 is included in EL27151 and overlap part of Skyfall anomaly. Exploration on the tenement was made by Seven State Mine, they conducted a study of past exploration work and of reconnaissance auger drilling. The auger drilling aim was to examine gold content in soil but no anomalous values was found (CR1987-0278).

EL4904 is included in El27151 and overlap part of Skyfall anomaly. Exploration on the tenement was made by Seven State Mine, they conducted a study of past exploration work and of reconnaissance auger drilling. The auger drilling aim was to examine gold content in soil but no anomalous values was found (CR1987-0278).

EL5383 is included in El27151 and overlap part of Skyfall anomaly. Exploration on the tenement was made by Seven State Mine, they conducted a study of past exploration work and of reconnaissance

Figure 2: Uranium rock-chips and stream sediment historic results over EL27151.

Table 1 Historic drilling done in EL27151

| Report number | Lease | Company (ies) | Commodity | Focus | Data available | Results of Exploration |
| --- | --- | --- | --- | --- | --- | --- |
| CR1982-0125 | EL1768 | Euralba Mining | Zn, Pb | Base metals associated with Dorisvale Fault | 10 percussion/diamond holes with logs details | Best U/Pb/Zn values are below the oxidized zone in leached shale, mudstone and siltstone with chert beds near base (no U). Best values were up to 47 ppm for uranium, up to 20000 ppm for zinc and up to 48000 ppm for lead. |
| CR1983-0043 | EL1768, EL1832 | Esso Australia | Massive sulphide | NW-SE trend indicated by geochemical data from previous Euralba drilling (1 to 10) | 3 vertical diamonds holes with logs details | Sulphides are associated with sediment breccia, mainly as fractured filling and in porous or brecciated zones. Inferred carbonate-hosted epigenetic type deposit. uranium (only up to 47 ppm), lead (up to 20000 ppm) and zinc (up to 48000 ppm over an interval of approximately 45m thick) |

Table 2 Historic geophysics survey done in EL27151

| Report number | Lease | Company | Commodity | Focus | Data available | Results of Exploration |
| --- | --- | --- | --- | --- | --- | --- |
| CR1980-0101 & CR1980-0102 | EL1768 & EL1832 | A.O. (Australia) | Pb, Zn and Diamonds | Dorisvale Fault area | Input and I.P. Survey | Two conductive zones and some isolated anomalies, relatively shallow chargeable bodies |
| CR1981-0089 | EL1768 | Ashton Mining Ltd | U, Pb, Zn and Diamonds | McArthur River or Rammelsberg type deposit and possibility exists for a sandstone/silstone U mineralization (Pb-Zn and U mineralisation in lower Cambrian Sediments along the Dorisvale monocline) | Airborne Magnetic and Gamma Ray Spectrometer | U anomalies identified in the Bamboo Creek area along the Dorisvale Fault. 50 anomalies identified (>3bg background). All anomalies associated with Cambrian rocks. |
| CR1983-0026 | EL1848 & EL1847 | Baroid Australia Pty. Ltd. | Pb, Zn, Au, Ba and Diamonds | Focus on vein deposits of barite, gold in carbonate rock, Pb/Au/Cu mineralisation of the MVT within dolomites of the Banyan Formation and to detect diamonds. | Radiometric Survey | Method not useful |
| CR1993-0143 & CR1994-0444 & CR1994-0789 | EL7626 | Poseidon exploration LTd. | Pc, Zn, Au and Diamonds | Shale and carbonate hosted Pb/Zn mineralization, diamonds and epithermal Au | Airborne Magnetic and Radiometric Survey | Strong U anomaly identified, coincident with anomalous U rockchip (outcrop of silicified and ferruginous basal Tindall Limestone). Electrical methods not effective. |
| CR1994-0027 | EL7796 | Stockdale prospecting Ltd. | Diamonds | Kimberlites | Aerial Magnetic Survey | Two anomalies identified with dipole signature but sample results indicate both are likely to be sinkholes. |
| CR1994-0106 | EL7939 | Poseidon exploration LTd. | Diamonds | Kimberlites | Aerial and Ground Magnetic Survey | Discrete dipolar magnetic anomaly detected by multiclient airborne surveys considered to be attributable to shallow sourced magnetic noise, with no underlying form. |
| CR1994-0320 | EL7629 | Stockdale prospecting Ltd. | Diamonds | Kimberlites | Aerial and Ground Magnetic Survey | One aerial magnetic anomaly detected, but follow up ground magnetics identified it as surface noise. |

Knightfall:

| Report number | Lease | Company | Commodity | Focus | Data available | Results of Exploration |
| --- | --- | --- | --- | --- | --- | --- |
| CR1998-0744 & CR1998-0034 | EL9628 | Stockdale Prospecting Ltd | Diamonds | Dorisvale Fault area | Aeromagnetic | Small aeromagnetic survey undertaken, no features of interest were observed. |
| CR1984-0051 | EL2327 | CRA Exploration Ltd | Uranium | Uranium Radiometric Anomalies | Airborne Radiometric | Eighteen airborne radiometric line anomalies identified. Follow up rock chip geochemistry indicated low U enrichment (max. 32ppm U) associated with laterite horizons developed over thin sediments of Cretaceous age (Mullaman Beds). |