BEETALOO W-1:  CUTTINGS SAMPLE DESCRIPTIONS

| Depth From (m) | Depth To (m) | % | Lithology and Shows |
| --- | --- | --- | --- |
| 22" Conductor set at 17.0mMDRT. Beetaloo W-1 was spudded with SLR Rig 185 at 21:00hrs on 22nd July 2016 from 17.0mMDRT. |  |  |  |
| 17 | 20 | 100 | CLAYSTONE: medium red to red brown, orange red, very fine arenaceous, moderately calcareous, common lithics, friable to moderately hard, blocky to sub blocky. *Sample heavily contaminated with cement |
| 20 | 30 | 70 20 10 | CLAYSTONE: medium yellow, orange to orange yellow, minor off white, kaolinitic, trace lithics, silty in part, hard, blocky to sub blocky. SILTSTONE: red to red brown, orange, arenaceous to argillaceous, common lithics, firm to moderately hard, occasionally very hard. SANDSTONE: light yellow orange, clear to translucent, Fe staining, fine to medium, moderately sorted, sub angular to sub rounded, nil cement, trace light yellow orange argillaceous matrix, minor lithics, occasionally friable to mod hard aggregates, poor visual porosity, no fluorescence. |
| 30 | 40 | 30 70 | CLAYSTONE: medium yellow, orange to orange yellow, minor off white, kaolinitic, trace lithics, silty in part, hard, blocky to sub blocky. SANDSTONE: light yellow orange, moderately red orange, light yellow grey, very fine to coarse, moderately to poorly sorted, sub angular to dominantly sub rounded, trace round grains, trace weak siliceous cement, minor light yellow orange silty matrix, minor lithics, minor friable to slight moderately hard aggregates, generally loose grains, fair to good inferred porosity, no fluorescence. |
| 40 | 50 | 10 90 | CLAYSTONE: medium yellow, orange to orange yellow, minor off white, kaolinitic, trace lithics, silty in part, hard, blocky to sub blocky. SANDSTONE: clear to translucent, light yellow orange, moderately red orange, light yellow grey, very fine to coarse, trace very coarse grains, moderately sorted, sub rounded to round, trace weak siliceous cement, common light yellow orange silty matrix, minor friable to minor moderately hard aggregates, generally loose grains, poor visual porosity, good inferred porosity, no fluorescence. |
| 50 | 60 | 100 | SANDSTONE: clear to translucent, light pink, fine to coarse, moderately sorted, sub rounded to round, weak siliceous cement, trace off white argillaceous matrix, rare lithics, generally loose clean grains, fair to good inferred porosity, no fluorescence. |
| 60 | 70 | 100 | SANDSTONE: clear to translucent, light pink, medium to very coarse, poorly sorted, sub rounded to round, minor sub angular, weak siliceous cement, nil matrix, rare lithics, generally loose clean grains, fair to good inferred porosity, no fluorescence. |
| 70 | 80 | 100 | SANDSTONE: clear to translucent, light orange, coarse to very coarse, medium in part, moderately sorted, sub rounded to round, minor sub angular, weak siliceous cement, nil matrix, rare lithics, generally loose clean grains, fair to good inferred porosity, no fluorescence. |
| 80 | 90 | 100 | SANDSTONE: light orange to light yellow, clear to translucent in part, coarse to very coarse, medium in part, moderately sorted, sub rounded to round, minor sub angular, weak siliceous cement, nil matrix, rare lithics, generally loose clean grains, fair to good inferred porosity, no fluorescence. |
| 90 | 100 | 80 20 | SANDSTONE: light red pink, clear to translucent in part, fine to medium, coarse in part, moderately sorted, sub rounded to round, minor sub angular, weak siliceous cement, nil matrix, rare lithics, generally loose clean grains, fair to good inferred porosity, no fluorescence. CLAYSTONE: light to medium red, light yellow orange, minor off white, light to medium purple in part, silty in part, minor locally siliceous, minor lithics, firm to occasionally dispersive, sub blocky to occasionally amorphous. |
| 100 | 110 | 80 20 | SANDSTONE: light red pink, clear to translucent in part, fine to medium, coarse in part, moderately sorted, sub rounded to round, minor sub angular, weak siliceous cement, nil matrix, rare lithics, generally loose clean grains, fair to good inferred porosity, no fluorescence. CLAYSTONE: light red purple, minor yellow orange, minor off white, silty in part, minor locally siliceous, occasional lithics, firm to occasionally dispersive, sub blocky to locally amorphous. |
| 110 | 113 | 70 30 | SANDSTONE: light red pink, clear to translucent, fine to medium, coarse in part, moderately sorted, sub rounded to round, minor sub angular, weak siliceous cement, minor light yellow orange argillaceous matrix, rare lithics, generally loose clean grains, fair inferred porosity, no fluorescence. CLAYSTONE: light red purple, minor dark translucent grey, dominantly siliceous grading to Chert, common lithics, hard to very hard, sub blocky to blocky. |
| 113 | 205.7 |  | Total lost circulation from 113mMDRT to section TD at 205.7mMDRT. |
| 205.7 | 210 | 90 10 | SILTSTONE: medium red brown, occasionally light brown, arenaceous, trace micro micaceous, minor to rare lithics, moderately hard to hard, blocky to sub blocky. SANDSTONE: light brown, rare off white, fine to occasionally very fine, well sorted, sub angular to minor sub rounded, moderately calcareous cement, occasionally to rare off white argillaceous matrix, minor dark lithics, moderately hard to hard, very poor visual porosity, no fluorescence. |
| 210 | 220 | 90 10 | SILTSTONE: medium red brown, occasionally light brown, arenaceous, trace micro mica, rare lithics, moderately hard to hard, blocky to sub blocky. SANDSTONE: light brown, minor off white, fine to occasionally very fine, well sorted, sub angular to sub rounded, moderately calcareous cement, occasional to rare off white argillaceous matrix, rare dark lithics, moderately hard to hard, very poor visual porosity, no fluorescence. |
| 220 | 230 | 60 10 30 | SILTSTONE: medium red brown, occasionally light brown, arenaceous, trace micro mica, rare lithics, moderately hard to hard, blocky to sub blocky. SANDSTONE: light brown, minor off white, fine to occasionally very fine, well sorted, sub angular to sub rounded, moderately calcareous cement, occasional to rare off white argillaceous matrix, rare dark lithics, moderately hard to hard, very poor visual porosity, no fluorescence. LIMESTONE: off white, very light yellow, very light grey, light brown grey, micritic to lutitic, argillaceous, microcrystalline to occasionally crystalline, hard to very hard. |
| 230 | 240 | 10 Trace 90 | SILTSTONE: medium red brown, rare light brown, arenaceous, trace micro mica, rare lithics, moderately hard to hard, blocky to sub blocky. SANDSTONE: light grey, minor off white, fine to very fine, well sorted, sub angular to sub rounded, moderately calcareous cement, occasional to rare off white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. LIMESTONE: off white, very light yellow, very light grey, light brown grey, micritic to lutitic, argillaceous, microcrystalline to minor crystalline, hard to very hard. |
| 240 | 250 | 50 10 40 | SILTSTONE: medium red brown, medium grey, rare light brown, arenaceous, trace micro mica, rare lithics, moderately hard to hard, blocky to sub blocky. SANDSTONE: light grey, rare off white, fine to very fine, well sorted, sub angular to sub rounded, moderately calcareous cement, occasional to rare off white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. LIMESTONE: off white, very light yellow, very light grey, light brown grey, micritic to lutitic, argillaceous, microcrystalline, hard to very hard. |
| 250 | 260 | Trace 100 | SILTSTONE: medium red brown, rare light brown, arenaceous, trace micro mica, rare lithics, moderately hard to hard, blocky to sub blocky. LIMESTONE: very light yellow, very light grey, off white, light brown grey, micritic to lutitic, argillaceous, rare lithics, microcrystalline, hard to very hard. |
| 260 | 270 | Trace 100 | SILTSTONE: medium red brown, rare light brown, arenaceous, trace micro mica, rare lithics, moderately hard to hard, blocky to sub blocky. LIMESTONE: very light yellow, off white in part, very light grey, light brown grey, micritic to lutitic, argillaceous, rare dark veining, microcrystalline to crystalline, hard to very hard. |
| 270 | 280 | 100 | LIMESTONE: very light yellow, off white in part, very light grey, light brown grey, micritic to lutitic, argillaceous, rare dark veining, minor dark lithics, trace pyrite nodules, microcrystalline to crystalline, hard to very hard. |
| 280 | 290 | 100 | LIMESTONE: very light yellow to off white, occasionally very light grey, minor medium to dark grey, light brown grey, micritic to sparitic, arenaceous to argillaceous, occasional dark veining, minor dark lithics, crystalline to minor microcrystalline, hard to very hard. |
| 290 | 300 | 100 | LIMESTONE: light yellow to off white, occasionally very light grey, micritic to sparitic, arenaceous to argillaceous, occasional dark veining, minor calcite grains, crystalline to minor microcrystalline, hard to very hard. |
| 300 | 310 | 100 | LIMESTONE: light yellow to off white, occasionally very light grey, micritic to sparitic, arenaceous to argillaceous, occasional dark veining, minor calcite grains, rare pyrite nodules, crystalline to cryptocrystalline, hard to very hard. |
| 310 | 320 | 100 | LIMESTONE: light yellow to off white, occasionally very light grey, micritic to sparitic, arenaceous to argillaceous, occasional dark veining, minor calcite grains, rare dark lithics, crystalline to cryptocrystalline, hard to very hard. |
| 320 | 330 | 90 10 | LIMESTONE: medium yellow orange to moderate yellow brown, light yellow to off white, micritic to sparitic, trace lutitic, arenaceous to argillaceous, occasional dark veining, minor calcite grains, rare pyrite nodules, crystalline to minor cryptocrystalline, hard to very hard, dull yellow mineral fluorescence. SILTSTONE: medium to occasionally light grey, dolomitic, argillaceous in part, hard to very hard, blocky to sub blocky. |
| 330 | 340 | 95 5 | LIMESTONE: dark yellow orange to moderate yellow brown, light yellow to off white, minor light yellow grey, micritic to sparitic, trace lutitic, arenaceous to argillaceous, occasional dark veining, minor calcite grains, rare pyrite nodules, crystalline to minor cryptocrystalline, hard to very hard, dull yellow mineral fluorescence. SILTSTONE: medium to occasionally light grey, dolomitic, argillaceous in part, trace lithics, hard to very hard, blocky to sub blocky. |
| 340 | 350 | 100 Trace | LIMESTONE: very light yellow to moderate yellow brown, occasionally off white, minor light yellow grey, micritic to lutitic, arenaceous to argillaceous, rare dark veining, rare dark lithics, rare calcite grains, crystalline to cryptocrystalline, hard to very hard, dull yellow mineral fluorescence. SILTSTONE: medium to occasionally light grey, minor dark grey, dolomitic, argillaceous in part, trace lithics, hard to very hard, blocky to sub blocky. |
| 350 | 360 | 100 | LIMESTONE: light yellow to minor moderate yellow brown, occasionally very light grey, light brown grey, micritic to sparitic, arenaceous to argillaceous, rare dark inclusions, rare calcite grains, crystalline to occasionally microcrystalline, rare cryptocrystalline, hard to very hard, dull yellow mineral fluorescence. |
| 360 | 370 | 100 | LIMESTONE: light yellow brown, tan in part, minor off white, micritic to sparitic, argillaceous to occasionally arenaceous, rare dark inclusions, minor calcite grains, microcrystalline to occasionally crystalline, hard to very hard, dull yellow mineral fluorescence. |
| 370 | 380 | 100 | LIMESTONE: dominantly light yellow brown, tan in part, occasionally translucent yellow brown, minor off white, micritic to sparitic, argillaceous to occasionally arenaceous, rare dark inclusions, minor calcite grains, microcrystalline to occasionally crystalline, hard to very hard, dull yellow mineral fluorescence. |
| 380 | 390 | 100 | LIMESTONE: dominantly light yellow brown, tan in part, occasionally translucent yellow brown, micritic to sparitic, argillaceous to occasionally arenaceous, trace dark inclusions, microcrystalline to occasionally crystalline, hard to very hard. |
| 390 | 400 | 100 | LIMESTONE: dominantly light yellow brown, tan in part, occasionally translucent yellow brown, rare off white, micritic to sparitic, argillaceous to occasionally arenaceous, trace dark inclusions, microcrystalline to occasionally crystalline, hard to very hard. |
| 400 | 410 | 90 10 | LIMESTONE: dominantly light yellow brown, tan in part, occasionally translucent yellow brown, rare off white, micritic to sparitic, argillaceous to occasionally arenaceous, trace dark inclusions, microcrystalline to occasionally crystalline, hard to very hard. SILTSTONE: light to medium grey, very light grey to occasionally off white, dolomitic, argillaceous in part, trace lithics, hard, blocky to sub blocky. |
| 410 | 420 | 70 30 | LIMESTONE: dominantly light yellow brown, tan in part, occasionally translucent yellow brown, rare off white, micritic to sparitic, argillaceous to occasionally arenaceous, trace dark inclusions, microcrystalline to occasionally crystalline, hard to very hard, dull yellow mineral fluorescence. SILTSTONE: medium red brown, medium to light orange red, light to medium grey, very light grey to occasionally off white, dolomitic, argillaceous in part, trace lithics, hard, blocky to sub blocky. |
| 420 | 430 | 90 10 | LIMESTONE: dominantly light yellow brown, tan in part, occasionally translucent yellow brown, rare off white, micritic to dominantly sparitic, argillaceous to commonly arenaceous, locally common dark inclusions, crystalline to cryptocrystalline, hard to very hard, dull yellow mineral fluorescence. SILTSTONE: medium red brown, light to medium grey in part, dolomitic, argillaceous in part, trace lithics, hard, blocky to sub blocky. |
| 430 | 440 | 100 Trace | LIMESTONE: light yellow brown, occasionally translucent yellow brown, rare off white, micritic to dominantly sparitic, argillaceous to commonly arenaceous, locally common dark inclusions, crystalline to cryptocrystalline, hard to very hard, minor dull yellow mineral fluorescence. SILTSTONE: medium red brown, light to medium grey, dolomitic, argillaceous in part, trace lithics, hard, blocky to sub blocky. |
| 440 | 450 | 50 50 | LIMESTONE: light yellow brown, occasionally translucent yellow brown, rare off white, micritic to dominantly sparitic, argillaceous to commonly arenaceous, locally common dark inclusions, crystalline to cryptocrystalline, hard to very hard, minor dull yellow mineral fluorescence. VOLCANICS: dark to very dark brown grey, dark grey black, minor dark to very dark brown red, very fine to occasionally fine, occasional to common black phenocrysts, minor feldspar phenocrysts, hard to very hard, blocky to minor sub blocky, massive. |
| 450 | 460 | 10 90 | LIMESTONE: light yellow brown, occasionally translucent yellow brown, rare off white, micritic to dominantly sparitic, argillaceous to commonly arenaceous, minor dark inclusions, crystalline to cryptocrystalline, hard to very hard, minor dull yellow mineral fluorescence. VOLCANICS: dark to very dark brown grey, dark grey black, minor dark to brown red, very fine to occasionally fine, minor dark phenocrysts, minor feldspar phenocrysts, trace pyrite nodules, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 460 | 470 | 100 | VOLCANICS: dark to very dark brown grey, dark grey black, minor dark brown red, very fine to fine, minor dark phenocrysts, minor feldspar phenocrysts, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 470 | 480 | 100 | VOLCANICS: dark to very dark brown grey, dark grey black, minor dark brown red, medium green in part, medium, fine to coarse in part, locally common feldspar phenocrysts, minor dark phenocrysts, rare green inclusions, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 480 | 490 | 100 | VOLCANICS: dark to very dark grey, dark grey black, minor dark brown red, medium green in part, medium, fine to coarse in part, very coarse in part, common dark phenocrysts, occasional feldspar phenocrysts, rare green inclusions, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 490 | 500 | 100 | VOLCANICS: dark to very dark grey, minor dark brown red, medium green in part, medium to coarse, occasionally fine, very coarse in part, common dark phenocrysts, feldspar increasing with depth, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 500 | 510 | 100 | VOLCANICS: dark to very dark grey, minor dark brown red, medium green in part, medium to coarse, occasionally fine, very coarse in part, common dark phenocrysts, locally common feldspar, minor calcite inclusions, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 510 | 520 | 100 | VOLCANICS: dark to very dark grey, dark brown red, medium green in part, occasionally off white, medium to coarse, occasionally fine, very coarse in part, common dark phenocrysts, locally common feldspar, minor calcite inclusions, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 520 | 530 | 100 | VOLCANICS: dark grey, dark brown red, off white, medium green in part, medium to coarse, occasionally fine, very coarse in part, common dark phenocrysts, locally common feldspar, minor calcite inclusions, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 530 | 540 | 100 | VOLCANICS: dominantly dark grey, dark brown red, off white, medium green in part, medium to coarse, minor fine, very coarse in part, occasional to common dark phenocrysts, locally common feldspar, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 540 | 550 | 100 | VOLCANICS: dominantly dark grey, minor dark brown red, off white, medium green in part, medium to coarse, minor fine, very coarse in part, occasional dark phenocrysts, minor feldspar, minor rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 550 | 560 | 100 | VOLCANICS: dark grey, minor dark brown red, off white, rare medium green, medium to coarse, minor fine, very coarse in part, occasional dark phenocrysts, minor feldspar, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| 560 | 570 | 100 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. |
| Spot | 576 | 70 30 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. SILTSTONE: dark red purple, light yellow white, minor light grey, argillaceous, dominantly siliceous, minor micro micaceous, hard to very hard, blocky to sub blocky. |
| 570 | 580 | 70 30 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. SILTSTONE: dark red purple, light yellow, medium to dark grey green, minor light grey, argillaceous, dominantly siliceous, locally micro micaceous, hard to very hard, blocky to sub blocky. |
| Spot | 582.5 | 40 60 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. SILTSTONE: dark red purple, occasionally light yellow, minor light grey, argillaceous, dominantly siliceous, locally micro micaceous, minor dark lithics, hard to very hard, blocky to sub blocky. |
| Spot | 585 | 30 70 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. SILTSTONE: dark red purple, minor light yellow to light grey, argillaceous, dominantly siliceous, locally micro micaceous, minor dark lithics, hard to very hard, blocky to sub blocky. |
| Spot | 588 | 20 80 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. SILTSTONE: dominantly dark red purple, medium to dark red brown, minor light yellow to light grey, argillaceous, occasionally siliceous, locally micro micaceous, rare dark lithics, hard to very hard, blocky to sub blocky. |
| 580 | 590 | 20 80 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. SILTSTONE: dominantly dark red purple, medium to dark red brown, minor light yellow to light grey, argillaceous, minor siliceous, locally micro micaceous, hard, blocky to sub blocky. |
| Spot | 592 | Trace 70 30 | VOLCANICS: dark grey, dark grey green, dark grey brown, very fine, minor fine to medium, occasional rock flour, hard to dominantly very hard, blocky to minor sub blocky, massive. SILTSTONE: dark to occasionally medium red brown, dark red purple in part, minor light yellow orange, argillaceous, locally grading to Claystone in part, micro micaceous, hard, sub fissile to sub blocky, occasionally blocky. CLAYSTONE: medium to occasionally light red brown, silty, grading to Siltstone in part, hard, sub blocky to blocky. |
| Spot | 594 | 40 60 | SILTSTONE: medium to dark red brown, occasional dark red purple in part, minor light yellow orange, argillaceous, locally grading to Claystone in part, micro micaceous, hard, sub fissile to sub blocky, occasionally blocky. CLAYSTONE: medium to occasionally light red brown, silty, grading to Siltstone in part, hard, sub blocky to blocky, minor sub fissile. |
|  |  |  | 14.75” Section TD at 594mMDRT. |
| 594 | 600 | 40 60 | LIMESTONE: white, translucent, very light grey, occasionally red brown stain, fine to medium grained, moderately sorted, angular, strong calcareous cement, trace very fine quartz grains, moderately hard to hard, tight visual porosity, no fluorescence. Dissolves vigorously in 10% HCl. CLAYSTONE: medium to dark reddish brown, slightly silty, trace micro mica, moderately hard to predominantly hard, sub fissile to sub blocky. |
| 600 | 605 | 40 60 | LIMESTONE: white, translucent, very light grey, occasionally red brown stain, fine to medium grained, moderately sorted, angular, strong calcareous cement, trace very fine quartz grains, moderately hard to hard, tight visual porosity, no fluorescence. Dissolves vigorously in 10% HCl. CLAYSTONE: medium to dark reddish brown, slightly silty, trace micro mica, moderately hard to predominantly hard, sub fissile to sub blocky. |
| 605 | 610 | 60 40 | LIMESTONE: white, translucent, very light grey, occasionally red brown stain, rare light greenish grey, fine to medium grained, moderately sorted, angular, strong calcareous cement, trace very fine quartz grains, moderately hard to hard, tight visual porosity, no fluorescence. Dissolves vigorously in 10% HCl. CLAYSTONE: medium to dark reddish brown, minor light greenish grey, slightly silty, trace micro mica, moderately hard to predominantly hard, sub fissile to sub blocky. |
| 610 | 615 | Trace 100 | LIMESTONE: white, very light grey, rare light greenish grey, fine grained, well sorted, strong calcareous cement, trace very fine quartz grains, moderately hard, tight visual porosity, no fluorescence. CLAYSTONE: light to medium greenish grey, light grey, trace micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 615 | 620 | 10 90 | SANDSTONE: white, translucent, clear, very fine to fine grained, sub round, well sorted, weak calcareous cement, friable to loose, fair inferred porosity, no fluorescence. CLAYSTONE: light to medium greenish grey, light grey, trace micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 620 | 625 | 100 | CLAYSTONE: light to medium greenish grey, light grey, moderately hard to hard, sub fissile to sub blocky. |
| 625 | 630 | 20 80 | SILTSTONE: medium to dark reddish brown, argillaceous grading to SILTY CLAYSTONE, trace micro mica, moderately hard to hard, sub blocky to sub fissile. CLAYSTONE: light to medium greenish grey, light grey, moderately hard to hard, sub fissile to sub blocky. |
| 630 | 635 | 100 | CLAYSTONE: light to medium greenish grey, light grey, moderately hard to hard, sub fissile to sub blocky. |
| 635 | 640 | 100 | CLAYSTONE: light to medium greenish grey, light grey, trace loose fine quartz grains, moderately hard to hard, sub fissile to sub blocky. |
| 640 | 645 | 10 90 | SANDSTONE: clear – translucent, very fine to fine, moderately sorted, sub angular to sub rounded, weak calcareous cement, minor silty matrix, trace nodular pyrite, hard, poor visual porosity, no fluorescent. CLAYSTONE: reddish brown, greenish grey to light greenish grey, trace micro mica, moderately hard, sub fissile to sub blocky. |
| 645 | 650 | 90 10 | CLAYSTONE: greenish grey to light greenish grey, trace micro mica, moderately hard to hard, sub fissile to sub blocky. SILSTONE: medium to dark reddish brown, moderately hard to hard, sub blocky to sub fissile. |
| 650 | 655 | 90 10 | CLAYSTONE: greenish grey to dark greenish grey, trace micro mica, moderately hard, sub fissile to sub blocky. SILSTONE: medium to dark reddish brown, moderately hard to hard, sub blocky to sub fissile. |
| 655 | 660 | 100 | CLAYSTONE: medium grey, greenish grey to dark greenish grey, minor reddish brown, trace micro mica, moderately hard, sub blocky to sub fissile. |
| 660 | 665 | 100 | CLAYSTONE: medium grey, greenish grey to dark greenish grey, minor reddish brown, trace micro mica, moderately hard, sub blocky to sub fissile. |
| 665 | 670 | 100 | CLAYSTONE: medium grey, greenish grey, minor reddish brown, silty grading to SILTY CLAYSTONE trace micro mica, moderately hard, sub fissile to sub blocky. |
| 670 | 675 | 40 60 | SANDSTONE: clear to translucent, light grey, very fine to fine, well sorted, sub angular to sub rounded, weak calcareous cement, moderately strong siliceous cement, argillaceous matrix, moderately hard, very poor visual porosity, no fluorescent. CLAYSTONE: greenish grey to light greenish grey, silty grading to SILTSTONE, trace micro mica, moderately hard, sub fissile to sub blocky. |
| 675 | 680 | 40 60 | SILSTONE: medium to dark reddish brown, argillaceous, micro mica, moderately hard to hard, sub fissile to sub blocky. CLAYSTONE: greenish grey to light greenish grey, trace micro mica, moderately hard, sub fissile to sub blocky. |
| 680 | 685 | 90 10 | CLAYSTONE: medium grey, greenish grey, silty grading to SILTY CLAYSTONE, trace micro mica, moderately hard, sub fissile to sub blocky. SILTSTONE: reddish brown, moderately hard, sub blocky to sub fissile. |
| 685 | 690 | 20 80 | CLAYSTONE: light to medium greenish grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SILTSTONE: reddish brown, argillaceous in parts grading to SILTY CLAYSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 690 | 695 | 20 80 | CLAYSTONE: light to medium greenish grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SILTSTONE: reddish brown, argillaceous in parts grading to SILTY CLAYSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 695 | 700 | 20 60 20 | CLAYSTONE: light to medium greenish grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SILTSTONE: reddish brown, argillaceous in parts grading to SILTY CLAYSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: clear, translucent, light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, trace calcareous cement, trace light grey argillaceous matrix, tight visual porosity, no fluorescence. |
| 700 | 705 | 20 30 50 | CLAYSTONE: light to medium greenish grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SILTSTONE: reddish brown, argillaceous grading to SILTY CLAYSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: clear, translucent, light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, local calcareous cement, minor light grey argillaceous matrix, tight visual porosity, no fluorescence. |
| 705 | 710 | 20 20 60 | CLAYSTONE: light to medium greenish grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SILTSTONE: reddish brown, argillaceous in parts grading to SILTY CLAYSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: clear, translucent, light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, local calcareous cement, minor light grey argillaceous matrix, tight visual porosity, no fluorescence. |
| 710 | 715 | 30 10 60 | CLAYSTONE: medium grey, medium greenish grey, silty, in parts grading to ARGILLACEOUS SILTSTONE trace micro mica, rare nodular pyrite, moderately hard, sub blocky to sub fissile. SILTSTONE: reddish brown, argillaceous in parts grading to SILTY CLAYSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: clear, translucent, light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, local calcareous cement, minor light grey argillaceous matrix, tight visual porosity, no fluorescence. |
| 715 | 720 | 50 50 | CLAYSTONE: medium grey, medium greenish grey, silty, in parts grading to ARGILLACEOUS SILTSTONE trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: clear, light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, minor weak calcareous cement, moderately hard, tight visual porosity, no fluorescence. |
| 720 | 725 | 40 60 | CLAYSTONE: medium grey, medium greenish grey, silty, grading to ARGILLACEOUS SILTSTONE trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: clear, light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, minor weak calcareous cement, moderately hard, tight visual porosity, no fluorescence. |
| 725 | 730 | 40 60 | CLAYSTONE: medium to dark grey, silty, in parts grading to ARGILLACEOUS SILTSTONE, moderately hard, sub blocky to sub fissile. SANDSTONE: clear, light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, minor weak calcareous cement, moderately hard, tight visual porosity, no fluorescence. |
| 730 | 735 | 70 30 | CLAYSTONE: medium to dark grey, silty, in parts grading to ARGILLACEOUS SILTSTONE, moderately hard, sub blocky to sub fissile. SANDSTONE: light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, no fluorescence. |
| 735 | 740 | 80 20 | CLAYSTONE: medium to dark grey, greenish grey, silty grading to ARGILLACEOUS SILTSTONE, moderately hard to hard, blocky to sub fissile. SANDSTONE: light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, no fluorescence. |
| 740 | 745 | 80 20 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, silty grading to ARGILLACEOUS SILTSTONE, moderately hard to hard, blocky to sub fissile. SANDSTONE: light grey, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, no fluorescence. |
| 745 | 750 | 85 15 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, silty grading to ARGILLACEOUS SILTSTONE, moderately hard to hard, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, no fluorescence. |
| 750 | 755 | 90 10 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, silty grading to ARGILLACEOUS SILTSTONE, moderately hard to hard, soft to amorphous in part, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, no fluorescence. |
| 755 | 760 | 20 50 30 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, trace micromica, moderately hard to hard, soft to amorphous in part, blocky to sub fissile. SILTSTONE: medium to dark grey brown, common arenaceous, moderately hard to hard, blocky to sub blocky. SANDSTONE: light grey, off white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, no fluorescence. |
| 760 | 765 | 10 50 40 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, trace micromica, moderately hard to hard, blocky to sub fissile. SILTSTONE: predominantly medium to dark grey brown, occasionally medium green grey, common arenaceous grading to very fine SANDSTONE moderately hard to hard, blocky to sub blocky, sub fissile in part. SANDSTONE: light grey, off white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, trace mineral fluorescence. |
| 765 | 770 | 10 60 30 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, trace micromica, moderately hard to hard, soft to amorphous in part, blocky to sub fissile. SILTSTONE: predominantly medium to dark grey brown, common arenaceous grading to very fine SANDSTONE, moderately hard to hard, blocky to sub blocky, sub fissile in part. SANDSTONE: light grey, off white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, moderately hard, tight visual porosity, no fluorescence. |
| 770 | 775 | 10 70 20 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, trace micromica, moderately hard to hard, blocky to sub fissile. SILTSTONE: predominantly medium to dark grey brown, common arenaceous grading to very fine SANDSTONE, moderately hard to hard, blocky to sub blocky. SANDSTONE: light grey, off white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, rare orange stains, moderately hard, brittle in part, tight visual porosity, no fluorescence. |
| 775 | 780 | 10 80 10 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, trace micromica, moderately hard to hard, blocky to sub fissile. SILTSTONE: predominantly medium to dark grey brown, common arenaceous grading to very fine SANDSTONE, moderately hard to hard, blocky to sub blocky. SANDSTONE: light grey, off white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, com sucrose texture, moderately hard, brittle in part, tight visual porosity, no fluorescence. |
| 780 | 785 | 70 30 | CLAYSTONE: medium to dark grey, greenish grey, rare light grey, trace micromica, moderately hard to hard, blocky to sub fissile. SILTSTONE: predominantly medium to dark grey brown, common arenaceous grading to very fine SANDSTONE, moderately hard to hard, blocky to sub blocky. |
| 785 | 790 | 100 | CLAYSTONE: medium to dark grey, greenish grey, minor brownish grey, rare light grey, arenaceous grading to very fine SILTSTONE in part, trace micromica, moderately hard to hard, blocky to sub fissile. |
| 790 | 795 | 100 | CLAYSTONE: predominantly medium to dark grey, greenish grey, minor brownish grey, minor light grey, arenaceous grading to very fine SILTSTONE in part, trace micromica, moderately hard to hard, blocky to sub fissile. |
| 795 | 800 | 100 | CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 800 | 805 | 100 | CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 805 | 810 | 100 | CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 810 | 815 | 20 80 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 815 | 820 | 80 20 | SANDSTONE: white, translucent, clear, very light grey, very fine to medium grained, moderately sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 820 | 825 | 30 70 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 825 | 830 | 20 80 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 830 | 835 | 20 80 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 835 | 840 | 40 60 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 840 | 845 | 10 20 70 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. SILTSTONE: light to medium grey, slightly arenaceous, trace micro mica, moderately hard to hard, blocky to sub fissile. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 845 | 850 | 20 10 70 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. SILTSTONE: light to medium grey, slightly arenaceous, trace micro mica, moderately hard to hard, blocky to sub fissile. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 850 | 855 | 20 20 60 | SANDSTONE: white, translucent, clear, very light grey, very fine to predominantly fine grained, weak sorted, sub angular to sub round, moderately strong siliceous cement, trace calcareous cement, minor white argillaceous matrix, moderately hard to hard, very poor visual porosity, no fluorescence. SILTSTONE: light to medium grey, slightly arenaceous, trace micro mica, moderately hard to hard, blocky to sub fissile. CLAYSTONE: light to medium grey, light to medium greenish grey, silty in part occasionally grading to SILTSTONE, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 855 | 860 | 90 10 | SANDSTONE: white, very light grey, translucent, clear, very fine to medium grained, trace coarse, poor sorting, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor visual porosity, no fluorescence. CLAYSTONE: light to medium grey, slightly silty in part, rare micro mica, moderately hard, sub blocky to sub fissile. |
| 860 | 865 | 80 10 10 | SANDSTONE: white, very light grey, translucent, clear, very fine to medium grained, trace coarse, poor sorting, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor visual porosity, no fluorescence. SILTSTONE: light to medium grey, very finely arenaceous grading to very fine SANDSTONE, trace micro mica, moderately hard, blocky to sub blocky. CLAYSTONE: light to medium grey, slightly silty in part, rare micro mica, moderately hard, sub blocky to sub fissile. |
| 865 | 870 | 90 10 | SANDSTONE: white, very light grey, translucent, clear, very fine to medium grained, fair sorting, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor visual porosity, no fluorescence. SILTSTONE: light to medium grey, very finely arenaceous grading to very fine SANDSTONE, trace micro mica, moderately hard, blocky to sub blocky. |
| 870 | 875 | 90 10 | SANDSTONE: white, very light grey, translucent, clear, very fine to medium grained, trace coarse, poor sorting, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor visual porosity, no fluorescence. SILTSTONE: light to medium grey, very finely arenaceous grading to very fine SANDSTONE, trace micro mica, moderately hard, blocky to sub blocky. |
| 875 | 880 | 100 | SANDSTONE: white, translucent, clear, minor light grey, fine to coarse, poor sorting, sub angular to occasionally sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 880 | 885 | 100 | SANDSTONE: white, translucent, clear, minor light grey, fine to coarse, poor sorting, sub angular to occasionally sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 885 | 890 | 100 | SANDSTONE: translucent, clear, white, fine to coarse grained, predominantly medium, sub angular to predominantly sub round, fair to moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 890 | 895 | 100 | SANDSTONE: translucent, clear, white, fine to coarse grained, predominantly medium, sub angular to predominantly sub round, fair to moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 895 | 900 | 100 | SANDSTONE: translucent, clear, white, fine to coarse grained, predominantly medium, sub angular to predominantly sub round, fair to moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 900 | 905 | 100 | SANDSTONE: translucent, clear, white, fine to coarse grained, predominantly medium, sub angular to predominantly sub round, fair to moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 905 | 910 | 100 | SANDSTONE: translucent, clear, white, fine to coarse grained, predominantly medium, sub angular to predominantly sub round, fair to moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 910 | 915 | 100 | SANDSTONE: translucent, clear, white, fine to coarse grained, predominantly medium, sub angular to predominantly sub round, fair to moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 915 | 920 | 20 80 | SILTSTONE: light to medium grey, medium greenish grey, rare micro mica, trace lithics, moderately hard to hard, fissile to sub fissile. SANDSTONE: translucent, clear, white, fine to predominantly medium, sub angular to predominantly sub round, fair to moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 920 | 925 | 10 90 | SILTSTONE: light to medium grey, medium greenish grey, rare micro mica, trace lithics, moderately hard to hard, fissile to sub fissile. SANDSTONE: translucent, clear, white, fine to medium grained, sub angular to predominantly sub round, moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 925 | 930 | 10 90 | SILTSTONE: light to medium grey, medium greenish grey, rare micro mica, trace lithics, moderately hard to hard, fissile to sub fissile. SANDSTONE: translucent, clear, white, fine to medium grained, sub angular to predominantly sub round, moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 930 | 935 | 10 90 | SILTSTONE: light to medium grey, medium greenish grey, rare micro mica, trace lithics, moderately hard to hard, fissile to sub fissile. SANDSTONE: translucent, clear, white, fine to coarse grained, sub angular to predominantly sub round, poorly sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 935 | 940 | Trace 100 | SILTSTONE: light to medium grey, medium greenish grey, rare micro mica, trace lithics, moderately hard to hard, fissile to sub fissile. SANDSTONE: translucent, clear, white, fine to medium grained, sub angular to predominantly sub round, moderately sorted, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 940 | 945 | Trace 100 | SILTSTONE: light to medium grey, medium greenish grey, rare micro mica, trace lithics, moderately hard to hard, fissile to sub fissile. SANDSTONE: translucent, clear, white, fine to coarse predominantly medium grained, sub angular to predominantly sub round, fair sorting, minor siliceous cement, trace white argillaceous matrix, friable to moderately hard aggregates, poor to fair visual porosity, no fluorescence. |
| 945 | 950 | 100 | SANDSTONE: translucent, clear, white, fine to medium grained, moderately sorted, sub angular to sub round, minor siliceous cement, rare white argillaceous matrix, friable to moderately hard aggregates, predominantly disaggregated quartz grains, poor to fair inferred porosity, no fluorescence. |
| 950 | 955 | 100 | SANDSTONE: translucent, clear, white, fine to medium grained, moderately sorted, sub angular to sub round, minor siliceous cement, rare white argillaceous matrix, friable to moderately hard aggregates, predominantly disaggregated quartz grains, poor to fair inferred porosity, no fluorescence. |
| 955 | 960 | 100 | SANDSTONE: translucent, clear, white, fine to medium grained, moderately sorted, sub angular to sub round, minor siliceous cement, rare white argillaceous matrix, friable to moderately hard aggregates, predominantly disaggregated quartz grains, poor to fair inferred porosity, no fluorescence. |
| 960 | 965 | 100 | SANDSTONE: translucent, clear, white, fine to medium grained, trace coarse, moderately sorted, sub angular to sub round, minor siliceous cement, rare white argillaceous matrix, friable to moderately hard aggregates, predominantly disaggregated quartz grains, poor to fair inferred porosity, no fluorescence. |
| 965 | 970 | 100 | SANDSTONE: translucent, clear, white, fine to medium grained, moderately sorted, sub angular to sub round, minor siliceous cement, rare white argillaceous matrix, friable to moderately hard aggregates, predominantly disaggregated quartz grains, poor to fair inferred porosity, no fluorescence. |
| 970 | 975 | Trace 100 | SILTSTONE: dark grey to medium light grey, light brown to moderate brown in parts, trace dark brown, dominantly argillaceous, trace arenaceous, trace micro mica, firm to trace moderately hard, fissile, occasionally sub fissile. SANDSTONE: clear to opaque, translucent, off white, dominantly very fine to fine, minor medium, moderately sorted, sub rounded to minor sub angular, trace angular, minor weak siliceous cement, trace off white argillaceous matrix, dominantly loose, minor friable aggregates, poor to very poor visual porosity, poor inferred porosity, no fluorescence. |
| 975 | 980 | Trace 100 | SILTSTONE: dark grey to medium light grey, light brown to moderate brown in parts, trace dark brown, dominantly argillaceous, trace arenaceous, trace micro mica, moderately hard, fissile, occasionally sub fissile. SANDSTONE: off white to opaque, clear to translucent, very fine to fine, minor medium, moderately to rare moderately well sorted, sub rounded to occasionally sub angular, trace weak siliceous cement, trace off white argillaceous matrix, minor light brown to translucent orange lithics, dominantly clean and loose, friable aggregates, poor visual porosity, poor inferred porosity, no fluorescence. |
| 980 | 985 | Trace 100 | SILTSTONE: dark grey to medium light grey, light brown to moderate brown in parts, trace dark brown, dominantly argillaceous, trace arenaceous, trace micro mica, firm to trace moderately hard, fissile, occasionally sub fissile. SANDSTONE: clear to translucent, opaque, off white, very fine to fine, common medium, moderately sorted, sub angular to minor sub rounded, trace angular, trace moderate to weak siliceous cement, minor off white argillaceous matrix, trace light brown to translucent orange lithics, minor fractured grains, dominantly loose, trace friable aggregates, poor visual porosity, poor inferred porosity, no fluorescence. |
| 985 | 990 | 60 40 | CLAYSTONE: medium grey to medium light grey, minor light bluish grey, very light grey in parts, trace light olive grey, dominantly argillaceous, minor silty, grading to SILTSTONE in parts, occasionally micro mica, firm, occasionally moderately hard, fissile, occasionally sub fissile. SANDSTONE: clear to translucent, opaque, off white, very fine to fine, common medium, moderately sorted, sub angular to minor sub rounded, trace angular, trace moderate to weak siliceous cement, minor off what argillaceous matrix, trace light brown to translucent orange lithics, minor fractured grains, dominantly loose, trace friable aggregates, poor visual porosity, poor inferred porosity, no fluorescence. |
| 990 | 995 | 40 60 | CLAYSTONE: light olive grey to medium grey, medium light grey to light grey in part, trace light brownish grey, dominantly argillaceous, trace silty grading to SILTSTONE in part, minor micro mica, moderately hard, fissile to minor sub fissile. SANDSTONE: clear to opaque translucent, off white, very fine to trace fine, moderately well sorted, sub-round to minor round, trace sub-angular, rare weak siliceous cement, rare off white to very light grey argillaceous matrix, dominantly loose, minor fractured grains, rare friable aggregates, poor visible porosity, poor inferred porosity, no fluorescence. |
| 995 | 1000 | 60 40 | CLAYSTONE: medium grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: white, very light grey, rare translucent, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1000 | 1005 | 90 10 | CLAYSTONE: medium grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: white, very light grey, rare translucent, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1005 | 1010 | 80 20 | CLAYSTONE: medium grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: white, very light grey, rare translucent, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1010 | 1015 | 80 20 | CLAYSTONE: medium grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: white, very light grey, rare translucent, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1015 | 1020 | 70 30 | CLAYSTONE: medium grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: white, very light grey, rare translucent, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1020 | 1025 | 60 40 | CLAYSTONE: medium grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: white, very light grey, rare translucent, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1025 | 1030 | 70 30 | CLAYSTONE: medium grey, medium dark grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, white, fine to medium grained, moderately sorted, sub angular to sub round, weak to moderately strong siliceous cement, minor white argillaceous matrix, friable to moderately hard aggregates, poor to fair inferred porosity, Fluorescence; 1028m (spot sample) 5% very dull yellow spotted, no direct cut, very faint white thin ring residue. |
| 1030 | 1035 | 30 70 | CLAYSTONE: medium grey, medium dark grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, white, fine to medium predominantly fine grained, grained, moderately sorted, sub angular to sub round, weak siliceous cement, minor white argillaceous matrix, friable aggregates, poor to fair inferred porosity, no fluorescence. |
| 1035 | 1040 | 60 40 | CLAYSTONE: medium grey, medium dark grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, white, fine to medium predominantly fine grained, grained, moderately sorted, sub angular to sub round, weak siliceous cement, minor white argillaceous matrix, friable aggregates, poor to fair inferred porosity, no fluorescence. |
| 1040 | 1045 | 100 | CLAYSTONE: medium grey, medium dark grey, medium brownish grey, very finely arenaceous, trace micro mica, moderately hard, sub blocky to sub fissile. |
| 1045 | 1050 | 70 30 | CLAYSTONE: medium brown, medium brownish grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: translucent, clear, white, fine grained, well sorted, sub angular to sub round, weak siliceous cement, rare white argillaceous matrix, predominantly loose / friable, fair inferred porosity, Fluorescence: trace dull yellow spotted, no cut or crush cut, trace faint white residue. |
| 1050 | 1055 | 90 10 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: translucent, clear, white, fine grained, well sorted, sub angular to sub round, weak siliceous cement, minor white argillaceous matrix, predominantly loose / friable, poor to fair inferred porosity, Fluorescence: trace very dull yellow spotted, no cut, slow faint white crush cut, very faint white film residue. |
| 1055 | 1060 | 90 10 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: translucent, clear, white, fine grained, well sorted, sub angular to sub round, weak siliceous cement, minor white argillaceous matrix, predominantly loose / friable, poor to fair inferred porosity, Fluorescence: trace very dull yellow spotted, no cut, slow faint white crush cut, very faint white film residue. |
| 1060 | 1065 | 90 10 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: translucent, clear, white, fine grained, well sorted, sub angular to sub round, weak siliceous cement, minor white argillaceous matrix, predominantly loose / friable, poor to fair inferred porosity, Fluorescence: trace very dull yellow spotted, no cut, slow faint white crush cut, very faint white film residue. |
| 1065 | 1070 | 80 20 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: white, translucent, clear, very fine to fine grained, moderately well sorted, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor inferred porosity, no fluorescence. |
| 1070 | 1075 | 40 60 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: white, translucent, clear, very fine to fine grained, moderately well sorted, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor inferred porosity, no fluorescence. |
| 1075 | 1080 | 70 30 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: white, translucent, clear, very fine to fine grained, moderately well sorted, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor inferred porosity, no fluorescence. |
| 1080 | 1085 | 80 20 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: white, translucent, clear, very fine to fine grained, trace medium, moderately well sorted, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor inferred porosity, no fluorescence. |
| 1085 | 1090 | 70 30 | CLAYSTONE: medium brown, medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: white, translucent, clear, very fine to fine grained, trace loose medium grains, moderately well sorted, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor inferred porosity, no fluorescence. |
| 1090 | 1095 | 70 30 | CLAYSTONE: medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: white, translucent, clear, very fine to fine grained, trace loose medium grains, moderately well sorted, sub angular to sub round, moderately strong siliceous cement, common white argillaceous matrix, moderately hard, very poor inferred porosity, no fluorescence. |
| 1095 | 1100 | 70 30 | CLAYSTONE: medium brownish grey, medium to dark grey, silty grading to argillaceous SILTSTONE, rare micro mica, moderately hard, blocky to sub blocky. SANDSTONE: off white, translucent, trace translucent light brown, rare to trace clear, very fine, trace fine grains, moderately well sorted, sub rounded to sub angular, common moderate to trace weak siliceous cement, common off white argillaceous matrix, dominantly friable, trace moderately hard, tight to very poor visual porosity, no fluorescence. |
| 1100 | 1105 | 70 30 | CLAYSTONE: olive grey to brownish grey, medium to light grey, trace medium dark grey, minor very light grey, trace silty grading to SILTSTONE, minor micromica, firm to trace moderately hard, rare soft, dominantly sub fissile to minor sub blocky, trace fissile. SANDSTONE: off white, translucent, trace translucent light brown, rare to trace clear, very fine, trace fine grains, moderately well sorted, sub rounded to sub angular, common moderate to trace weak siliceous cement, common off white argillaceous matrix, dominantly friable, trace moderately hard, tight to very poor visual porosity, no fluorescence. |
| 1105 | 1110 | 80 20 | CLAYSTONE: olive grey to brownish grey, medium to light grey, trace medium dark grey, minor very light grey, silty grading to SILTSTONE, minor micro mica, firm to trace moderately hard, rare soft, dominantly sub-fissile to minor sub-blocky, trace fissile. SANDSTONE: off white to very light grey, frosted, translucent in part, dominantly very fine aggregates grade to arenaceous SILTSTONE, trace fine grains, moderately well sorted, sub-round to sub-angular, com moderately to trace strong siliceous cement, com off white to very light grey argillaceous matrix, friable to com moderately hard, tight visual porosity, poor to very poor inferred porosity, no fluorescence. |
| 1110 | 1115 | 90 10 | CLAYSTONE: light brownish grey to brownish grey, minor olive grey, medium to medium light grey, trace medium dark grey, very light grey in part, silty grade to SILTSTONE in part, minor micro mica, trace sorted, minor firm to trace moderately hard, sub-fissile to sub-blocky, trace fissile. SANDSTONE: off white to very light grey, frosted, translucent in part, dominantly very fine aggregates grade to arenaceous SILTSTONE, trace fine grains, moderately well sorted, sub-round to sub-angular, com moderately to trace strong siliceous cement, com off white to very light grey argillaceous matrix, friable to com moderately hard, tight visual porosity, poor to very poor inferred porosity, no fluorescence. |
| 1115 | 1120 | 80 20 | CLAYSTONE: light brownish grey to brownish grey, minor olive grey, medium to medium light grey, trace medium dark grey, very light grey in part, silty grade to SILTSTONE in part, minor micro mica, trace sorted, minor firm to trace moderately hard, sub-fissile to sub-blocky, trace fissile. SANDSTONE: off white to very light grey, translucent in part, trace translucent light brown, very fine grade to arenaceous SILTSTONE, moderately well sorted, sub-round to minor sub-angular, com moderately to minor weak siliceous cement, com to abundant off white argillaceous matrix, rare black lithics, moderately hard, minor friable, nil to tight visual porosity, no fluorescence. |
| 1120 | 1125 | 60 40 | CLAYSTONE: light brownish grey to olive grey, trace brownish grey, light olive grey in part, medium grey to medium light grey, trace silty grade to SILTSTONE in part, minor to com micro mica, firm to occasionally moderately hard, sub-fissile to sub-blocky. SANDSTONE: off white to very light grey, translucent in part, trace translucent light brown, very fine grade to arenaceous SILTSTONE, moderately well sorted, sub-round to minor sub-angular, com moderately to minor weak siliceous cement, com to abundant off white argillaceous matrix, rare black lithics, moderately hard, minor friable, nil to tight visual porosity, no fluorescence. |
| 1125 | 1130 | 50 50 | CLAYSTONE: light brownish grey to olive grey, trace brownish grey, light olive grey in part, medium grey to medium light grey, trace silty grade to SILTSTONE in part, minor to com micro mica, firm to occasionally moderately hard, sub-fissile to sub-blocky. SANDSTONE: off white to very light grey, trace translucent, trace light grey, very fine aggregates, moderately well sorted, sub-round to sub-angular, com weak to minor moderately siliceous cement, abundant off white argillaceous matrix, trace black lithics, friable to com moderately hard, trace unconsolidated, nil to tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1130 | 1135 | 40 60 | CLAYSTONE: olive grey to light brownish grey, medium light grey to medium dark grey, trace light brown grey, trace silty grade to SILTSTONE in part, minor to occasionally micro mica, firm, trace soft, rare moderately hard, sub-fissile to sub-blocky. SANDSTONE: off white to very light grey, trace translucent, trace light grey, very fine aggregates, moderately well sorted, sub-round to sub-angular, com weak to minor moderately siliceous cement, abundant off white argillaceous matrix, trace black lithics, friable to com moderately hard, trace unconsolidated, nil to tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1135 | 1140 | 60 40 | CLAYSTONE: olive grey to light brownish grey, medium light grey to medium dark grey, trace light brown grey, trace silty grade to SILTSTONE in part, minor to occasionally micro mica, firm, trace soft, rare moderately hard, sub-fissile to sub-blocky. SANDSTONE: off white, very light grey, translucent in part, trace milky, very fine aggregates, moderately well sorted, sub-round to minor sub-angular, com moderately to weak siliceous cement, com to abundant off white argillaceous matrix, minor black lithics, moderately hard, trace friable, nil to tight visual porosity, no fluorescence. |
| 1140 | 1145 | 60 40 | CLAYSTONE: olive grey to light brownish grey, medium light grey to medium dark grey, trace light brown grey, trace silty grade to SILTSTONE in part, minor to occasionally micro mica, firm, trace soft, rare moderately hard, sub-fissile to sub-blocky, trace blocky. SANDSTONE: off white, very light grey, translucent in part, trace milky, very fine aggregates, moderately well sorted, sub-round to minor sub-angular, com moderately to weak siliceous cement, com to abundant off white argillaceous matrix, minor black lithics, moderately hard, trace friable, nil to tight visual porosity, no fluorescence. |
| 1145 | 1150 | 80 20 | CLAYSTONE: olive grey to light brownish grey, medium light grey to medium dark grey, trace light brown grey, trace silty grade to SILTSTONE in part, minor to occasionally micro mica, firm, trace soft, rare moderately hard, sub-fissile to sub-blocky, trace fissile. SANDSTONE: off white to very light grey, light grey, trace translucent, rare translucent light brown, very fine aggregates, moderately well sorted, sub-round to occasionally sub-angular, com moderately siliceous cement, com off white argillaceous matrix, trace black lithics, dominantly moderately hard, trace friable, nil to tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1150 | 1155 | 90 10 | CLAYSTONE: medium dark to medium grey, common dark grey, trace light grey, light brownish grey in part, trace silty grade to SILTSTONE in part, minor micro mica, firm to moderately hard, sub-fissile to sub-blocky, trace fissile. SANDSTONE: off white to very light grey, light grey, trace translucent, rare translucent light brown, very fine aggregates, moderately well sorted, sub-round to occasionally sub-angular, com moderately siliceous cement, com off white argillaceous matrix, trace black lithics, dominantly moderately hard, trace friable, nil to tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1155 | 1160 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, occasionally translucent to clear, very fine to fine grained, well sorted, sub angular to sub round, strong siliceous cement, minor white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1160 | 1165 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, occasionally translucent to clear, very fine to fine grained, well sorted, sub angular to sub round, strong siliceous cement, minor white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1165 | 1170 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, occasionally translucent to clear, very fine to fine grained, well sorted, sub angular to sub round, strong siliceous cement, minor white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1170 | 1175 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, occasionally translucent to clear, very fine to fine grained, well sorted, sub angular to sub round, strong siliceous cement, minor white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1175 | 1180 | 100 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. |
| 1180 | 1185 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, translucent to clear, very fine to fine grained, well sorted, sub angular to sub round, strong siliceous cement, minor white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1185 | 1190 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, translucent to clear, very fine to fine grained, well sorted, sub angular to sub round, strong siliceous cement, minor white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1190 | 1195 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, translucent, clear, very fine to predominantly fine grained, trace medium, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1195 | 1200 | 80 20 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, translucent, clear, very fine to predominantly fine grained, trace medium, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1200 | 1205 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, translucent, clear, very fine to predominantly fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1205 | 1210 | 90 10 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, translucent, clear, very fine to predominantly fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1210 | 1215 | 80 20 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, locally silty grading to argillaceous SILTSTONE, trace micro mica, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, translucent, clear, very fine to predominantly fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1215 | 1220 | 80 20 | CLAYSTONE: medium grey to medium dark grey, occasionally medium brownish grey, locally silty grading to argillaceous SILTSTONE, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1220 | 1225 | 90 10 | CLAYSTONE: medium grey to medium dark grey, occasionally medium brownish grey, locally silty grading to argillaceous SILTSTONE, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1225 | 1230 | 80 20 | CLAYSTONE: medium grey to medium dark grey, occasionally medium brownish grey, locally silty grading to argillaceous SILTSTONE, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1230 | 1235 | 70 30 | CLAYSTONE: medium grey to medium dark grey, occasionally medium brownish grey, locally silty grading to argillaceous SILTSTONE, moderately hard, sub blocky to sub fissile, trace fissile. SANDSTONE: white, very light grey, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1235 | 1240 | 80 20 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, occasionally light greenish grey, locally silty grading to argillaceous SILTSTONE, moderately hard, sub blocky to sub fissile. SANDSTONE: white, translucent, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1240 | 1245 | 80 20 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, occasionally light greenish grey, locally silty grading to argillaceous SILTSTONE, moderately hard, sub blocky to sub fissile. SANDSTONE: white, translucent, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1245 | 1250 | 70 30 | CLAYSTONE: medium grey to medium dark grey, medium brownish grey, occasionally light greenish grey, locally silty grading to argillaceous SILTSTONE, moderately hard, sub blocky to sub fissile. SANDSTONE: white, translucent, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1250 | 1255 | 70 30 | CLAYSTONE: dark to medium dark grey, occasionally medium to medium light grey, minor light olive grey, light grey in parts, trace reddish brown, trace silty, trace micro mica, firm, minor moderately hard, trace soft, sub fissile to minor sub blocky, rare fissile. SANDSTONE: white, translucent, very fine to occasionally fine grained, sub angular to occasionally sub round, moderately well sorted, moderately strong siliceous cement, common white argillaceous matrix, hard, tight to very poor visual porosity, no fluorescence. |
| 1255 | 1260 | 60 40 | CLAYSTONE: dark to medium dark grey, occasionally medium to medium light grey, minor light olive grey, light grey in parts, trace reddish brown, trace silty, trace micro mica, firm, minor moderately hard, trace soft, sub fissile to minor sub blocky, rare fissile. SANDSTONE: translucent to minor clear, off white to very light grey, trace white, very fine to fine, trace medium, moderately well sorted, minor weak to moderately siliceous cement, common off white argillaceous matrix, rare black lithics, friable to moderately hard, hard aggregates, tight to very poor visual porosity, poor inferred porosity, no fluorescence. |
| 1260 | 1265 | 90 10 | CLAYSTONE: dark to medium grey, medium light grey to light grey, minor light brown to trace light olive grey, trace to minor silty grad to SILTSTONE, trace micro mica, firm to trace moderately hard, sub fissile, minor sub blocky, trace fissile. SANDSTONE: translucent to minor clear, off white to very light grey, trace white, very fine to fine, trace medium, moderately well sorted, minor weak to moderately siliceous cement, common off white argillaceous matrix, rare black lithics, friable to moderately hard, trace hard aggregates, tight to very poor visual porosity, poor inferred porosity, no fluorescence. |
| 1265 | 1270 | 80 20 | CLAYSTONE: dark to medium grey, medium light grey to light grey, minor light brown to trace light olive grey, trace to minor silty grad to SILTSTONE, trace micro mica, firm to trace moderately hard, sub fissile, minor sub blocky, trace fissile. SANDSTONE: very light grey to off white, minor light grey, very fine to trace fine, rare medium, moderately well sorted, minor moderate to trace strong siliceous cement, common off white argillaceous matrix, trace black lithics, moderately hard, tight to very poor visual porosity, very poor to poor inferred porosity, no fluorescence. |
| 1270 | 1275 | 80 20 | CLAYSTONE: light-medium light grey, medium grey in part, occasionally dark to medium dark grey, trace to minor silty grade to SILTSTONE, minor micro mica, firm to com moderately hard, sub-fissile, minor sub-blocky, trace fissile. SANDSTONE: very light grey to off white, minor light grey, very fine to trace fine, rare medium, moderately well sorted, minor moderate to trace strong siliceous cement, common off white argillaceous matrix, trace black lithics, moderately hard, trace friable, tight to very poor visual porosity, very poor to poor inferred porosity, no fluorescence. |
| 1275 | 1280 | 70 30 | CLAYSTONE: light-medium light grey, medium grey in part, occasionally dark to medium dark grey, trace to minor silty grade to SILTSTONE, minor micro mica, firm to com moderately hard, sub-fissile, minor sub-blocky, trace fissile. SANDSTONE: light grey to very light grey, minor off white to translucent, dominantly very fine trace grade to arenaceous SILTSTONE, trace fine, moderately to moderately well sorted, sub-angular to sub-round, moderately siliceous cement in part, com off white to light grey argillaceous matrix, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1280 | 1285 | 80 20 | CLAYSTONE: light grey to medium light grey, com medium grey to dark grey, minor to trace silty grade to SILTSTONE in part, occasionally micro mica, firm to com moderately hard, sub-fissile, minor sub-blocky, trace fissile. SANDSTONE: light grey to very light grey, minor off white to translucent, dominantly very fine trace grade to arenaceous SILTSTONE, trace fine, moderately to moderately well sorted, sub-angular to sub-round, moderately siliceous cement in part, com off white to light grey argillaceous matrix, moderately hard aggregates, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. |
| 1285 | 1290 | 80 20 | CLAYSTONE: light grey to medium light grey, com medium grey to dark grey, minor to trace silty grade to SILTSTONE in part, occasionally micro mica, firm to com moderately hard, sub-fissile, minor sub-blocky, trace fissile. SANDSTONE: light grey to very light grey, minor off white to translucent, dominantly very fine trace grade to arenaceous SILTSTONE, trace fine, moderately to moderately well sorted, sub-angular to sub-round, moderately siliceous cement in part, common off white to light grey argillaceous matrix, moderately hard aggregates, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. |
| 1290 | 1295 | 80 20 | CLAYSTONE: light grey to medium light grey, com medium grey to dark grey, minor to trace silty grade to SILTSTONE in part, occasionally micro mica, firm to com moderately hard, sub-fissile, minor sub-blocky, trace fissile. SANDSTONE: off white to very light grey, trace white, minor translucent, very fine to trace fine, grade to arenaceous SILTSTONE in part, moderately sorted, sub-angular to sub-round, moderately to trace strong siliceous cement, common off white argillaceous matrix, moderately hard to trace hard, nil to tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1295 | 1300 | 80 20 | CLAYSTONE: light grey to medium light grey, com medium grey to dark grey, minor to trace silty grade to SILTSTONE in part, occasionally micro mica, firm to com moderately hard, sub-fissile, minor sub-blocky, trace fissile. SANDSTONE: off white to very light grey, trace white, minor translucent, very fine to trace fine, grade to arenaceous SILTSTONE in part, moderately sorted, sub-angular to sub-round, moderately to trace strong siliceous cement, common off white argillaceous matrix, trace black lithics, moderately hard to trace hard, nil to tight visual porosity, no fluorescence. |
| 1300 | 1305 | 70 30 | CLAYSTONE: dark to medium dark grey, medium grey in part, trace light grey, trace light brownish grey, minor micro mica, rare disseminated pyrite, moderately hard, firm in part, sub-fissile, sub-blocky in part, trace fissile. SANDSTONE: off white to very light grey, trace white, minor translucent, very fine to trace fine, grade to arenaceous SILTSTONE in part, moderately sorted, sub-angular to sub-round, moderately to trace strong siliceous cement, common off white argillaceous matrix, trace black lithics, moderately hard to trace hard, nil to tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1305 | 1308 | 70 30 | CLAYSTONE: dark to medium dark grey, medium grey in part, trace light grey, trace light brownish grey, minor micro mica, rare disseminated pyrite, moderately hard, firm in part, sub-fissile, sub-blocky in part, trace fissile. SANDSTONE: translucent to clear, off white to very light grey, very fine to trace fine, rare medium aggregates, poor to moderately sorted, sub-round, minor sub-angular, minor moderate to weak siliceous cement, minor off white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1308 | 1310 | 90 10 | CLAYSTONE: medium grey to medium light grey, dark grey in part, very light to light grey, minor greyish black, trace silty, trace micro mica, moderately hard to trace hard, sub-fissile to fissile, minor sub-blocky to trace blocky. SANDSTONE: translucent to clear, off white to very light grey, very fine to trace fine, rare medium aggregates, poor to moderately sorted, sub-round, minor sub-angular, minor moderate to weak siliceous cement, minor off white argillaceous matrix, moderately hard aggregates, very poor visual porosity, no fluorescence. |
| 1310 | 1312 | 90 10 | CLAYSTONE: dark to medium dark grey, medium grey in part, trace light grey, trace light brownish grey, minor micro mica, rare disseminated pyrite, moderately hard, firm in part, sub-fissile, sub-blocky in part, trace fissile. SANDSTONE: off white to very light grey, trace clear to translucent, very fine to minor fine, trace grade to arenaceous SILTSTONE, moderately sorted, sub-round to trace sub-angular, common moderately siliceous cement, minor to common off white argillaceous matrix, moderately hard to trace friable, tight visual porosity, no fluorescence. |
| 1312 | 1314 | 90 10 | CLAYSTONE: dark to medium dark grey, medium grey in part, trace light grey, trace light brownish grey, minor micro mica, rare disseminated pyrite, moderately hard, firm in part, sub-fissile, sub-blocky in part, trace fissile. SANDSTONE: off white to very light grey, trace clear to translucent, very fine to minor fine, trace grade to arenaceous SILTSTONE, moderately sorted, sub-round to trace sub-angular, common moderately siliceous cement, minor to common off white argillaceous matrix, moderately hard to trace friable, tight visual porosity, no fluorescence. |
| 1314 | 1315 | 90 10 | CLAYSTONE: dark to medium dark grey, medium grey in part, trace light grey, trace light brownish grey, minor micro mica, rare disseminated pyrite, moderately hard, firm in part, sub-fissile, sub-blocky in part, trace fissile. SANDSTONE: off white to very light grey, trace clear to translucent, very fine to minor fine, trace grade to arenaceous SILTSTONE, moderately sorted, sub-round to trace sub-angular, common moderately siliceous cement, minor to common off white argillaceous matrix, moderately hard to trace friable, tight visual porosity, no fluorescence. |
| 1315 | 1316 | 90 10 | CLAYSTONE: olive grey-brownish grey, light olive grey in part, common dark to medium dark grey, occasionally medium grey, trace light brownish grey, minor micro mica, rare disseminated pyrite, moderately hard, firm in part, sub-fissile, sub-blocky in part, trace fissile. SANDSTONE: off white to very light grey, trace clear to translucent, very fine to minor fine, trace grade to arenaceous SILTSTONE, moderately sorted, sub-round to trace sub-angular, common moderately siliceous cement, minor to common off white argillaceous matrix, moderately hard to trace friable, tight visual porosity, no fluorescence. |
| 1316 | 1318 | 90 10 | CLAYSTONE: olive grey-brownish grey, light olive grey in part, common dark to medium dark grey, occasionally medium grey, trace light brownish grey, trace micro mica, rare disseminated pyrite, moderately hard, firm in part, sub-fissile, sub-blocky in part, trace fissile. SANDSTONE: off white to very light grey, trace clear to translucent, very fine to minor fine, trace grade to arenaceous SILTSTONE, moderately sorted, sub-round to trace sub-angular, common moderately siliceous cement, minor to common off white argillaceous matrix, moderately hard to trace friable, tight visual porosity, no fluorescence. |
| 1318 | 1320 | 80 20 | CLAYSTONE: medium dark grey, medium brownish grey, slightly silty, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, trace quartz overgrowths, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1320 | 1325 | 60 40 | CLAYSTONE: medium dark grey, medium brownish grey, slightly silty, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, trace quartz overgrowths, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1325 | 1330 | 70 30 | CLAYSTONE: medium dark grey, medium brownish grey, slightly silty, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, trace quartz overgrowths, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1330 | 1335 | 70 30 | CLAYSTONE: medium dark grey, medium brownish grey, slightly silty, trace micro mica, moderately hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, trace quartz overgrowths, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1335 | 1340 | 80 20 | CLAYSTONE: medium dark grey to dark grey, slightly silty, rare micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, clear, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1340 | 1345 | 80 20 | CLAYSTONE: medium dark grey to dark grey, slightly silty, rare micro mica, trace pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, clear, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1345 | 1350 | 60 40 | CLAYSTONE: medium dark grey to dark grey, slightly silty, rare micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, clear, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1350 | 1355 | 80 20 | CLAYSTONE: medium dark grey to dark grey, slightly silty, rare micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, clear, very fine to predominantly fine grained, sub angular to occasionally sub round, strong siliceous cement, rare white argillaceous matrix, moderately hard aggregates, tight to very poor visual porosity, no fluorescence. |
| 1355 | 1360 | 60 40 | CLAYSTONE: medium to dark grey, occasionally medium to dark brownish grey, trace micro mica, trace nodular pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, very fine to predominantly fine grained, trace medium, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, trace off white argillaceous matrix, predominantly dis-aggregated quartz grains, very poor to poor inferred porosity, no fluorescence. |
| 1360 | 1365 | 50 50 | CLAYSTONE: medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1365 | 1370 | 40 60 | CLAYSTONE: medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1370 | 1375 | 40 60 | CLAYSTONE: medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1375 | 1380 | 60 40 | CLAYSTONE: medium grey to medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, off white, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1380 | 1385 | 50 50 | CLAYSTONE: medium grey to medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, off white, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1385 | 1390 | 20 80 | CLAYSTONE: medium grey to medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, off white, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1390 | 1395 | 60 40 | CLAYSTONE: medium grey to medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, off white, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1395 | 1400 | 70 30 | CLAYSTONE: medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, off white, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1400 | 1405 | 80 20 | CLAYSTONE: medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, off white, clear, very fine to fine grained, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1405 | 1410 | 90 10 | CLAYSTONE: medium dark grey, trace micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: very light grey, translucent, off white, clear, very fine to fine grained, trace medium quartz grains, sub angular to sub round, moderately well sorted, moderately strong siliceous cement, minor off white argillaceous matrix, tight to very poor visual porosity, no fluorescence. |
| 1410 | 1415 | 100 | CLAYSTONE: medium dark grey to dark grey, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 1415 | 1420 | 100 | CLAYSTONE: medium dark grey to dark grey, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 1420 | 1425 | 100 | CLAYSTONE: medium dark grey to dark grey, trace micro mica, moderately hard to hard, blocky to sub fissile. |
| 1425 | 1430 | 90 10 | CLAYSTONE: medium dark grey to dark grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: translucent, very light grey, clear, very fine to fine grained, sub angular to sub rounded, moderately sorted, strong siliceous cement in fine grained aggregates, rare white argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1430 | 1435 | 80 20 | CLAYSTONE: medium dark grey to dark grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: translucent, very light grey, clear, very fine to fine grained, sub angular to sub rounded, moderately sorted, strong siliceous cement in fine grained aggregates, rare white argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1435 | 1440 | 60 40 | CLAYSTONE: medium dark grey to dark grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: translucent, very light grey, clear, very fine to fine grained, sub angular to sub rounded, moderately sorted, strong siliceous cement in fine grained aggregates, rare white argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1440 | 1445 | 40 60 | CLAYSTONE: medium dark grey to dark grey, trace micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: translucent, very light grey, clear, very fine to fine grained, sub angular to sub rounded, moderately sorted, strong siliceous cement in fine grained aggregates, rare white argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1445 | 1450 | 20 80 | CLAYSTONE: medium dark grey to dark grey, rare micro mica, moderately hard, blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, fine to coarse predominantly medium, poor sorting, sub angular to sub rounded, strong siliceous cement in fine grained aggregates, minor white argillaceous matrix, trace pyrite, moderately hard aggregates, medium to coarse grains are dis-aggregated, tight to very poor visual porosity, no fluorescence. |
| 1450 | 1455 | 10 90 | CLAYSTONE: medium dark grey to dark grey, rare micro mica, moderately hard, blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, fine to coarse predominantly medium, poor sorting, sub angular to sub rounded, strong siliceous cement in fine grained aggregates, minor white argillaceous matrix, trace pyrite, moderately hard aggregates, medium to coarse grains are dis-aggregated, tight to very poor visual porosity, no fluorescence. |
| 1455 | 1460 | 100 | SANDSTONE: translucent, clear, very light grey, fine to coarse predominantly medium, poor sorting, sub angular to sub rounded, strong siliceous cement in fine grained aggregates, minor white argillaceous matrix, moderately hard aggregates, medium to coarse grains are dis-aggregated, tight to very poor visual porosity, no fluorescence. |
| 1460 | 1465 | 20 80 | CLAYSTONE: medium dark grey to dark grey, rare micro mica, moderately hard, blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, fine to coarse predominantly medium, poor sorting, sub angular to sub rounded, strong siliceous cement in fine grained aggregates, minor white argillaceous matrix, trace pyrite, moderately hard aggregates, medium to coarse grains are dis-aggregated, tight to very poor visual porosity, no fluorescence. |
| 1465 | 1470 | 40 60 | CLAYSTONE: medium dark grey to dark grey, rare micro mica, moderately hard, blocky to sub fissile. SANDSTONE: translucent, clear, very light grey, fine to coarse predominantly medium, poor sorting, sub angular to sub rounded, strong siliceous cement in fine grained aggregates, minor white argillaceous matrix, trace pyrite, moderately hard aggregates, medium to coarse grains are dis-aggregated, tight to very poor visual porosity, no fluorescence. |
| 1470 | 1475 | 100 | CLAYSTONE: very dark grey to dark grey black, silty grading to argillaceous SILTSTONE, moderately hard to hard, blocky to sub fissile. |
| 1475 | 1480 | 80 20 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. SANDSTONE: very light brown, very light brownish grey, very fine to fine grained, well sorted, sub angular to sub rounded, strong siliceous cement, minor very light brown argillaceous matrix, moderately hard to hard, tight visual porosity, no fluorescence. |
| 1480 | 1485 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1485 | 1490 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1490 | 1495 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1495 | 1500 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1500 | 1505 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1505 | 1510 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1510 | 1515 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1515 | 1520 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1520 | 1525 | 100 | CLAYSTONE: very dark grey black, silty in part grading to argillaceous SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
|  |  |  | Core 1 Cut from 1525m MDRT |
| 1525 | 1526 Spot | 100 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1526 | 1527 | 100 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1527 | 1528 Spot | 100 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1528 | 1529 Spot | 100 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1529 | 1530 | 100 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1530 | 1532 Spot | 100 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. |
| 1532 | 1533 | 90 10 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. SANDSTONE: translucent, clear, white, very fine to fine grained, trace medium, moderately sorted, sub angular, minor moderately strong siliceous cement, common white argillaceous matrix, rare quartz overgrowths, friable to moderately hard, tight visual porosity, no fluorescence. |
| 1533 | 1536 | 90 10 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. SANDSTONE: translucent, clear, white, very fine to fine grained, trace medium, moderately sorted, sub angular, minor moderately strong siliceous cement, common white argillaceous matrix, rare quartz overgrowths, friable to moderately hard, tight visual porosity, no fluorescence. |
| 1536 | 1539 | 90 10 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. SANDSTONE: translucent, clear, white, very fine to fine grained, trace medium, moderately sorted, sub angular, minor moderately strong siliceous cement, common white argillaceous matrix, rare quartz overgrowths, friable to moderately hard, tight visual porosity, no fluorescence. |
| 1539 | 1542 | 90 10 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. SANDSTONE: translucent, clear, white, very fine to fine grained, trace medium, moderately sorted, sub angular, minor moderately strong siliceous cement, common white argillaceous matrix, rare quartz overgrowths, friable to moderately hard, tight visual porosity, no fluorescence. |
| 1542 | 1545 | 80 20 | CLAYSTONE: very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub blocky occasionally sub fissile. SANDSTONE: translucent, clear, white, very fine to fine grained, moderately well sorted, sub angular, minor moderately strong siliceous cement, common white argillaceous matrix, rare quartz overgrowths, friable to moderately hard, tight visual porosity, no fluorescence. |
| 1545 | 1548 | 80 20 | CLAYSTONE: dark grey to very dark grey, very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: translucent, clear, white, very fine to fine grained, trace medium, moderately sorted, sub angular, minor moderately strong siliceous cement, common white argillaceous matrix, rare quartz overgrowths, friable to moderately hard, tight visual porosity, no fluorescence. |
| 1548 | 1551 | 90 10 | CLAYSTONE: dark grey to very dark grey, very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: translucent, clear, white, very fine to fine grained, trace medium, moderately sorted, sub angular, minor moderately strong siliceous cement, common white argillaceous matrix, rare quartz overgrowths, friable to moderately hard, tight visual porosity, no fluorescence. |
|  | 1553 Spot | 60 40 | CLAYSTONE: dark grey to very dark grey, very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: translucent, clear, medium to coarse grained, moderately sorted, sub rounded, trace siliceous cement, trace white argillaceous matrix, predominantly loose quartz grains, poor to fair inferred porosity, no fluorescence. |
|  | 1553.5 Spot | 60 40 | CLAYSTONE: dark grey to very dark grey, very dark grey black, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: translucent, clear, medium to coarse grained, moderately sorted, sub rounded, trace siliceous cement, trace white argillaceous matrix, predominantly loose quartz grains, poor to fair inferred porosity, no fluorescence. |
| 1551 | 1554 | 70 30 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: translucent, clear, white, fine to occasionally medium grained, moderately sorted, sub angular to sub rounded, moderately strong siliceous cement in fine grained aggregates, minor white argillaceous matrix, very poor to poor visual porosity, no fluorescence. |
|  | 1555 Spot | 100 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. |
| 1554 | 1557 | 90 10 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: translucent, clear, white, fine to occasionally medium grained, moderately sorted, sub angular to sub rounded, moderately strong siliceous cement in fine grained aggregates, minor white argillaceous matrix, very poor to poor visual porosity, no fluorescence. |
|  | 1558.5 Spot | 100 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. |
|  | 1559 Spot | 100 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. |
| 1557 | 1560 | 90 10 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: light grey, translucent, clear, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, common white argillaceous matrix, very poor visual porosity, no fluorescence. |
|  | 1561 Spot | 80 20 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: light grey, translucent, clear, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, common white argillaceous matrix, very poor visual porosity, no fluorescence. |
|  | 1562 Spot | 60 40 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: light grey, translucent, clear, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, common white argillaceous matrix, very poor visual porosity, no fluorescence. |
| 1560 | 1563 | 70 30 | CLAYSTONE: dark grey black, very dark grey, silty in part grading to SILTSTONE, rare micro mica, hard, blocky to sub fissile. SANDSTONE: light grey, translucent, clear, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, common white argillaceous matrix, very poor visual porosity, no fluorescence. |
|  |  |  | Core 1 1525 – 1563.1m MDRT Cut 38.1m Recovered 36.12m (95%) |
| 1563 | 1566 | 40 60 | CLAYSTONE: dark brownish black to greyish black, dark grey, silty grade to SILTSTONE in part, common micro mica, rare pyrite nodules, moderately hard to hard, sub-fissile to trace fissile, common sub blocky. SANDSTONE: light grey, translucent, clear, white, very fine to fine, well sorted, sub angular to sub rounded, moderately strong siliceous cement, common white argillaceous matrix, very poor visual porosity, no fluorescence. |
| 1566 | 1569 | 30 70 | CLAYSTONE: dark brownish black to greyish black, dark grey, silty grade to SILTSTONE in part, common micro mica, rare pyrite nods, moderately hard to hard, sub fissile to trace fissile, common sub blocky. SANDSTONE: light brown to translucent light brown, translucent to trace clear, minor off white, very fine to fine, rare medium, moderately sorted, sub angular to minor sub-rounded, trace angular, common moderately to trace strong siliceous cement, common light brown argillaceous matrix, minor black lithics, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1569 | 1572 | 30 70 | CLAYSTONE: dark brownish black to greyish black, dark grey, silty grade to SILTSTONE in part, common micro mica, rare pyrite nods, moderately hard to hard, sub fissile to trace fissile, common sub blocky. SANDSTONE: light brown to translucent light brown, translucent to trace clear, minor off white, very fine to fine, rare medium, moderately sorted, sub angular to minor sub rounded, trace angular, common moderately to trace strong siliceous cement, common light brown argillaceous matrix, minor black lithics, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1572 | 1575 | 30 70 | CLAYSTONE: dark grey to greyish black, trace dark brownish grey, common silty grade to SILTSTONE in part, common Mica flakes, minor micro mica, moderately hard to hard, sub fissile to minor sub blocky, trace fissile. SANDSTONE: light brown to translucent light brown, translucent to trace clear, minor off white, very fine to fine, rare medium, moderately sorted, sub angular to minor sub rounded, trace angular, common moderately to trace strong siliceous cement, common light brown argillaceous matrix, minor black lithics, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1575 | 1581 | 40 60 | CLAYSTONE: dark grey to greyish black, trace dark brownish grey, common silty grade to SILTSTONE in part, common mica flakes, minor micro mica, moderately hard to hard, sub-fissile to minor sub blocky, trace fissile. SANDSTONE: light brown to translucent light brown, trace off white, trace opaque, very fine to minor fine, moderately sorted, angular to sub angular, minor sub rounded, common strong siliceous cement, common light brown argillaceous matrix, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1581 | 1584 | 50 50 | CLAYSTONE: dark grey to greyish black, trace dark brownish grey, common silty grade to SILTSTONE in part, common Mica flakes, minor micro mica, moderately hard to hard, sub fissile to minor sub blocky, trace fissile. SANDSTONE: light brown to translucent light brown, trace off white, trace opaque, very fine to minor fine, moderately sorted, angular to sub angular, minor sub rounded, common strong siliceous cement, common light brown argillaceous matrix, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1584 | 1587 | 60 40 | CLAYSTONE: dark grey to greyish black, trace dark brownish grey, common silty grade to SILTSTONE in part, common mica flakes, minor micro mica, moderately hard to hard, sub fissile to minor sub blocky, trace fissile. SANDSTONE: light brown to translucent light brown, trace off white, trace opaque, very fine to minor fine, moderately sorted, angular to sub angular, minor sub-rounded, common strong siliceous cement, common light brown argillaceous matrix, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1587 | 1590 | 70 30 | CLAYSTONE: very dark grey to greyish black, trace dark brownish grey, silty grading to SILSTONE, trace mica flakes, common micro mica, moderately hard to hard, trace firm, sub blocky to common sub fissile, trace fissile. SANDSTONE: light brown to translucent light brown, trace off white, trace opaque, very fine to minor fine, moderately sorted, angular to sub angular, minor sub rounded, common strong siliceous cement, common light brown argillaceous matrix, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1590 | 1593 | 100 | CLAYSTONE: very dark grey to greyish black, trace dark brownish grey, silty grading to SILSTONE, trace mica flakes, common micro mica, moderately hard to hard, trace firm, sub blocky to common sub fissile, trace fissile. |
| 1593 | 1596 | 90 10 | CLAYSTONE: very dark grey to greyish black, trace dark brownish grey, silty grading to SILSTONE, trace mica flakes, common micro mica, moderately hard to hard, trace firm, sub blocky to common sub fissile, trace fissile. SANDSTONE: light brown to translucent light brown, trace off white, trace opaque, very fine to minor fine, moderately sorted, angular to sub angular, minor sub rounded, common strong siliceous cement, common light brown argillaceous matrix, moderately hard to hard, minor disaggregated grains, tight to very poor visual porosity, no fluorescence. |
| 1596 | 1599 | 100 | CLAYSTONE: very dark grey, greyish black, silty grading to SILTSTONE, trace micro mica, moderately hard to hard, sub blocky to sub fissile. |
| 1599 | 1602 | 90 10 | CLAYSTONE: very dark grey to greyish black, trace dark brownish grey, silty grading to SILSTONE, trace mica flakes, common micro mica, moderately hard to hard, trace firm, sub blocky to common sub fissile, trace fissile. SANDSTONE: off white to white, very light brown, translucent, very fine to minor fine, moderately sorted, angular to sub angular, minor sub rounded, strong siliceous cement, common off white argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1602 | 1605 | 90 10 | CLAYSTONE: dark grey to very dark grey, dark grey black, very finely arenaceous, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, off white to white, translucent, very fine to fine grained, moderately well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common off white to very light grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1605 | 1608 | 80 20 | CLAYSTONE: dark grey to very dark grey, dark grey black, very finely arenaceous, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, off white to white, translucent, very fine to fine grained, moderately well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common off white to very light grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1608 | 1611 | 90 10 | CLAYSTONE: dark grey to very dark grey, dark grey black, very finely arenaceous, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, off white to white, translucent, very fine to fine grained, moderately well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common off white to very light grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1611 | 1614 | 90 10 | CLAYSTONE: dark grey to very dark grey, dark grey black, very finely arenaceous, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, off white to white, translucent, very fine to fine grained, moderately well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common off white to very light grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1614 | 1617 | 80 20 | CLAYSTONE: dark grey to very dark grey, dark grey black, silty in part, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, occasionally off white to white, light brownish grey, very fine to occasionally fine grained, moderately well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common off white to very light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1617 | 1620 | 80 20 | CLAYSTONE: dark grey to very dark grey, dark grey black, silty in part, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, occasionally off white to white, light brownish grey, very fine to occasionally fine grained, moderately well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common off white to very light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1620 | 1623 | 80 20 | CLAYSTONE: dark grey to very dark grey, dark grey black, silty in part, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, occasionally off white to white, light brownish grey, very fine to occasionally fine grained, moderately well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common off white to very light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1623 | 1626 | 70 30 | CLAYSTONE: very dark grey to dark grey black, very finely arenaceous in part, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, light brownish grey, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1626 | 1629 | 70 30 | CLAYSTONE: very dark grey to dark grey black, very finely arenaceous in part, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, light brownish grey, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1629 | 1632 | 80 20 | CLAYSTONE: very dark grey to dark grey black, very finely arenaceous in part, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, light brownish grey, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1632 | 1635 | 90 10 | CLAYSTONE: very dark grey to dark grey black, very finely arenaceous in part, rare micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light grey, light brownish grey, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1635 | 1638 | 100 | CLAYSTONE: very dark grey to dark grey black, very finely arenaceous, minor micro mica, moderately hard to hard, blocky to sub fissile. |
| 1638 | 1641 | 90 10 | CLAYSTONE: dark grey black, silty, minor micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1641 | 1644 | 100 | CLAYSTONE: dark grey black, silty, minor micro mica, moderately hard to hard, blocky to sub fissile. |
| 1644 | 1647 | 90 10 | CLAYSTONE: dark grey black, silty, minor micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1647 | 1650 | 100 | CLAYSTONE: dark grey black, silty, minor micro mica, moderately hard to hard, blocky to sub fissile. |
| 1650 | 1653 | 90 10 | CLAYSTONE: dark grey black, silty, minor micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1653 | 1656 | 100 | CLAYSTONE: dark grey black, silty, minor micro mica, moderately hard to hard, blocky to sub fissile. |
| 1656 | 1659 | 90 10 | CLAYSTONE: dark grey black, silty, common micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1659 | 1662 | 90 10 | CLAYSTONE: dark grey black, silty, common micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1662 | 1665 | 90 10 | CLAYSTONE: dark grey black, silty, common micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1665 | 1668 | 90 10 | CLAYSTONE: dark grey black, silty, common micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1668 | 1671 | 90 10 | CLAYSTONE: dark grey black, silty, common micro mica, moderately hard to hard, blocky to sub fissile. SANDSTONE: very light brownish grey, very light grey, off white, very fine to fine grained, moderately well sorted, sub angular to sub rounded, strong siliceous cement, common off white to light brownish grey argillaceous matrix, moderately hard to hard, tight to very poor visual porosity, no fluorescence. |
| 1671 | 1674 | 100 | CLAYSTONE: black, very dark grey black, silty in part, common micro mica, hard, blocky to sub blocky, occasionally sub fissile. |
| 1674 | 1677 | 100 | CLAYSTONE: black, very dark grey black, silty in part, common micro mica, hard, blocky to sub blocky, occasionally sub fissile. |
| 1677 | 1680 | 100 | CLAYSTONE: black, very dark grey black, silty in part, common micro mica, hard, blocky to sub blocky, occasionally sub fissile. |
| 1680 | 1683 | 100 | CLAYSTONE: black, very dark grey black, silty in part, common micro mica, hard, blocky to sub blocky, occasionally sub fissile. |
| 1683 | 1686 | 100 | CLAYSTONE: black, very dark grey black, silty in part, common micro mica, hard, blocky to sub blocky, occasionally sub fissile. |
| 1686 | 1689 | 100 | CLAYSTONE: black, very dark grey, silty in part, minor micro mica, hard, blocky to sub blocky, occasionally sub fissile. Trace 5mm tabular cuttings/cavings . |
| 1689 | 1692 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1692 | 1695 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1695 | 1698 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1698 | 1701 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1701 | 1704 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1704 | 1707 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1707 | 1710 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1710 | 1713 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1713 | 1716 | 100 | CLAYSTONE: very dark grey to very dark grey black, silty in part, common micro mica, trace nodular pyrite, hard, blocky to sub blocky, occasionally sub fissile. |
| 1716 | 1719 | 100 | CLAYSTONE: dark grey to grey black, trace very dark grey brown, trace medium light grey, silty in parts, grading to SILTSTONE, common micro mica, moderately hard to hard, sub blocky to occasionally sub fissile. |
| 1719 | 1722 | 100 trace | CLAYSTONE: dark grey to grey black, trace very dark grey brown, trace medium light grey, silty in parts, grading to SILTSTONE, common micro mica, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: light grey to translucent, opaque in parts, very fine to trace fine, moderately well sorted, sub angular to trace angular, strong siliceous cement, light grey argillaceous matrix, hard to moderately hard, nil to tight visual porosity, no fluorescence. |
| 1722 | 1725 | 90 10 | CLAYSTONE: dark grey to grey black, trace very dark grey brown, trace medium light grey, silty in parts, grading to SILTSTONE, common micro mica, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: light grey to medium light grey, translucent to trace opaque, very fine, trace fine, moderately well sorted, sub angular to trace sub rounded, common strong siliceous cement, common light argillaceous matrix, common black lithics, hard to trace very hard, nil to tight visual porosity, no fluorescence. |
| 1725 | 1728 | 90 10 | CLAYSTONE: dark grey to grey black, trace very dark grey brown, trace medium light grey, silty in parts, grading to SILTSTONE, common micro mica, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: light grey to medium light grey, translucent to trace opaque, very fine, trace fine, moderately well sorted, sub angular to trace sub rounded, common strong siliceous cement, common light argillaceous matrix, common black lithics, hard to trace very hard, nil to tight visual porosity, no fluorescence. |
| 1728 | 1731 | 80 20 | CLAYSTONE: dark grey to grey black, brownish black, silty grading to SILTSTONE in parts, common micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: light grey to medium light grey, translucent to trace opaque, very fine, trace fine, moderately well sorted, sub angular to trace sub rounded, common strong siliceous cement, common light argillaceous matrix, common black lithics, hard to trace very hard, nil to tight visual porosity, no fluorescence. |
| 1731 | 1734 | 70 30 | CLAYSTONE: dark grey to grey black, brownish black, silty grading to SILTSTONE in parts, common micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: light brown to light grey, off white in parts, minor opaque, trace clear, translucent in parts, very fine to trace fine, moderately well sorted, angular to sub angular, common strong siliceous cement, common off white to light grey argillaceous matrix, moderately hard to hard, trace disaggregated grains, nil to tight visual porosity, no fluorescence. |
| 1734 | 1737 | 80 20 | CLAYSTONE: dark grey to grey black, brownish black, silty grading to SILTSTONE in parts, common micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: light brown to light grey, off white in parts, minor opaque, trace clear, translucent in parts, very fine to trace fine, moderately well sorted, angular to sub angular, common strong siliceous cement, common off white to light grey argillaceous matrix, moderately hard to hard, trace disaggregated grains, nil to tight visual porosity, no fluorescence. |
| 1737 | 1740 | 70 30 | CLAYSTONE: dark grey to grey black, brownish black, silty grading to SILTSTONE in parts, common micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: light brown to light grey, off white in parts, minor opaque, trace clear, translucent in parts, very fine to trace fine, moderately well sorted, angular to sub angular, common strong siliceous cement, common off white to light grey argillaceous matrix, moderately hard to hard, trace disaggregated grains, nil to tight visual porosity, no fluorescence. |
| 1740 | 1743 | 70 30 | CLAYSTONE: dark grey to grey black, brownish black, silty grading to SILTSTONE in parts, common micro mica, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: light brown to light grey, off white in parts, minor opaque, trace clear, translucent in parts, very fine to trace fine, moderately well sorted, angular to sub angular, common strong siliceous cement, common off white to light grey argillaceous matrix, moderately hard to hard, trace disaggregated grains, nil to tight visual porosity, no fluorescence. |
| 1743 | 1746 | 60 40 | CLAYSTONE: dark grey, brownish grey, medium dark grey in parts, silty in parts, com micro mica, rare nodular pyrite, moderately hard to hard, sub fissile to minor sub blocky. SANDSTONE: light brown to light grey, off white in parts, minor opaque, trace clear, translucent in parts, very fine to trace fine, moderately well sorted, angular to sub angular, common strong siliceous cement, common off white to light grey argillaceous matrix, moderately hard to hard, trace disaggregated grains, nil to tight visual porosity, no fluorescence. |
| 1746 | 1749 | 70 30 | CLAYSTONE: dark grey, brownish grey, medium dark grey in parts, silty in parts, com micro mica, moderately hard to hard, sub fissile to minor sub blocky. SANDSTONE: light brown to light grey, off white in parts, minor opaque, trace clear, translucent in parts, very fine to trace fine, moderately well sorted, angular to sub angular, common strong siliceous cement, common off white to light grey argillaceous matrix, moderately hard to hard, trace disaggregated grains, nil to tight visual porosity, no fluorescence. |
| 1749 | 1752 | 60 40 | CLAYSTONE: dark grey, brownish grey, medium dark grey in parts, silty in parts, com micro mica, moderately hard to hard, sub fissile to minor sub blocky. SANDSTONE: light grey, off white, translucent, trace medium light grey, very fine to trace fine, moderately well sorted, sub angular to angular, minor sub rounded, common strong siliceous cement, common off white to light brown argillaceous matrix, moderately hard to hard, nil to trace visual porosity, no fluorescence. |
| 1752 | 1755 | 60 40 | CLAYSTONE: dark grey, brownish grey, medium dark grey in parts, silty in parts, com micro mica, moderately hard to hard, sub fissile to minor sub blocky. SANDSTONE: light grey, off white, translucent, trace medium light grey, very fine to trace fine, moderately well sorted, sub angular to angular, minor sub rounded, common strong siliceous cement, common off white to light brown argillaceous matrix, moderately hard to hard, nil to trace visual porosity, no fluorescence. |
| 1755 | 1758 | 70 30 | CLAYSTONE: dark grey, brownish grey, medium dark grey in parts, silty in parts, com micro mica, moderately hard to hard, sub fissile to minor sub blocky. SANDSTONE: light grey, off white, translucent, trace medium light grey, very fine to trace fine, moderately well sorted, sub angular to angular, minor sub rounded, common strong siliceous cement, common off white to light brown argillaceous matrix, moderately hard to hard, nil to trace visual porosity, no fluorescence. |
| 1758 | 1761 | 70 30 | CLAYSTONE: dark grey to grey black, minor brownish grey, black, silty in parts, common micro mica, moderately hard to trace hard, sub blocky to sub fissile. SANDSTONE: light grey, off white, translucent, trace medium light grey, very fine to trace fine, moderately well sorted, sub angular to angular, minor sub rounded, common strong siliceous cement, common off white to light brown argillaceous matrix, moderately hard to hard, nil to trace visual porosity, no fluorescence. |
| 1761 | 1764 | 60 40 | CLAYSTONE: dark grey to grey black, minor brownish grey, black, silty in parts, common micro mica, moderately hard to trace hard, sub blocky to sub fissile. SANDSTONE: light grey, off white, translucent, trace medium light grey, very fine to trace fine, moderately well sorted, sub angular to angular, minor sub rounded, common strong siliceous cement, common off white to light brown argillaceous matrix, moderately hard to hard, nil to trace visual porosity, no fluorescence. |
| 1764 | 1767 | 40 60 | CLAYSTONE: dark grey to grey black, minor brownish grey, black, silty in parts, common micro mica, moderately hard to trace hard, sub blocky to sub fissile. SANDSTONE: light grey, off white, translucent, trace medium light grey, very fine to trace fine, moderately well sorted, sub angular to angular, minor sub rounded, common strong siliceous cement, common off white to light brown argillaceous matrix, moderately hard to hard, nil to trace visual porosity, no fluorescence. |
| 1767 | 1770 | 40 60 | CLAYSTONE: dark grey to grey black, minor brownish grey, black, silty in parts, common micro mica, moderately hard to trace hard, sub blocky to sub fissile. SANDSTONE: light grey, off white, translucent, trace medium light grey, very fine to trace fine, moderately well sorted, sub angular to angular, minor sub rounded, common strong siliceous cement, common off white to light brown argillaceous matrix, moderately hard to hard, nil to trace visual porosity, no fluorescence. |
| 1770 | 1773 | 100 | SANDSTONE: translucent, clear, white, very light grey, common pink orange stain, fine to medium, trace coarse, poorly sorted, angular to sub angular, strong siliceous cement, common white argillaceous matrix, hard aggregates, common loose disaggregated grains, tight inferred porosity, no fluorescence. |
| 1773 | 1776 | 100 | SANDSTONE: translucent, clear, trace pink stain, white, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1776 | 1779 | 100 | SANDSTONE: translucent, clear, trace pink stain, white, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1779 | 1782 | 100 | SANDSTONE: translucent, clear, trace pink stain, white, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1782 | 1785 | 100 | SANDSTONE: translucent, clear, white, trace pink stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1785 | 1788 | 100 | SANDSTONE: translucent, clear, white, trace pink stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1788 | 1791 | 10 100 | SILTSTONE: moderate reddish orange, moderate reddish brown, pinkish brown, very find arenaceous, argillaceous in part grading to CLAYSTONE, firm, dispersive in part, blocky. SANDSTONE: translucent, clear, white, trace pinkish brown stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1791 | 1794 | 100 | SANDSTONE: translucent, clear, white, common pinkish brown stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1794 | 1797 | 100 | SANDSTONE: translucent, clear, white, common pinkish brown stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1797 | 1800 | 10 90 | SILTSTONE: moderate reddish orange, moderate reddish brown, very find arenaceous, argillaceous in part grading to CLAYSTONE, firm, dispersive in part, blocky. SANDSTONE: translucent, clear, white, common pinkish brown stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1800 | 1803 | 100 | SANDSTONE: translucent, clear, white, common pinkish brown stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1803 | 1806 | 100 | SANDSTONE: translucent, clear, white, common pinkish brown stain, fine to coarse, poorly sorted, angular shards to sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1806 | 1809 | 100 | SANDSTONE: translucent, clear, white, pale yellow in part, fine to coarse, occasional very coarse, poorly sorted, angular to sub angular, minor strong siliceous cement, minor white argillaceous matrix, minor moderately hard to hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1809 | 1812 | 100 | SANDSTONE: translucent, clear, white, trace pale yellow, fine to coarse, occasional very coarse, trace very fine grains, poorly to very poorly sorted, angular to sub angular, trace sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard to moderately hard aggregates, dominantly disaggregated quartz grains, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1812 | 1815 | 100 | SANDSTONE: clear, translucent, white, rare pale yellow, fine to coarse, minor very coarse, poorly sorted, angular to sub angular, minor strong siliceous cement, minor white argillaceous matrix, minor moderately hard to hard aggregates, trace very hard, dominantly disaggregated quartz grains, tight visual to very poor inferred porosity, no fluorescence. |
| 1815 | 1818 | 100 | SANDSTONE: translucent, clear, white, pale yellow in part, fine to coarse, occasional very coarse, poorly sorted, angular to sub angular, minor strong siliceous cement, minor white argillaceous matrix, hard to moderately hard aggregates, dominantly disaggregated quartz grains, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1818 | 1821 | 100 | SANDSTONE: translucent, clear, white, pale yellow in part, fine to coarse, occasional very coarse, poorly sorted, angular to sub angular, minor strong siliceous cement, minor white argillaceous matrix, hard to moderately hard aggregates, dominantly disaggregated quartz grains, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 1821 | 1824 | 90 10 | SANDSTONE: translucent, clear, white, pale yellow in part, fine to coarse, occasional very coarse, poorly sorted, angular to sub angular, minor strong siliceous cement, minor white argillaceous matrix, hard to moderately hard aggregates, dominantly disaggregated quartz grains, tight visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: moderately reddish brown to moderately red, medium grey to medium dark grey, trace dark grey, trace dark red brown, argillaceous grading to silty CLAYSTONE in part, minor very fine arenaceous, firm to trace soft, sub blocky to minor sub fissile. |
| 1824 | 1827 | 90 10 | SANDSTONE: clear to translucent, opaque, white, very fine to coarse, occasionally very coarse, poorly to very poorly sorted, sub angular to angular, common strong siliceous cement, occasional white argillaceous matrix, hard, common disaggregated quartz grains, very poor inferred porosity, no fluorescence. SILTSTONE: moderately reddish brown to moderately red, medium grey to medium dark grey, trace dark grey, trace dark red brown, argillaceous grading to silty CLAYSTONE in part, minor very fine arenaceous, firm to trace soft, sub blocky to minor sub fissile. |
| 1827 | 1830 | 100 | SANDSTONE: translucent, clear, white, very fine to predominantly medium to coarse, poor sorting, angular shards to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, minor quartz overgrowths, hard, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1830 | 1833 | 100 | SANDSTONE: translucent, clear, white, very fine to predominantly medium to coarse, poor sorting, angular shards to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, minor quartz overgrowths, hard, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1833 | 1836 | 100 | SANDSTONE: translucent, clear, white, very fine to predominantly medium to coarse, poor sorting, angular shards to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, minor quartz overgrowths, hard, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1836 | 1839 | 100 | SANDSTONE: translucent, clear, white, very fine to predominantly medium to coarse, poor sorting, angular shards to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, minor quartz overgrowths, hard, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1839 | 1842 | 10 90 | SILTSTONE: moderate reddish orange, moderate reddish brown, very find arenaceous, argillaceous in part grading to CLAYSTONE, firm, dispersive in part, blocky. SANDSTONE: translucent, clear, white, common orange pink stain, very fine to predominantly medium, trace coarse, fair sorting, angular to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1842 | 1845 | 100 | SANDSTONE: translucent, clear, white, common orange pink stain, very fine to predominantly medium, trace coarse, fair sorting, angular to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1845 | 1848 | 100 | SANDSTONE: translucent, clear, white, common orange pink stain, very fine to predominantly medium, trace coarse, fair sorting, angular to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1848 | 1851 | 100 | SANDSTONE: translucent, clear, white, common orange pink stain, very fine to predominantly medium, trace coarse, fair sorting, angular to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1851 | 1854 | 100 | SANDSTONE: translucent, clear, white, common orange pink stain, very fine to predominantly medium, trace coarse, fair sorting, angular to sub angular, occasionally sub rounded, strong siliceous cement, minor white argillaceous matrix, hard aggregates, common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1854 | 1857 | 100 | SANDSTONE: translucent, clear, white, trace red pink stain, fine to coarse, predominately medium, minor very fine, rare very course, poor sorting, angular to sub angular, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1857 | 1860 | 100 | SANDSTONE: translucent, clear, white, trace red pink stain, fine to coarse, minor very fine, trace very course, poor sorting, angular to sub angular, trace sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1860 | 1863 | 100 | SANDSTONE: translucent, clear, white, rare red pink stain, fine to coarse, minor very fine, predominately medium grains, poor to moderately sorted, angular to sub angular, trace sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1863 | 1866 | 20 80 | SILTSTONE: reddish brown to moderate red, moderately red orange, trace dark brownish grey, very fine arenaceous, argillaceous grading to CLAYSTONE in parts, firm to trace moderately hard, sub blocky to minor sub fissile. SANDSTONE: translucent, clear, white, rare red pink stain, fine to coarse, minor very fine, predominately medium grains, poor to moderately sorted, angular to sub angular, trace sub rounded, minor strong siliceous cement, minor white argillaceous matrix, hard aggregates common disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1866 | 1869 | 30 70 | SILTSTONE: reddish brown to moderate red, moderately red orange to orange brown, trace dark brownish grey, minor very fine arenaceous, commonly argillaceous grading to silty CLAYSTONE in parts, firm to trace moderately hard, commonly soft, sub blocky to minor sub fissile. SANDSTONE: translucent, clear, off white to very light grey, rare red pink stain, fine to coarse, minor very fine, predominately medium grains, poor to moderately sorted, angular to sub angular, trace sub rounded, minor strong siliceous cement, minor off white argillaceous matrix, hard aggregates in part, predominantly disaggregated quartz grains, tight to very poor inferred porosity, no fluorescence. |
| 1869 | 1872 | 15 85 | SILTSTONE: reddish brown to moderate red, moderately red orange to orange brown, trace dark brownish grey, minor very fine arenaceous, commonly argillaceous grading to silty CLAYSTONE in parts, firm to trace moderately hard, sub blocky to minor sub fissile. SANDSTONE: translucent to clear, opaque in part, off white to very light grey, minor medium red orange stain, fine to coarse, dominantly medium, poorly sorted, sub angular to minor angular, sub rounded in part, minor strong siliceous cement, rare off white to very light grey argillaceous matrix, moderately hard to had aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1872 | 1875 | 100 | SANDSTONE: translucent to clear, opaque in part, off white to very light grey in part, minor medium red brown stain, fine to coarse, dominantly medium grained, poor sort, sub angular to sub rounded, common angular grains and shards, minor strong siliceous cement, trace off white to medium red brown argillaceous matrix, moderately hard to occasionally hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1875 | 1878 | 100 | SANDSTONE: clear to translucent, off white, minor medium red brown Fe staining, fine to medium, minor coarse, moderately sort, angular to sub angular, strong to moderate siliceous cement, trace medium red brown argillaceous matrix, moderately hard to occasionally hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1878 | 1881 | 100 | SANDSTONE: clear to translucent, off white, rare medium red brown Fe staining, fine to medium, moderately well sort, angular to sub angular, strong to moderate siliceous cement, trace medium red brown argillaceous matrix, occasional to minor rock flour, moderately hard to occasionally hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1881 | 1884 | 100 | SANDSTONE: clear to translucent, off white, rare medium red brown Fe staining, fine to medium, moderately well sort, angular to sub angular, strong to moderate siliceous cement, trace medium red brown argillaceous matrix, occasional to minor rock flour, moderately hard to occasionally hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1884 | 1887 | 95 5 | SANDSTONE: clear to translucent, commonly off white to very light grey, rare medium red brown Fe staining, fine to medium, moderately well sorted, angular to sub angular, strong to moderate siliceous cement, trace medium red brown argillaceous matrix, occasional to minor rock flour, moderately hard to occasionally hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. SILTSTONE: reddish brown to dark reddish brown, moderately red orange to orange brown, trace dark brownish grey, commonly very finely arenaceous, minor argillaceous, moderately hard, sub blocky to minor sub fissile. |
| 1887 | 1890 | 70 30 | SANDSTONE: clear to translucent, commonly off white to very light grey, rare medium red brown Fe staining, rare pale yellowish grey, fine to medium, moderately well sorted, angular to sub angular, strong to moderate siliceous cement, minor off white to medium red brown argillaceous matrix, rare friable to dominantly moderately hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. SILTSTONE: reddish brown to dark reddish brown, moderately red orange to orange brown, trace dark brownish grey, commonly very finely arenaceous, argillaceous grading to silty CLAYSTONE in part, moderately hard, minor soft to firm, sub blocky to minor sub fissile. |
| 1890 | 1893 | 40 60 | SANDSTONE: clear to translucent, commonly off white, rare medium red brown Fe staining, fine to medium, moderately well sorted, angular to sub angular, strong to moderate siliceous cement, minor medium red brown argillaceous matrix, rare moderately hard aggregates, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. SILTSTONE: reddish brown to dark reddish brown, moderately red orange to orange brown, trace dark brownish grey, arenaceous to argillaceous, grading to silty CLAYSTONE in part, minor lithics, moderately hard, minor soft to firm, sub blocky to minor sub fissile. |
| 1893 | 1896 | 100 | SANDSTONE: clear to translucent, commonly off white, fine to occasionally medium, well sorted, angular to sub angular, strong to moderate siliceous cement, minor off white argillaceous matrix, common rock flour, rare moderately hard, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. |
| 1896 | 1899 | 90 10 | SANDSTONE: clear to translucent, commonly off white, fine to medium, minor coarse, moderately sorted, angular to sub angular, strong to moderate siliceous cement, minor off white argillaceous matrix, common rock flour, rare moderately hard, dominantly disaggregated quartz grains, very poor inferred porosity, no fluorescence. SILTSTONE: reddish brown to dark reddish brown, moderately red orange to orange brown, minor dark grey, arenaceous to argillaceous, grading to silty CLAYSTONE in part, minor lithics, moderately hard, minor soft to firm, sub blocky to minor sub fissile. |
| 1899 | 1902 | 100 | SANDSTONE: clear to translucent, minor red brown, occasionally off white, medium to coarse, minor fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor red brown silty matrix, disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1902 | 1905 | 80 20 | SANDSTONE: clear to translucent, minor red brown, occasionally off white, medium to coarse, minor fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor red brown silty matrix, disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. SILTSTONE: medium to occasionally dark red brown, occasional to minor argillaceous, minor dark lithics, firm to occasionally moderately hard, dispersive to soft in part, sub blocky. |
| 1905 | 1908 | 80 20 | SANDSTONE: clear to translucent, occasionally off white, minor red brown, medium to coarse, minor fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor red brown silty matrix, disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. SILTSTONE: medium to occasionally dark red brown, medium red orange, occasional to minor argillaceous, minor dark lithics, firm to occasionally moderately hard, dispersive to soft in part, sub blocky. |
| 1908 | 1911 | 90 10 | SANDSTONE: clear to translucent, occasionally off white, minor red brown, light to medium grey in part, medium to coarse, minor fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor red brown silty matrix, occasional localised dark lithics, disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. SILTSTONE: medium to occasionally dark red brown, medium red orange, minor light grey, locally siliceous, occasional to minor argillaceous, minor dark lithics, firm to occasionally moderately hard, dispersive to soft in part, sub blocky. |
| 1911 | 1914 | 100 | SANDSTONE: clear to translucent, occasionally off white, trace red brown FE staining, light to medium grey in part, medium to coarse, minor fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor red brown silty matrix, occasional localised dark lithics, disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1914 | 1917 | 70 30 | SANDSTONE: clear to translucent, occasionally off white, minor red brown, light to medium grey in part, medium to coarse, minor fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor red brown silty matrix, occasional localised dark lithics, disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. SILTSTONE: 1) light red purple, arenaceous, occasional lithics, moderately hard to hard, sub blocky. 2) medium to occasionally dark red brown, medium red orange, locally siliceous, occasional to minor argillaceous, minor dark lithics, firm to occasionally moderately hard, dispersive to soft in part, sub blocky. |
| 1917 | 1920 | 100 | SANDSTONE: clear to translucent, occasionally off white, fine to medium, moderately well sorted, sub angular to occasionally sub rounded, strong siliceous cement, minor red brown silty matrix, occasional localised dark lithics, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1920 | 1923 | 100 | SANDSTONE: off white, clear to translucent, rare light grey, fine to medium, moderately well sorted, sub angular to occasionally sub rounded, strong siliceous cement, minor red brown silty matrix, occasional localised dark lithics, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1923 | 1926 | 100 | SANDSTONE: clear to translucent, occasionally off white, fine to medium, minor coarse, moderately well sorted, sub angular to sub rounded, strong siliceous cement, minor red brown silty matrix, occasional localised dark lithics, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1926 | 1929 | 90 10 | SANDSTONE: clear to translucent, occasionally off white, medium to coarse, moderately well sorted, angular to sub angular, strong siliceous cement, minor red brown silty matrix, occasional localised dark lithics, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. SILTSTONE: medium to occasionally dark red brown, medium red orange, red purple in part, locally siliceous, occasional to minor argillaceous, minor dark lithics, firm to occasionally moderately hard, dispersive to soft in part, sub blocky. |
| 1929 | 1932 | 80 20 | SANDSTONE: clear to translucent, minor off white, fine to coarse, moderately well sorted, angular to sub angular, strong siliceous cement, rare off white argillaceous matrix, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. SILTSTONE: medium to occasionally dark red brown, medium red orange, locally siliceous, occasional to minor argillaceous, minor dark lithics, firm to occasionally moderately hard, dispersive in part, sub blocky. |
| 1932 | 1935 | 100 Trace | SANDSTONE: clear to translucent, minor off white, medium to coarse, occasionally fine, moderately well sorted, sub angular to sub rounded, strong siliceous cement, rare off white argillaceous matrix, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. SILTSTONE: medium to occasionally dark red brown, medium red orange, locally siliceous, occasional to minor argillaceous, minor dark lithics, firm to occasionally moderately hard, dispersive in part, sub blocky. *Very dark grey siltstone cavings observed – Kyalla. |
| 1935 | 1938 | 100 | SANDSTONE: clear to translucent, rare off white, fine to medium, well sorted, angular to occasionally sub rounded, strong siliceous cement, rare off white argillaceous matrix, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1938 | 1941 | 100 | SANDSTONE: clear to translucent, rare off white, fine to medium, well sorted, angular to occasionally sub rounded, strong siliceous cement, rare off white argillaceous matrix, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1941 | 1944 | 100 | SANDSTONE: clear to translucent, occasionally off white to very light grey, rare medium to dark red brown staining, fine to medium, occasionally coarse, moderately well sorted, angular to occasionally sub rounded, strong siliceous cement, rare off white to very light grey argillaceous matrix, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor inferred porosity, no fluorescence. |
| 1944 | 1947 | 90 10 | SANDSTONE: clear to translucent, occasionally off white to very light grey, rare medium to dark red brown staining, fine to medium, occasionally coarse, moderately well sorted, angular to occasionally sub rounded, strong siliceous cement, rare off white to very light grey argillaceous matrix, loose clean disaggregated quartz grains, hard to moderately hard in part, very poor to poor inferred porosity, no fluorescence. SILTSTONE: medium to dark red brown, medium to dark red orange, rare medium grey brown, siliceous in part, minor argillaceous, occasional dark lithics, firm to moderately hard, rare dispersive, sub blocky. |
| 1947 | 1950 | 70 30 | SANDSTONE: clear to translucent, occasionally off white to very light grey, common medium to dark red brown staining, fine to medium, occasionally coarse, moderately well sorted, angular to occasionally sub rounded, strong siliceous cement, rare off white to very light grey argillaceous matrix, loose disaggregated quartz grains, hard to moderately hard in part, very poor to poor inferred porosity, no fluorescence. SILTSTONE: medium to dark red brown, medium to dark red orange, rare medium grey brown, siliceous in part, minor argillaceous, occasional dark lithics, firm to moderately hard, rare dispersive, sub blocky. |
| 1950 | 1953 | 100 Trace | SANDSTONE: clear to translucent, common off white, rare medium to dark red brown staining, fine to medium, occasionally coarse, well sorted, angular to sub rounded, strong siliceous cement, minor off white to very light grey argillaceous matrix, dominantly disaggregated quartz grains, hard to moderately hard in part, very poor to poor inferred porosity, no fluorescence. SILTSTONE: medium to dark red brown, medium to dark red orange, rare medium grey brown, siliceous in part, minor argillaceous, occasional dark lithics, firm to moderately hard, rare dispersive, sub blocky. |
| 1953 | 1956 | 95 5 | SANDSTONE: clear to translucent, common off white, rare medium to dark red brown staining, fine to medium, occasionally coarse, well sorted, angular to sub rounded, strong siliceous cement, minor off white to very light grey argillaceous matrix, trace to locally common dark lithics, dominantly disaggregated quartz grains, moderately hard to hard in part, minor friable, very poor inferred porosity, no fluorescence. SILTSTONE: medium to dark red brown, medium to dark red orange, rare medium grey brown, minor argillaceous, occasional dark lithics, firm to moderately hard, commonly soft to dispersive, sub blocky. |
| 1956 | 1959 | 60 40 | SANDSTONE: clear to translucent, common off white to very light grey, minor pale yellowish grey, pale pinkish red in part, fine to medium, occasionally coarse, moderately sorted, sub angular to sub rounded, common strong siliceous cement, minor off white to very light grey argillaceous to occasionally silty matrix, trace dark lithics, moderately hard to hard, disaggregated quartz grains, poor visual porosity, no fluorescence. SILTSTONE: 1) medium to occasionally dark red brown, medium red orange, locally siliceous, argillaceous in part, minor dark lithics, moderately hard, soft to dispersive in part, sub blocky to minor sub fissile. 2) light red purple, arenaceous, minor argillaceous, occasional lithics, moderately hard to hard, sub blocky to rare sub fissile. |
| 1959 | 1962 | 70 30 | SANDSTONE: clear to translucent, occasionally off white, minor red brown staining in part, fine to medium, moderately well sorted, sub angular to minor sub rounded, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, trace dark lithics, moderately hard to hard, disaggregated quartz grains, poor visual and inferred porosity, no fluorescence. SILTSTONE: medium to minor dark red brown, medium red orange, argillaceous in part, minor dark lithics, moderately hard, soft to dispersive in part, sub blocky to minor sub fissile. |
| 1962 | 1965 | 100 Trace | SANDSTONE: clear to translucent, minor medium red brown staining in part, fine to medium, moderately well sorted, sub angular to minor sub rounded, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, trace dark lithics, moderately hard to hard, disaggregated quartz grains, poor visual and inferred porosity, no fluorescence. SILTSTONE: medium to minor dark red brown, medium red orange, argillaceous in part, minor dark lithics, moderately hard, soft to dispersive in part, sub blocky to minor sub fissile. |
| 1965 | 1968 | 100 Trace | SANDSTONE: clear to translucent, minor medium red brown staining in part, fine to medium, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, trace dark lithics, moderately hard to hard, disaggregated quartz grains, poor visual and inferred porosity, no fluorescence. SILTSTONE: medium to minor dark red brown, medium red orange, argillaceous in part, minor dark lithics, moderately hard, soft to dispersive in part, sub blocky to minor sub fissile. |
| 1968 | 1971 | 50 50 | SANDSTONE: clear to translucent, minor off white, fine to medium, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, trace dark lithics, moderately hard to hard, disaggregated quartz grains, poor visual and inferred porosity, no fluorescence. SILTSTONE: medium to dark grey, medium grey brown, minor red brown, arenaceous in part, minor micro mica, trace pyrite, moderately hard to occasionally hard, sub blocky. |
| 1971 | 1974 | 70 30 | SANDSTONE: clear to translucent, light pink, minor off white, medium to occasionally coarse, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, trace dark lithics, hard to occasionally very hard, rare disaggregated quartz grains, poor visual and inferred porosity, no fluorescence. SILTSTONE: 1) dark grey, medium grey brown, arenaceous in part, occasionally siliceous, minor micro mica, trace pyrite, moderately hard to occasionally hard, sub blocky. 2) medium red brown, minor argillaceous, rare pyrite nodules, moderately hard to occasionally hard, sub blocky. |
| 1974 | 1977 | 100 Trace | SANDSTONE: clear to translucent, minor off white, medium to occasionally coarse, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, trace dark lithics, dominantly disaggregated quartz grains, hard to occasionally very hard in part, poor visual and inferred porosity, no fluorescence. SILTSTONE: 1) dark grey, medium grey brown, arenaceous in part, occasionally siliceous, minor micro mica, trace pyrite, moderately hard to occasionally hard, sub blocky. 2) medium red brown, minor argillaceous, rare pyrite nodules, moderately hard to occasionally hard, sub blocky. |
| 1977 | 1980 | 100 Trace | SANDSTONE: clear to translucent, minor off white, fine to dominantly medium, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, trace dark lithics, dominantly disaggregated quartz grains, hard to occasionally very hard in part, poor visual and inferred porosity, no fluorescence. SILTSTONE: 1) medium grey, medium grey brown, arenaceous in part, occasionally siliceous, minor micro mica, trace pyrite, moderately hard to occasionally hard, sub blocky. 2) medium red brown, minor argillaceous, rare pyrite nodules, moderately hard to occasionally hard, sub blocky. |
| 1980 | 1983 | 40 60 | SANDSTONE: clear to translucent, minor off white, fine to dominantly medium, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, occasional to common green lithics, disaggregated quartz grains, hard to occasionally very hard, poor visual and inferred porosity, no fluorescence. SILTSTONE: dark to occasionally medium grey, medium grey brown, argillaceous, occasionally arenaceous, minor micro mica, moderately hard to occasionally hard, sub blocky. |
| 1983 | 1986 | 30 70 | SANDSTONE: clear to translucent, minor off white, fine to dominantly medium, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous to occasionally silty matrix, occasional to common green lithics, disaggregated quartz grains, hard to occasionally very hard, poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, argillaceous, minor arenaceous, micro mica, rare pyrite nodules, moderately hard to occasionally hard, sub blocky. |
| 1986 | 1989 | 80 20 | SANDSTONE: clear to translucent, light grey, medium, occasionally fine to rare coarse, moderately sorted, angular to sub rounded, strong siliceous cement, minor off white argillaceous matrix, occasional lithics, loose disaggregated quartz grains, hard to very hard in part, poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, argillaceous, minor arenaceous, micro mica, rare pyrite nodules, moderately hard to occasionally hard, sub blocky. |
| 1989 | 1992 | 90 10 | SANDSTONE: off white to light grey, clear to translucent, medium to fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor off white argillaceous matrix, occasional lithics, loose disaggregated quartz grains, hard to very hard in part, poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, argillaceous, minor arenaceous, common micro mica, trace pyrite nodules, moderately hard to occasionally hard, sub blocky. |
| 1992 | 1995 | 80 20 | SANDSTONE: off white to light grey, clear to translucent, medium to minor fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor off white argillaceous matrix, occasional lithics, loose disaggregated quartz grains, hard to very hard in part, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, argillaceous, minor arenaceous, common micro mica, trace pyrite nodules, minor lithics, moderately hard to occasionally hard, sub blocky, |
| 1995 | 1998 | 90 10 | SANDSTONE: off white to light grey, clear to translucent, medium to minor fine, rare coarse, moderately sorted, angular to occasionally sub angular, strong siliceous cement, minor off white argillaceous matrix, occasional lithics, loose disaggregated quartz grains, hard to very hard in part, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, common micro mica, minor lithics, moderately hard to occasionally hard, sub blocky to occasionally sub fissile. |
| 1998 | 2001 | 70 30 | SANDSTONE: off white to very light grey, clear to translucent, medium to minor fine, rare coarse, moderately sorted, angular to occasionally sub angular, minor sub rounded, strong siliceous cement, minor off white to very light grey argillaceous matrix, occasional lithics, rare Siltstone laminations, com disaggregated quartz grains, hard to very hard in part, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, rare grading to very fine Sandstone, common micro mica, minor lithics, moderately hard to occasionally hard, sub blocky to occasionally sub fissile. |
| 2001 | 2004 | 90 10 | SANDSTONE: clear to translucent, off white to very light grey in part, fine to medium, rare coarse, moderately well sorted, angular to sub angular, minor sub rounded, strong siliceous cement, minor off white to very light grey argillaceous matrix, trace lithics, com disaggregated quartz grains, hard to very hard in part, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, occasional to locally common micro mica, minor lithics, rare nodular pyrite, moderately hard to occasionally hard, sub blocky to occasionally sub fissile. |
| 2004 | 2007 | 70 30 | SANDSTONE: clear to translucent, off white to very light grey in part, fine to medium, rare coarse, moderately well sorted, angular to sub angular, minor sub rounded, strong siliceous cement, minor off white to very light grey argillaceous matrix, trace lithics, com disaggregated quartz grains, hard to very hard in part, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, occasional to locally common micro mica, minor lithics, rare nodular pyrite, moderately hard to occasionally hard, sub blocky to occasionally sub fissile. |
| 2007 | 2010 | 60 40 | SANDSTONE: clear to translucent, commonly off white to very light grey, fine to medium, rare very fine, moderately well sorted, angular to sub rounded, strong siliceous cement, minor off white to very light grey argillaceous matrix, minor disaggregated quartz grains, hard to very hard in part, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, rare grading to very fine Sandstone in part, common micro mica, moderately hard to hard, sub blocky to sub fissile. |
| 2010 | 2013 | 60 40 | SANDSTONE: clear to translucent, commonly off white to very light grey, medium, minor fine to coarse, moderately sorted, angular to sub angular, strong siliceous cement, minor off white to very light grey argillaceous matrix, minor disaggregated quartz grains, dominantly hard to very hard in part, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, common micro mica, moderately hard to hard, sub blocky to sub fissile. |
| 2013 | 2016 | 70 30 | SANDSTONE: off white to very light grey, clear to translucent in part, fine to dominantly medium, minor coarse, moderately sorted, angular to sub rounded, strong siliceous cement, minor off white to very light grey argillaceous matrix, moderately hard to hard in part, minor disaggregated quartz grains, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, common micro mica, trace pyrite nodules, moderately hard to hard, sub blocky to sub fissile. |
| 2016 | 2019 | 70 30 | SANDSTONE: off white to very light grey, clear to translucent, occasionally pale brown grey, fine to dominantly medium, minor coarse, moderately sorted, angular to sub angular, strong siliceous cement, minor off white to very light grey argillaceous matrix, trace disseminated pyrite, predominantly moderately hard to occasionally hard, minor disaggregated quartz grains, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, rare dark brown grey, argillaceous, minor arenaceous, common micro mica, minor to locally common pyrite nodules, moderately hard to hard, sub blocky to sub fissile. |
| 2019 | 2022 | 70 30 | SANDSTONE: off white to very light grey, clear to translucent, occasional pale brown grey mottling, fine to dominantly medium, minor coarse, moderately sorted, angular to sub angular, strong siliceous cement, minor off white to very light grey argillaceous to occasionally silty matrix, trace silty laminations, predominantly hard to occasionally moderately hard, occasional disaggregated quartz grains, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, rare dark brown grey, argillaceous, minor arenaceous, common micro mica, trace pyrite nodules, moderately hard to hard, sub blocky to sub fissile. |
| 2022 | 2025 | 80 20 | SANDSTONE: translucent to clear, off white to very light grey, occasional pale brown grey mottling, fine to dominantly medium, moderately well sorted, angular to sub angular, strong siliceous cement, minor off white to very light grey argillaceous to occasionally silty matrix, trace silty laminations, predominantly hard to occasionally moderately hard, occasional disaggregated quartz grains, poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, rare dark brown grey, argillaceous, minor arenaceous grading to very fine Sandstone in part, rare pyrite nodules, common micro mica, moderately hard to hard, sub blocky to sub fissile. |
| 2025 | 2028 | 90 10 | SANDSTONE: translucent to clear, minor off white to very light grey, fine to dominantly medium, moderately well sorted, sub angular to angular, strong siliceous cement, trace off white to very light grey argillaceous matrix, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, minor arenaceous, rare pyrite nodules, common micro mica, moderately hard to hard, sub blocky to sub fissile. |
| 2028 | 2031 | 80 20 | SANDSTONE: clear to translucent, off white to very light grey in part, fine to dominantly medium, moderately well sorted, sub angular to angular, strong siliceous cement, trace off white to very light grey argillaceous matrix, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, minor medium grey, argillaceous, minor arenaceous, common micro mica, hard to occasionally moderately hard, sub blocky to sub fissile in part. |
| 2031 | 2034 | 70 30 | SANDSTONE: clear to translucent, off white to very light grey in part, medium to coarse, occasionally fine, moderately sorted, sub angular to dominantly angular, strong siliceous cement, trace off white to very light grey argillaceous matrix, rare pyrite nodules, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally very dark grey, minor medium grey, argillaceous, minor arenaceous, common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2034 | 2037 | 90 10 | SANDSTONE: clear to translucent, off white to very light grey in part, medium to coarse, occasionally fine, moderately sorted, sub angular to dominantly angular, strong siliceous cement, occasional off white argillaceous matrix, trace disseminated pyrite, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally very dark grey, minor medium grey, argillaceous to occasionally arenaceous, common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2037 | 2040 | 100 Trace | SANDSTONE: clear to translucent, off white to very light grey in part, medium to occasionally fine, moderately sorted, sub angular to dominantly angular, strong siliceous cement, occasional off white argillaceous matrix, trace disseminated pyrite, occasional rock flour, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally very dark grey, minor medium grey, argillaceous to occasionally arenaceous, common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2040 | 2043 | 70 30 | SANDSTONE: clear to translucent, off white to very light grey in part, medium to occasionally fine, moderately sorted, sub angular to dominantly angular, strong siliceous cement, occasional off white argillaceous matrix, trace disseminated pyrite, minor silty laminations, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally very dark grey, minor medium grey, argillaceous to occasionally arenaceous, common micro mica, occasional disseminated pyrite, moderately hard to hard, sub fissile to sub blocky. |
| 2043 | 2046 | 20 80 | SANDSTONE: clear to translucent, off white to very light grey in part, medium to occasionally fine, moderately sorted, sub angular to dominantly angular, strong siliceous cement, occasional off white argillaceous matrix, trace disseminated pyrite, minor silty laminations, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, dark grey brown, minor medium grey, dominantly argillaceous, grading to Claystone in part, minor arenaceous, common micro mica, hard to moderately hard, minor dispersive, sub fissile to sub blocky. |
| 2046 | 2049 | 30 70 | SANDSTONE: clear to translucent, off white to very light grey in part, fine to coarse, dominantly medium, moderately sorted, sub angular to dominantly angular, strong siliceous cement, occasional off white argillaceous matrix, minor nodular pyrite, minor silty laminations, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium brown grey, dark grey, dominantly argillaceous, grading to Claystone in part, minor arenaceous, common micro mica, hard to moderately hard, minor dispersive, sub fissile to sub blocky. *Minor Sandstone and Siltstone cavings observed. |
| 2049 | 2052 | 80 20 | SANDSTONE: clear to translucent, off white to very light grey in part, fine to coarse, dominantly medium, moderately sorted, sub angular to dominantly angular, strong siliceous cement, minor to locally common off white argillaceous matrix, minor nodular pyrite, occasional rock flour, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium brown grey, dark grey, dominantly argillaceous, grading to Claystone in part, minor arenaceous, common micro mica, hard to moderately hard, minor dispersive, sub fissile to minor sub blocky. |
| 2052 | 2055 | 80 20 | SANDSTONE: clear to translucent, off white to very light grey in part, fine, occasionally medium to rare very fine, dominantly medium, moderately sorted, sub angular to dominantly angular, strong siliceous cement, minor to locally common off white argillaceous matrix, minor silty inclusions, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium brown grey, dark grey, locally arenaceous to dominantly argillaceous, grading to Claystone in part, minor arenaceous, common micro mica, hard to moderately hard, minor dispersive, sub fissile to minor sub blocky. |
| 2055 | 2058 | 60 40 | SANDSTONE: clear to translucent, off white to very light grey in part, very fine to medium, dominantly fine, moderately sorted, sub angular to dominantly angular, strong siliceous cement, minor off white argillaceous matrix, rare pyrite nodules, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium brown grey, dark grey, argillaceous, locally arenaceous, grading to Claystone in part, common micro mica, hard to moderately hard, minor dispersive, sub fissile to minor sub blocky. |
| 2058 | 2061 | 80 20 | SANDSTONE: clear to translucent, minor off white to very light grey in part, medium, occasionally fine, well sorted, angular to minor sub rounded, strong siliceous cement, minor off white argillaceous matrix, rare pyrite nodules, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally medium grey, argillaceous, locally arenaceous, rare siliceous, grading to Claystone in part, common micro mica, hard to moderately hard, minor dispersive, sub fissile to minor sub blocky. |
| 2061 | 2064 | 80 20 | SANDSTONE: clear to translucent, minor off white to very light grey in part, medium to coarse, moderately well sorted, angular to minor sub rounded, strong siliceous cement, minor off white argillaceous matrix, rare pyrite nodules, minor rock flour, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, locally arenaceous, rare siliceous, grading to Claystone in part, common micro mica, hard to moderately hard, minor dispersive, sub fissile to minor sub blocky. |
| 2064 | 2067 | 100 trace | SANDSTONE: clear to translucent, minor off white to very light grey in part, medium to occasionally coarse, minor fine, moderately well sorted, angular to minor sub angular, strong siliceous cement, trace off white argillaceous matrix, rare disseminated pyrite, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, locally arenaceous, rare siliceous, common to abundant micro mica, hard to moderately hard, sub fissile to minor sub blocky. |
| 2067 | 2070 | 90 10 | SANDSTONE: off white to light grey, clear to translucent, medium to occasionally coarse, minor fine, moderately well sorted, angular to minor sub angular, rare rounded, strong siliceous cement, trace off white argillaceous matrix, rare disseminated pyrite, dominantly disaggregated quartz grains, moderately hard to dominantly hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, occasionally arenaceous, rare siliceous, common to abundant micro mica, hard to moderately hard, sub fissile to minor sub blocky. |
| 2070 | 2073 | 100 Trace | SANDSTONE: clear to translucent, off white, fine to medium, well sorted, angular to minor sub angular, rare rounded, strong siliceous cement, trace off white argillaceous matrix, rare disseminated pyrite, dominantly disaggregated quartz grains, friable to moderately hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, occasionally arenaceous, rare siliceous, common to abundant micro mica, hard to moderately hard, sub fissile to minor sub blocky. |
| 2073 | 2076 | 100 Trace | SANDSTONE: clear to translucent, fine to medium, well sorted, angular to minor sub angular, strong siliceous cement, trace off white argillaceous matrix, rare disseminated pyrite, dominantly clean loose disaggregated quartz grains, minor friable to moderately hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, argillaceous, occasionally arenaceous, rare siliceous, common to abundant micro mica, hard to moderately hard, sub fissile to minor sub blocky. |
| 2076 | 2079 | 100 Trace | SANDSTONE: clear to translucent, rare light grey, fine to medium, well sorted, angular to minor sub angular, strong siliceous cement, occasional off white argillaceous matrix, rare disseminated pyrite, dominantly clean loose disaggregated quartz grains, minor friable to moderately hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: very dark to dark grey, occasionally medium grey, argillaceous, occasionally arenaceous, rare siliceous, common to abundant micro mica, hard to moderately hard, sub fissile to minor sub blocky. |
| 2079 | 2082 | 60 40 | SANDSTONE: off white to light grey, clear to translucent, fine to medium, well sorted, angular to minor sub angular, strong siliceous cement, occasional off white argillaceous matrix, rare disseminated pyrite, dominantly clean loose disaggregated quartz grains, minor friable to moderately hard, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium grey, dark grey brown, argillaceous, minor grading to Claystone, rare arenaceous, rare siliceous, common to abundant micro mica, hard to moderately hard, sub fissile to minor sub blocky. |
| 2082 | 2085 | 70 30 | SILTSTONE: dark to medium grey, dark grey brown, argillaceous, minor arenaceous with rare grading to very fine Sandstone, common to abundant micro mica, hard to moderately hard, sub fissile to minor sub blocky. SANDSTONE: off white to light grey, clear to translucent, minor pale brown grey, fine to medium, well sorted, angular to minor sub angular, strong siliceous cement, occasional off white argillaceous matrix, rare disseminated pyrite, friable to moderately hard, minor clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. |
| 2085 | 2088 | 80 20 | SILTSTONE: dark to medium grey, dark grey brown, argillaceous, minor grading to Claystone, minor arenaceous, common to abundant micro mica, rare nodular pyrite, hard to moderately hard, sub fissile to minor sub blocky. SANDSTONE: off white to light grey, pale brown grey in part, clear to translucent, fine to medium, well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, minor pale brown grey argillaceous matrix, rare disseminated pyrite, friable to moderately hard, clean loose disaggregated quartz grains in part, very poor visual porosity, poor inferred porosity, no fluorescence. |
| 2088 | 2091 | 90 10 | SILTSTONE: dark to medium grey, dark grey brown, rare dark grey black, argillaceous, minor grading to Claystone, occasionally arenaceous, minor to locally common micro mica, rare nodular pyrite, hard to moderately hard, sub fissile to sub blocky. SANDSTONE: off white to light grey, pale brown grey in part, occasionally light to medium grey, minor clear to translucent, very fine to medium, well sorted, angular to sub angular, occasionally sub rounded, strong siliceous cement, common pale brown grey argillaceous matrix, rare disseminated pyrite, friable to moderately hard, very poor visual porosity, no fluorescence. |
| 2091 | 2094 | 80 20 | SILTSTONE: dark to medium grey, dark grey brown, rare dark grey black, argillaceous, minor grading to Claystone, occasionally arenaceous, minor to locally common micro mica, rare nodular and disseminated pyrite, hard to moderately hard, sub fissile to sub blocky. SANDSTONE: light to occasionally med grey, minor clear to translucent, pale brown grey in part, fine to medium, moderately well sorted, angular to sub rounded, strong siliceous cement, common pale brown grey argillaceous matrix, rare disseminated pyrite, rare silty laminations, friable to moderately hard, very poor visual porosity, no fluorescence. |
| 2094 | 2097 | 80 20 | SILTSTONE: dark to medium grey, dark grey brown, rare dark grey black, argillaceous to arenaceous, minor to locally common micro mica, rare nodular and disseminated pyrite, hard to moderately hard, sub fissile to sub blocky. SANDSTONE: light to occasionally medium grey, minor clear to translucent, pale brown grey in part, fine to medium, moderately well sorted, angular to sub rounded, strong siliceous cement, common pale brown grey argillaceous matrix, rare disseminated pyrite, rare silty laminations, friable to moderately hard, very poor visual porosity, no fluorescence. |
| 2097 | 2100 | 60 40 | SILTSTONE: dark to medium grey, dark grey brown, rare dark grey black, argillaceous, minor grading to Claystone, occasionally arenaceous, minor to locally common micro mica, hard to moderately hard, sub fissile to sub blocky. SANDSTONE: light to occasionally medium grey, minor clear to translucent, pale brown grey in part, fine to medium, moderately well sorted, angular to sub rounded, strong siliceous cement, common pale brown grey argillaceous matrix, friable to moderately hard, very poor visual porosity, no fluorescence. |
| 2100 | 2103 | 60 40 | SANDSTONE: off white to light grey, light to medium grey, minor clear to translucent, pale brown grey in part, fine to medium, moderately well sorted, angular to sub rounded, strong siliceous cement, common pale brown grey argillaceous matrix, rare disseminated pyrite, rare silty laminations, moderately hard, very poor visual porosity, no fluorescence. SILTSTONE: dark to medium grey, dark brown grey, argillaceous, minor grading to Claystone, occasionally arenaceous, trace siliceous, common micro mica, hard to moderately hard, sub fissile to sub blocky. |
| 2103 | 2106 | 70 30 | SANDSTONE: off white to light grey, clear to translucent, minor pale brown grey, fine to medium, moderately well sorted, angular to sub rounded, strong siliceous cement, common pale brown grey argillaceous matrix, minor lithics, rare disseminated pyrite, moderately hard to hard, occasional clean loose disaggregated quartz grains, poor visual porosity, no fluorescence. SILTSTONE: dark to medium grey, dark brown grey, argillaceous grading to Claystone in part, occasionally arenaceous, minor to locally common micro mica, hard to moderately hard, sub fissile to sub blocky. |
| 2106 | 2109 | 10 90 | SANDSTONE: off white to light grey, clear to translucent, minor pale brown, fine to medium, well sorted, angular to sub angular, strong siliceous cement, minor light brown to off white argillaceous matrix, minor lithics, hard to moderately hard, minor clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally medium grey, dark brown grey in part, argillaceous to arenaceous, common micro mica, hard to occasionally moderately hard, sub fissile to minor sub blocky. |
| 2109 | 2112 | 20 80 | SANDSTONE: light brown to light grey, off white, clear to translucent, medium to coarse, occasionally fine, well sorted, angular to sub angular, strong siliceous cement, minor light brown to off white argillaceous matrix, minor lithics, hard to moderately hard, minor clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, dark brown grey in part, arenaceous to occasionally argillaceous, common micro mica, hard to occasionally moderately hard, sub fissile to minor sub blocky. |
| 2112 | 2115 | 80 20 | SANDSTONE: clear to translucent, off white, medium to coarse, well sorted, angular to minor sub angular, strong siliceous cement, minor light brown to off white argillaceous matrix, minor lithics, hard, minor clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, occasionally medium grey brown, argillaceous to arenaceous, common micro mica, hard to occasionally moderately hard, sub fissile to minor sub blocky. |
| 2115 | 2118 | 80 20 | SANDSTONE: clear to translucent, light grey, medium to coarse, well sorted, angular to minor sub angular, strong siliceous cement, minor light brown to off white argillaceous matrix, trace nodular pyrite, minor lithics, hard, minor clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, dark grey brown, argillaceous to arenaceous, common micro mica, hard to occasionally moderately hard, sub fissile to minor sub blocky. |
| 2118 | 2121 | 100 Trace | SANDSTONE: off white, clear to translucent, fine to coarse, poorly sorted, angular to sub angular, strong siliceous cement, rare off white argillaceous matrix, minor lithics, occasional rock flour, hard to very hard, clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, dark grey brown, argillaceous to arenaceous, common micro mica, hard to occasionally moderately hard, sub fissile to minor sub blocky. |
| 2121 | 2124 | 80 20 | SANDSTONE: off white, clear to translucent, fine to medium, moderately sorted, angular to sub angular, strong siliceous cement, rare off white argillaceous matrix, minor lithics, occasional rock flour, hard to very hard, clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium grey brown, dark grey, argillaceous to arenaceous, minor micro mica, hard to occasionally moderately hard, sub fissile to minor sub blocky. |
| 2124 | 2127 | 60 40 | SANDSTONE: off white to light grey, clear to translucent in part, medium to occasionally fine, moderately sorted, angular to sub angular, strong siliceous cement, rare off white argillaceous matrix, minor lithics, occasional rock flour, hard to very hard, clean loose disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, argillaceous to arenaceous, common micro mica, hard to occasionally moderately hard, sub fissile to minor sub blocky. |
| 2127 | 2130 | 50 50 | SANDSTONE: off white to light grey, medium grey in part, minor clear to translucent, medium to occasionally fine, moderately sorted, angular to sub angular, strong siliceous cement, rare off white argillaceous matrix, minor lithics, rare disseminated pyrite, hard to very hard, disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, argillaceous to arenaceous, common micro mica, hard to moderately hard, sub fissile to occasionally sub blocky. |
| 2130 | 2133 | 70 30 | SANDSTONE: off white, light to medium grey, minor clear to translucent, medium to occasionally fine, moderately sorted, angular to sub angular, strong siliceous cement, rare off white argillaceous matrix, occasional dark lithics, hard to very hard, disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, minor dark grey brown, argillaceous to dominantly arenaceous, common micro mica, hard to moderately hard, sub fissile to occasionally sub blocky. |
| 2133 | 2136 | 90 10 | SANDSTONE: off white, light grey, minor clear to translucent, medium to occasionally coarse, moderately sorted, angular to sub angular, strong siliceous cement, rare off white argillaceous matrix, minor glauconite, occasional dark lithics, hard to very hard, disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, minor dark grey brown, arenaceous, minor argillaceous, common micro mica, hard to moderately hard, sub fissile to occasionally sub blocky. |
| 2136 | 2139 | 70 30 | SANDSTONE: off white, light grey, minor clear to translucent, medium to occasionally coarse, moderately sorted, angular to sub angular, strong siliceous cement, occasional to locally common off white argillaceous matrix, minor glauconite, occasional dark lithics, hard to very hard, disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, minor dark grey brown, arenaceous, minor argillaceous, common to abundant micro mica, hard to occasionally moderately hard, sub fissile to occasionally sub blocky. |
| 2139 | 2142 | 90 10 | SANDSTONE: off white, light grey, minor clear to translucent, rare medium grey brown, medium to occasionally coarse, fine in part, moderately sorted, angular to sub angular, strong siliceous cement, occasional to locally common off white argillaceous matrix, occasional dark lithics, hard to very hard, disaggregated quartz grains, very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, minor dark grey brown, arenaceous, minor argillaceous, common to abundant micro mica, occasional mica flakes, hard to occasionally moderately hard, sub fissile to occasionally sub blocky. |
| 2142 | 2145 | 90 10 | SANDSTONE: off white, light grey, minor clear to translucent, rare medium grey brown, medium to occasionally coarse, fine in part, moderately sorted, angular to sub angular, strong siliceous cement, occasional to locally common off white argillaceous matrix, occasional dark lithics, hard to occasionally very hard, minor disaggregated quartz grains, very poor to tight visual and inferred porosity, no fluorescence. SILTSTONE: dark grey, minor dark grey brown, arenaceous, minor argillaceous, common to abundant micro mica, occasional mica flakes, hard to occasionally moderately hard, sub blocky to sub fissile. |
| 2145 | 2148 | 90 10 | SANDSTONE: off white, light grey, minor clear to translucent, rare medium grey brown, medium to occasionally coarse, fine in part, moderately sorted, angular to sub angular, strong siliceous cement, occasional to locally common off white argillaceous matrix, occasional dark lithics, hard to occasionally very hard, minor disaggregated quartz grains, very poor to tight visual and inferred porosity, no fluorescence. SILTSTONE: dark grey to grey black, minor brownish black, common arenaceous, minor argillaceous, common micro mica, moderately hard to hard, sub fissile to minor fissile, trace sub blocky. |
| 2148 | 2151 | 60 40 | SANDSTONE: clear, opaque, off white, translucent, medium to occasional fine grains, moderately sorted, sub angular to angular, strong siliceous cement, minor off white argillaceous matrix, minor dark lithics, hard to occasionally very hard, minor disaggregated quartz grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to grey black, minor brownish black, common arenaceous, minor argillaceous, common micro mica, moderately hard to hard, sub fissile to minor fissile, trace sub blocky. |
| 2151 | 2154 | 80 20 | SANDSTONE: clear, opaque, off white, translucent, medium to occasional fine grains, moderately sorted, sub angular to angular, strong siliceous cement, minor off white argillaceous matrix, minor dark lithics, hard to occasionally very hard, minor disaggregated quartz grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: grey black to very dark grey, arenaceous, argillaceous in part, abundant micro mica, moderately hard to hard, fissile to sub fissile, trace sub blocky. |
| 2154 | 2157 | 80 20 | SANDSTONE: clear, translucent, opaque, off white, fine to occasional very fine grains, moderately sorted, angular to sub angular, strong siliceous cement, trace off white argillaceous matrix, minor dark grey lithics, hard to very hard, minor disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: grey black to very dark grey, arenaceous, argillaceous in part, abundant micro mica, moderately hard to hard, fissile to sub fissile, trace sub blocky. |
| 2157 | 2160 | 80 20 | SANDSTONE: clear, translucent, opaque, off white, fine to medium grains, moderately sorted, angular to sub angular, strong siliceous cement, trace off white argillaceous matrix, minor dark grey lithics, hard to very hard, minor disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: grey black to very dark grey, arenaceous, argillaceous in part, abundant micro mica, moderately hard to hard, fissile to sub fissile, trace sub blocky. |
| 2160 | 2163 | 90 10 | SANDSTONE: clear, translucent, opaque, off white, fine, occasionally very fine to medium, moderately sorted, angular to sub angular, strong siliceous cement, trace off white argillaceous cement, minor dark grey lithics, hard to very hard, minor disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: very dark grey, brownish grey, greyish black in part, arenaceous, minor argillaceous, common micro mica, sub fissile to minor fissile, sub blocky in part. |
| 2163 | 2166 | 80 20 | SANDSTONE: off white, translucent, minor clear to opaque, trace light grey, very fine to fine, minor medium, moderately to moderately well sorted, sub angular to minor sub rounded, trace angular, strong siliceous cement, minor off white argillaceous matrix, rare glauconite grains, hard to very hard in part, common disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: very dark grey, brownish grey, greyish black in part, arenaceous, minor argillaceous, common micro mica, sub fissile to minor fissile, sub blocky in part. |
| 2166 | 2169 | 90 10 | SANDSTONE: off white, translucent, minor clear to opaque, trace light grey, very fine to medium, moderately to moderately well sorted, sub angular to minor sub rounded, trace angular, strong siliceous cement, minor off white argillaceous matrix, occasional rock flour, hard to very hard in part, common disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to grey black, brownish grey to brownish black, dominantly arenaceous, minor argillaceous, common micro mica, moderately hard to hard, sub fissile to sub blocky, trace fissile. |
| 2169 | 2172 | 40 60 | SANDSTONE: off white, translucent, minor clear to opaque, trace light grey, fine to medium, moderately to moderately well sorted, sub angular to minor sub rounded, trace angular, strong siliceous cement, minor off white argillaceous matrix, rare glauconite grains, hard to very hard in part, common disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey brown, dark grey in part, argillaceous to minor arenaceous, common micro micaceous, firm to hard, dispersive in part, sub blocky, minor blocky to sub fissile. |
| 2172 | 2175 | 60 40 | SANDSTONE: off white to light brown, light grey, rare clear to translucent, fine to medium, rare coarse, moderately sorted, sub angular, trace angular, strong siliceous cement, minor off white argillaceous matrix, rare glauconite grains, hard to very hard in part, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey brown, dark grey in part, arenaceous, minor argillaceous, common micro micaceous, minor mica flecks, firm to hard, sub blocky, minor blocky to sub fissile. |
| 2175 | 2178 | 20 80 | SANDSTONE: off white to light brown, light grey, rare clear to translucent, very fine to medium, moderately sorted, sub angular, trace angular, strong siliceous cement, minor off white argillaceous matrix, rare glauconite grains, occasional rock flour, hard to very hard in part, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to occasionally grey brown, very dark grey in part, arenaceous in part, common to abundant micro micaceous, minor mica flecks, firm to hard, sub blocky, minor blocky to sub fissile. |
| 2178 | 2181 | 30 70 | SANDSTONE: light grey to occasionally off white, minor light brown, light grey, rare clear to translucent, very fine to fine, occasionally medium, moderately sorted, sub angular, trace angular, strong siliceous cement, minor off white argillaceous matrix, rare glauconite grains, trace dark lithics, occasional rock flour, hard to very hard in part, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to occasionally grey brown, very dark grey in part, arenaceous in part, common to abundant micro micaceous, minor mica flecks, firm to hard, sub blocky, minor blocky to sub fissile. |
| 2181 | 2184 | 20 80 | SANDSTONE: light to occasionally medium grey, minor off white, minor light brown, very fine to fine, occasionally medium, moderately sorted, sub angular, trace angular, strong siliceous cement, minor off white argillaceous matrix, rare glauconite grains, occasional rock flour, trace dark lithics, hard to occasionally very hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, dark grey brown, arenaceous in part, common to abundant micro micaceous, minor mica flecks, moderately hard to hard, sub blocky, minor blocky. |
| 2184 | 2187 | 10 90 | SANDSTONE: light to occasionally medium grey, minor off white, minor light brown, very fine to dominantly fine, moderately sorted, sub angular, trace angular, strong siliceous cement, minor off white argillaceous matrix, rare glauconite grains, trace dark lithics, hard to occasionally very hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, dark grey brown, locally arenaceous, abundant micro micaceous, minor mica flecks, moderately hard to hard, sub blocky, minor blocky. |
| 2187 | 2190 | 90 10 | SANDSTONE: clear to translucent, off white, light grey, fine to medium, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare glauconite grains, hard to occasionally very hard, occasional clean loose disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, dark grey brown, minor arenaceous, abundant micro micaceous, minor mica flecks, moderately hard to hard, sub blocky, minor blocky. |
| 2190 | 2193 | 90 10 | SANDSTONE: light grey, clear to translucent, off white in part, medium to occasionally fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare glauconite grains, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, occasionally dark grey brown, minor arenaceous, common micro micaceous, minor mica flecks, moderately hard to hard, sub blocky, minor blocky. |
| 2193 | 2196 | 70 30 | SANDSTONE: off white to light grey, rare clear to translucent, fine to occasionally medium, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, occasionally dark grey brown, minor arenaceous, common to abundant micro micaceous, minor mica flecks, moderately hard to hard, sub blocky, minor blocky. |
| 2196 | 2199 | 50 50 | SANDSTONE: off white to light grey brown, rare clear to translucent, fine to occasionally medium, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare mica flakes, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, occasionally dark grey brown, minor arenaceous, common to abundant micro micaceous, minor mica flecks, moderately hard to dominantly hard, sub fissile to sub blocky. |
| 2199 | 2202 | 20 80 | SANDSTONE: off white to light grey brown, rare clear to translucent, fine to occasionally medium, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare mica flakes, minor localised dark lithics, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally medium grey, occasionally dark grey brown, minor arenaceous, minor siliceous, common to abundant micro micaceous, minor mica flecks, moderately hard to dominantly hard, sub fissile to sub blocky. |
| 2202 | 2205 | 10 90 | SANDSTONE: off white to light grey, occasionally medium grey, fine to occasionally very fine, medium in part, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare mica flakes, minor localised dark lithics, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, occasionally dark grey brown, arenaceous, common to abundant micro micaceous, minor mica flecks, moderately hard to dominantly hard, sub blocky. |
| 2205 | 2208 | 10 90 | SANDSTONE: off white to light grey, fine to occasionally very fine, medium in part, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare mica flakes, minor localised dark lithics, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, occasionally dark grey brown, minor argillaceous, occasional to common micro micaceous, moderately hard to dominantly hard, sub blocky. |
| 2208 | 2211 | 10 90 | SANDSTONE: light grey to occasionally off white, fine to very fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare mica flakes, minor localised dark lithics, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, occasionally dark grey brown, minor argillaceous, occasional to common micro micaceous, rare disseminated pyrite, moderately hard to dominantly hard, sub blocky. |
| 2211 | 2214 | 20 80 | SANDSTONE: light grey medium grey, very fine to fine, moderately sorted, angular to occasionally sub angular, strong siliceous cement, trace off white argillaceous matrix, rare mica flakes, trace dark lithics, hard to very hard, loose disaggregated grains in part, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: medium to dark grey brown, commonly arenaceous, micro micaceous, rare disseminated pyrite, moderately hard to dominantly hard, sub blocky. |
| 2214 | 2217 | 90 10 | SANDSTONE: clear to translucent, off white, medium, occasionally fine to coarse, moderately sorted, angular to minor sub angular, strong siliceous cement, trace to nil off white argillaceous matrix, hard to very hard, loose clean disaggregated grains, poor to very poor inferred porosity, very poor to tight vis porosity, no fluorescence. SILTSTONE: dark grey, medium to dark grey brown, commonly arenaceous, micro micaceous, moderately hard to dominantly hard, sub blocky. |
| 2217 | 2220 | 90 10 | SANDSTONE: clear to translucent, off white, medium, occasionally fine to coarse, moderately sorted, angular to minor sub angular, strong siliceous cement, trace to nil off white argillaceous matrix, hard to very hard, loose clean disaggregated grains, poor to very poor inferred porosity, very poor to tight vis porosity, no fluorescence. SILTSTONE: dark grey to brownish grey, arenaceous, minor argillaceous, common micro mica, moderately hard to hard, sub fissile to minor sub blocky. |
| 2220 | 2223 | 95 5 | SANDSTONE: off white, translucent, clear, minor light to very light grey, fine to minor medium, trace very fine, rare coarse, moderately sorted, angular to sub angular, minor sub rounded, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to brownish grey, arenaceous, minor argillaceous, common micro mica, moderately hard to hard, sub fissile to minor sub blocky. |
| 2223 | 2226 | 95 5 | SANDSTONE: off white, translucent, clear, minor light to very light grey, fine to minor medium, trace very fine, rare coarse, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to brownish grey, arenaceous, minor argillaceous, common micro mica, moderately hard to hard, sub fissile to minor sub blocky. |
| 2226 | 2229 | 70 30 | SANDSTONE: off white, translucent, clear, minor light to very light grey, fine to minor medium, trace very fine, rare coarse, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, olive grey, medium grey in part, minor brownish grey, argillaceous, minor arenaceous, micro mica in part, moderately hard to hard, sub fissile to sub blocky. |
| 2229 | 2232 | 60 40 | SANDSTONE: off white, translucent, clear, very light grey, opaque, fine to trace medium, trace very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, olive grey, medium grey in part, minor brownish grey, argillaceous, minor arenaceous, micro mica in part, moderately hard to hard, sub fissile to sub blocky. |
| 2232 | 2235 | 20 80 | SANDSTONE: off white, translucent, clear, very light grey, opaque, fine to trace medium, trace very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: medium grey to medium dark grey, olive grey to brownish grey, dark grey, dominantly arenaceous, minor argillaceous, micro mica in part, moderately hard to hard, sub fissile to sub blocky. |
| 2235 | 2238 | 10 90 | SANDSTONE: off white, translucent, clear, very light grey, opaque, fine to trace medium, trace very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark to very dark grey, dark grey brown, occasionally argillaceous to arenaceous, minor to locally common micro mica, moderately hard to hard, sub blocky to minor sub fissile. |
| 2238 | 2241 | 5 95 | SANDSTONE: off white, translucent, clear, very light grey, opaque, fine to trace medium, trace very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, dark grey brown, occasionally argillaceous to minor arenaceous, minor to locally common micro mica, moderately hard to hard, sub blocky to minor sub fissile. |
| 2241 | 2244 | 10 90 | SANDSTONE: light to medium grey, fine to medium, trace very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark to occasionally medium brown grey, dark grey, arenaceous, locally common micro mica, rare pyrite nodules, moderately hard to hard, sub blocky to minor sub fissile. |
| 2244 | 2247 | Trace 100 | SANDSTONE: light to medium grey, fine to medium, trace very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, common disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to dark brown grey, arenaceous, locally common micro mica, hard to occasionally very hard, sub blocky to minor sub fissile. |
| 2247 | 2250 | 10 90 | SANDSTONE: light grey to off white, fine to medium, minor very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, hard to minor very hard, minor disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to dark brown grey, argillaceous to dominantly arenaceous, locally common micro mica, hard to occasionally very hard, sub blocky to minor sub fissile. |
| 2250 | 2253 | 10 90 | SANDSTONE: light grey to off white, fine to medium, minor very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, minor pyrite nodules, hard to minor very hard, minor disaggregated grains, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, dark brown grey, argillaceous to dominantly arenaceous, common micro mica, hard to occasionally very hard, sub blocky to minor sub fissile. |
| 2253 | 2256 | 20 80 | SANDSTONE: off white to light grey , fine to very fine, occasionally medium, minor very fine, moderately sorted, angular to sub angular, strong siliceous cement, minor off white argillaceous matrix, minor pyrite nodules, dominantly disaggregated grains, hard to minor very hard in part, tight to very poor visual porosity, poor to very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium to very dark grey, dark brown grey, argillaceous to dominantly arenaceous, common micro mica, hard to occasionally very hard, sub blocky to minor sub fissile. |
| 2256 | 2259 | 40 60 | SANDSTONE: light brown to light grey, occasionally light to medium brown grey, fine to occasionally medium, moderately well sorted, angular to occasionally sub rounded, moderate to occasionally strong siliceous cement, occasional to common light brown grey silty matrix, locally common dark lithics, friable to moderately hard, hard in part, poor visual porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium to very dark grey, dark brown grey, argillaceous to dominantly arenaceous, common micro mica, hard to occasionally very hard, sub blocky to minor sub fissile. |
| 2259 | 2262 | 40 60 | SANDSTONE: light brown to light grey, occasionally light to medium brown grey, fine to occasionally medium, moderately well sorted, angular to sub angular, moderate to occasionally strong siliceous cement, occasional to common light brown grey silty matrix, locally common dark lithics, friable to moderately hard, hard in part, poor visual porosity, no fluorescence. SILTSTONE: medium to dominantly dark grey, dark brown grey, dominantly arenaceous, common micro mica, minor mica flakes, hard to occasionally very hard, sub blocky to blocky. |
| 2262 | 2265 | 50 50 | SANDSTONE: light grey to off white, minor translucent, minor medium brown grey, fine to occasionally medium, moderately well sorted, angular to sub angular, moderate strong siliceous cement, occasional off white argillaceous matrix, minor dark lithics, dominantly disaggregated grains, friable to moderately hard in part, poor to very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: medium to dominantly dark grey, dark brown grey, dominantly arenaceous, common micro mica, minor mica flakes, hard to minor very hard, sub blocky to blocky. |
| 2265 | 2268 | 70 30 | SANDSTONE: off white to light grey, minor translucent, minor light brown grey, fine to occasionally medium, moderately well sorted, angular to sub angular, moderately strong siliceous cement, occasional off white argillaceous matrix, trace glauconite grains, minor dark lithics, dominantly disaggregated loose grains, friable to hard in part, poor to very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: medium to dominantly dark grey, dark brown grey, dominantly arenaceous, common micro mica, minor mica flakes, hard to minor very hard, sub blocky to blocky. |
| 2268 | 2271 | 20 80 | SANDSTONE: light grey to light grey brown, fine to occasionally medium, moderately well sorted, angular to sub angular, moderately strong siliceous cement, occasional off white argillaceous matrix, trace glauconite grains, minor dark lithics, dominantly disaggregated loose grains, friable to hard in part, poor to very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey, occasionally medium grey, arenaceous, occasionally grading to very fine Sandstone, common micro mica, minor mica flakes, hard to minor very hard, sub blocky to blocky. |
| 2271 | 2274 | 5 95 | SANDSTONE: light grey to light grey brown, fine to occasionally medium, moderately well sorted, angular to sub angular, moderately strong siliceous cement, occasional off white argillaceous matrix, trace glauconite grains, minor dark lithics, dominantly disaggregated loose grains, friable to hard in part, poor to very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey brown, dark grey, arenaceous, occasionally grading to very fine Sandstone, common micro mica, minor mica flakes, occasional lithics, hard to minor very hard, sub blocky to blocky. |
| 2274 | 2277 | 20 80 | SANDSTONE: light brown grey, very fine to fine, well sorted, angular to sub angular, moderately strong siliceous cement, occasional off white argillaceous matrix, minor dark lithics, silty in part, dominantly disaggregated loose grains, friable to hard in part, poor to very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey to dark grey brown, arenaceous, occasionally grading to very fine Sandstone, common micro mica, minor mica flakes, occasional lithics, hard to occasionally moderately hard, sub blocky to blocky. |
| 2277 | 2280 | 20 80 | SANDSTONE: light brown grey, very fine to fine, occasionally medium, well sorted, angular to sub angular, moderately strong siliceous cement, occasional off white argillaceous matrix, rare disseminated pyrite, minor dark lithics, silty in part, dominantly disaggregated loose grains, friable to hard in part, poor to very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark grey to dark grey brown, arenaceous, occasionally grading to very fine Sandstone, rare argillaceous, common micro mica, minor mica flakes, occasional lithics, hard to occasionally moderately hard, sub blocky to blocky. |
| 2280 | 2283 | Trace 100 | SANDSTONE: light brown grey, very fine to fine, occasionally medium, well sorted, angular to sub angular, moderately strong siliceous cement, occasional off white argillaceous matrix, rare disseminated pyrite, minor dark lithics, silty in part, dominantly disaggregated loose grains, friable to hard in part, poor to very poor visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: very dark grey, dark grey in part, argillaceous to arenaceous, common micro mica, occasional lithics, moderately hard to hard, sub blocky to occasionally blocky. |
| 2283 | 2286 | 100 | SILTSTONE: very dark to dark grey, argillaceous to arenaceous, trace disseminated pyrite, common micro mica, occasional lithics, moderately hard to hard, sub blocky to occasionally blocky. |
| 2286 | 2289 | 100 | SILTSTONE: very dark to dark grey, argillaceous to arenaceous, occasional disseminated pyrite, occasional to locally common micro mica, occasional lithics, moderately hard to hard, sub blocky to occasionally blocky. |
| 2289 | 2292 | 10 90 | SANDSTONE: off white, light grey to medium light grey in part, trace translucent, very fine to minor fine, moderately sorted, sub angular to minor sub rounded, trace angular, strong siliceous cement, trace off white argillaceous matrix, hard to very hard, trace disaggregated grains, nil to tight visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, dark brown grey, trace very dark grey, arenaceous, minor argillaceous, minor to locally common micro mica, trace disseminated and nodular pyrite, moderately hard to hard, sub fissile to sub blocky. |
| 2292 | 2295 | 60 40 | SANDSTONE: off white, light grey to medium light grey in part, trace translucent, very fine to minor fine, moderately sorted, sub angular to minor sub rounded, trace angular, strong siliceous cement, trace off white argillaceous matrix, hard to very hard, trace disaggregated grains, nil to tight visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to dark brown grey, trace very dark grey, arenaceous, minor argillaceous, minor to locally common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2295 | 2298 | 40 60 | SANDSTONE: off white, light grey to medium light grey in part, trace translucent, very fine to minor fine, moderately sorted, sub angular to minor sub rounded, trace angular, strong siliceous cement, trace off white argillaceous matrix, hard to very hard, trace disaggregated grains, nil to tight visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to dark brown grey, trace brown grey, trace very dark grey, arenaceous, minor argillaceous, minor to locally common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2298 | 2301 | 30 70 | SANDSTONE: off white, light grey to medium light grey in part, trace translucent, very fine to minor fine, moderately sorted, sub angular to minor sub rounded, trace angular, strong siliceous cement, trace off white argillaceous matrix, hard to very hard, trace disaggregated grains, nil to tight visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to medium dark grey, brown grey, trace medium grey, dominantly arenaceous, minor argillaceous, minor locally common micro mica, moderately hard to hard, trace very hard, sub fissile to sub blocky. |
| 2301 | 2304 | 60 40 | SANDSTONE: light grey to off white, medium light grey in part, translucent to frosted, dominantly fine, trace very fine, trace medium grains, moderately to moderately well sorted, sub angular to minor sub rounded, strong siliceous cement, trace off white argillaceous cement, moderately hard to hard, trace very hard, common disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey to medium dark grey, brown grey, trace medium grey, dominantly arenaceous, minor argillaceous, minor locally common micro mica, moderately hard to hard, trace very hard, sub fissile to sub blocky. |
| 2304 | 2307 | 20 80 | SANDSTONE: light grey to off white, medium light grey in part, translucent to frosted, dominantly fine, trace very fine, trace medium grains, moderately to moderately well sorted, sub angular to minor sub rounded, strong siliceous cement, trace off white argillaceous cement, moderately hard to hard, trace very hard, common disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: brown grey, dark grey to medium dark grey, medium grey in part, arenaceous, argillaceous in part, minor locally common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2307 | 2310 | 10 90 | SANDSTONE: light grey to medium light grey, off white, translucent, fine to minor very fine, moderately to moderately well sorted, sub angular to sub rounded, strong siliceous cement, trace off white argillaceous matrix, moderately hard to hard, minor very hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: brown grey, dark grey to medium dark grey, medium grey in part, arenaceous, argillaceous in part, minor locally common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2310 | 2313 | 20 80 | SANDSTONE: light grey to medium light grey, off white, translucent, fine to minor very fine, moderately to moderately well sorted, sub angular to sub rounded, strong siliceous cement, trace off white argillaceous matrix, moderately hard to hard, minor very hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: brown grey, dark grey to medium dark grey, medium grey in part, arenaceous, argillaceous in part, minor locally common micro mica, moderately hard to hard, sub fissile to sub blocky. |
| 2313 | 2316 | Trace 100 | SANDSTONE: light grey to medium light grey, off white, translucent, fine to minor very fine, moderately to moderately well sorted, sub angular to sub rounded, strong siliceous cement, trace off white argillaceous matrix, moderately hard to hard, minor very hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey, medium grey to medium dark grey, dark grey brown, dominantly argillaceous, minor arenaceous, trace micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to sub fissile. |
| 2316 | 2319 | Trace 100 | SANDSTONE: light grey to off white, translucent in part, fine to minor medium, occasionally very fine, moderately well sorted, sub angular, strong siliceous cement, trace off white argillaceous matrix, moderately hard to hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark grey brown, medium grey in part, dark grey brown, dominantly argillaceous, minor arenaceous, occasional micro mica, moderately hard to hard, sub blocky to minor blocky. |
| 2319 | 2322 | 5 95 | SANDSTONE: light to medium grey, occasionally off white, fine to minor medium, occasionally very fine, moderately well sorted, sub angular, strong siliceous cement, trace off white argillaceous matrix, moderately hard to hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to medium grey, dark grey brown, arenaceous to minor argillaceous, occasional micro mica, hard to occasionally moderately hard, sub blocky to minor sub fissile. |
| 2322 | 2325 | Trace 100 | SANDSTONE: light to medium grey, occasionally off white, fine to minor medium, occasionally very fine, moderately well sorted, sub angular, strong siliceous cement, trace off white argillaceous matrix, moderately hard to hard, occasional disaggregated grains, tight to very poor visual porosity, very poor inferred porosity, no fluorescence. SILTSTONE: dark to medium grey, dark grey brown in part, dominantly arenaceous, occasional micro mica, minor pyrite nodules, hard to occasionally moderately hard, sub blocky to minor sub fissile. |
| 2325 | 2328 | 60 40 | SANDSTONE: off white to light grey, very fine to minor medium, moderately sorted, sub angular to sub rounded, moderate to strong calcareous cement, occasional off white argillaceous matrix, dominantly disaggregated grains, occasionally friable to moderately hard, very poor to tight visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium grey, dark grey brown in part, dominantly arenaceous, occasional micro mica, minor pyrite nodules, hard to occasionally moderately hard, sub blocky to minor sub fissile. |
| 2328 | 2331 | 40 60 | SANDSTONE: off white to light grey, very fine to fine, moderately sorted, sub angular to occasionally sub rounded, moderate to strong calcareous and siliceous cement, occasional off white argillaceous matrix, dominantly disaggregated grains, occasionally friable to moderately hard, very poor to tight visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium grey, dark grey brown in part, arenaceous, grading to very fine Sandstone in part, occasional micro mica, rare pyrite nodules, hard to occasionally moderately hard, sub blocky to minor sub fissile. |
| 2331 | 2334 | 50 50 | SANDSTONE: off white to mottled light grey, very fine to fine, moderately sorted, sub angular to occasionally sub rounded, moderate to strong calcareous and siliceous cement, occasional off white argillaceous matrix, occasional localised dark lithics, dominantly disaggregated grains, occasionally friable to moderately hard, very poor to tight visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: dark to medium grey, dark grey brown in part, arenaceous, grading to very fine Sandstone in part, occasional micro mica, rare pyrite nodules, hard to occasionally moderately hard, sub blocky to minor sub fissile. |
| 2334 | 2337 | 40 60 | SANDSTONE: light grey, minor medium grey, very fine, grading to arenaceous Siltstone in part, well sorted, sub angular to occasionally sub rounded, moderate to strong calcareous and siliceous cement, occasional off white argillaceous matrix, locally common dark specks, disaggregated grains, dominantly friable to moderately hard, very poor to tight visual porosity, poor inferred porosity, no fluorescence. SILTSTONE: medium grey, dark grey in part, arenaceous, commonly grading to very fine Sandstone, occasional micro mica, rare pyrite nodules, hard to occasionally moderately hard, sub blocky to minor sub fissile. |
| 2337 | 2340 | 30 70 | SANDSTONE: light grey, minor medium grey, very fine to rare fine, grading to arenaceous Siltstone in part, well sorted, sub angular to occasionally sub rounded, moderate to strong siliceous cement, occasional off white argillaceous matrix, locally common dark specks, dominantly hard, very poor to tight visual porosity, no fluorescence. SILTSTONE: light to dark grey, arenaceous, commonly grading to very fine Sandstone, siliceous, occasional micro mica, rare pyrite nodules, hard, sub blocky to minor sub fissile. |
| 2340 | 2343 | 10 90 | SANDSTONE: light grey, minor medium grey, very fine to rare fine, grading to arenaceous Siltstone in part, well sorted, sub angular to occasionally sub rounded, moderate to strong calcareous cement, occasional off white argillaceous matrix, locally common dark specks, dominantly hard, very poor to tight visual porosity, no fluorescence. SILTSTONE: medium to light grey, occasionally dark grey, arenaceous, commonly grading to very fine Sandstone, siliceous, occasional micro mica, hard, sub blocky to minor sub fissile. |
| 2343 | 2346 | 10 90 | SANDSTONE: light grey, minor medium grey, very fine to rare fine, grading to arenaceous Siltstone in part, well sorted, sub angular to occasionally sub rounded, moderate to strong calcareous cement, occasional off white argillaceous matrix, locally common dark specks, dominantly hard, very poor to tight visual porosity, no fluorescence. SILTSTONE: light to medium grey, minor dark grey, arenaceous, commonly grading to very fine Sandstone, siliceous, occasional micro mica, hard, sub blocky to blocky. |
| 2346 | 2349 | 5 95 | SANDSTONE: light grey, minor medium grey, very fine to rare fine, grading to arenaceous Siltstone in part, well sorted, sub angular to occasionally sub rounded, moderate calcareous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, very poor to tight visual porosity, no fluorescence. SILTSTONE: light to medium grey, minor dark grey, arenaceous, commonly grading to very fine Sandstone, siliceous, rare micro mica, hard, sub blocky to blocky. |
| 2349 | 2352 | 20 80 | SANDSTONE: light grey, minor medium grey, minor clear to translucent, very fine to rare fine, grading to arenaceous Siltstone in part, well sorted, sub angular to angular, moderate calcareous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, very poor to tight visual porosity, no fluorescence. SILTSTONE: light to dark grey, arenaceous, commonly grading to very fine Sandstone, siliceous, locally common micro mica, hard, sub blocky to blocky. |
| 2352 | 2355 | 10 90 | SANDSTONE: light grey, minor medium grey, minor clear to translucent, very fine to fine, occasionally medium, minor grading to arenaceous Siltstone in part, well sorted, sub angular to angular, moderate calcareous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, common disaggregated grains, very poor to tight visual porosity, no fluorescence. SILTSTONE: medium to dark grey, occasionally light grey, arenaceous, commonly grading to very fine Sandstone, siliceous, locally common micro mica, hard, sub blocky to blocky. |
| 2355 | 2358 | 5 95 | SANDSTONE: light grey, minor medium grey, minor clear to translucent, very fine to fine, occasionally medium, minor grading to arenaceous Siltstone in part, well sorted, sub angular to angular, moderate calcareous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, common disaggregated grains, very poor to tight visual porosity, no fluorescence. SILTSTONE: medium to occasionally dark grey, occasionally light grey, arenaceous, minor siliceous, occasional to common micro mica, hard to occasionally moderately hard, sub blocky to blocky. |
| 2358 | 2361 | 10 90 | SANDSTONE: light grey, minor medium grey, minor clear to translucent, fine to minor medium, well sorted, sub angular to angular, moderate siliceous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, minor disaggregated grains, poor to very poor visual porosity, no fluorescence. SILTSTONE: dark to medium grey, occasionally light grey, argillaceous to occasionally arenaceous, minor siliceous, occasional to common micro mica, hard to occasionally moderately hard, sub blocky to blocky. |
| 2361 | 2364 | 5 95 | SANDSTONE: light grey, minor medium grey, minor clear to translucent, fine to minor medium, well sorted, sub angular to angular, moderate to strong siliceous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, minor disaggregated grains, poor to very poor visual porosity, no fluorescence. SILTSTONE: dark to medium grey, occasionally light grey, argillaceous to occasionally arenaceous, minor siliceous, common micro mica, rare nodular pyrite, hard to occasionally moderately hard, sub blocky to blocky. |
| 2364 | 2367 | Trace 100 | SANDSTONE: light grey, minor medium grey, minor clear to translucent, fine to minor medium, well sorted, sub angular to angular, moderate to strong siliceous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, minor disaggregated grains, poor to very poor visual porosity, no fluorescence. SILTSTONE: dark to medium grey, minor very dark grey, argillaceous to minor arenaceous, minor siliceous, common micro mica, rare nodular pyrite, hard to occasionally moderately hard, sub blocky, rare blocky to sub fissile. |
| 2367 | 2370 | Trace 100 | SANDSTONE: light grey, minor medium grey, minor clear to translucent, fine to minor medium, well sorted, sub angular to angular, moderate to strong siliceous cement, minor off white argillaceous matrix, trace dark lithics, dominantly hard, minor disaggregated grains, poor to very poor visual porosity, no fluorescence. SILTSTONE: dark grey, minor medium to very dark grey, argillaceous to occasionally arenaceous, minor siliceous, common micro mica, trace nodular pyrite, hard to occasionally moderately hard, sub blocky, rare blocky to sub fissile. |
| 2370 | 2373 | 100 | SILTSTONE: dark grey, minor medium to very dark grey, argillaceous to occasionally arenaceous, minor siliceous, locally common micro mica, trace nodular pyrite, hard to occasionally moderately hard, sub blocky to blocky. |
| 2373 | 2376 | 100 | SILTSTONE: medium to occasionally dark brown grey, minor light to medium grey, arenaceous, minor siliceous, minor micro mica, trace nodular pyrite, hard to occasionally moderately hard, sub blocky to blocky. |
| 2376 | 2379 | 100 | SILTSTONE: medium to occasionally dark brown grey, minor light to medium grey, commonly arenaceous, rare grading to very fine Sandstone, minor argillaceous, minor siliceous, minor micro mica, rare nodular pyrite, hard to occasionally moderately hard, sub blocky to blocky. |
| 2379 | 2382 | 100 | SILTSTONE: medium to occasionally dark brown grey, minor light to medium grey, commonly arenaceous, rare grading to very fine SANDSTONE, minor argillaceous, minor siliceous, minor micro mica, minor dark specks, rare nodular pyrite, hard to occasionally moderately hard, sub blocky to blocky. |
| 2382 | 2385 | 100 | SILTSTONE: medium to occasionally dark brown grey, minor light to medium grey, commonly arenaceous, rare grading to very fine SANDSTONE, minor argillaceous, rare siliceous, minor micro mica, minor dark specks, rare nodular pyrite, hard to occasionally moderately hard, sub blocky to blocky. |
| 2385 | 2388 | 100 | SILTSTONE: dark grey, medium dark grey, trace grey brown, arenaceous trace grading to very fine SANDSTONE, argillaceous in part, rare siliceous common micro mica, hard to minor very hard, trace moderately hard, sub fissile to occasionally sub blocky. |
| 2388 | 2391 | 100 | SILTSTONE: dark grey, medium dark grey, trace grey brown, arenaceous trace grading to very fine SANDSTONE, argillaceous in part, common micro mica, hard to minor very hard, trace moderately hard, sub fissile to occasionally sub blocky. |
| 2391 | 2394 | 100 | SILTSTONE: dark grey, medium dark grey, trace grey brown, arenaceous trace grading to very fine SANDSTONE, argillaceous in part, common micro mica, hard to minor very hard, trace moderately hard, sub fissile to occasionally sub blocky. |
| 2394 | 2397 | 100 | SILTSTONE: dark grey, medium dark grey, trace light grey, trace grey brown, arenaceous trace grading to very fine SANDSTONE, argillaceous in part, common micro mica, hard to minor very hard, trace moderately hard, sub fissile to occasionally sub blocky. |
| 2397 | 2400 | Trace 100 | SANDSTONE: (Trace) off white to light grey, minor medium light grey, very fine commonly grading to arenaceous SILTSTONE, moderately well sorted, sub rounded, strong siliceous cement, off white argillaceous matrix, hard to very hard, nil to tight visual porosity, no fluorescence. SILTSTONE: dark grey to medium dark grey, medium to medium light grey in part, trace light grey, minor brownish grey, arenaceous trace grading to very fine SANDSTONE, minor argillaceous, trace black lithics, minor micro mica, moderately hard to hard, sub fissile to minor sub blocky. |
| 2400 | 2403 | Trace 100 | SANDSTONE: (Trace) off white to light grey, minor medium light grey, very fine commonly grading to arenaceous SILTSTONE, moderately well sorted, sub rounded, strong siliceous cement, off white argillaceous matrix, hard to very hard, nil to tight visual porosity, no fluorescence. SILTSTONE: medium grey to medium dark grey, dark grey in part, trace light grey, minor grey brown, arenaceous rare to trace grading to very fine SANDSTONE, occasional argillaceous, trace black lithics, minor locally common micro mica, moderately hard to hard, sub fissile to common fissile. |
| 2403 | 2406 | 100 | SILTSTONE: dark grey to medium dark grey, medium to medium light grey in part, trace light grey, minor brownish grey, arenaceous trace grading to very fine SANDSTONE, minor argillaceous, trace black lithics, minor micro mica, moderately hard to hard, sub fissile to minor sub blocky. |
| 2406 | 2409 | Trace 100 | SANDSTONE: (Trace) off white to light grey, minor medium light grey, very fine commonly grading to arenaceous SILTSTONE, moderately well sorted, sub rounded, strong siliceous cement, off white argillaceous matrix, hard to very hard, nil to tight visual porosity, no fluorescence. SILTSTONE: medium grey to medium dark grey, dark grey in part, trace light grey, minor grey brown, arenaceous rare to trace grading to very fine SANDSTONE, occasionally argillaceous, trace black lithics, minor locally common micro mica, moderately hard to hard, sub fissile to common fissile. |
| 2409 | 2412 | 100 | SILTSTONE: medium grey, medium dark grey, dark grey in part, trace light grey, minor grey brown, arenaceous rare to trace grading to very fine SANDSTONE, occasionally argillaceous, trace black lithics, minor locally common micro mica, moderately hard to hard, sub fissile to common fissile. |
| 2412 | 2415 | 100 | SILTSTONE: dark grey to very dark grey, medium grey, trace light grey, minor grey brown, arenaceous trace grading to very fine SANDSTONE, argillaceous in part, common micro mica, trace black lithics, moderately hard to hard, sub blocky to sub fissile. |
| 2415 | 2418 | 100 | SILTSTONE: light brown grey to brownish grey, dark grey, medium grey in part, trace light grey, common argillaceous, arenaceous in part, micro mica in part, moderately hard to hard, sub blocky to minor sub fissile. |
| 2418 | 2421 | 100 | SILTSTONE: dark grey brown, dark to occasionally medium grey, minor light grey, arenaceous, argillaceous in part, common micro mica, trace localised lithics, moderately hard to dominantly hard, sub blocky to minor sub fissile. |
| 2421 | 2424 | Trace 100 | SANDSTONE: (Trace) off white to light grey, very fine, well sorted, sub angular to sub rounded, moderately to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace lithics, moderately hard, tight visual porosity, no fluorescence. SILTSTONE: dark grey brown, dark to occasionally medium grey, minor light grey, arenaceous, argillaceous in part, common micro mica, trace localised lithics, moderately hard to dominantly hard, sub blocky to minor sub fissile. |
| 2424 | 2427 | Trace 100 | SANDSTONE: (Trace) off white to light grey, very fine to occasionally fine, well sorted, sub angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace lithics, moderately hard, tight visual porosity, no fluorescence. SILTSTONE: dark to occasionally medium grey, dark grey brown in part, minor light grey, argillaceous to dominantly arenaceous, locally common micro mica, trace localised lithics, moderately hard to dominantly hard, sub blocky to minor sub fissile. |
| 2427 | 2430 | Trace 100 | SANDSTONE: (Trace) off white to light grey, very fine to occasionally fine, well sorted, angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. SILTSTONE: dark to medium grey, minor light grey, arenaceous, rare argillaceous, locally common micro mica, trace localised lithics, moderately hard to dominantly hard, sub blocky to minor sub fissile. |
| 2430 | 2433 | 5 95 | SANDSTONE: (Trace) off white to light grey, very fine to occasionally fine, grading to arenaceous Siltstone in part, well sorted, angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. SILTSTONE: dark to medium grey, minor light grey, arenaceous, grading to very fine Sandstone in part, rare argillaceous, locally common micro mica, trace localised lithics, moderately hard to dominantly hard, sub blocky to minor sub fissile. |
| 2433 | 2436 | 100 | SILTSTONE: dark grey, occasionally medium grey, minor light grey, arenaceous, grading to very fine Sandstone in part, rare argillaceous, locally common micro mica, minor pyrite nodules, trace localised lithics, moderately hard to dominantly hard, sub blocky to minor sub fissile. |
| 2436 | 2439 | 100 | SILTSTONE: dark grey, minor medium grey, rare dark grey brown, arenaceous, grading to very fine Sandstone in part, rare argillaceous, dominantly micro micaceous, minor pyrite nodules, trace localised lithics, moderately hard to dominantly hard, sub blocky to sub fissile. |
| 2439 | 2442 | 100 Trace | SILTSTONE: dark grey, minor medium grey, rare dark grey brown, argillaceous to arenaceous, dominantly micro micaceous, minor pyrite nodules, trace localised lithics, moderately hard to dominantly hard, sub blocky to sub fissile. SANDSTONE: (Trace) light to occasionally medium grey, off white in part, very fine to occasionally fine, grading to arenaceous Siltstone in part, well sorted, angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2442 | 2445 | 100 Trace | SILTSTONE: dark grey, minor medium grey, rare dark grey brown, argillaceous to dominantly arenaceous, occasionally to locally common micro micaceous, minor pyrite nodules, trace localised lithics, moderately hard to dominantly hard, sub blocky to sub fissile. SANDSTONE: (Trace) light to occasionally medium grey, off white in part, very fine to occasionally fine, commonly grading to arenaceous Siltstone, well sorted, angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2445 | 2448 | 95 5 | SILTSTONE: dark to medium grey, rare dark grey brown, argillaceous to dominantly arenaceous, occasionally to locally common micro micaceous, minor pyrite nodules, trace localised lithics, moderately hard to dominantly hard, sub blocky to sub fissile. SANDSTONE: light grey brown, off white, very fine, commonly grading to arenaceous Siltstone, well sorted, angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2448 | 2451 | 100 Trace | SILTSTONE: medium to dark grey, dark grey brown, argillaceous to dominantly arenaceous, occasionally to locally common micro micaceous, minor pyrite nodules, trace localised lithics, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: (Trace) light grey brown, off white, very fine, commonly grading to arenaceous Siltstone, well sorted, angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2451 | 2454 | 100 Trace | SILTSTONE: medium to dark grey, dark grey brown, argillaceous to dominantly arenaceous, occasionally to locally common micro micaceous, minor mica flakes, minor pyrite nodules, trace localised lithics, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: (Trace) light grey brown, off white, light grey in part, very fine, commonly grading to arenaceous Siltstone, well sorted, angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2454 | 2457 | 100 Trace | SILTSTONE: medium grey brown, medium to dark grey, argillaceous to dominantly arenaceous, occasionally to locally common micro micaceous, minor mica flakes, trace lithics, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: (Trace) off white, light grey brown, light grey in part, very fine, commonly grading to arenaceous Siltstone, well sorted, angular to minor sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2457 | 2460 | 95 5 | SILTSTONE: medium grey brown, medium to dark grey, argillaceous to dominantly arenaceous, occasionally to locally common micro micaceous, minor mica flakes, rare pyrite nodules, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. SANDSTONE: off white to light grey, light grey brown, very fine, commonly grading to arenaceous Siltstone, well sorted, angular, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2460 | 2463 | 100 Trace | SILTSTONE: medium grey brown, medium to dark grey, argillaceous to dominantly arenaceous, occasionally to locally common micro micaceous, minor mica flakes, rare pyrite nodules, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. SANDSTONE: (Trace) medium grey, occasionally light to dark grey, very fine, commonly grading to arenaceous Siltstone, well sorted, angular, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, trace dark specks, moderately hard, tight visual porosity, no fluorescence. |
| 2463 | 2466 | 100 Trace | SILTSTONE: medium to dark grey, medium grey brown, argillaceous to arenaceous, occasionally to locally common micro micaceous, minor mica flakes, rare pyrite nodules, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. SANDSTONE: (Trace) medium grey, occasionally light grey, very fine, commonly grading to arenaceous Siltstone, well sorted, angular, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. |
| 2466 | 2469 | 100 Trace | SILTSTONE: medium to dark grey, medium grey brown, argillaceous to arenaceous, occasionally grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, trace disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. SANDSTONE: (Trace) medium grey, occasionally light grey, very fine, commonly grading to arenaceous Siltstone, well sorted, angular, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. |
| 2469 | 2472 | 100 Trace | SILTSTONE: medium to dark grey, light grey, light grey brown, argillaceous to arenaceous, occasionally grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, trace disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. SANDSTONE: (Trace) medium grey, occasionally light grey, very fine, commonly grading to arenaceous Siltstone, well sorted, angular, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. |
| 2472 | 2475 | 95 5 | SILTSTONE: light to medium grey, light grey brown, arenaceous, commonly grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, trace disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. SANDSTONE: off white to light grey, very fine, commonly grading to arenaceous Siltstone, well sorted, sub angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. |
| 2475 | 2478 | 100 | SILTSTONE: light to medium grey, light grey brown, arenaceous, commonly grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, trace disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. |
| 2478 | 2481 | 100 | SILTSTONE: light to medium grey, light grey brown, arenaceous, commonly grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, trace disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. |
| 2481 | 2484 | 5 95 | SANDSTONE: off white to light grey, very fine, commonly grading to arenaceous Siltstone, well sorted, sub angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. SILTSTONE: light to medium grey, light grey brown, arenaceous, commonly grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, rare disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. |
| 2484 | 2487 | 5 95 | SANDSTONE: off white to light grey, very fine, commonly grading to arenaceous Siltstone, well sorted, sub angular to sub rounded, moderate to occasionally strong siliceous cement, minor to occasional off white argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. SILTSTONE: light to medium grey, light grey brown, arenaceous, commonly grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, trace disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. |
| 2487 | 2490 | 100 | SILTSTONE: medium to dark grey, light grey brown, arenaceous, commonly grading to very fine Sandstone, occasionally to locally common micro micaceous, minor mica flakes, trace disseminated pyrite, trace lithics, moderately hard to hard, sub blocky to sub fissile, minor fissile. |
| 2490 | 2493 | 100 | SILTSTONE: very dark grey to dark grey, medium grey to trace medium light grey, arenaceous trace grading to very fine Sandstone, argillaceous, common micro mica, minor locally common disseminated pyrite, moderately hard, hard to trace very hard, sub blocky to minor sub fissile. |
| 2493 | 2496 | 100 | SILTSTONE: very dark grey to dark grey, medium grey to trace medium light grey, arenaceous trace grading to very fine Sandstone, argillaceous, common micro mica, minor locally common disseminated pyrite, moderately hard, hard to trace very hard, sub blocky to minor sub fissile. |
| 2496 | 2499 | 5 95 | SANDSTONE: light grey to off white, very fine grading to arenaceous Siltstone, moderately well sorted, sub rounded, strong siliceous cement, off white argillaceous matrix, hard to very hard, nil visual porosity, no fluorescence. SILTSTONE: very dark grey to dark grey, medium grey to trace medium light grey, arenaceous trace grading to very fine Sandstone, argillaceous, common micro mica, minor locally common disseminated pyrite, moderately hard, hard to trace very hard, sub blocky to minor sub fissile. |
| 2499 | 2502 | 100 | SILTSTONE: very dark grey to dark grey, medium grey to trace medium light grey, arenaceous trace grading to very fine Sandstone, argillaceous, common micro mica, minor locally common disseminated pyrite, moderately hard, hard to trace very hard, sub blocky to minor sub fissile. |
| 2502 | 2505 | 100 | SILTSTONE: dark grey to medium dark grey, medium grey, trace grey brown, arenaceous, common argillaceous, locally common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, sub fissile in part. |
| 2505 | 2508 | 100 | SILTSTONE: dark grey to medium dark grey, medium grey, trace medium light grey, trace grey brown, arenaceous, common argillaceous, locally common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, sub fissile in part. |
| 2508 | 2511 | 95 5 | SILTSTONE: dark to minor medium grey, trace grey brown, arenaceous, commonly argillaceous, locally common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, sub fissile in part. SANDSTONE: off white to light grey brown, very fine grading to arenaceous Siltstone, well sorted, sub angular, strong to moderately siliceous cement, rare off white argillaceous matrix, hard to very hard, tight visual porosity, no fluorescence. |
| 2511 | 2514 | 95 5 | SILTSTONE: dark to minor light grey, rare grey brown, arenaceous, commonly argillaceous, grading to Claystone in part, locally common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, sub fissile in part. SANDSTONE: off white to light grey brown, very fine grading to arenaceous Siltstone, well sorted, sub angular, strong to moderately siliceous cement, rare off white argillaceous matrix, hard to very hard, tight visual porosity, no fluorescence. |
| 2514 | 2517 | 100 Trace | SILTSTONE: medium grey, occasionally dark to minor light grey, rare grey brown, arenaceous, commonly argillaceous, grading to Claystone in part, locally common micro mica, trace disseminated pyrite, moderately hard to dominantly hard, sub blocky, sub fissile in part. SANDSTONE: off white to light grey brown, very fine grading to arenaceous Siltstone, well sorted, sub angular, strong to moderately siliceous cement, rare off white argillaceous matrix, hard to very hard, tight visual porosity, no fluorescence. |
| 2517 | 2520 | 95 5 | SILTSTONE: medium grey, minor dark to light grey, arenaceous to minor argillaceous, grading to Claystone in part, locally common micro mica, trace disseminated pyrite, moderately hard to dominantly hard, sub blocky, sub fissile in part. SANDSTONE: off white to light grey brown, very fine to occasionally fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderately siliceous cement, rare off white argillaceous matrix, hard to very hard, tight visual porosity, no fluorescence. |
| 2520 | 2523 | 100 Trace | SILTSTONE: medium grey, minor dark to light grey, arenaceous to minor argillaceous, minor grading to Claystone in part, locally common micro mica, trace disseminated pyrite, moderately hard to dominantly hard, sub fissile, occasionally sub blocky to fissile. SANDSTONE: (Trace) off white to light grey brown, very fine to occasionally fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous cement, rare off white argillaceous matrix, hard to very hard, tight visual porosity, no fluorescence. |
| 2523 | 2526 | 95 5 | SILTSTONE: medium grey, minor dark to light grey, arenaceous to minor argillaceous, minor grading to Claystone in part, locally common micro mica, trace disseminated pyrite, moderately hard to dominantly hard, sub fissile, occasionally sub blocky to fissile. CLAYSTONE: very dark grey, minor very dark grey brown, silty in part, minor micro mica, moderately hard, occasionally firm to hard, sub fissile to occasionally fissile. |
| 2526 | 2529 | 95 5 | SILTSTONE: dark to medium grey, rare light grey, dominantly argillaceous, arenaceous in part, minor grading to Claystone in part, locally common micro mica, trace disseminated pyrite, moderately hard to dominantly hard, sub fissile, occasionally sub blocky to fissile. CLAYSTONE: very dark grey, minor very dark grey brown, silty in part, minor micro mica, rare disseminated pyrite, moderately hard, occasionally firm to hard, sub fissile to occasionally fissile. |
| 2529 | 2532 | 100 | SILTSTONE: light to medium grey, commonly arenaceous, grading to very fine Sandstone in part, argillaceous, in part, locally common micro mica, trace disseminated pyrite, moderately hard to dominantly hard, sub fissile, occasionally sub blocky to fissile. |
| 2532 | 2535 | 100 | SILTSTONE: dark to occasionally very dark grey, occasionally medium grey, argillaceous to arenaceous, grading to very fine Sandstone in part, locally micro micaceous, trace disseminated pyrite, moderately hard to hard, sub fissile, occasionally sub blocky to fissile. |
| 2535 | 2538 | 90 10 | SILTSTONE: dark to occasionally very dark grey, occasionally medium grey, argillaceous to dominantly arenaceous, grading to very fine Sandstone in part, locally micro micaceous, trace disseminated pyrite, locally common dark specks, moderately hard to hard, sub fissile, occasionally sub blocky to fissile. SANDSTONE: light grey, very fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous cement, rare off white argillaceous matrix, hard to very hard, tight visual porosity, no fluorescence. |
| 2538 | 2541 | 100 Trace | SILTSTONE: dark to occasionally very dark grey, occasionally medium grey, argillaceous to arenaceous, grading to very fine Sandstone in part, locally micro micaceous, locally common disseminated pyrite, locally common dark specks, moderately hard to hard, sub fissile, occasionally sub blocky to fissile. SANDSTONE: (Trace) light grey, very fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous cement, rare off white argillaceous matrix, hard to very hard, tight visual porosity, no fluorescence. |
| 2541 | 2544 | 80 20 | SILTSTONE: dark to occasionally very dark grey, occasionally medium grey, dominantly arenaceous, grading to very fine Sandstone, locally micro micaceous, locally common disseminated pyrite, locally common dark specks, moderately hard to hard, sub fissile, occasionally sub blocky to fissile. SANDSTONE: medium to dark mottled grey, light grey, very fine to minor fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, disaggregated grains, hard to very hard, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2544 | 2547 | 90 10 | SILTSTONE: dark grey, minor medium grey, dominantly arenaceous, commonly grading to very fine Sandstone, locally micro micaceous, locally common disseminated pyrite, locally common dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to dark mottled grey, medium to dark brown grey in part, very fine to fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, disaggregated grains, hard to very hard, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2547 | 2550 | 90 10 | SILTSTONE: mottled dark to medium grey, minor very dark grey, dominantly arenaceous, commonly grading to very fine Sandstone, locally common micro micaceous, locally common disseminated pyrite, locally common dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to dark mottled grey, medium to dark brown grey in part, very fine to fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, disaggregated grains, hard to very hard, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2550 | 2553 | 90 10 | SILTSTONE: medium to dark grey, minor very dark grey, argillaceous to dominantly arenaceous, grading to very fine Sandstone, locally common micro micaceous, locally common disseminated pyrite, locally common dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to dark mottled grey, medium to dark brown grey in part, very fine to fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, disaggregated grains, hard to very hard, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2553 | 2556 | 80 20 | SILTSTONE: medium to dark grey, minor very dark grey, dominantly arenaceous, commonly grading to very fine Sandstone, locally common micro micaceous, rare disseminated pyrite, minor dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to dark mottled grey, medium to dark brown grey in part, clear to translucent in part, very fine to minor fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, disaggregated grains, hard to very hard, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2556 | 2559 | 95 5 | SILTSTONE: medium grey, minor light to rare dark grey, dominantly arenaceous, commonly grading to very fine Sandstone, locally common micro micaceous, minor nodular and disseminated pyrite, minor dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to dark mottled grey, medium to dark brown grey in part, clear to translucent in part, very fine to minor fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, disaggregated grains, hard to very hard, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2559 | 2562 | 80 20 | SILTSTONE: medium grey, minor light to rare dark grey, dominantly arenaceous, commonly grading to very fine Sandstone, rare argillaceous, locally micro micaceous, minor nodular and disseminated pyrite, minor dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to dark mottled grey, light brown grey in part, very fine to fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous cement, rare off white argillaceous matrix, common dark specks, common disaggregated grains, hard to very hard in part, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2562 | 2565 | 80 20 | SILTSTONE: medium to dark grey, minor light grey, dominantly arenaceous, commonly grading to very fine Sandstone, argillaceous in part, locally micro micaceous, minor dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to occasionally dark mottled brown grey, very fine to fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, common disaggregated grains, hard to very hard in part, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2565 | 2568 | 90 10 | SILTSTONE: dark to medium grey, minor medium grey brown grey, dominantly arenaceous, argillaceous in part, locally micro micaceous, occasional dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to occasionally dark mottled brown grey, very fine to fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, common disaggregated grains, hard to very hard in part, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2568 | 2571 | 30 65 5 | CLAYSTONE: very dark grey to minor grey black, minor dark grey, silty in part, occasional to common disseminated pyrite, minor micro micaceous, firm to moderately hard, sub blocky to occasionally sub fissile. SILTSTONE: dark to medium grey, minor medium grey brown grey, dominantly arenaceous, argillaceous in part, locally micro micaceous, occasional dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. SANDSTONE: medium to occasionally dark mottled brown grey, very fine to fine, grading to arenaceous Siltstone in part, well sorted, sub angular, strong to moderate siliceous and minor calcareous cement, rare off white argillaceous matrix, common dark specks, common disaggregated grains, hard to very hard in part, tight visual porosity, very poor inferred porosity, no fluorescence. |
| 2571 | 2574 | 70 30 | CLAYSTONE: very dark grey to grey black, minor dark grey, silty in part, occasional to common disseminated pyrite, micro micaceous in part, firm to moderately hard, sub blocky to occasionally sub fissile. SILTSTONE: dark to medium grey, minor medium grey brown, argillaceous to occasionally arenaceous, locally micro micaceous, occasional dark specks, moderately hard to hard, sub blocky to occasionally sub fissile. |
| 2574 | 2577 | 60 40 | CLAYSTONE: dark to medium grey, occasionally very dark grey, silty in part, occasional to common disseminated pyrite, micro micaceous in part, firm to moderately hard, occasionally hard to very hard, sub blocky to occasionally sub fissile. SILTSTONE: dark to medium grey, minor medium grey brown, argillaceous to occasionally arenaceous, locally micro micaceous, moderately hard to hard, sub blocky to occasionally sub fissile. |
| 2577 | 2580 | 70 30 | CLAYSTONE: very dark to medium grey, occasionally light grey, silty in part, occasional to common disseminated pyrite, micro micaceous in part, firm to moderately hard, occasionally hard to very hard, sub blocky to occasionally sub fissile. SILTSTONE: dark to medium grey, minor medium grey brown, argillaceous to occasionally arenaceous, locally micro micaceous, moderately hard to hard, sub blocky to occasionally sub fissile. |
| 2580 | 2583 | 50 50 | CLAYSTONE: dark to medium grey, occasionally light grey, silty in part, occasional to common disseminated pyrite, micro micaceous in part, moderately hard to hard, minor very hard, sub blocky to occasionally sub fissile. SILTSTONE: medium grey, occasionally dark grey, minor medium grey brown, argillaceous to occasionally arenaceous, locally micro micaceous, moderately hard to hard, sub blocky to occasionally sub fissile. |
| 2583 | 2586 | 20 80 | CLAYSTONE: dark to medium grey, occasionally light grey, silty in part, minor disseminated pyrite, micro micaceous in part, moderately hard to hard, minor very hard, sub blocky to occasionally sub fissile. SILTSTONE: medium grey, occasionally light to dark grey, minor medium grey brown, arenaceous, grading to very fine Sandstone in part, minor argillaceous, locally micro micaceous, moderately hard to dominantly hard, sub blocky, minor sub fissile. |
| 2586 | 2589 | 20 80 | CLAYSTONE: very dark grey to dark grey, trace grey black, trace silty, minor disseminated pyrite, trace micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: dark to medium dark grey, medium grey in part, trace dark brown, arenaceous, common argillaceous, locally common micro mica, moderately hard to hard, sub blocky to minor sub fissile. |
| 2589 | 2592 | 30 70 | CLAYSTONE: very dark grey to dark grey, trace grey black, trace silty, minor disseminated pyrite, trace micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: dark grey, medium to medium dark grey, trace dark brownish grey, arenaceous, common argillaceous trace grading to silty Claystone, trace micro mica, moderately hard to hard, dominantly sub blocky, trace sub fissile. |
| 2592 | 2595 | 50 50 | CLAYSTONE: dark grey to very dark grey, silty in part, minor disseminated pyrite, trace micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: dark grey, medium grey in part, trace dark brownish grey, common argillaceous trace grading to silty Claystone, arenaceous in part, trace micro mica, moderately hard to hard, dominantly sub blocky, trace sub fissile. |
| 2595 | 2598 | 10 90 | CLAYSTONE: dark grey to very dark grey, silty in part, occasional disseminated pyrite, trace micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: dark to medium grey, trace dark brownish grey, commonly argillaceous, minor grading to silty Claystone, arenaceous in part, occasionally micro micaceous, moderately hard to dominantly hard, sub blocky, trace sub fissile. |
| 2598 | 2601 | 40 60 | CLAYSTONE: very dark grey to dark grey, very dark grey brown, silty in part, occasional disseminated pyrite, trace micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: dark to medium grey, trace dark brownish grey, commonly argillaceous, minor grading to silty Claystone, arenaceous in part, occasionally micro micaceous, moderately hard to dominantly hard, sub blocky, trace sub fissile. |
| 2601 | 2604 | 40 60 | CLAYSTONE: very dark grey to dark grey, very dark grey brown, silty in part, minor disseminated and rare nodular pyrite, trace micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: dark to medium grey, trace dark brownish grey, dominantly argillaceous, commonly grading to Claystone, occasionally micro micaceous, moderately hard to dominantly hard, sub blocky, trace sub fissile. |
| 2604 | 2607 | 60 40 | CLAYSTONE: very dark grey, dark grey black in part, very dark grey brown, minor silty in part, minor disseminated and rare nodular pyrite, trace micro mica, moderately hard to hard, dominantly sub fissile to occasionally sub blocky. SILTSTONE: dark to medium grey, trace dark brownish grey, dominantly argillaceous, commonly grading to Claystone, occasionally micro micaceous, hard to occasionally very hard, minor moderately hard, sub blocky to sub fissile. |
| 2607 | 2610 | 60 40 | CLAYSTONE: very dark grey, dark grey black in part, very dark grey brown, minor silty in part, minor disseminated and rare nodular pyrite, trace micro mica, moderately hard to hard, dominantly sub fissile to occasionally sub blocky. SILTSTONE: dark to dominantly very dark grey, trace dark brownish grey, dominantly argillaceous, commonly grading to Claystone, occasionally micro micaceous, hard to occasionally very hard, minor moderately hard, sub blocky to sub fissile. |
| 2610 | 2613 | 80 20 | CLAYSTONE: very dark grey to black, dark grey black in part, very dark grey brown, minor silty in part, rare nodular pyrite, occasional to locally common micro mica, hard to occasionally very hard, dominantly sub fissile to occasionally sub blocky. SILTSTONE: dark to very dark grey, trace dark brown grey, minor medium grey, dominantly argillaceous, commonly grading to Claystone, occasionally micro micaceous, hard to occasionally very hard, minor moderately hard, sub blocky to sub fissile. |
| 2613 | 2616 | 70 30 | CLAYSTONE: very dark grey to black, dark grey black in part, very dark grey brown in part, locally common silty, rare nodular pyrite, occasional to locally common micro mica, hard to occasionally very hard, dominantly sub fissile to fissile in part. SILTSTONE: dark to very dark grey, commonly dark brown grey, minor medium grey, dominantly argillaceous, commonly grading to Claystone, occasionally micro micaceous, hard to occasionally very hard, minor moderately hard, sub fissile to occasionally sub blocky. |
| 2616 | 2619 | 100 | CLAYSTONE: very dark grey to black, dark grey black in part, very dark grey brown, locally occasionally silty, rare disseminated pyrite, occasional to locally common micro mica, hard to occasionally very hard, dominantly sub fissile to fissile in part. |
| 2619 | 2622 | 100 | CLAYSTONE: very dark grey to black, dark grey black in part, very dark grey brown, minor dark grey, locally occasionally silty, rare disseminated pyrite, occasional to locally common micro mica, hard to occasionally very hard, sub fissile to occasionally fissile, minor sub blocky. |
| 2622 | 2625 | 100 | CLAYSTONE: very dark brown grey to grey black, dark grey black in part, minor dark grey, rare disseminated pyrite, occasional to locally common micro mica, firm to moderately hard, occasionally hard, sub fissile to occasionally fissile, minor sub blocky. |
| 2625 | 2628 | 70 30 | CLAYSTONE: very dark brown grey, dark grey in part, rare disseminated pyrite, occasional to locally common micro mica, firm to moderately hard, occasionally hard, sub fissile to occasionally fissile, minor sub blocky. SILTSTONE: medium to occasionally light grey, minor dark grey, dominantly argillaceous, grading to Claystone in part, occasionally micro micaceous, hard to occasionally very hard, minor moderately hard, sub fissile to occasionally sub blocky. |
| 2628 | 2631 | 80 20 | CLAYSTONE: dark grey, dark brown grey in part, rare disseminated pyrite, occasional micro mica, firm to moderately hard, occasionally hard, sub blocky to sub fissile. SILTSTONE: medium to occasionally light grey, minor dark grey, dominantly argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, hard to occasionally very hard, minor moderately hard, sub fissile to occasionally sub blocky. |
| 2631 | 2634 | 70 30 | CLAYSTONE: dark to occasionally medium grey, rare very dark grey, rare disseminated pyrite, occasional micro mica, firm to moderately hard, occasionally hard, sub blocky to sub fissile. SILTSTONE: medium to occasionally light grey, minor dark grey, dominantly argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard to occasionally very hard, minor moderately hard, sub fissile to occasionally sub blocky. |
| 2634 | 2637 | 70 30 | CLAYSTONE: dark to occasionally medium grey, rare very dark grey, medium grey brown in part, rare disseminated pyrite, occasional micro mica, firm to moderately hard, occasionally hard, sub fissile to occasionally sub blocky. SILTSTONE: medium to occasionally light grey, minor dark grey, dominantly argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard to occasionally very hard, minor moderately hard, sub blocky. |
| 2637 | 2640 | 90 10 | CLAYSTONE: dark to light grey brown, dark to light grey, silty in part, rare disseminated pyrite, occasional micro mica, firm to moderately hard, occasionally hard, sub fissile to occasionally sub blocky. SILTSTONE: medium to occasionally light grey, minor dark grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard to occasionally very hard, minor moderately hard, sub blocky. |
| 2640 | 2643 | 90 10 | CLAYSTONE: medium to commonly dark grey, medium grey brown in part, silty in part, rare disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: medium to occasionally light grey, minor dark grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard, minor moderately hard, sub blocky. |
| 2643 | 2646 | 80 20 | CLAYSTONE: medium to commonly dark grey, medium grey brown in part, silty in part, occasionally grading to Siltstone, rare disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub fissile to occasionally sub blocky. SILTSTONE: medium to occasionally light grey, minor dark grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard, minor moderately hard, sub blocky, minor blocky. |
| 2646 | 2649 | 80 20 | CLAYSTONE: medium to commonly dark grey, medium grey brown in part, silty in part, commonly grading to Siltstone, rare disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub blocky to sub fissile. SILTSTONE: medium to occasionally dark grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard, minor moderately hard, sub blocky, minor blocky. |
| 2649 | 2652 | 90 10 | CLAYSTONE: dark grey, occasionally medium grey, medium grey brown in part, locally silty, rare disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub blocky to sub fissile. SILTSTONE: dark to medium grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard, minor moderately hard, sub blocky to occasionally blocky. |
| 2652 | 2655 | 95 5 | CLAYSTONE: dark grey, occasionally medium grey, medium grey brown in part, locally silty, rare localised disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub blocky to sub fissile. SILTSTONE: dark to medium grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard, minor moderately hard, sub blocky to occasionally blocky. |
| 2655 | 2658 | 80 20 | CLAYSTONE: light to medium brown grey, occasionally medium to rare dark grey, locally silty, rare localised disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub blocky to sub fissile. SILTSTONE: medium to dark grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard, minor moderately hard, sub blocky to occasionally blocky. |
| 2658 | 2661 | 90 10 | CLAYSTONE: light to dark grey, light to medium grey brown, locally silty, rare localised disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub blocky to sub fissile. SILTSTONE: medium to dark grey, argillaceous, grading to Claystone in part, minor siliceous, occasionally micro micaceous, minor disseminated pyrite, hard, minor moderately hard, sub blocky to occasionally blocky. |
| 2661 | 2664 | 90 10 | CLAYSTONE: dark grey, minor medium grey, locally silty, rare localised disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub blocky to occasionally sub fissile. SILTSTONE: medium to dark grey, argillaceous, grading to Claystone in part, minor siliceous, locally common micro micaceous, minor disseminated pyrite, hard to occasionally moderately hard, sub blocky to occasionally blocky. |
| 2664 | 2667 | 90 10 | CLAYSTONE: dark grey, minor medium grey, locally silty, rare localised disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, sub blocky to occasionally sub fissile, minor blocky. SILTSTONE: medium to dark grey, argillaceous, grading to Claystone in part, minor siliceous, locally common micro micaceous, minor disseminated pyrite, hard to occasionally moderately hard, sub blocky to commonly blocky. |
| 2667 | 2670 | 80 20 | CLAYSTONE: dark grey, minor medium grey, medium grey brown in part, locally silty, rare localised disseminated pyrite, occasional to locally common micro mica, moderately hard to hard, minor firm, sub blocky to occasionally sub fissile, minor blocky. SILTSTONE: medium to dark grey, argillaceous, dominantly grading to Claystone, minor siliceous, locally common micro micaceous, minor disseminated pyrite, hard to occasionally moderately hard, sub blocky to commonly blocky. |
| 2670 | 2673 | 90 10 | CLAYSTONE: dark grey, minor silty, rare localised disseminated pyrite, minor micro mica, moderately hard to hard, sub blocky to occasionally sub fissile, minor blocky. SILTSTONE: medium to dark grey, argillaceous, dominantly grading to Claystone, locally common micro micaceous, hard to occasionally moderately hard, sub blocky to commonly blocky. |
| 2673 | 2676 | 80 20 | CLAYSTONE: dark to minor medium grey, minor silty, rare localised disseminated pyrite, minor micro mica, moderately hard to hard, sub blocky to occasionally sub fissile, minor blocky. SILTSTONE: medium to dark grey, argillaceous, dominantly grading to Claystone, locally common micro micaceous, minor mica flakes, hard to occasionally moderately hard, sub blocky to commonly blocky. |
| 2676 | 2679 | 70 30 | CLAYSTONE: dark to minor medium grey, minor silty, rare localised disseminated pyrite, minor micro mica, moderately hard to hard, sub blocky to occasionally sub fissile, minor blocky. SILTSTONE: light to medium grey, arenaceous in part, grading to very fine Sandstone in part, locally common micro micaceous, minor mica flakes, rare disseminated pyrite, hard to occasionally moderately hard, sub blocky to commonly blocky. |
| 2679 | 2682 | 20 80 | SILTSTONE: medium grey to medium light grey, argillaceous trace grading to silty Claystone, common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. CLAYSTONE: dark to minor medium grey, minor silty, rare localised disseminated pyrite, minor micro mica, moderately hard to hard, sub blocky to occasionally sub fissile, minor blocky. |
| 2682 | 2685 | 90 10 | CLAYSTONE: dark to minor medium grey, minor silty, rare localised disseminated pyrite, minor micro mica, moderately hard to hard, sub blocky to occasionally sub fissile, minor blocky. SILTSTONE: medium grey to medium light grey, argillaceous trace grading to silty Claystone, common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. |
| 2685 | 2688 | 80 20 | CLAYSTONE: dark to medium grey, occasional to minor silty, minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. SILTSTONE: medium grey to medium light grey, argillaceous trace grading to silty Claystone, common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. |
| 2688 | 2691 | 80 20 | CLAYSTONE: dark to medium grey, occasional to minor silty, minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. SILTSTONE: medium grey, medium light grey, argillaceous grading to silty Claystone, common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. |
| 2691 | 2694 | 90 10 | CLAYSTONE: dark grey, trace medium dark grey, minor silty, minor mica, rare nodular pyrite, moderately hard to hard, sub blocky to trace sub fissile. SILTSTONE: medium grey, medium light grey, argillaceous trace grading to silty Claystone, common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. |
| 2694 | 2697 | 90 10 | CLAYSTONE: dark grey, trace medium dark grey, minor silty, minor mica, rare nodular pyrite, moderately hard to hard, sub blocky to trace sub fissile. SILTSTONE: medium to medium light grey, argillaceous minor grading to silty Claystone, trace arenaceous, minor micro mica, moderately hard to hard, sub blocky, trace sub fissile. |
| 2697 | 2700 | 100 | CLAYSTONE: dark grey, trace medium dark grey, minor silty, minor mica, rare nodular pyrite, moderately hard to hard, sub blocky to trace sub fissile. |
| 2700 | 2703 | 90 10 | CLAYSTONE: dark grey, trace medium dark grey, minor silty, minor mica, rare nodular pyrite, moderately hard to hard, sub blocky to trace sub fissile. SILTSTONE: medium to medium light grey, trace light grey, argillaceous minor grading to silty Claystone, trace arenaceous, minor micro mica, moderately hard to hard, sub blocky, trace sub fissile. |
| 2703 | 2706 | 95 5 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. SILTSTONE: medium to medium light grey, trace medium dark grey, dominantly argillaceous, grading to silty CLAYSTONE in part, trace micro mica, moderately hard to hard, sub blocky to trace sub fissile. |
| 2706 | 2709 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. |
| 2709 | 2712 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. |
| 2712 | 2715 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. |
| 2715 | 2718 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. |
| 2718 | 2721 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. |
| 2721 | 2724 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. |
| 2724 | 2727 | 90 10 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. LIMESTONE: off white to very light grey, soft to firm, dominantly lutitic, argillaceous in part, microcrystalline, trace disseminated pyrite. |
| 2727 | 2730 | 95 5 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace very dark grey, silty in part, minor micro mica, moderately hard to hard, trace firm, sub blocky, trace sub fissile. LIMESTONE: off white to very light grey, soft to firm, dominantly lutitic, argillaceous in part, microcrystalline, trace disseminated pyrite. |
| 2730 | 2733 | 100 trace trace | CLAYSTONE: dark grey to medium dark grey, silty in part, micro micaceous, moderately hard to hard, trace firm, sub blocky, trace sub fissile. SILTSTONE: medium grey to medium dark grey, moderately hard to hard, sub blocky to sub fissile, argillaceous in part. LIMESTONE: off white to very light grey, soft to firm, dominantly lutitic, argillaceous in part, microcrystalline, trace disseminated pyrite. |
| 2733 | 2736 | 100 trace trace | CLAYSTONE: dark grey, medium dark grey in part, silty in part, micro micaceous, moderately hard to hard, trace firm, sub blocky, trace sub fissile. LIMESTONE: off white to very light grey, soft to firm, dominantly lutitic, argillaceous in part, microcrystalline, trace disseminated pyrite. SANDSTONE: off white to very light grey, predominantly quartz grains, rounded to well-rounded grains, moderately sorted, well cemented, hard, poor inferred porosity, no fluorescence. |
| 2736 | 3739 | 100 | CLAYSTONE: dark grey, minor medium dark grey to medium grey in part, commonly micro micaceous, trace carbonaceous specks, arenaceous in part, firm to hard, sub blocky to sub fissile. |
| 3739 | 2742 | 100 | CLAYSTONE: medium dark grey to dark grey, common micro mica, trace carbonaceous specks, arenaceous in part, hard, sub blocky to sub fissile. |
| 2742 | 2745 | 100 | CLAYSTONE: medium dark grey to dark grey, common micro mica, trace carbonaceous specks, arenaceous in part, hard, sub blocky to sub fissile. |
| 2745 | 2748 | 95 5 | CLAYSTONE: medium dark grey, medium grey, very finely arenaceous, common micro mica, rare disseminated pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: off white to very light grey, moderately to well sorted, sub rounded to rounded, weak siliceous cement, common very light grey argillaceous matrix, trace carbonaceous specks, friable to moderately hard, poor visual porosity, no fluorescence. |
| 2748 | 2751 | 95 5 | CLAYSTONE: medium dark grey to medium grey, very finely arenaceous, common micro mica, rare disseminated pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: off white to very light grey, moderately well sorted, sub angular to sub rounded, weak siliceous cement, common very light grey argillaceous matrix, trace carbonaceous specks, friable to moderately hard, poor visual porosity, no fluorescence. |
| 2751 | 2754 | 90 10 | CLAYSTONE: medium dark grey to dark grey, medium grey, very finely arenaceous, common micro mica, rare disseminated pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: off white to very light grey, moderately well sorted, sub angular to sub rounded, weak siliceous cement, common very light grey argillaceous matrix, trace carbonaceous specks, friable to moderately hard, poor visual porosity, no fluorescence. |
| 2754 | 2757 | 80 20 | CLAYSTONE: medium dark grey, medium grey, very finely arenaceous, common micro mica, rare disseminated pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: off white to very light grey, moderately well sorted, sub angular to sub rounded, weak siliceous cement, common very light grey argillaceous matrix, trace carbonaceous specks, friable to moderately hard, poor visual porosity, no hydrocarbon fluorescence. |
| 2757 | 2760 | 80 20 | CLAYSTONE: medium dark grey, medium grey, very finely arenaceous, common micro mica, rare disseminated pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: off white to very light grey, very fine to fine, grading to SILSTONE, moderately well sorted, sub angular to sub rounded, weak siliceous cement, common very light grey argillaceous matrix, trace carbonaceous specks, friable to moderately hard, poor visual porosity, no hydrocarbon fluorescence. |
| 2760 | 2763 | 90 10 | CLAYSTONE: medium grey, medium dark grey, very fine arenaceous, common micro mica, rare pyrite, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: off white to very light grey, very fine to fine, grading to SILSTONE, moderately well sorted, sub angular to sub rounded, weak siliceous cement, common very light grey argillaceous matrix, trace carbonaceous specks, friable to moderately hard, poor visual porosity, no hydrocarbon fluorescence. |
| 2763 | 2766 | 100 | CLAYSTONE: dark grey, medium to medium dark grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. |
| 2766 | 2769 | 90 10 | CLAYSTONE: Dark grey, medium to medium dark grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: light grey, medium light grey, very fine to trace fine, moderately well sorted, sub angular to sub rounded, weak salicaceous and calcareous cement, light grey argillaceous matrix, moderately hard to hard, very poor to poor visible porosity, no fluorescence. |
| 2769 | 2772 | 80 20 | CLAYSTONE: dark grey, medium to medium dark grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: light grey, medium light grey, very fine to trace fine, moderately well sorted, sub angular to sub rounded, weak salicaceous and calcareous cement, light grey argillaceous matrix, moderately hard to hard, very poor to poor visible porosity, no fluorescence. |
| 2772 | 2775 | 80 20 | CLAYSTONE: dark grey, medium to medium dark grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: light grey, medium light grey, very fine to trace fine, moderately well sorted, sub angular to sub rounded, weak salicaceous and calcareous cement, light grey argillaceous matrix, moderately hard to hard, very poor to poor visible porosity, no fluorescence. |
| 2775 | 2778 | 90 10 | CLAYSTONE: dark grey, medium to medium dark grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: light grey, medium light grey, very fine to fine, moderately well sorted, sub angular to sub rounded, weak salicaceous and calcareous cement, light grey argillaceous matrix, moderately hard to hard, very poor to poor visible porosity, no fluorescence. |
| 2778 | 2781 | 90 10 | CLAYSTONE: dark grey, medium to medium dark grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: light grey, medium light grey, very fine to fine, moderately well sorted, sub angular to sub rounded, weak salicaceous and calcareous cement, light grey argillaceous matrix, moderately hard to hard, very poor to poor visible porosity, no fluorescence. |
| 2781 | 2784 | 100 | CLAYSTONE: medium grey to dark grey, medium grey in part, silty in part, trace to minor micro mica, trace disseminated pyrite, moderately hard to minor hard, sub blocky to trace sub fissile. |
| 2784 | 2787 | 100 | CLAYSTONE: medium dark grey, medium grey to medium light grey, silty in part, trace to minor micro mica, trace disseminated pyrite, moderately hard to minor hard, sub blocky to trace sub fissile. |
| 2787 | 2790 | 95 5 | CLAYSTONE: medium grey to dark grey, minor medium light grey, silty in part, trace to minor micro mica, trace disseminated pyrite, moderately hard to minor hard, sub blocky to trace sub fissile. SANDSTONE: light grey, medium light grey, trace off white, very fine to trace fine, moderately well sorted, sub angular to sub rounded, weak salicaceous and calcareous cement, light grey argillaceous matrix, moderately hard to hard, very poor to poor visible porosity, no fluorescence. |
| 2790 | 2793 | 100 trace | CLAYSTONE: medium grey to dark grey, minor medium light grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to trace sub fissile. SANDSTONE: light grey, medium light grey, trace off white, very fine to trace fine, moderately well sorted, sub angular to sub rounded, weak salicaceous and calcareous cement, light grey argillaceous matrix, moderately hard to hard, very poor to poor visible porosity, no fluorescence, |
| 2793 | 2796 | 80 20 | CLAYSTONE: medium dark grey to dark grey, minor medium light grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to trace sub fissile. SANDSTONE: light grey to minor off white, medium light grey in part, very fine grading to arenaceous SILTSTONE in part, moderately sorted, sub angular to sub rounded, trace calcareous and siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 2796 | 2799 | 90 10 | CLAYSTONE: medium dark grey to dark grey, minor medium to medium light grey, dark brownish grey in part, silty, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, minor sub fissile. SANDSTONE: light grey to minor off white, medium light grey in part, very fine grading to arenaceous SILTSTONE in part, moderately sorted, sub angular to sub rounded, trace calcareous and siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 2799 | 2802 | 90 10 | CLAYSTONE: Medium dark grey to dark grey, minor medium to medium light grey, trace dark brownish grey, silty, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, occasionally sub fissile. SANDSTONE: light grey to minor off white, medium light grey in part, very fine grading to arenaceous SILTSTONE, moderately sorted, sub angular to sub rounded, trace siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 2802 | 2805 | 100 | CLAYSTONE: medium dark grey to dark grey, trace dark brownish grey, medium light grey in part, silty in part, trace to minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. |
| 2805 | 2808 | 100 | CLAYSTONE: medium dark grey to dark grey, trace dark brownish grey, trace medium light grey, silty in part, minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. |
| 2808 | 2811 | 90 10 | CLAYSTONE: medium grey to dark grey,, trace dark brownish grey, medium light grey in part, silty in part, trace to minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: light grey to off white, very fine common grading to arenaceous SILTSTONE, moderately sorted, sub angular to sub rounded, trace siliceous and calcareous cement, off white argillaceous matrix, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2811 | 2814 | 90 10 | CLAYSTONE: medium dark grey to dark grey, trace dark brownish grey, trace medium light grey, silty in part, minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: very light grey, off white, very fine to fine commonly grading to arenaceous SILTSTONE, moderately well sorted, sub angular to sub rounded, trace siliceous and calcareous cement, off white argillaceous matrix, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2814 | 2817 | 90 10 | CLAYSTONE: medium dark grey, medium grey, minor dark grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, very fine, well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, friable to firm, occasionally moderately hard, very poor visual porosity, no fluorescence. |
| 2817 | 2820 | 90 10 | CLAYSTONE: medium dark grey, medium grey, minor dark grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, very fine, well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, friable to firm, occasionally moderately hard, very poor visual porosity, no fluorescence. |
| 2820 | 2823 | 90 10 | CLAYSTONE: medium dark grey, medium grey, minor dark grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, very fine, well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, friable to firm, occasionally moderately hard, very poor visual porosity, no fluorescence. |
| 2823 | 2826 | 90 10 | CLAYSTONE: medium dark grey, medium grey, minor dark grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, very fine, well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, friable to firm, occasionally moderately hard, very poor visual porosity, no fluorescence. |
| 2826 | 2829 | 90 10 | CLAYSTONE: medium dark grey, medium grey, minor dark grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, very fine, well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, friable to firm, occasionally moderately hard, very poor visual porosity, no fluorescence. |
| 2829 | 2832 | 100 trace | CLAYSTONE: medium dark grey, medium grey, minor dark grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, very fine, well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, friable to firm, occasionally moderately hard, very poor visual porosity, no fluorescence. |
| 2832 | 2835 | 100 trace | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, trace translucent, very fine to fine, moderately well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2835 | 2838 | 100 trace | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, trace translucent, very fine to fine, moderately well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2838 | 2841 | 90 10 | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, trace translucent, very fine to fine, moderately well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2841 | 2844 | 90 10 | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, trace translucent, very fine to fine, moderately well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2844 | 2847 | 100 Trace | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, trace translucent, very fine to fine, moderately well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2847 | 2850 | 80 20 | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, off white, trace translucent, very fine to fine, moderately well sorted, calcareous and siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2850 | 2853 | 90 10 | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, trace disseminated pyrite, moderately hard, blocky to sub fissile. SANDSTONE: very light grey to light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong calcareous and siliceous cement, common very light grey to off white argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2853 | 2856 | 80 20 | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, trace disseminated pyrite, moderately hard, blocky to sub fissile. SANDSTONE: very light grey to light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong calcareous and siliceous cement, common very light grey to off white argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2856 | 2859 | 90 10 | CLAYSTONE: medium dark grey, medium grey, slightly silty, common micro mica, trace disseminated pyrite, moderately hard, blocky to sub fissile. SANDSTONE: very light grey to light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong calcareous and siliceous cement, common very light grey to off white argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2859 | 2862 | 80 20 | CLAYSTONE: medium grey to medium dark grey, slightly silty, common micro mica, trace disseminated pyrite, moderately hard, blocky to sub fissile. SANDSTONE: very light grey to light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong calcareous and siliceous cement, common very light grey to off white argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2862 | 2865 | 80 20 | CLAYSTONE: medium grey to medium dark grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2865 | 2868 | 90 10 | CLAYSTONE: medium dark grey, medium grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2868 | 2871 | 90 10 | CLAYSTONE: medium grey, medium dark grey, slightly silty in part, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2871 | 2874 | 80 20 | CLAYSTONE: medium dark grey, medium grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2874 | 2877 | 80 20 | CLAYSTONE: medium dark grey, medium grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2877 | 2880 | 100 Trace | CLAYSTONE: medium dark grey, medium grey, slightly silty, minor micro mica, moderately hard, blocky to sub fissile. SANDSTONE: light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2880 | 2883 | 90 10 | CLAYSTONE: medium dark grey to dark grey, medium light grey to light grey, silty in part, minor localised common micro mica, rare disseminated pyrite, moderately hard to hard, dominantly sub blocky, minor sub fissile. SANDSTONE: light grey, off white, very fine to occasionally fine grained, moderately well sorted, strong siliceous cement, common very light grey argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2883 | 2886 | 95 5 | CLAYSTONE: medium dark grey to dark grey, medium light grey to light grey, silty in part, minor localised common micro mica, rare disseminated pyrite, moderately hard to hard, dominantly sub blocky, minor sub fissile. SANDSTONE: off white to light grey, very fine common grading to arenaceous SILTSTONE, trace fine, moderately sorted, sub rounded to sub angular, minor siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 2886 | 2889 | 100 Trace | CLAYSTONE: medium dark grey, dark grey, trace light grey, silty in part, minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. SANDSTONE: off white to light grey, very fine common grading to arenaceous SILTSTONE, trace fine, moderately sorted, sub rounded to sub angular, minor siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 2889 | 2892 | 100 Trace | CLAYSTONE: medium dark grey, dark grey, trace light grey, silty in part, minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. SANDSTONE: off white to light grey, very fine common grading to arenaceous SILTSTONE, trace fine, moderately sorted, sub rounded to sub angular, minor siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 2892 | 2895 | 90 10 | CLAYSTONE: medium grey, medium dark grey to minor dark grey, silty in part, trace micro mica, trace disseminated pyrite, moderately hard, trace hard, sub blocky to occasional sub fissile. SANDSTONE: off white to light grey, very fine common grading to arenaceous SILTSTONE, moderately sorted, sub rounded to sub angular, minor siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 2895 | 2898 | 95 5 | CLAYSTONE: medium grey, medium dark grey to minor dark grey, silty in part, trace micro mica, trace disseminated pyrite, moderately hard, hard in part, sub blocky to occasionally sub fissile. SANDSTONE: light grey, off white, very light grey in part, very fine, moderately sorted, sub rounded, trace siliceous cement, off white argillaceous matrix, moderately hard, tight to very poor visual porosity, no fluorescence. |
| 2898 | 2901 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, silty in part, minor to commonly micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile. |
| 2901 | 2904 | 100 | CLAYSTONE: dark grey, medium dark grey to medium grey, silty in part, minor to commonly micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky, trace sub fissile |
| 2904 | 2907 | 100 | CLAYSTONE: medium to dark grey, medium dark grey in part, trace light grey, silty in part, minor micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to trace sub fissile. |
| 2907 | 2910 | 100 Trace | CLAYSTONE: medium to dark grey, minor medium dark grey, trace light grey, minor silty, common micro mica, trace disseminated pyrite, moderately hard to minor hard, sub blocky to trace sub fissile. SANDSTONE: off white to very light grey, trace light grey, very fine to trace fine, grading to arenaceous SILTSTONE in part, moderately sorted, sub rounded, minor siliceous cement, off white argillaceous matrix, trace dark lithics, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2910 | 2913 | 80 20 | CLAYSTONE: medium to dark grey, minor medium dark grey, trace light grey, minor silty, common micro mica, trace disseminated pyrite, moderately hard to minor hard, sub blocky to trace sub fissile. SANDSTONE: off white to very light grey, trace light grey, very fine to trace fine, minor grading to arenaceous SILTSTONE, moderately sorted, sub rounded, minor siliceous cement, off white argillaceous matrix, trace dark lithics, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2913 | 2916 | 90 10 | CLAYSTONE: dark grey, medium dark grey to medium grey, trace medium light grey, minor silty, minor locally common micro mica, trace disseminated pyrite, moderately hard, occasional hard, sub blocky to minor sub fissile. SANDSTONE: off white to very light grey, trace light grey, very fine to trace fine, minor grading to arenaceous SILTSTONE, moderately sorted, sub rounded, minor siliceous cement, off white argillaceous matrix, trace dark lithics, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2916 | 2919 | 100 Trace | CLAYSTONE: dark grey, medium dark grey to medium grey, trace medium light grey, minor silty, minor locally common micro mica, trace disseminated pyrite, moderately hard, occasional hard, sub blocky to minor sub fissile. SANDSTONE: off white to very light grey, trace light grey, very fine to trace fine, minor grading to arenaceous SILTSTONE, moderately sorted, sub rounded, minor siliceous cement, off white argillaceous matrix, trace dark lithics, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2919 | 2922 | 100 Trace | CLAYSTONE: dark grey, medium dark grey to medium grey, trace medium light grey, minor silty, minor locally common micro mica, trace disseminated pyrite, moderately hard, occasional hard, sub blocky to minor sub fissile. SANDSTONE: off white to very light grey, trace light grey, very fine to trace fine, minor grading to arenaceous SILTSTONE, moderately sorted, sub rounded, minor siliceous cement, off white argillaceous matrix, trace dark lithics, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2922 | 2925 | 90 10 | CLAYSTONE: medium dark grey to dark grey, medium grey, trace medium light grey, minor silty, minor locally common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to minor sub fissile. SANDSTONE: off white to very light grey, trace light grey, very fine grading to arenaceous SILTSTONE in part, moderately sorted, sub rounded to sub angular, trace siliceous cement, off white argillaceous matrix, trace dark lithics, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2925 | 2928 | 90 10 | CLAYSTONE: dark grey, medium grey in part, rare brownish grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to trace sub fissile. SANDSTONE: off white to very light grey, trace light grey, very fine grading to arenaceous SILTSTONE in part, moderately sorted, sub rounded to sub angular, trace siliceous cement, off white argillaceous matrix, trace dark lithics, moderately hard to hard, tight to very poor visible porosity, no fluorescence. |
| 2928 | 2931 | 100 | CLAYSTONE: dark grey, medium grey in part, rare brownish grey, silty in part, minor to common micro mica, trace disseminated pyrite, moderately hard to hard, sub blocky to trace sub fissile. |
| 2931 | 2934 | 100 | CLAYSTONE: medium dark grey to dark grey, minor medium grey, rare grey brown, silty in part, trace to minor micro mica, rare disseminated pyrite, moderately hard to hard, sub blocky, trace bub fissile. |
| 2934 | 2937 | 100 | CLAYSTONE: medium dark grey, dark grey, trace dark grey black, trace micro mica, moderately hard, blocky to sub fissile. |
| 2937 | 2940 | 100 | CLAYSTONE: medium dark grey, dark grey, trace dark grey black, trace micro mica, moderately hard, blocky to sub fissile. |
| 2940 | 2943 | 90 10 | CLAYSTONE: medium dark grey, dark grey, trace dark grey black, trace micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, very fine to fine grained, well sorted, sub angular to sub rounded, grading to arenaceous SILTSTONE in part, moderately strong siliceous cement, common light grey argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. |
| 2943 | 2946 | 90 10 | CLAYSTONE: medium grey, medium dark grey, trace dark grey, trace micro mica, moderately hard, blocky to sub fissile. SANDSTONE: very light grey, very fine to fine grained, well sorted, sub angular to sub rounded, grading to arenaceous SILTSTONE in part, moderately strong siliceous cement, common light grey argillaceous matrix, moderately hard, tight visual porosity, no fluorescence. |
| 2946 | 2949 | 100 | CLAYSTONE: medium grey, medium dark grey, trace dark grey, trace micro mica, moderately hard, blocky to sub fissile. |
| 2949 | 2952 | 100 | CLAYSTONE: medium dark grey, medium grey, trace dark grey, trace micro mica, moderately hard, blocky to sub fissile. |
| 2952 | 2955 | 100 | CLAYSTONE: medium dark grey, medium grey, trace dark grey, trace micro mica, moderately hard, blocky to sub fissile. |
| 2955 | 2958 | 100 | CLAYSTONE: medium dark grey, medium grey, trace dark grey, trace micro mica, moderately hard, blocky to sub fissile. |
| 2958 | 2961 | 100 | CLAYSTONE: medium dark grey, medium grey, trace dark grey, trace micro mica, moderately hard, blocky to sub fissile. |
| 2961 | 2964 | 100 | CLAYSTONE: medium dark grey, medium grey, trace dark grey, trace micro mica, moderately hard, blocky to sub fissile. |
| 2964 | 2967 | 100 | CLAYSTONE: very dark grey, medium grey, trace micro mica, moderately hard, blocky to sub fissile. |
| 2967 | 2970 | 100 | CLAYSTONE; very dark grey, very dark grey black, common micro mica, moderately hard to occasionally hard, blocky to sub blocky, occasionally sub fissile. |
| 2970 | 2973 | 100 | CLAYSTONE; very dark grey, very dark grey black, common micro mica, moderately hard to occasionally hard, blocky to sub blocky, occasionally sub fissile. |
| 2973 | 2976 | 100 | CLAYSTONE; very dark grey, very dark grey black, common micro mica, moderately hard to occasionally hard, blocky to sub blocky, occasionally sub fissile. |
| 2976 | 2977.5 Spot | 100 | CLAYSTONE: very dark grey black, minor to common micro mica, moderately hard to occasionally hard, blocky to sub blocky occasionally sub fissile. |
| 2976 | 2979 | 100 | CLAYSTONE: very dark grey black, minor to common micro mica, moderately hard to occasionally hard, blocky to sub blocky occasionally sub fissile. |
| 2979 | 2982 | 100 | CLAYSTONE: very dark grey black, minor to common micro mica, moderately hard to occasionally hard, blocky to sub blocky occasionally sub fissile. |
| 2982 | 2985 | 100 | CLAYSTONE: very dark grey black, minor to common micro mica, moderately hard to occasionally hard, blocky to sub blocky occasionally sub fissile. |
| 2985 | 2988 | 100 | CLAYSTONE: very dark grey black, minor to common micro mica, moderately hard to occasionally hard, blocky to sub blocky occasionally sub fissile. |
| 2988 | 2991 | 100 | CLAYSTONE: very dark grey to dark grey, minor micro mica, moderately hard, blocky to sub fissile. |
| 2991 | 2994 | 100 | CLAYSTONE: medium dark grey to dark grey, minor micro mica, moderately hard, blocky to sub fissile. |
| 2994 | 2997 | 100 | CLAYSTONE: very dark grey to dark grey black, common micro mica, rare disseminated pyrite, moderately hard, blocky to sub fissile. |
| 2997 | 3000 | 100 | CLAYSTONE: very dark grey to grey black, minor to common micro mica, minor disseminated pyrite, moderately hard to hard, sub blocky to occasional sub fissile. |
| 3000 | 3003 | 100 | CLAYSTONE: very dark grey to grey black, dark grey in part, minor to occasional micro mica, minor disseminated pyrite, moderately hard to common hard, sub blocky, minor sub fissile. |
| 3003 | 3006 | 100 | CLAYSTONE: very dark grey to grey black, dark grey in part, minor to occasional micro mica, minor disseminated pyrite, moderately hard to hard, trace very hard, sub blocky, minor sub fissile. |
| 3006 | 3009 | 100 | CLAYSTONE: grey black, very dark grey to minor dark grey, trace micro mica, minor disseminated pyrite, hard to very hard, minor moderately hard, dominantly sub blocky to trace sub fissile. |
| 3009 | 3012 | 100 | CLAYSTONE: grey black, very dark grey to minor dark grey, trace micro mica, minor disseminated pyrite, hard to very hard, minor moderately hard, dominantly sub blocky to trace sub fissile. |
| 3012 | 3015 | 100 | CLAYSTONE: very dark grey to dark grey, minor grey black, trace disseminated pyrite, rare micro mica, moderately hard to dominantly hard, sub blocky, trace sub fissile. |
| 3015 | 3018 | 100 | CLAYSTONE: very dark grey to dark grey, minor grey black, trace disseminated pyrite, rare micro mica, minor moderately hard to dominantly hard, sub blocky, trace sub fissile. |
| 3018 | 3021 | 100 | CLAYSTONE: dark grey to very dark grey, grey black in part, trace micro mica, trace disseminated pyrite, hard to very hard, sub blocky. |
| 3021 | 3024 | 100 | CLAYSTONE: dark grey to very dark grey, grey black in part, trace micro mica, trace disseminated pyrite, hard to very hard, trace moderately hard, sub blocky. |
| 3024 | 3027 | 100 | CLAYSTONE: dark to very dark grey, grey black in part, trace to minor micro mica, trace disseminated pyrite, dominantly hard, trace moderately hard, sub blocky. |
| 3027 | 3030 | 100 | CLAYSTONE: dark to very dark grey, grey black in part, trace micro mica, trace disseminated pyrite, dominantly hard, trace moderately hard, sub blocky. |
| 3030 | 3033 | 100 | CLAYSTONE: very dark grey to grey black, trace dark grey, trace micro mica, rare disseminated pyrite, dominantly hard, minor moderately hard, trace very hard, sub blocky. |
| 3033 | 3036 | 100 | CLAYSTONE: very dark grey to grey black, trace dark grey, trace micro mica, rare disseminated pyrite, dominantly hard, minor moderately hard, trace very hard, sub blocky, rare blocky. |
| 3036 | 3039 | 100 | CLAYSTONE: grey black, very dark grey, minor dark grey, trace disseminated pyrite, rare to trace micro mica, hard to trace very hard, minor moderately hard, sub blocky. |
| 3039 | 3042 | 100 | CLAYSTONE: grey black, very dark grey, minor dark grey, trace disseminated pyrite, rare to trace micro mica, hard to very hard, minor moderately hard, sub blocky. |
| 3042 | 3045 | 100 | CLAYSTONE: dark grey to very dark grey, grey black, trace micro mica, trace disseminated pyrite, hard, mod hard in part, trace very hard, sub blocky, trace sub fissile. |
| 3045 | 3048 | 100 | CLAYSTONE: dark grey to very dark grey, grey black, trace micro mica, trace disseminated pyrite, hard, mod hard in part, trace very hard, sub blocky, minor sub fissile. |
| 3048 | 3051 | 100 | CLAYSTONE: dark to very dark grey, grey black, trace disseminated pyrite, rare micro mica, rare nodular pyrite, hard to very hard, sub blocky, trace sub fissile. |
| 3051 | 3054 | 100 | CLAYSTONE: dark to very dark grey, grey black, trace disseminated pyrite, rare micro mica, rare nodular pyrite, hard to very hard, sub blocky, trace sub fissile. |
| 3054 | 3057 | 100 | CLAYSTONE: grey black, very dark grey, trace disseminated pyrite, trace micro mica, rare nodular pyrite, hard to very hard, sub blocky, trace sub fissile. |
| 3057 | 3060 | 100 | CLAYSTONE: grey black, very dark grey, trace disseminated pyrite, trace micro mica, rare nodular pyrite, hard to very hard, sub blocky, trace sub fissile. |
| 3060 | 3063 | 100 Trace | CLAYSTONE: very dark grey, grey black, trace disseminated pyrite, rare to trace micro mica, hard to very hard, sub blocky, trace sub fissile. SANDSTONE: off white, light grey, very light grey, translucent, trace clear, fine to medium, trace very fine, poor sorting, sub angular, strong siliceous cement, off white argillaceous matrix, hard to very hard, poor to fair visual porosity, no fluorescence. |
| 3063 | 3066 | 100 Trace | CLAYSTONE: very dark grey, grey black, trace disseminated pyrite, rare to trace micro mica, hard to very hard, sub blocky, trace sub fissile. SANDSTONE: off white, light grey, very light grey, translucent, trace clear, fine to medium, trace very fine, poor sorting, sub angular, strong siliceous cement, off white argillaceous matrix, hard to very hard, poor to fair visual porosity, no fluorescence. |
| 3066 | 3069 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, trace disseminated pyrite, hard, blocky to sub blocky, minor sub fissile. |
| 3069 | 3072 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, trace disseminated pyrite, hard, blocky to sub blocky, minor sub fissile. |
| 3072 | 3075 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, trace disseminated pyrite, hard, blocky to sub blocky, minor sub fissile. |
| 3075 | 3078 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, trace disseminated pyrite, hard, blocky to sub blocky, minor sub fissile. |
| 3078 | 3081 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, trace disseminated pyrite, hard, blocky to sub blocky, minor sub fissile. |
| 3081 | 3084 | 100 | CLAYSTONE: very dark grey to very dark grey black, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3084 | 3087 | 100 Trace | CLAYSTONE: very dark grey, very dark grey black, trace micro mica, trace disseminated pyrite, hard, blocky to sub blocky, minor sub fissile. SANDSTONE: translucent, clear, light grey, fine grained, well sorted, sub angular, trace siliceous cement, minor quartz overgrowths, hard, tight visual porosity, no fluorescence. |
| 3087 | 3090 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3090 | 3093 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3093 | 3096 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3096 | 3099 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3099 | 3102 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3102 | 3105 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3105 | 3108 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3108 | 3111 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3111 | 3114 | 100 Trace | CLAYSTONE: very dark grey black to very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. SANDSTONE: very light grey, translucent, very fine to fine grained, well sorted, sub angular, strong siliceous cement, minor light grey argillaceous matrix, trace quartz overgrowths, hard, tight visual porosity, no fluorescence. |
| 3114 | 3117 | 100 Trace | CLAYSTONE: very dark grey very dark grey black, trace micro mica, hard, blocky to sub blocky, minor sub fissile. SANDSTONE: very light grey, translucent, very fine to fine grained, well sorted, sub angular, strong siliceous cement, minor light grey argillaceous matrix, trace quartz overgrowths, hard, tight visual porosity, no fluorescence. |
| 3117 | 3120 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3120 | 3123 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3123 | 3126 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3126 | 3129 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3129 | 3132 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3132 | 3135 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3135 | 3138 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3138 | 3141 | 100 | CLAYSTONE: very dark grey black, very dark grey, trace micro mica, hard, blocky to sub blocky, minor sub fissile. |
| 3141 | 3144 | 80 20 | CLAYSTONE: very dark grey to greyish black, trace to minor disseminated pyrite, hard to very hard, sub blocky to trace sub fissile. SANDSTONE: light grey to off white, translucent, minor clear, very fine to fine, sub rounded, common calcareous cement, off white argillaceous cement, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 3144 | 3147 | 70 30 | CLAYSTONE: medium grey to light grey, trace to minor disseminated pyrite, hard to very hard, sub blocky to trace sub fissile. SANDSTONE: light grey to off white, translucent, minor clear, very fine to fine, sub rounded, common calcareous cement, off white argillaceous cement, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 3147 | 3150 | 70 30 | CLAYSTONE: medium grey, occasional medium light grey, trace calcareous fragments, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: light grey to very light grey, off white, translucent, minor clear, very fine to fine, sub rounded, common calcareous cement, off white argillaceous cement, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 3150 | 3153 | 70 30 | CLAYSTONE: medium grey, occasional medium light grey, trace calcareous fragments, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: off white, translucent, trace light grey to very light grey, minor opaque, very fine to fine, sub rounded to trace sub angular, common calcareous cement, off white argillaceous cement, moderately hard, tight to very poor visible porosity, no fluorescence. |
| 3153 | 3156 | 80 20 | CLAYSTONE: medium grey, occasional medium light grey, trace calcareous fragments, moderately hard to hard, sub blocky to sub fissile. SANDSTONE: light grey to off white, translucent, trace clear, very fine to fine, moderately sorted, sub rounded, common calcareous cement, off white argillaceous matrix, moderately hard, tight to very poor porosity, no fluorescence. |
| 3156 | 3159 | 100 | CLAYSTONE: medium to medium light grey, light grey in part, trace medium dark grey, trace calcareous fragments, rare disseminated pyrite, hard to very hard, sub blocky to minor sub fissile. |
| 3159 | 3162 | 100 | CLAYSTONE: medium grey to medium light grey, light grey in part, trace medium dark grey, trace micro mica, hard, trace very hard, sub blocky to sub fissile. |
| 3162 | 3165 | 100 | CLAYSTONE: medium to light grey, trace very light grey, minor medium dark grey, trace micro mica, hard to very hard, sub blocky to occasional sub fissile. |
| 3165 | 3168 | 100 | CLAYSTONE: medium grey to light grey, very light grey in part, trace medium dark grey, trace micro mica, hard to minor very hard, sub blocky to occasional sub fissile. |
| 3168 | 3171 | 100 | CLAYSTONE: medium grey to light grey, very light grey in part, trace medium dark grey, trace micro mica, hard to very hard, sub blocky to sub fissile. |
| 3171 | 3173 | 100 | CLAYSTONE: medium grey to light grey, very light grey in part, trace medium dark grey, trace micro mica, rare disseminated pyrite, hard to minor very hard, sub blocky to sub fissile. |