- Haddington Resources Limited

- Liberator and Long Island

- Geophysical Survey

- Processing Report

December 2009

Table of Contents

1.	Datum Specification	3

2.	Parallax	4

3.	Magnetic processing	5

3.1	Processing Flow	5

3.2	Compensation	5

3.3	Magnetic Model	6

3.4	Diurnal Base Value	6

3.5	Tie Line levelling Method	6

3.6	Micro-levelling Method	7

3.7	Interpolation Method	7

4.	Radiometric Processing	8

4.1	Processing Flow	8

4.2	Window Energy Limits	8

4.3	Spectral Stripping Ratios	9

4.4	Aircraft, Cosmic Backgrounds and Height Attenuation coefficients	9

4.5	Conversion to Ground Concentrations	9

4.6	Tie Line Levelling	10

4.7	Micro-levelling Method	10

4.8	Interpolation Method	10

5.	Elevation Processing	11

5.1	Processing Flow	11

5.2	Tie Line levelling Method	11

5.3	Micro-levelling Method	11

5.4	Interpolation Method	12

6.	Deliverable Items	13

6.1	Magnetic and elevation Located Data file	14

6.2	Radiometric Located Data file	15

6.3	256 Radiometric Located Data file	17

7.	Magnetic Data Processing Flow Chart	19

8.	Elevation Data Processing Flow Chart	20

9.	Radiometric Processing Flow Chart	21

# Datum Specification

The output survey coordinates are based on the Geocentric Datum of Australia 1994 (GDA94), zone 52.

It has the following parameters:

Projection name:	Map Grid of Australia

Datum:			Geocentric Datum of Australia (GDA94)

Reference Frame:	ITRF92 (International Terrestrial Reference 1992)

Epoch:			1994.0

Ellipsoid:		GRS80

Semi-major axis:	6.378.137.0 metres

Inverse flattening:	298,257222101

False Northing:		10,000,000 m N

False Easting:		500,000 m E

Scale Factor:		0.9996

# Parallax

Parallax corrections were applied as follows:

- 0 fiducials (at 20 hertz) for magnetics data.

- 0 fiducials (at 20 hertz) for elevation data.

- 0.25 seconds for radiometrics

# Magnetic processing

## Processing Flow

The diurnal base station data was checked for spikes and steps, and suitably filtered prior to the removal of diurnal variations from the aircraft magnetic data.

The diurnal data was filtered with a second difference filter to identify and remove spikes of less than 0.05nT. A second smoothing filter, a 13 point moving average filter is used to reduce noise levels.

The filtered diurnal are then applied to the survey data by synchronising the diurnal data time with the aircraft survey time. The average diurnal base station value was added to the survey data.

An eighth difference filter was run on the raw magnetic survey data in order to identify any remaining spikes in the data, which were manually edited from the data.

The X and Y positioning of the data was then checked for spikes before applying the IGRF correction.  Any spikes in the positions were manually edited.

The IGRF 2005 (updated to (2009.87) correction was calculated at each data point taking into account the height above sea level using the gps altitude. This regional magnetic gradient was subtracted from the survey data points.

The data was then tie-line levelled and micro-levelled.

## Compensation

The data was compensated post flight using a 16 term model based on the work done by C.D. Hardwick.

Magnetic compensation sequences were flown after acquisition completed and after routine maintenance was performed, as required. The resulting coefficients were used for post flight magnetic compensation:

| Aircraft | Date | Flight | StDev (UnC) | StDev (Cmp) | IR |
| --- | --- | --- | --- | --- | --- |
| VH-AVN | 10/11/09 | 1 | 4.698 | 0.146 | 32.11 |

UNC: Standard deviation of uncompensated TMI (nT)

CMP: Standard deviation of compensated TMI (nT)

IR: Improvement ratio (UNC/CMP)

A qualitative measure of the effectiveness of the compensation is to examine the standard deviation of the high passed total magnetic field before and after the compensation coefficients have been applied. The higher the improvement ratio the better the compensation model has removed the aircraft induced manoeuvre signal. This may not always the case, as a very clean aircraft will have a low improvement ratio. Also if the manoeuvres are smaller than specified then the IR ratio will be small. After reviewing the data, the compensation was deemed to be successful.

## Magnetic Model

IGRF was removed using the N-value corrected gps altitude.

The magnetic model for the centre of the area is detailed below:

| Model | IGRF 2005 updated to 2009.87 |
| --- | --- |
| Declination | 3.57941 degrees |
| Inclination | -41.67359 degrees |
| Field strength | 46947.88 nT |
| Grid zone | 52 |
| Grid central meridian | 129.00000 degrees |
| Input latitude | -13.59216 degrees |
| Input longitude | 131.36366 degrees |
| Grid convergence | 0.55548 degrees |
| Grid magnetic angle | 4.13489 degrees |
| Secular variation | -0.01150 degrees |

## Diurnal Base Value

The average diurnal base value was       46,962.96 nT

## Tie Line levelling Method

Tie line levelling was applied to the data by least squares minimisation, using a polynomial fit of order 0, of the differences in elevation values at the crossover points of the survey traverse and tie line data.

The least squares tie line levelling process employs a two pass Gauss-Seidel iterative scheme. The essential steps in this process are:

In the first pass the tie lines were first adjusted to minimise, in the least squares sense, the crossover values with the traverse line values being held constant.

The second pass held the levelled tied line values constant, and minimised in the least squares sense, the crossover values with traverses.

The DC correction values to be applied to the traverse lines and tie lines were then applied to the magnetic data.

To reduce the effects of radar altimeter and gps errors on the recorded elevation data at the crossover points, data having a radar altimeter difference greater than 15 metres in a radius of 50 metres on the traverse or tie lines were excluded from the tying process.

## Micro-levelling Method

Micro-levelling techniques were then selectively applied to the tie line levelled data to remove minor residual variations in profile intensity  Selective micro-levelling was applied in order to leave unaffected any data having no residual levelling artefacts.  Selective micro-levelling proceeds using the following steps:

Areas of interest that required micro-levelling were identified through the use of image processing visualisation.

Polygons were used to define areas requiring micro-levelling.

“Pseudo-ties” were constructed from the gridded data by extracting traverses from the grid normal to the flight direction.

Line dependent artefacts were removed from the pseudo lines using custom filters.

Crossover values were calculated between traverse lines and pseudo tie lines.

The traverse lines were adjusted in the pre-defined sections to minimise the crossover values.

This process was repeated in order to remove various wavelength line dependent artefacts from the pseudo-ties.  The object of each micro-levelling iteration was to produce a smooth control surface to which the traverse lines are levelled. This control surface was provided through the use of “pseudo-ties”.

## Interpolation Method

The interpolation used is a minimum curvature algorithm. The algorithm is based on the worked published by Briggs 1974, Briggs I. C.: Machine contouring using minimum curvature. Geophysics. Vol. 39, No. 1. February 1974. pp. 39-48.

The algorithm has been modified to include a tension parameter based on the work published by Smith and Wessel  Smith, W. H. F, and P. Wessel, 1990, Gridding with continuous curvature splines in tension, Geophysics 55, 293-305.

A tension factor of 0 was used to interpolate the data.

The mesh size for data interpolation was 10 x 10 metres.

# Radiometric Processing

## Processing Flow

The processing steps radiometric data were as follows:

###### Application of necessary parallax corrections to data

###### Check radar altimeter data for spikes

###### NASVD spectral smoothing

- Examine the output to determine the number of components required.

- Select 7 components for spectral reconstruction.

###### Standard 256 channel radiometric corrections:

- Dead-time correction performed on 256 channel data.

- Check if energy recalibration required

- Remove 256 channel aircraft and cosmic  backgrounds from the data

- Remove background radon from window data using Minty’s method (1996)

- Perform STP height corrected spectral stripping

- Perform STP height correction of window data to specified survey height (25m).

###### Micro-levelling

###### .

## Window Energy Limits

The energy bounds for the windows were

| Window Name | Energy Range (Mev) |
| --- | --- |
| Potassium | 1.374 – 1.566 |
| Thorium | 2.416 – 2.799 |
| Uranium | 1.662 – 1.854 |
| Total Count | 0.414 – 2.799 |

## Spectral Stripping Ratios

The stripping ratios used in the processing were:

| Alpha | 0.277 |
| --- | --- |
| Beta | 0.408 |
| Gamma | 0.776 |
| a | 0.045 |
| b | 0.001 |
| g | 0.000 |

## Aircraft, Cosmic Backgrounds and Height Attenuation coefficients

|  | Total Count | Potassium | Uranium | Thorium |
| --- | --- | --- | --- | --- |
| Aircraft Bkg | 399..3 | 44.3 | 18.5 | 6.0 |
| Cosmic Bkg | 1.484 | 0.084 | 0.075 | 0.059 |
| Height Attn | -0.007434 | -0.009432 | -0.008428 | -0.007510 |

The above values are for the windows only. During processing the equivalent 256 channel aircraft and cosmic backgrounds are removed.

## Conversion to Ground Concentrations

|  | Total Count Cps to Dose rate | Potassium Cps to Percent | Uranium Cps to PPM | Thorium Cps to PPM |
| --- | --- | --- | --- | --- |
| Conversion Factor | 86.042 | 350.696 | 22.740 | 16.456 |

## Tie Line Levelling

No tie line levelling was applied.

## Micro-levelling Method

Micro-levelling techniques were then selectively applied to the tie line levelled data to remove minor residual variations in profile intensity  Selective micro-levelling was applied in order to leave unaffected any data having no residual levelling artifacts.  Selective micro-levelling proceeds using the following steps:

Areas of interest that required micro-levelling were identified through the use of image processing visualisation.

Polygons were used to define areas requiring micro-levelling.

“Pseudo-ties” were constructed from the gridded data by extracting traverses from the grid normal to the flight direction.

Line dependent artefacts were removed from the pseudo lines using custom filters.

Crossover values were calculated between traverse lines and pseudo tie lines.

The traverse lines were adjusted in the pre-defined sections to minimise the crossover values.

This process was repeated in order to remove various wavelength line dependent artefacts from the pseudo-ties.  The object of each micro-levelling iteration was to produce a smooth control surface to which the traverse lines are levelled. This control surface was provided through the use of “pseudo-ties”.

## Interpolation Method

The interpolation used is a minimum curvature algorithm. The algorithm is based on the worked published by Briggs 1974, Briggs I. C.: Machine contouring using minimum curvature. Geophysics. Vol. 39, No. 1. February 1974. pp. 39-48.

The algorithm has been modified to include a tension parameter based on the work published by Smith and Wessel  Smith, W. H. F, and P. Wessel, 1990, Gridding with continuous curvature splines in tension, Geophysics 55, 293-305.

A tension factor of 0 was used to interpolate the data.

The mesh size for data interpolation was 25 x 25 metres

# Elevation Processing

## Processing Flow

The processing steps for digital elevation data were as follows:

###### Application of necessary parallax corrections to data

###### Calculation of raw digital elevation data by subtracting the radar altimeter from the gps altitude

###### Tie line levelling

###### Micro-levelling

## Tie Line levelling Method

Tie line levelling was applied to the data by least squares minimisation, using a polynomial fit of order 0, of the differences in elevation values at the crossover points of the survey traverse and tie line data.

The least squares tie line levelling process employs a two pass Gauss-Seidel iterative scheme. The essential steps in this process are:

In the first pass the tie lines were first adjusted to minimise, in the least squares sense, the crossover values with the traverse line values being held constant.

The second pass held the levelled tied line values constant, and minimised in the least squares sense, the crossover values with traverses.

The DC correction values to be applied to the traverse lines and tie lines were then applied to the magnetic data.

To reduce the effects of radar altimeter and gps errors on the recorded elevation data at the crossover points, data having a radar altimeter difference greater than 15 metres in a radius of 50 metres on the traverse or tie lines were excluded from the tying process.

## Micro-levelling Method

Micro-levelling techniques were then selectively applied to the tie line levelled data to remove minor residual variations in profile intensity  Selective micro-levelling was applied in order to leave unaffected any data having no residual levelling artefacts.  Selective micro-levelling proceeds using the following steps:

Areas of interest that required micro-levelling were identified through the use of image processing visualisation.

Polygons were used to define areas requiring micro-levelling.

“Pseudo-ties” were constructed from the gridded data by extracting traverses from the grid normal to the flight direction.

Line dependent artefacts were removed from the pseudo lines using custom filters.

Crossover values were calculated between traverse lines and pseudo tie lines.

The traverse lines were adjusted in the pre-defined sections to minimise the crossover values.

This process was repeated in order to remove various wavelength line dependent artefacts from the pseudo-ties.  The object of each micro-levelling iteration was to produce a smooth control surface to which the traverse lines are levelled. This control surface was provided through the use of “pseudo-ties”.

## Interpolation Method

The interpolation used is a minimum curvature algorithm. The algorithm is based on the worked published by Briggs 1974, Briggs I. C.: Machine contouring using minimum curvature. Geophysics. Vol. 39, No. 1. February 1974. pp. 39-48..

The algorithm has been modified to include a tension parameter based on the work published by Smith and Wessel  Smith, W. H. F, and P. Wessel, 1990, Gridding with continuous curvature splines in tension, Geophysics 55, 293-305.

A tension factor of 0 was used to interpolate the data.

The mesh size for data interpolation was 10 x 10 metres.

# Deliverable Items

The deliverable items included all digital data. The located data conformed to ASEG-GDF format and the gridded data was suppled in ERMapper format. The description of the located data is below.

There were two area processed:

Liberator

Long Island

Located data supplied in ASEG GDF

| File name | Definition |
| --- | --- |
| *_magdtm | Magnetics and Elevation data |
| *_rads | Radiometric data |
| *_rad256 | Raw 256 channel data |

Gridded data supplied in ER Mapper format

| File name | Definition | Units |
| --- | --- | --- |
| *_tmi | Final magnetic gridded data | nT |
| *_elev | Final dtm gridded data | m |
| *_tot | Final radiometric dose rate gridded data | nGy/hr |
| *_pot | Final radiometric potassium gridded data | percent |
| *_ura | Final radiometric uranium gridded data | ppm |
| *_th | Final radiometric thorium gridded data | ppm |

* denotes the area name

Also provided were grids of both areas combined

## Magnetic and elevation Located Data file

COMM

COMM  Baigent Geosciences Pty. Ltd.

COMM  -----------------------------

COMM

COMM  LOCATED DATA

COMM  -------------

COMM Area  :   Liberator

COMM Company Flown by:   Thomson Aviation Pty. Ltd.

COMM Company Flown for:  Haddington Resources

COMM Company Processed:  Baigent Geosciences Pty. Ltd.

COMM

COMM AIRBORNE SURVEY EQUIPMENT:

COMM -------------------------

COMM

COMM Aircraft                         : Airtractor

COMM Magnetometer                     : Geometrics G822 Cesium  Vapour

COMM Magnetometer Resolution          : 0.001 nT

COMM Magnetometer Compensation        : Post Flight

COMM Magnetometer Sample Interval     : 20 Hz, Approx 3.00 metres

COMM Data Acquisition                 : GeoOZ Model 2007

COMM Spectrometer                     : Radiation Solutions RS 500

COMM Crystal Size                     : 64 lt downward array

COMM Spectrometer Sample Interval     : 0.5 Seconds (approx 30 metres)

COMM GPS Navigation System            : Novatel 951R GPS Receiver

COMM

COMM

COMM

COMM

COMM AIRBORNE SURVEY SPECIFICATIONS

COMM

COMM Flight Line Direction            :    045 - 225  degrees

COMM Flight Line Separation           :           50  metres

COMM Tie Line Direction               :    135 - 315  degrees

COMM Tie Line Separation              :           500 metres

COMM Terrain Clearance                :       30 metres (MTC)

COMM

COMM

COMM Survey flown                     :        November 2009

COMM

COMM

COMM Flight path calculated from differentially

COMM corrected GPS Data using a Novatel 951R GPS Receiver.

COMM

COMM

COMM Grid notation refers to GDA/MGA Zone  52

COMM

COMM

COMM MAGNETIC DATA CORRECTIONS:

COMM -------------------------

COMM Diurnal variations removed

COMM IGRF(2005) updated to 2009.87 removed

COMM Average survey base station value added to datum

COMM

COMM RADIOMETRIC CORRECTIONS AND COEFFICIENTS:

COMM ----------------------------------------

COMM Spectral data preprocessed using NASVD

COMM Data has been corrected for aircraft and cosmic backgrounds.

COMM Height corrected to a constant datum of 35 metres,

COMM minimum height of 20 and a maximum of 300 metres.

COMM Data has also been corrected for radon using the method described by Minty

COMM and corrected for channel interaction.

COMM

COMM                   Tot.Count    Potassium   Uranium     Thorium

COMM Arcft Bkg           155.53        23.58      6.047         0

COMM Cosmic Bkg          0.986        0.0514      0.041       0.0549

COMM Height Attn        0.007434    0.009432     0.008428    0.007510

COMM CPS to equivalents   57.789     230.992     19.669      11.896

COMM

COMM

COMM STRIPPING RATIOS:

COMM ----------------

COMM   Alpha = 0.276,  Beta = 0.418, Gamma = 0.759,

COMM   a = 0.048,  b = 0.003,  g = 0.001

COMM

COMM  Field Name                    Format         Units     Null Value

COMM

COMM  BGS Job Number               a5

COMM  Line number                  a8

COMM  Flight number                i4

COMM  Flight date                  a8

COMM  fiducial                      f12.1                         -999999.0

COMM  mga_east                      f10.2          METRES         -99999.00

COMM  mga_north                     f11.2          METRES         -99999.00

COMM  wgs84_lat                     f12.7          DEGREES        -99.000000

COMM  wgs84_long                    f13.7          DEGREES        -999.00000

COMM  raw_mag                       f10.3          nT             -9999

COMM  mag_gammas                    f10.3          nT             -9999.000

COMM  diurnal_gammas                f10.3          nT             -9999.000

COMM  igrf_gammas                   f10.3          nT             -9999.000

COMM  mag_level                     f10.3          nT             -9999.000

COMM  rad_alt                       f8.2           METRES         -999.00

COMM  gps_height                    f8.2           METRES         -999.00

COMM  dtm                           f8.2           METRES         -999.00

## Radiometric Located Data file

COMM

COMM  Baigent Geosciences Pty. Ltd.

COMM  -----------------------------

COMM

COMM  LOCATED DATA

COMM  -------------

COMM Area  :   Liberator

COMM Company Flown by:   Thomson Aviation Pty. Ltd.

COMM Company Flown for:  Haddington Resources

COMM Company Processed:  Baigent Geosciences Pty. Ltd.

COMM

COMM AIRBORNE SURVEY EQUIPMENT:

COMM -------------------------

COMM

COMM Aircraft                         : Airtractor

COMM Magnetometer                     : Geometrics G822 Cesium  Vapour

COMM Magnetometer Resolution          : 0.001 nT

COMM Magnetometer Compensation        : Post Flight

COMM Magnetometer Sample Interval     : 20 Hz, Approx 3.00 metres

COMM Data Acquisition                 : GeoOZ Model 2007

COMM Spectrometer                     : Radiation Solutions RS 500

COMM Crystal Size                     : 64 lt downward array

COMM Spectrometer Sample Interval     : 0.5 Seconds (approx 30 metres)

COMM GPS Navigation System            : Novatel 951R GPS Receiver

COMM

COMM

COMM

COMM

COMM AIRBORNE SURVEY SPECIFICATIONS

COMM

COMM Flight Line Direction            :    045 - 225  degrees

COMM Flight Line Separation           :           50  metres

COMM Tie Line Direction               :    135 - 315  degrees

COMM Tie Line Separation              :           500 metres

COMM Terrain Clearance                :       30 metres (MTC)

COMM

COMM

COMM Survey flown                     :        November 2009

COMM

COMM

COMM Flight path calculated from differentially

COMM corrected GPS Data using a Novatel 951R GPS Receiver.

COMM

COMM

COMM Grid notation refers to GDA/MGA Zone  52

COMM

COMM

COMM MAGNETIC DATA CORRECTIONS:

COMM -------------------------

COMM Diurnal variations removed

COMM IGRF(2005) updated to 2009.87 removed

COMM Average survey base station value added to datum

COMM

COMM RADIOMETRIC CORRECTIONS AND COEFFICIENTS:

COMM ----------------------------------------

COMM Spectral data preprocessed using NASVD

COMM Data has been corrected for aircraft and cosmic backgrounds.

COMM Height corrected to a constant datum of 30 metres,

COMM minimum height of 10 and a maximum of 300 metres.

COMM Data has also been corrected for radon using the method described by Minty

COMM and corrected for channel interaction.

COMM

COMM                   Tot.Count    Potassium   Uranium     Thorium

COMM Arcft Bkg           399.3        44.3       18.5         6.0

COMM Cosmic Bkg          1.484        0.084      0.075       0.059

COMM Height Attn        0.007434    0.009432     0.008428    0.007510

COMM CPS to equivalents   86.042     350.696     22.740      16.456

COMM

COMM

COMM STRIPPING RATIOS:

COMM ----------------

COMM   Alpha = 0.276,  Beta = 0.418, Gamma = 0.759,

COMM   a = 0.048,  b = 0.003,  g = 0.001

COMM

COMM  Field Name                    Format         Units     Null Value

COMM

COMM  BGS Job Number               a5

COMM  Line number                  a8

COMM  Flight number                i4

COMM  Flight date                  a8

COMM  fiducial                      f12.1                         -999999.0

COMM  mga_east                      f10.2          METRES         -99999.00

COMM  mga_north                     f11.2          METRES         -99999.00

COMM  wgs84_lat                     f12.7          DEGREES        -99.000000

COMM  wgs84_long                    f13.7          DEGREES        -999.00000

COMM  rad_alt                       f8.2           METRES         -999.00

COMM  temp_air_deg_c                f5.1           DEG            -9.0

COMM  baro_pressure                 f8.2           hPa            -999.0

COMM  gps_height                    f8.2           METRES         -999.00

COMM  live_time                     f6.0           MSEC           -9999

COMM  raw_tot_cps                   f8.0           CPS            -99.

COMM  raw_pot_cps                   f7.0           CPS            -99.

COMM  raw_ura_cps                   f7.0           CPS            -99.

COMM  raw_th_cps                    f7.0           CPS            -99.

COMM  cosmicd_cps                   f6.0           CPS            -999

COMM  dose_rate                     f10.4          nGy/h          -999.0000

COMM  pot_percent                   f9.4           PERCENT        -99.0000

COMM  ura_ppm                       f9.4           PPM            -99.

COMM  th_ppm                        f9.4           PPM            -99.000

## 256 Radiometric Located Data file

COMM

COMM  Baigent Geosciences Pty. Ltd.

COMM  -----------------------------

COMM

COMM  LOCATED DATA

COMM  -------------

COMM Area  :   Liberator

COMM Company Flown by:   Thomson Aviation Pty. Ltd.

COMM Company Flown for:  Haddington Resources

COMM Company Processed:  Baigent Geosciences Pty. Ltd.

COMM

COMM AIRBORNE SURVEY EQUIPMENT:

COMM -------------------------

COMM

COMM Aircraft                         : Airtractor

COMM Magnetometer                     : Geometrics G822 Cesium  Vapour

COMM Magnetometer Resolution          : 0.001 nT

COMM Magnetometer Compensation        : Post Flight

COMM Magnetometer Sample Interval     : 20 Hz, Approx 3.00 metres

COMM Data Acquisition                 : GeoOZ Model 2007

COMM Spectrometer                     : Radiation Solutions RS 500

COMM Crystal Size                     : 64 lt downward array

COMM Spectrometer Sample Interval     : 0.5 Seconds (approx 30 metres)

COMM GPS Navigation System            : Novatel 951R GPS Receiver

COMM

COMM

COMM

COMM

COMM AIRBORNE SURVEY SPECIFICATIONS

COMM

COMM Flight Line Direction            :    045 - 225  degrees

COMM Flight Line Separation           :           50  metres

COMM Tie Line Direction               :    135 - 315  degrees

COMM Tie Line Separation              :           500 metres

COMM Terrain Clearance                :       30 metres (MTC)

COMM

COMM

COMM Survey flown                     :        November 2009

COMM

COMM

COMM Flight path calculated from differentially

COMM corrected GPS Data using a Novatel 951R GPS Receiver.

COMM

COMM

COMM Grid notation refers to GDA/MGA Zone  52

COMM

COMM

COMM MAGNETIC DATA CORRECTIONS:

COMM -------------------------

COMM Diurnal variations removed

COMM IGRF(2005) updated to 2009.87 removed

COMM Average survey base station value added to datum

COMM

COMM RADIOMETRIC CORRECTIONS AND COEFFICIENTS:

COMM ----------------------------------------

COMM Spectral data preprocessed using NASVD

COMM Data has been corrected for aircraft and cosmic backgrounds.

COMM Height corrected to a constant datum of 30 metres,

COMM minimum height of 10 and a maximum of 300 metres.

COMM Data has also been corrected for radon using the method described by Minty

COMM and corrected for channel interaction.

COMM

COMM                   Tot.Count    Potassium   Uranium     Thorium

COMM Arcft Bkg           399.3        44.3       18.5         6.0

COMM Cosmic Bkg          1.484        0.084      0.075       0.059

COMM Height Attn        0.007434    0.009432     0.008428    0.007510

COMM CPS to equivalents   86.042     350.696     22.740      16.456

COMM

COMM

COMM STRIPPING RATIOS:

COMM ----------------

COMM   Alpha = 0.276,  Beta = 0.418, Gamma = 0.759,

COMM   a = 0.048,  b = 0.003,  g = 0.001

COMM

COMM  Field Name                    Format         Units     Null Value

COMM

COMM  BGS Job Number               a5

COMM  Line number                  a8

COMM  Flight number                i4

COMM  Flight date                  a8

COMM  fiducial                      f12.1                         -999999.0

COMM  mga_east                      f10.2          METRES         -99999.00

COMM  mga_north                     f11.2          METRES         -99999.00

COMM  wgs84_lat                     f12.7          DEGREES        -99.000000

COMM  wgs84_long                    f13.7          DEGREES        -999.00000

COMM  rad_alt                       f8.2           METRES         -999.00

COMM  temp_air_deg_c                f5.1           DEG            -9.0

COMM  baro_pressure                 f8.2           hPa            -999.0

COMM  gps_height                    f8.2           METRES         -999.00

COMM  live_time                     f6.0           MSEC           -9999

COMM  cosmicd_cps                   f6.0           CPS            -999

COMM  256 channel raw spectra    256f6.0           CPS            -9

# Magnetic Data Processing Flow Chart

Import Raw Aircraft

Data

Check, despike and noise checks

Flight Path Plots

Import Raw Diurnal

Despike and Filter

Diurnally corrected data

Remove IGRF

Interpolate

Tie Line Level

Interpolate

Micro Level

(Selective)

Image

Finished ?

Final

Located Data

Grided Data

No

Yes

Production of

Images and maps

Apply filtered diurnal

# Elevation Data Processing Flow Chart

Import Raw Aircraft

Data

Check, despike and noise checks

Flight Path Plots

Create dtm channel

GPS - radalt

Interpolate

Tie Line Level

Interpolate

Micro Level

(Selective)

Image

Finished ?

Final

Located Data

Grided Data

No

Yes

Production of

Images and maps

# Radiometric Processing Flow Chart

Import Raw Aircraft

Data

Check, despike and noise checks

Flight Path Plots

Spectral Smoothing

NASVD

Interpolate

Radiometric Corrections

Interpolate

Micro Level (Selective)

Image

Finished ?

Final

Located Data

Grided Data

No

Yes

Production of

Images and maps