14th November, 2017

Mike Tait

Department of Infrastructure, Planning and Logistics

Northern Territory Government

P.O. Box 61, Palmerston NT 0831

Email: mike.tait@nt.gov.au

Dear Mike,

This letter is the follow-on from our meeting on Thursday the 2nd of November, where we discussed repair work to the road north of Mount Wells. Firstly, thank you for taking the time to meet with us and providing clarity on our way forward.

Attached are some photographs showing the damaged culvert, and a sketch showing our suggested method of repairing the road so large trucks can access safely. Our preferred option is to clear away the western side of the culvert embankments down to the level of the creek crossing, and then to make the slopes shallow enough for large vehicles to use. This will require cutting back the creek banks so the slope is shallow enough for trucks and trailers to cross without difficulty. The base of the creek appears firm, and therefore the crossing should survive future wet season run-off. We plan to leave the culvert and drainage open so the creek can drain normally.

We decided against the other option of filling in the crossing, as that would have likely resulted in a major wash-out due to heavy rainfall in the next wet season. This would have rendered the crossing impassable for all vehicles. That would be a safety issue if we were still active on our tenement, and to anyone else using the road.

The major culvert wash-out in question is located about 220 meters north of the gate into the old Mount Wells tin mine and battery. The culvert co-ordinates are approximately:  -130 30’ 10” South Latitude, and 1310 43’ 12” East Longitude. The total length of the culvert embankments is about 50 meters, and the vertical extent of the wash-out is 2-4 meters. As noted above, we plan to push the western part of the embankments about 4 meters to the east (on to the existing embankments), thereby straightening the road somewhat. Then we will cut back the creek banks, so the slopes into and out of the creek are gentle enough for large vehicle access. The cut backs will extend anywhere from 25m to 45m in both directions from the creek.

We anticipate doing the repairs to this creek crossing in late May or early June, 2018. A local contractor will be hired, who is familiar with basic road design, and what needs to be done for a safe and proper job. Also, we would like to accept your offer of signage to alert road users that road works are in progress. We will call you and arrange for the appropriate signs and a time to pick them up, and an estimated time to return them after the job is complete.

There are two or three other small creek wash-outs along the Mt Wells – McKinlay Road, to where we turn off onto the old Mavis haul road, which accesses our tenements. The old Mavis road has five or six creek crossings in various stages of disrepair. All the repairs to creek crossings are relatively minor compared to the work required to fix the washed-out culvert job near Mt Wells. However, we will ensure that all road repairs are made as safe as possible for large trucks and regular vehicle traffic.

We hope this information and data meets your requirements, and we look forward to receiving your approval for the repair work as soon as possible.

Kind regards,

Jim McGregor-Dawson

Exploration Manager

Ismins Pty Ltd

Attachments

One satellite image showing location of washed-out culvert just north of Mt wells.

Two photos of the damaged culvert and the bypass track.

One hand drawn sketch showing the damaged culvert and the proposed repair.