JRV_NTDL4_MAGN2019A.txt – additional text

- Units of measurement:

SI Units – mag sus units. 10-3

Standards International – Centimetre-gram-second system of units.

- Unit:

TERRA-KT-10SC. TERRAPLUS MAGNETIC SUSCEPTIBILITY & CONDUCTIVITY METER KT 10

- Reference and Calibration:

Three Mag. Sus units are sent to;

“GeoResults” – Peter McMullen

32 Tuckeroo Parade

Helena Valley WA 6056

peter.m@georesults.com.au

mobile phone: 0428 147 973

Units are calibrated min. once every 6 months, max. once every 12 months.

- Correction factors applied

The Terra-KT-10SC units have setting to accommodate for various core sizes. As measurements are taken on various PQ, HQ and NQ core profiles the units are adjusted accordingly.

Settings are adjusted for measuring Chip samples.

- All measurements taken on full core on the outer curved surface.

- Measurements were taken using the “average” settings over a regular 1.0 metre interval.