# PURPOSE

The purpose of this document is to describe how a specific gravity (density) measurement is taken.

Specific Gravity measurements are taken to aid the resource geologist in calculating the tonnage of a mineralised zone. It is important that the specific gravity measurement is taken accurately, as any error will directly result in an error in estimating the gold content within a mineralised zone.

The apparent specific gravity is the weight of a given volume of rock divided by the weight of an equal volume of water, making no allowances for voids or air spaces in the volume of rock measured. In other words, this is the ratio of the bulk density of a rock to the mass of a unit volume of water.

# SCOPE

This procedure applies to Geologists and Field Technicians involved in the determination of rock density.

The aim is to get a representation of apparent specific gravity values for all rock types. It is important for resource geology work that a data base of specific gravity measurements be collated for all deposits. Samples need to representative of;

- weathered rock through to fresh rock

- all lithology’s, including waste

- all mineralized styles

# PREPARATION

For measurements to be accurate it is important that,

- the scales are set up on an appropriate static surface

- a purpose built SG Station is provided, it is to be used as the static surface

- there is no wind disturbing the scales

- known specific gravity standards are used to check the accuracy of the scales

- samples are of an appropriate size for the apparatus

- samples are dry before placing into water

Scales should be set up so that the weight of the core can be measured both in air and immersed in water. Air displacement from samples as a result of water immersion is negligible for solid core. The scales should be placed on a static surface and not on a core rack with rollers, as movement of the rollers may cause the wire (string) to touch the rollers and result in false weight readings (giving lower SG’s). A wooden or cardboard box should be placed over the scales to prevent weight fluctuations and inaccuracies due to wind movement, if being carried out in windy/breezy conditions. The scales are to be zeroed and leveled before weighing commences.

PPE

- Safety Boots

- Safety Glasses

- Sun protection (optional)

- Gloves (optional)

- Long sleeve shirt and long trousers with reflective stripping

# PROCEDURE

| ITEM/ WHO | KEY STEPS | CLARIFICATION | REF DOC |
| --- | --- | --- | --- |
| Geologist | Obtain the samples for specific gravity testing or if logging core select the sections of core for specific gravity (SG) testing during logging. | Obtain the required samples from the field, limit the weight/size of the sample to less than 2kg. Ensure to record where the samples are collected from (Easting, Northing, RL) using a GPS. All co-ordinates should be recorded in GDA94 MGA grid. If logging the geologist will select pieces of core between 20-30cm in length to be measured for specific gravity. This may mean in some cases that you need to use a hammer to break the core to get a small enough piece. This can be in 2 pieces, as in the case of an ore zone, however no piece is allowed to be less than 10cm. The from and to depths need to be marked on the core as well as the letters SG in BLUE PEN (so that the person requested to undertake the testing can easily identify it). See figure below. |  |
|  | SAMPLE SELECTION | For drill core unless otherwise instructed 2 specific gravity samples need to be taken from each core tray. Keeping in mind we require measurements for all lithology’s Attempt to measure all mineralized zones and various styles of. The measurements should always be taken after the core has been photographed, but before the core has been cut and sampled whenever possible. This prevents core marked for SG determination being missed, ensures the section marked for SG is captured on the core photo and ensures you are working with core that is more intact as it has not been broken up during core cutting and sampling. Please ensure that specific gravity samples are taken from all examples of the geology; do not discriminate against the more barren lithology’s. If collecting field samples or samples from pits which are considered fresh, use this procedure to determine the SG. |  |
|  | Density Measurement | Ensure that the scales are zeroed before starting. Each sample needs to be first weighed in air (placed on top of the scales) and, then transferred to the core cradle that is immersed in water and weighed again with the sample completely covered with water. Ensure the weight of the basket has been taken into account when taking the measurements. Ensure no part of the sample or the line holding the basket is in contact with any surfaces as this will affect the readings Record the both weights on the ‘Specific Gravity Record Sheet’ and forward results to the Database Administrator. |  |
|  | Specific Gravity (Density) Calculation | Use the following formula to calculate the SG S.G. = Weight in Air (Weight in Air – Weight in water) e.g. S.G. = 1.92 (1.92 – 1.20) = 2.67 Note: all measurements are to be recorded to 2 significant figures |  |

At the end of each day or session the scales are to be wiped clean and dry, packed up, placed back in the appropriate box and stored in a secure dry location, to prevent damage. The scales are a delicate measuring device and need to be treated as such.

Only rest/sit the scales on the ground if the under balance hook is attached, if the hook is not in contact with the surface. Allows remove the underbalance hook from the scales in order for the scales to sit flat on the surface when it is removed from the SG station.

DISCLAIMER:

- I have read this document

- I have under stood the instructions

- I have understood that I am to use the required PPE

- I have been trained in the use of this item

| Name | Date | Signature |
| --- | --- | --- |

| Name | Date | Signature |
| --- | --- | --- |